APPENDIX

Fig. 19 illustrates the conceptual model for the determination of the variation of preconsolidation pressure pc and initial void ratio eini with depth, along with the pertinent equations. In the geologic past (Fig. 19a), the entire soil profile was normally consolidated (NC). The clay became overconsolidated by the removal of soil due to erosion of the top D meters of the profile, to end up to the present state (Fig. 19b).

(a)                                                                           (b)

Fig. 19: Schematic showing the mechanism of development of overconsolidation due to erosion

The larger the D is, the higher the preconsolidation pressure pc becomes, which is in essence the mean effective stress p when the soil was normally consolidated, i.e., pNC. The water table is assumed to be at the ground surface at both states, and thus the buoyant unit weight is used for the calculations of the vertical effective stress σv. For the estimation of the buoyant unit weight, the specific gravity Gs of the soil particles was assumed to be equal to 2.7. The coefficient of earth pressure at rest K0, which is needed for the calculation of the horizontal effective stresses and, hence, the mean effective stress p, is calculated using the extension of  the Jaky (1944) formula proposed by Mayne & Kulhawy (1982) that accounts for the effect of the overconsolidation ratio OCR:

Ko=1-sinφcOCRsinφc    			   (10)

OCR=σv,NC'σv'  						   (11)

Fig. 20: Idealization of void ratio vs mean effective stress response of clays.

The calculation of the corresponding initial void ratio relies on the bilinear conceptual model (Roscoe et al. 1958) shown Fig. 20, based on which:

(12)

where κ (=Cs/ln(10)) and λ (=Cc/ln(10)) are the inclinations of the unloading-reloading line (elastic path) and normal consolidation line, respectively, in the e-lnp space. The void ratio eini increases as the mean effective stress decreases from its normal consolidation value (=pc) to its present in-situ value pini. The normal consolidation line is unique for a given clay layer, with N being its ordinate at a reference pressure of 100kPa. In this study, the N value is taken equal to 1.2.