Appendix

Flowchart 1: Workflow for Bell State Fidelity Characterization, including Hardware Execution, Error Mitigation, and Analysis.

| Layout | Run | Job ID |
| --- | --- | --- |
| [1, 4] | Original Run | czw7g5gnhqag008te950 |
| [2, 3] | 1 | czy8qedqnmvg008w2560 |
| [2, 3] | 2 | czy8qxqkzhn0008d7fh0 |
| [2, 3] | 3 | czy8r28kzhn0008d7fhg |
| [2, 3] | 4 | czy8rbsqnmvg008w259g |
| [2, 3] | 5 | czy8rgjrxz8g008f4ghg |
| [7, 8] | 1 | czy8rv3rxz8g008f4gp0 |
| [7, 8] | 2 | czy8rzvqnmvg008w25f0 |
| [7, 8] | 3 | czy8sand8drg008hvpgg |
| [7, 8] | 4 | czy8sme6rr3g008me9ag |
| [7, 8] | 5 | czy8sxz6rr3g008me9c0 |

Table 4: Job ID record on the IBM Kyiv Quantum Hardware.

Visual Studio Code Miniconda Environment Phyton Code

pip install qiskit qiskit-ibm-runtime matplotlib numpy scipy pandas

# --- Run this code block ONCE securely ---

from qiskit_ibm_runtime import QiskitRuntimeService

# Replace the placeholder below with your REAL API token from IBM Quantum

my_api_token = "PASTE YOUR IBM QUANTUM API TOKEN HERE"

try:

# Save the account information for Qiskit to use later

# Use channel='ibm_quantum' for the cloud service

# overwrite=True allows updating if you saved before

QiskitRuntimeService.save_account(channel="ibm_quantum", token=my_api_token, overwrite=True)

print("--- IBM Quantum Account Saved Successfully! ---")

print("You can now run the main experiment script.")

except Exception as e:

print(f"--- ERROR saving account: {e} ---")

print("Please double-check your API token and try again.")

# --- End of secure setup code ---

# --- Enhanced Experiment Code: bell_state_multi_run.py ---

# --- UPDATE: TARGET_LAYOUTS=[[2, 3], [7, 8]], NUM_RUNS=5, Handles NaN/Type/ProbDist from M3, Requires EXPLICIT TARGET_LAYOUTS, Includes Multiple Runs, Hardware Targets, Calibration Data, Noise Sim, M3 Readout Mitigation ---

# Imports

import datetime

import traceback

import time # For potential delays between runs

import json # For saving complex data like calibration

import numpy as np

import matplotlib.pyplot as plt

from scipy.stats import binomtest

# Qiskit imports

from qiskit import QuantumCircuit, transpile, QuantumRegister

from qiskit.visualization import circuit_drawer, plot_histogram

# Removed: from qiskit.providers.models import BackendProperties

from qiskit.quantum_info import hellinger_fidelity # Example metric

# Qiskit Runtime and Aer

from qiskit_ibm_runtime import QiskitRuntimeService, SamplerV2 as RuntimeSampler, Session, Options

from qiskit_aer import AerSimulator

from qiskit_aer.noise import NoiseModel, depolarizing_error, ReadoutError # For noise simulation

# Qiskit Result Handling

from qiskit.result import marginal_counts

# Measurement Mitigation using M3 library

try:

import mthree # For Measurement Mitigation

except ImportError:

print("Warning: 'mthree' package not found. Readout mitigation will be disabled.")

print("Install with: pip install mthree")

mthree = None # Set to None if not found

# --- Configuration Parameters ---

# === Benchmarking Settings ===

TARGET_BACKENDS = ["ibm_kyiv"] # <<< MODIFY AS NEEDED

# --- IMPORTANT: TARGET_LAYOUTS ---

# Set to test layouts [2, 3] and [7, 8]

TARGET_LAYOUTS = [[2, 3], [7, 8]] # <<< MODIFIED as requested

# --- Number of Runs ---

NUM_RUNS = 5 # <<< MODIFIED to 5 runs per target

SUBMISSION_DELAY = 5 # Increased delay slightly for more jobs

# === General Settings ===

SHOTS = 4096

# === Feature Flags ===

ENABLE_CALIBRATION_LOGGING = True

ENABLE_NOISE_SIMULATION = False

ENABLE_READOUT_MITIGATION = True if mthree else False # Only enable if mthree is imported

# === Noise Simulation Settings (if ENABLE_NOISE_SIMULATION=True) ===

NOISE_SIM_GATE_ERROR_RATE = 0.005

NOISE_SIM_READOUT_ERROR_RATE = 0.01

NOISE_SIM_CLIMATES = {

"Low Noise": {"gate": 0.001, "readout": 0.005},

"Medium Noise": {"gate": 0.005, "readout": 0.01},

}

# === Data Storage ===

all_results = []

calibration_data_log = {}

# --- The Quantum Circuit (Bell State |Φ⁺⟩) ---

qc_bell = QuantumCircuit(2, 2, name="Bell_State_Test")

qc_bell.h(0)

qc_bell.cx(0, 1)

qc_bell.measure([0, 1], [0, 1])

print("--- Enhanced Bell State Fidelity Experiment ---")

print(f"Circuit: Bell State |Φ⁺⟩")

print(qc_bell.draw(output='text'))

# --- Helper Function to Calculate Metrics (NaN Safe) ---

# ... (calculate_metrics function remains the same as v6) ...

def calculate_metrics(counts_or_probs_dict, shots):

"""Calculates probabilities, P(Corr), P(Anti) from counts or probabilities.

Handles potential non-numeric values and NaNs."""

metrics = {'counts': {}, 'probs': {}, 'p_corr': np.nan, 'p_anti': np.nan}

is_probs = isinstance(shots, float) and shots == 1.0

processed_numeric_dict = {'00': 0.0, '01': 0.0, '10': 0.0, '11': 0.0}

if isinstance(counts_or_probs_dict, dict):

for k in processed_numeric_dict.keys():

raw_value = counts_or_probs_dict.get(k, 0)

try:

numeric_value = float(raw_value)

if np.isnan(numeric_value): processed_numeric_dict[k] = np.nan if is_probs else 0.0

else: processed_numeric_dict[k] = numeric_value

except (ValueError, TypeError): processed_numeric_dict[k] = np.nan if is_probs else 0.0

else: print(f"Warning: Invalid input type ({type(counts_or_probs_dict)}) to calculate_metrics."); return metrics

if is_probs:

metrics['probs'] = processed_numeric_dict

metrics['counts'] = {k: 'N/A (from probs)' for k in processed_numeric_dict}

else:

metrics['counts'] = processed_numeric_dict

valid_counts = [v for v in metrics['counts'].values() if not np.isnan(v)]; total_counts = sum(valid_counts)

if total_counts > 0 and not np.isnan(total_counts): metrics['probs'] = {k: (v / total_counts) if not np.isnan(v) else 0.0 for k, v in metrics['counts'].items()}

else: metrics['probs'] = {k: 0.0 for k in processed_numeric_dict}

try:

p00 = metrics['probs'].get('00', 0.0); p00 = p00 if not np.isnan(p00) else np.nan

p01 = metrics['probs'].get('01', 0.0); p01 = p01 if not np.isnan(p01) else np.nan

p10 = metrics['probs'].get('10', 0.0); p10 = p10 if not np.isnan(p10) else np.nan

p11 = metrics['probs'].get('11', 0.0); p11 = p11 if not np.isnan(p11) else np.nan

metrics['p_corr'] = p00 + p11; metrics['p_anti'] = p01 + p10

if np.isnan(metrics['p_corr']): metrics['p_corr'] = np.nan

if np.isnan(metrics['p_anti']): metrics['p_anti'] = np.nan

except Exception as calc_e: print(f"Warning: Error during final metric calculation: {calc_e}"); metrics['p_corr'] = np.nan; metrics['p_anti'] = np.nan

return metrics

# --- Helper Function to Get Specific Calibration Data ---

backend = None # Define globally for access in helper function

def get_calibration_for_qubits(properties, layout: list):

"""Extracts relevant calibration data for a given qubit layout."""

# ... (Function body remains the same as v6 - uses global 'backend') ...

if properties is None or layout is None or len(layout) != 2: print("Warning: Cannot get calibration data - properties or layout invalid."); return {}

q0, q1 = layout[0], layout[1]; cal_data = {}

def safe_get(func, *args):

try: return func(*args)

except: return None

cal_data['T1_q0'] = safe_get(properties.t1, q0); cal_data['T1_q1'] = safe_get(properties.t1, q1)

cal_data['T2_q0'] = safe_get(properties.t2, q0); cal_data['T2_q1'] = safe_get(properties.t2, q1)

cal_data['ReadoutError_q0'] = safe_get(properties.readout_error, q0); cal_data['ReadoutError_q1'] = safe_get(properties.readout_error, q1)

cal_data['Frequency_q0'] = safe_get(properties.frequency, q0); cal_data['Frequency_q1'] = safe_get(properties.frequency, q1)

cx_error = None; gate_list = []

if hasattr(properties, 'gates'): gate_list = properties.gates

for gate in gate_list:

gate_name = getattr(gate, 'gate', getattr(gate, 'name', None))

if gate_name == 'cx' and hasattr(gate, 'qubits') and set(gate.qubits) == set([q0, q1]):

cx_error = safe_get(properties.gate_error, gate_name, gate.qubits)

if cx_error is not None: break

cal_data['CNOT_error'] = cx_error

cal_data['H_error_q0'] = safe_get(properties.gate_error, 'h', [q0])

if cal_data['H_error_q0'] is None and backend is not None:

try:

if hasattr(backend,'basis_gates') and 'sx' in backend.basis_gates: cal_data['H_error_q0'] = safe_get(properties.gate_error, 'sx', [q0])

except Exception: pass

return cal_data

# --- 1. Ideal Simulation (Baseline) ---

print("\n--- Running Ideal Simulation (AerSimulator) ---")

# ... (Ideal simulation code remains the same) ...

ideal_sim_results = {'backend': 'Ideal (Sim)', 'layout': 'N/A', 'run': 0, 'job_id': 'N/A', 'timestamp': str(datetime.datetime.now())}

try:

aer_sim = AerSimulator(); ideal_tqc = transpile(qc_bell, aer_sim)

job_ideal = aer_sim.run(ideal_tqc, shots=SHOTS); result_ideal = job_ideal.result()

ideal_raw_counts = result_ideal.get_counts(ideal_tqc)

ideal_metrics = calculate_metrics(ideal_raw_counts, SHOTS)

ideal_sim_results.update(ideal_metrics)

print(f"Ideal Result: P(Corr) = {ideal_sim_results['p_corr']:.4f}, P(Anti) = {ideal_sim_results['p_anti']:.4f}")

all_results.append(ideal_sim_results)

except Exception as e: print(f"Error during ideal simulation: {e}"); traceback.print_exc()

# --- 2. Noise Simulation (Optional) ---

if ENABLE_NOISE_SIMULATION:

print("\n--- Running Controlled Noise Simulations (AerSimulator) ---")

# ... (Noise simulation code remains the same) ...

for climate_name, noise_params in NOISE_SIM_CLIMATES.items():

print(f"  Simulating climate: '{climate_name}'")

noise_sim_results = {'backend': f'Sim ({climate_name})', 'layout': 'N/A', 'run': 0, 'job_id': 'N/A', 'timestamp': str(datetime.datetime.now())}

try:

noise_model=NoiseModel(); dep_error_cx = depolarizing_error(noise_params["gate"], 2); noise_model.add_all_qubit_quantum_error(dep_error_cx, ['cx'])

p0g1 = noise_params["readout"]; p1g0 = noise_params["readout"]; readout_error = ReadoutError([[1 - p1g0, p1g0], [p0g1, 1 - p0g1]]); noise_model.add_all_qubit_readout_error(readout_error)

aer_sim_noisy = AerSimulator(noise_model=noise_model); noisy_tqc = transpile(qc_bell, aer_sim_noisy)

job_noise = aer_sim_noisy.run(noisy_tqc, shots=SHOTS); result_noise = job_noise.result(); noise_raw_counts = result_noise.get_counts(noisy_tqc)

noise_metrics = calculate_metrics(noise_raw_counts, SHOTS); noise_sim_results.update(noise_metrics)

print(f"  Result: P(Corr) = {noise_sim_results['p_corr']:.4f}, P(Anti) = {noise_sim_results['p_anti']:.4f}")

all_results.append(noise_sim_results)

except Exception as e: print(f"  Error during noise simulation for '{climate_name}': {e}"); traceback.print_exc()

# --- 3. Hardware Execution ---

print("\n--- Running on Real IBM Quantum Hardware ---")

mitigators = {}

backend = None # Ensure backend is defined before the loop

try:

print("Connecting to IBM Quantum service...")

service = QiskitRuntimeService()

print("Connected.")

# === Loop through Target Backends ===

for backend_name in TARGET_BACKENDS:

print(f"\n=== Processing Backend: {backend_name} ===")

backend = None # Reset backend for each loop iteration

try:

backend = service.backend(backend_name) # Assign to outer scope variable

print(f"Acquired backend '{backend_name}' (Status: {backend.status().status_msg})")

# --- Query and Print Coupling Map ---

if hasattr(backend, 'coupling_map') and backend.coupling_map:

print(f"\nAvailable CNOT connections (qubit pairs) on {backend_name}:")

connected_pairs = []

for pair in backend.coupling_map:

sorted_pair = sorted(pair)

if sorted_pair not in connected_pairs:

connected_pairs.append(sorted_pair); # Optionally print: print(f"  - {sorted_pair}")

print(f"  (Found {len(connected_pairs)} unique pairs - check list above if needed)")

if not connected_pairs: print("  - No connected pairs found.")

else: print(f"Warning: Could not retrieve coupling map for {backend_name}.")

# === Check TARGET_LAYOUTS Configuration ===

if not TARGET_LAYOUTS or not isinstance(TARGET_LAYOUTS, list) or not all(isinstance(p, list) and len(p)==2 for p in TARGET_LAYOUTS):

print("\nERROR: TARGET_LAYOUTS not set correctly. Edit script with valid pair(s)."); continue

backend_properties = None

if ENABLE_CALIBRATION_LOGGING:

# ... (Calibration retrieval code remains the same) ...

try:

print(f"Retrieving calibration properties for {backend_name}...")

backend_properties = backend.properties(refresh=True)

if backend_properties:

calibration_data_log[backend_name] = {"retrieval_time": str(datetime.datetime.now()), "full_properties": None}

print("Calibration data retrieved.")

else: print("Warning: Backend properties retrieval returned None."); backend_properties = None

except Exception as cal_e: print(f"Warning: Could not retrieve calibration data for {backend_name}: {cal_e}"); backend_properties = None

# === Loop through EXPLICIT Target Layouts ===

for initial_layout in TARGET_LAYOUTS: # Iterates through e.g., [[2, 3], [7, 8]]

current_initial_layout = list(initial_layout) if isinstance(initial_layout, (list, tuple)) else None

if current_initial_layout is None or len(current_initial_layout) != 2:

print(f"Skipping invalid layout configuration: {initial_layout}"); continue

layout_key = str(current_initial_layout)

print(f"\n--- Testing Explicit Layout: {current_initial_layout} on {backend_name} ---")

tqc = None

print("Transpiling circuit for layout", current_initial_layout, "...")

try:

tqc = transpile(qc_bell, backend=backend, initial_layout=current_initial_layout, optimization_level=1)

print(f"Transpilation successful for layout {current_initial_layout}.")

except Exception as transpile_e: print(f"Error during transpilation for layout {current_initial_layout}: {transpile_e}"); traceback.print_exc(); continue

layout_cal_data = {}

if ENABLE_CALIBRATION_LOGGING and backend_properties:

# ... (Specific calibration logging remains the same) ...

try:

layout_cal_data = get_calibration_for_qubits(backend_properties, current_initial_layout)

if backend_name not in calibration_data_log: calibration_data_log[backend_name] = {}

if "layouts" not in calibration_data_log[backend_name]: calibration_data_log[backend_name]["layouts"] = {}

calibration_data_log[backend_name]["layouts"][layout_key] = layout_cal_data

print(f"Specific calibration data logged for layout {current_initial_layout}.")

except Exception as spec_cal_e: print(f"Warning: Could not extract/log specific calibration for layout {current_initial_layout}: {spec_cal_e}")

# === M3 Readout Mitigation Calibration ===

mitigator_key = f"{backend_name}_{layout_key}"

if ENABLE_READOUT_MITIGATION and mitigator_key not in mitigators:

# ... (M3 calibration logic remains the same) ...

if not mthree: print("M3 library not available, skipping mitigation."); ENABLE_READOUT_MITIGATION = False

else:

print(f"Starting Readout Calibration (mthree) for layout {current_initial_layout}...")

try:

mit = mthree.M3Mitigation(backend); mit.cals_from_system(current_initial_layout, shots=SHOTS)

mitigators[mitigator_key] = mit; print("M3 Readout calibration prepared.")

except Exception as mit_cal_e: print(f"Error during mthree readout mitigation calibration for layout {current_initial_layout}: {mit_cal_e}"); traceback.print_exc()

# === Loop for Multiple Runs ===

print(f"Starting {NUM_RUNS} execution runs for layout {current_initial_layout}...")

run_job_ids = []

for run_num in range(NUM_RUNS):

print(f"  Submitting Run {run_num + 1}/{NUM_RUNS}...")

run_results = {'backend': backend_name, 'layout': current_initial_layout, 'run': run_num + 1, 'job_id': 'N/A', 'timestamp': str(datetime.datetime.now())}

try:

with Session(backend=backend) as session:

sampler = RuntimeSampler() # Initialize without session

pub = (tqc,)

job_hw = sampler.run([pub], shots=SHOTS) # Run associates with session

job_id = job_hw.job_id(); run_results['job_id'] = job_id; run_job_ids.append(job_id)

print(f"  Run {run_num + 1} job submitted (ID: {job_id}). Waiting...")

result_hw = job_hw.result()

print(f"  Job {job_id} finished with status: {job_hw.status()}")

if len(result_hw) > 0:

pub_result = result_hw[0]; data_bin = pub_result.data

num_clbits = qc_bell.num_clbits

hardware_raw_counts_bin = {format(i, f'0{num_clbits}b'): 0 for i in range(1 << num_clbits)}

# --- Process Results (Handles String Keys) --- <<< Updated in v7

valid_counts_found = False # Flag

try:

creg_name = qc_bell.cregs[0].name

# print(f"Debug: Accessing classical register '{creg_name}'") # Optional Debug

if hasattr(data_bin, creg_name):

register_data = getattr(data_bin, creg_name)

hardware_raw_counts_str_keys = register_data.get_counts()

# print(f"Debug: Raw Str Counts from register '{creg_name}': {hardware_raw_counts_str_keys}") # Optional Debug

max_int_key = (1 << num_clbits) - 1

filtered_counts = {}

for key_str, value in hardware_raw_counts_str_keys.items():

try:

try: key_int = int(key_str, 2)

except ValueError: key_int = int(key_str)

if 0 <= key_int <= max_int_key: filtered_counts[key_int] = value; valid_counts_found = True

else: print(f"Warning: Ignoring unexpected outcome key {key_int} (from '{key_str}')")

except (ValueError, TypeError) as e_conv: print(f"Warning: Could not convert key '{key_str}'. Ignoring. Error: {e_conv}")

for key_int, value in filtered_counts.items(): hardware_raw_counts_bin[format(key_int, f'0{num_clbits}b')] = value

if not valid_counts_found and hardware_raw_counts_str_keys: print("Warning: No valid counts found after processing register data.")

elif not hardware_raw_counts_str_keys: print("Debug: Register data had no counts.")

else: print(f"Error: Could not find classical register '{creg_name}' in results data.")

except Exception as e_results: print(f"Error processing results data: {e_results}"); traceback.print_exc()

# --- End Process Results ---

raw_metrics = calculate_metrics(hardware_raw_counts_bin, SHOTS)

run_results.update(raw_metrics)

if np.isnan(raw_metrics['p_corr']): print("  Warning: Raw metrics calculation resulted in NaN.")

print(f"  Raw Results (Filtered): P(Corr)={raw_metrics['p_corr']:.4f}, P(Anti)={raw_metrics['p_anti']:.4f}")

# Apply M3 Readout Mitigation (Handles NaN/Type/ProbDistribution)

if ENABLE_READOUT_MITIGATION and mitigator_key in mitigators:

# ... (Mitigation application logic remains the same as v6) ...

mitigated_results_valid = False; mit_probs = {}; states = [format(i, f'0{num_clbits}b') for i in range(2**num_clbits)]

try:

mit = mitigators[mitigator_key]

# print(f"Debug M3 Inputs: Counts = {hardware_raw_counts_bin}, Layout = {current_initial_layout}") # Optional Debug

quasi_probs = mit.apply_correction(hardware_raw_counts_bin, current_initial_layout, return_mitigation_overhead=False)

mitigated_probs_vector_raw = quasi_probs.nearest_probability_distribution()

# print(f"Debug: Type returned by nearest_probability_distribution: {type(mitigated_probs_vector_raw)}")

if isinstance(mitigated_probs_vector_raw, (np.ndarray, list)):

# print("Debug: Handling mitigated result as numpy array/list.")

try:

mitigated_probs_vector = np.asarray(mitigated_probs_vector_raw, dtype=float)

if np.isnan(mitigated_probs_vector).any(): print("  Warning: NaN values detected in mthree vector."); mit_probs = {s: np.nan for s in states}

else: mit_probs = {states[i]: p for i, p in enumerate(mitigated_probs_vector)}

except (TypeError, ValueError) as conversion_error: print(f"  Warning: Could not convert array/list to float. Error: {conversion_error}"); mit_probs = {s: np.nan for s in states}

elif hasattr(mitigated_probs_vector_raw, 'get') or hasattr(mitigated_probs_vector_raw,'__getitem__'):

# print("Debug: Handling mitigated result as ProbDistribution-like object (dict access).")

temp_probs = {}

for i, state_str in enumerate(states):

prob = 0.0; raw_extracted_val = None

try:

raw_extracted_val = mitigated_probs_vector_raw.get(state_str)

if raw_extracted_val is None and state_str not in mitigated_probs_vector_raw:

try: raw_extracted_val = mitigated_probs_vector_raw[i]

except (IndexError, KeyError, TypeError): raw_extracted_val = mitigated_probs_vector_raw.get(i)

if raw_extracted_val is None: prob = 0.0

else: float_prob = float(raw_extracted_val); prob = float_prob if not np.isnan(float_prob) else np.nan

except (KeyError, IndexError): prob = 0.0

except (ValueError, TypeError): print(f"  Warning: Non-numeric value '{raw_extracted_val}' extracted for state '{state_str}'. Storing NaN."); prob = np.nan

except Exception as e_access: print(f"  Warning: Error accessing prob for state '{state_str}'. Storing NaN. Error: {e_access}"); prob = np.nan

temp_probs[state_str] = prob

mit_probs = temp_probs

else: print(f"  Warning: Unhandled type from nearest_probability_distribution: {type(mitigated_probs_vector_raw)}. Storing NaN."); mit_probs = {s: np.nan for s in states}

if any(np.isnan(p) for p in mit_probs.values()): print("  Warning: NaN values present after processing mitigated probs."); mitigated_results_valid = False

else:

mit_metrics = calculate_metrics(mit_probs, 1.0)

run_results['mitigated_probs'] = mit_metrics['probs']; run_results['mitigated_p_corr'] = mit_metrics['p_corr']; run_results['mitigated_p_anti'] = mit_metrics['p_anti']

if not np.isnan(mit_metrics['p_corr']):

total_hw_shots_approx = sum(v for v in hardware_raw_counts_bin.values() if isinstance(v,int))

run_results['mitigated_counts'] = {s: round(p * total_hw_shots_approx) for s, p in mit_probs.items() if not np.isnan(p)}

print(f"  M3 Readout Mitigation Applied: P(Corr)={mit_metrics['p_corr']:.4f}, P(Anti)={mit_metrics['p_anti']:.4f}")

mitigated_results_valid = True

else: print("  M3 Readout Mitigation resulted in NaN metrics after calculation."); run_results['mitigated_counts'] = {s: 'NaN' for s in states}; mitigated_results_valid = False

except Exception as mit_apply_e: print(f"  Warning: Could not apply mthree readout mitigation: {mit_apply_e}"); traceback.print_exc(); mitigated_results_valid = False

if not mitigated_results_valid:

num_clbits = qc_bell.num_clbits; states = [format(i, f'0{num_clbits}b') for i in range(2**num_clbits)]

run_results['mitigated_probs'] = {s: np.nan for s in states}; run_results['mitigated_p_corr'] = np.nan

run_results['mitigated_p_anti'] = np.nan; run_results['mitigated_counts'] = {s: 'NaN' for s in states}

else: print("  No results received from hardware job.")

# ... rest of run processing ...

all_results.append(run_results)

print(f"  Run {run_num + 1} processed.")

except Exception as run_e:

print(f"  --- ERROR during hardware run {run_num + 1} ---"); print(f"  An error occurred: {type(run_e).__name__}: {run_e}"); traceback.print_exc()

run_results['p_corr'] = 'ERROR'; all_results.append(run_results)

if run_num < NUM_RUNS - 1 and SUBMISSION_DELAY > 0: print(f"  Waiting {SUBMISSION_DELAY}s..."); time.sleep(SUBMISSION_DELAY)

except Exception as backend_e:

print(f"--- ERROR processing backend {backend_name} ---"); print(f"An error occurred: {type(backend_e).__name__}: {backend_e}"); traceback.print_exc()

except Exception as service_e:

print("\n--- ERROR connecting to IBM Quantum Service or during main hardware loop ---"); print(f"An error occurred: {type(service_e).__name__}: {service_e}"); traceback.print_exc()

# --- 4. Data Aggregation and Analysis (NaN Safe) ---

print("\n--- Aggregating and Analyzing Results ---")

# ... (Aggregation code remains the same as v6 - already handles NaN) ...

aggregated_results = {}

def get_layout_key(layout):

if isinstance(layout, list): return tuple(layout)

return layout

for res in all_results:

layout_tuple_key = get_layout_key(res.get('layout', 'N/A'))

key = f"{res.get('backend', 'Unknown')}_{str(layout_tuple_key)}"

is_valid_raw = 'p_corr' in res and isinstance(res['p_corr'], (float, np.floating)) and not np.isnan(res['p_corr'])

is_valid_mit = 'mitigated_p_corr' in res and isinstance(res['mitigated_p_corr'], (float, np.floating)) and not np.isnan(res['mitigated_p_corr'])

if key not in aggregated_results: aggregated_results[key] = {'p_corr_list': [], 'p_anti_list': [], 'mit_p_corr_list': [], 'mit_p_anti_list': [], 'count': 0, 'layout': res.get('layout', 'N/A')}

if is_valid_raw:

aggregated_results[key]['p_corr_list'].append(res['p_corr'])

aggregated_results[key]['p_anti_list'].append(res['p_anti'])

aggregated_results[key]['count'] += 1

if is_valid_mit:

aggregated_results[key]['mit_p_corr_list'].append(res['mitigated_p_corr'])

aggregated_results[key]['mit_p_anti_list'].append(res['mitigated_p_anti'])

print("\nSummary Statistics (Mean ± Std Dev - NaN runs excluded):")

sorted_keys = sorted(aggregated_results.keys())

for key in sorted_keys:

data = aggregated_results[key]

if data['count'] > 0:

valid_p_corr = [p for p in data['p_corr_list'] if not np.isnan(p)]; valid_p_anti = [p for p in data['p_anti_list'] if not np.isnan(p)]

valid_mit_p_corr = [p for p in data['mit_p_corr_list'] if not np.isnan(p)]; valid_mit_p_anti = [p for p in data['mit_p_anti_list'] if not np.isnan(p)]

num_valid_raw = len(valid_p_corr)

mean_p_corr = np.mean(valid_p_corr) if num_valid_raw > 0 else np.nan; std_p_corr = np.std(valid_p_corr) if num_valid_raw > 1 else 0

mean_p_anti = np.mean(valid_p_anti) if num_valid_raw > 0 else np.nan; std_p_anti = np.std(valid_p_anti) if num_valid_raw > 1 else 0

print(f"\nTarget: {key} ({num_valid_raw} valid runs)")

print(f"  Layout Used: {data['layout']}")

print(f"  Raw P(Corr): {mean_p_corr*100:.2f}% ± {std_p_corr*100:.2f}%")

print(f"  Raw P(Anti): {mean_p_anti*100:.2f}% ± {std_p_anti*100:.2f}%")

num_valid_mit = len(valid_mit_p_corr)

if num_valid_mit > 0:

mean_mit_p_corr = np.mean(valid_mit_p_corr); std_mit_p_corr = np.std(valid_mit_p_corr) if num_valid_mit > 1 else 0

mean_mit_p_anti = np.mean(valid_mit_p_anti); std_mit_p_anti = np.std(valid_mit_p_anti) if num_valid_mit > 1 else 0

print(f"  Mitigated P(Corr) ({num_valid_mit} valid runs): {mean_mit_p_corr*100:.2f}% ± {std_mit_p_corr*100:.2f}%")

print(f"  Mitigated P(Anti) ({num_valid_mit} valid runs): {mean_mit_p_anti*100:.2f}% ± {std_mit_p_anti*100:.2f}%")

elif ENABLE_READOUT_MITIGATION:

mit_attempted_and_failed = any( ('mitigated_p_corr' in res and np.isnan(res['mitigated_p_corr'])) for res in all_results if f"{res.get('backend', 'Unknown')}_{str(get_layout_key(res.get('layout', 'N/A')))}" == key)

if mit_attempted_and_failed: print("  Mitigated results: NaN or Error during processing for all valid runs.")

else: print("  Mitigated results: Not available (Check logs - Mitigation disabled, errored, or no valid runs?).")

# --- TODO: Add Correlation Analysis Code Here ---

print("\nPlaceholder for Calibration Correlation Analysis...")

# --- TODO: Add ZNE Implementation Here (Optional/Advanced) ---

print("\nPlaceholder for Zero-Noise Extrapolation (ZNE) Implementation...")

# --- 5. Save Detailed Results (NaN Safe) ---

# ... (Saving code remains the same as v6 - already NaN safe) ...

results_filename = f"bell_state_results_{datetime.datetime.now():%Y%m%d_%H%M%S}.json"

calibration_filename = f"calibration_log_{datetime.datetime.now():%Y%m%d_%H%M%S}.json"

try:

def convert_numpy_and_datetime(obj):

if isinstance(obj, np.integer): return int(obj)

elif isinstance(obj, np.floating): return None if np.isnan(obj) else float(obj)

elif isinstance(obj, np.ndarray): return obj.tolist()

elif isinstance(obj, (datetime.datetime, datetime.date)): return obj.isoformat()

elif isinstance(obj, np.bool_): return bool(obj)

elif isinstance(obj, np.complex_): return {'real': obj.real, 'imag': obj.imag}

try: return json.JSONEncoder().encode(obj)

except TypeError: return str(obj)

with open(results_filename, 'w') as f: json.dump(all_results, f, indent=4, default=convert_numpy_and_datetime)

print(f"\nDetailed run results saved to: {results_filename}")

if ENABLE_CALIBRATION_LOGGING:

with open(calibration_filename, 'w') as f:

log_to_save = {}

for bk, bk_data in calibration_data_log.items(): log_to_save[bk] = { "retrieval_time": bk_data.get("retrieval_time"), "layouts": bk_data.get("layouts", {}) }

json.dump(log_to_save, f, indent=4, default=convert_numpy_and_datetime)

print(f"Calibration data log saved to: {calibration_filename}")

except Exception as save_e: print(f"\nError saving results/calibration data: {save_e}")

# --- 6. Visualization (Example: Comparing aggregated results - NaN Safe) ---

print("\n--- Generating Aggregate Visualizations (Example) ---")

# ... (Plotting code remains the same as v6 - already handles NaN) ...

try:

valid_keys = [k for k, v in aggregated_results.items() if v['count'] > 0 and 'Ideal (Sim)' not in k]

if not valid_keys: print("No valid aggregated hardware data to plot.")

else:

labels = valid_keys

raw_p_anti_means = [np.mean([p for p in aggregated_results[k]['p_anti_list'] if not np.isnan(p)]) if [p for p in aggregated_results[k]['p_anti_list'] if not np.isnan(p)] else np.nan for k in labels]

raw_p_anti_stds = [np.std([p for p in aggregated_results[k]['p_anti_list'] if not np.isnan(p)]) if aggregated_results[k]['count'] > 1 else 0 for k in labels]

plot_mitigated = False; mit_p_anti_means=[]; mit_p_anti_stds=[]

if ENABLE_READOUT_MITIGATION:

valid_mit_data_exists = any(aggregated_results[k]['mit_p_anti_list'] and not np.isnan(aggregated_results[k]['mit_p_anti_list']).all() for k in labels if k in aggregated_results)

if valid_mit_data_exists:

mit_p_anti_means = [np.mean([p for p in aggregated_results[k]['mit_p_anti_list'] if not np.isnan(p)]) if [p for p in aggregated_results[k]['mit_p_anti_list'] if not np.isnan(p)] else np.nan for k in labels]

mit_p_anti_stds = [np.std([p for p in aggregated_results[k]['mit_p_anti_list'] if not np.isnan(p)]) if len(aggregated_results[k]['mit_p_anti_list']) > 1 else 0 for k in labels]

if not np.isnan(mit_p_anti_means).all(): plot_mitigated = True

x = np.arange(len(labels)); width = 0.35 if plot_mitigated else 0.7

fig, ax = plt.subplots(figsize=(max(10, len(labels)*1.2), 6))

valid_raw_indices = [i for i, mean in enumerate(raw_p_anti_means) if not np.isnan(mean)]

if valid_raw_indices: ax.bar(x[valid_raw_indices] - width/2 if plot_mitigated else x[valid_raw_indices], np.array(raw_p_anti_means)[valid_raw_indices], width, label='Raw P(Anti)', yerr=np.array(raw_p_anti_stds)[valid_raw_indices], capsize=4, alpha=0.8)

if plot_mitigated:

valid_mit_indices = [i for i, mean in enumerate(mit_p_anti_means) if not np.isnan(mean)]

if valid_mit_indices:

if len(valid_mit_indices) <= len(x): ax.bar(x[valid_mit_indices] + width/2, np.array(mit_p_anti_means)[valid_mit_indices], width, label='Mitigated P(Anti)', yerr=np.array(mit_p_anti_stds)[valid_mit_indices], capsize=4, alpha=0.8)

else: print("Warning: Mismatch in length/indices of mitigated data for plotting.")

ax.set_ylabel('Probability P(Anticorrelated)'); ax.set_title('Mean P(Anticorrelated) across Different Targets/Runs')

ax.set_xticks(x); ax.set_xticklabels([labels[i] for i in range(len(labels))], rotation=60, ha="right")

ax.legend(); ax.grid(axis='y', linestyle='--', alpha=0.7); ax.set_ylim(bottom=0)

fig.tight_layout(); agg_plot_filename = 'aggregated_p_anti_comparison.png'

plt.savefig(agg_plot_filename); print(f"Aggregated P(Anti) comparison plot saved as {agg_plot_filename}"); plt.close(fig)

except ImportError: print("\nWarning: Matplotlib/NumPy needed for aggregate visualization.")

except Exception as agg_plot_e: print(f"\nError during aggregate visualization: {agg_plot_e}"); traceback.print_exc()

print(f"\n--- Experiment Finished ({datetime.datetime.now()}) ---")

# --- End of enhanced experiment code ---

Experiment Results

--- Enhanced Bell State Fidelity Experiment --- Circuit: Bell State ¦Φ⁺⟩ ┌───┐ ┌─┐ q_0: ┤ H ├──■──┤M├─── └───┘┌─┴─┐└╥┘┌─┐ q_1: ─────┤ X ├─╫─┤M├ └───┘ ║ └╥┘ c: 2/═══════════╩══╩═ 0 1 --- Running Ideal Simulation (AerSimulator) --- Ideal Result: P(Corr) = 1.0000, P(Anti) = 0.0000 --- Running on Real IBM Quantum Hardware --- Connecting to IBM Quantum service... Connected. === Processing Backend: ibm_kyiv === Acquired backend 'ibm_kyiv' (Status: active) Available CNOT connections (qubit pairs) on ibm_kyiv: (Found 144 unique pairs - check list above if needed) Retrieving calibration properties for ibm_kyiv... ERROR:stevedore.extension:Could not load 'ibm_backend': cannot import name 'ProviderV1' from 'qiskit.providers' (c:\Users\JKDM\miniconda3\envs\cwq\Lib\site-packages\qiskit\providers\__init__.py) ERROR:stevedore.extension:Could not load 'ibm_dynamic_circuits': cannot import name 'ProviderV1' from 'qiskit.providers' (c:\Users\JKDM\miniconda3\envs\cwq\Lib\site-packages\qiskit\providers\__init__.py) ERROR:stevedore.extension:Could not load 'ibm_backend': cannot import name 'ProviderV1' from 'qiskit.providers' (c:\Users\JKDM\miniconda3\envs\cwq\Lib\site-packages\qiskit\providers\__init__.py) ERROR:stevedore.extension:Could not load 'ibm_dynamic_circuits': cannot import name 'ProviderV1' from 'qiskit.providers' (c:\Users\JKDM\miniconda3\envs\cwq\Lib\site-packages\qiskit\providers\__init__.py) Calibration data retrieved. --- Testing Explicit Layout: [2, 3] on ibm_kyiv --- Transpiling circuit for layout [2, 3] ... Transpilation successful for layout [2, 3]. Specific calibration data logged for layout [2, 3]. Starting Readout Calibration (mthree) for layout [2, 3]... M3 Readout calibration prepared. Starting 5 execution runs for layout [2, 3]... Submitting Run 1/5... Run 1 job submitted (ID: czy8qedqnmvg008w2560). Waiting... Job czy8qedqnmvg008w2560 finished with status: DONE Raw Results (Filtered): P(Corr)=0.9875, P(Anti)=0.0125 M3 Readout Mitigation Applied: P(Corr)=1.0000, P(Anti)=0.0000 Run 1 processed. Waiting 5s... Submitting Run 2/5... Run 2 job submitted (ID: czy8qxqkzhn0008d7fh0). Waiting... Job czy8qxqkzhn0008d7fh0 finished with status: DONE Raw Results (Filtered): P(Corr)=0.9790, P(Anti)=0.0210 M3 Readout Mitigation Applied: P(Corr)=0.9956, P(Anti)=0.0044 Run 2 processed. Waiting 5s... Submitting Run 3/5... Run 3 job submitted (ID: czy8r28kzhn0008d7fhg). Waiting... Job czy8r28kzhn0008d7fhg finished with status: DONE Raw Results (Filtered): P(Corr)=0.9849, P(Anti)=0.0151 M3 Readout Mitigation Applied: P(Corr)=1.0000, P(Anti)=0.0000 Run 3 processed. Waiting 5s... Submitting Run 4/5... Run 4 job submitted (ID: czy8rbsqnmvg008w259g). Waiting... Job czy8rbsqnmvg008w259g finished with status: DONE Raw Results (Filtered): P(Corr)=0.9844, P(Anti)=0.0156 M3 Readout Mitigation Applied: P(Corr)=1.0000, P(Anti)=0.0000 Run 4 processed. Waiting 5s... Submitting Run 5/5... Run 5 job submitted (ID: czy8rgjrxz8g008f4ghg). Waiting... Job czy8rgjrxz8g008f4ghg finished with status: DONE Raw Results (Filtered): P(Corr)=0.9866, P(Anti)=0.0134 M3 Readout Mitigation Applied: P(Corr)=1.0000, P(Anti)=0.0000 ERROR:stevedore.extension:Could not load 'ibm_backend': cannot import name 'ProviderV1' from 'qiskit.providers' (c:\Users\JKDM\miniconda3\envs\cwq\Lib\site-packages\qiskit\providers\__init__.py) ERROR:stevedore.extension:Could not load 'ibm_dynamic_circuits': cannot import name 'ProviderV1' from 'qiskit.providers' (c:\Users\JKDM\miniconda3\envs\cwq\Lib\site-packages\qiskit\providers\__init__.py) ERROR:stevedore.extension:Could not load 'ibm_backend': cannot import name 'ProviderV1' from 'qiskit.providers' (c:\Users\JKDM\miniconda3\envs\cwq\Lib\site-packages\qiskit\providers\__init__.py) ERROR:stevedore.extension:Could not load 'ibm_dynamic_circuits': cannot import name 'ProviderV1' from 'qiskit.providers' (c:\Users\JKDM\miniconda3\envs\cwq\Lib\site-packages\qiskit\providers\__init__.py) Run 5 processed. --- Testing Explicit Layout: [7, 8] on ibm_kyiv --- Transpiling circuit for layout [7, 8] ... Transpilation successful for layout [7, 8]. Specific calibration data logged for layout [7, 8]. Starting Readout Calibration (mthree) for layout [7, 8]... M3 Readout calibration prepared. Starting 5 execution runs for layout [7, 8]... Submitting Run 1/5... Run 1 job submitted (ID: czy8rv3rxz8g008f4gp0). Waiting... Job czy8rv3rxz8g008f4gp0 finished with status: DONE Raw Results (Filtered): P(Corr)=0.9148, P(Anti)=0.0852 M3 Readout Mitigation Applied: P(Corr)=1.0000, P(Anti)=0.0000 Run 1 processed. Waiting 5s... Submitting Run 2/5... Run 2 job submitted (ID: czy8rzvqnmvg008w25f0). Waiting... Job czy8rzvqnmvg008w25f0 finished with status: DONE Raw Results (Filtered): P(Corr)=0.9150, P(Anti)=0.0850 M3 Readout Mitigation Applied: P(Corr)=1.0000, P(Anti)=0.0000 Run 2 processed. Waiting 5s... Submitting Run 3/5... Run 3 job submitted (ID: czy8sand8drg008hvpgg). Waiting... Job czy8sand8drg008hvpgg finished with status: DONE Raw Results (Filtered): P(Corr)=0.9102, P(Anti)=0.0898 M3 Readout Mitigation Applied: P(Corr)=1.0000, P(Anti)=0.0000 Run 3 processed. Waiting 5s... Submitting Run 4/5... Run 4 job submitted (ID: czy8sme6rr3g008me9ag). Waiting... Job czy8sme6rr3g008me9ag finished with status: DONE Raw Results (Filtered): P(Corr)=0.9016, P(Anti)=0.0984 M3 Readout Mitigation Applied: P(Corr)=1.0000, P(Anti)=0.0000 Run 4 processed. Waiting 5s... Submitting Run 5/5... Run 5 job submitted (ID: czy8sxz6rr3g008me9c0). Waiting... Job czy8sxz6rr3g008me9c0 finished with status: DONE Raw Results (Filtered): P(Corr)=0.8977, P(Anti)=0.1023 M3 Readout Mitigation Applied: P(Corr)=1.0000, P(Anti)=0.0000 Run 5 processed. --- Aggregating and Analyzing Results --- Summary Statistics (Mean ± Std Dev - NaN runs excluded): Target: Ideal (Sim)_N/A (1 valid runs) Layout Used: N/A Raw P(Corr): 100.00% ± 0.00% Raw P(Anti): 0.00% ± 0.00% Mitigated results: Not available (Check logs - Mitigation disabled, errored, or no valid runs?). Target: ibm_kyiv_(2, 3) (5 valid runs) Layout Used: [2, 3] Raw P(Corr): 98.45% ± 0.30% Raw P(Anti): 1.55% ± 0.30% Mitigated P(Corr) (5 valid runs): 99.91% ± 0.18% Mitigated P(Anti) (5 valid runs): 0.09% ± 0.18% Target: ibm_kyiv_(7, 8) (5 valid runs) Layout Used: [7, 8] Raw P(Corr): 90.79% ± 0.70% Raw P(Anti): 9.21% ± 0.70% Mitigated P(Corr) (5 valid runs): 100.00% ± 0.00% Mitigated P(Anti) (5 valid runs): 0.00% ± 0.00% Placeholder for Calibration Correlation Analysis... Placeholder for Zero-Noise Extrapolation (ZNE) Implementation... Detailed run results saved to: bell_state_results_20250414_122032.json Calibration data log saved to: calibration_log_20250414_122032.json --- Generating Aggregate Visualizations (Example) --- Aggregated P(Anti) comparison plot saved as aggregated_p_anti_comparison.png --- Experiment Finished (2025-04-14 12:20:32.719663) ---