Supplement 2

Eigenoscillation periods estimated from the observation data

The natural vibration of ports and harbors along the Japanese coastline, which has a complex geometry, requires consideration of not only the vibration within the port but also the resonance effect due to the relationship with reflected waves from the adjacent coast. It is thus appropriate to extract the peak period from the observation data after removing the tidal component adopting spectral analysis or other methods. We analyzed the natural frequency period of a harbor by locating a tidal station using the method described below. The data used are tide level observation data from the Japan Meteorological Agency and the Port and Harbor Bureau of the Ministry of Land, Infrastructure, Transport and Tourism, as described in Section 2, as well as tide level anomaly data in which long-period components such as astronomical tides with periods longer than 300 minutes are removed with a high-pass filter. To remove tidal level changes due to atmospheric waves generated by volcanic eruptions, we used data for the 18 hours from 00:00 to 18:00 JST on January 15, 2022, with 4320 records per site. The data for this period were used to calculate the power spectral density and extract the peak period. Figure S2.1 shows an example of the power spectral analysis for Amami with a primary mode at 12.1 minutes and secondary mode at 15.1 minutes. Table S2.1 gives the results for tidal stations along the Pacific coast. Among peak periods detected within the period range of 2–128 minutes, the maximum period up to the tertiary mode is given in the table. “N/A” indicates that the intensity of the tertiary spectral peak was less than one-tenth that of the primary mode.

Fig. S2.1 Example of the power spectral density of the tide level before the volcanic meteotsunami observed at Amami.

Table S2.1 Eigenoscillation periods for the Japan Pacific coastline

| Station name | Latitude (deg) | Longitude (deg) | Mode 1 (min) | Mode 2 (min) | Mode 3 (min) |
| --- | --- | --- | --- | --- | --- |
| Hanasaki Kiritappu Kushiro Urakawa Tomakomai (East) Muroran Hachinohe Kuji Miyako Ohfunato Ayukawa Ishinomaki Onahama Kashima, Chiba Mera Odawara Okada Miyakejima Chichijima Uchiura, Numazu Irouzaki Shimizukou Omaezaki Maisaka Akabane Nagoya Toba Owase Kumano Urakami Kushimoto Shirahama Gobou Tannowa Wakayama Awayuki Kochi Komatsushima Murotomisaki Tosashimizu Uwajima Saiki, Oita Miyazaki Aburatsu Kagoshima Makurazaki Tanegashima Amami Naha Minamidaitoujima Hirara, Miyakojima Ishigakijima Yonagunijima | 43.22 43.05 42.98 42.10 42.63 42.21 40.32 40.20 39.65 39.02 38.30 38.40 36.93 35.93 34.92 35.23 34.78 34.05 27.10 35.02 34.62 35.02 34.60 34.68 34.60 35.08 34.48 34.08 33.93 33.55 33.48 33.68 33.85 34.33 34.22 33.77 33.50 34.02 33.27 32.78 33.23 32.95 31.54 31.58 31.60 31.27 30.47 28.32 26.22 25.87 24.82 24.33 24.45 | 145.57 145.08 144.37 142.46 141.62 140.57 141.32 141.80 141.97 141.75 141.50 141.27 140.90 140.70 139.82 139.15 139.38 139.55 142.20 138.88 138.85 138.52 138.22 137.62 137.18 136.88 136.82 136.20 136.17 135.90 135.77 135.38 135.17 135.18 135.15 134.60 133.57 134.58 134.17 132.97 132.55 131.97 131.27 131.42 130.57 130.30 130.97 129.53 127.67 131.23 125.28 124.17 122.95 | 33.0 73.1 27.7 15.3 37.9 >128.0 6.1 14.0 48.8 4.0 7.8 85.3 24.4 64.0 2.3 6.2 5.6 4.8 18.0 19.3 5.5 64.0 23.8 31.0 15.3 2.6 19.3 11.9 14.4 12.8 22.3 41.0 4.9 >128.0 >128.0 14.4 16.5 >128.0 8.9 20.1 16.8 73.1 26.3 18.6 >128.0 16.3 14.0 12.2 26.9 6.4 48.8 17.1 10.1 | 64.0 33.0 51.2 13.8 31.0 53.9 6.7 9.5 26.3 4.1 6.6 33.0 85.3 26.3 6.1 6.0 5.2 5.2 20.1 8.4 6.0 19.7 17.4 15.5 19.7 >128.0 41.0 18.6 2.2 25.6 12.0 7.6 5.1 10.7 41.0 11.9 93.1 35.3 9.8 17.7 29.3 19.0 32.0 26.3 16.8 18.3 16.0 15.1 64.0 2.5 36.6 19.0 2.6 | 17.7 3.9 36.6 6.0 24.4 N/A 2.0 21.8 13.0 3.6 8.3 2.2 20.9 N/A 5.7 2.2 4.8 4.6 14.4 26.3 5.1 35.3 14.8 41.0 48.8 78.8 22.8 32.0 2.2 14.4 14.8 17.4 10.4 13.8 25.6 9.5 26.9 51.2 7.8 4.4 73.1 11.6 37.9 37.9 23.8 13.0 7.8 7.0 19.3 2.5 21.3 8.5 N/A |