Vital Signs Measurement Skill Assessment Observation Form

STUDENT NAME/NO:

| Implementation Steps | Correct | Incorrect/incomplete | Not implemented |
| --- | --- | --- | --- |
| Measuring Body Temperature |  |  |  |
| Performing hand hygiene |  |  |  |
| Identifying patient |  |  |  |
| Evaluating the area where the temperature will be measured |  |  |  |
| Positioning the measuring device in the appropriate area/keeping it at the appropriate distance |  |  |  |
| Applying the process |  |  |  |
| Ensuring hand hygiene |  |  |  |
| Saving the transaction |  |  |  |
| Blood Pressure Measurement |  |  |  |
| Performing hand hygiene |  |  |  |
| Identifying patient |  |  |  |
| Selection of an appropriate area |  |  |  |
| Positioning the patient's arm/leg in the appropriate position |  |  |  |
| Palpating the brachial or popliteal artery; positioning the cuff 25 cm above the pulse |  |  |  |
| Placement of the stethoscope receiver over the brachial or popliteal artery and the earphones in the ears |  |  |  |
| Closing valve of pressure bulb; rapidly inflates cuff to 30 mmHg above systolic pressure |  |  |  |
| Releasing pressure bulb slowly at a rate of 2-3 mmHg/second; noting value on manometer when first sound heard |  |  |  |
| Deflating cuff gently and noting the point at which dampened/muffled sound disappears |  |  |  |
| Removing cuff from extremity |  |  |  |
| Proper cleaning of equipment |  |  |  |
| Ensuring hand hygiene |  |  |  |
| Saving the transaction |  |  |  |
| Evaluation of the Pulse from the Radial Artery |  |  |  |
| Perform hand hygiene |  |  |  |
| Identify patient |  |  |  |
| Selection of an appropriate area |  |  |  |
| Appropriate positioning of the patient's arm |  |  |  |
| Placing tips of first two fingers over groove along radial or thumb side of patients’ inner wrist |  |  |  |
| Compressing against radius to obliterate pulse, then slightly releasing to palpate pulse |  |  |  |
| Counting 30 seconds if the pulse is rhythmic and 60 seconds if abnormal |  |  |  |
| Saving the transaction |  |  |  |
| Evaluation of Respiration |  |  |  |
| Monitoring the rib cage without telling the patient |  |  |  |
| Counting 30 seconds if breathing is rhythmic and 60 seconds if breathing is abnormal |  |  |  |
| Saving the transaction |  |  |  |
| Evaluation of oxygen saturation (SpO2) |  |  |  |
| Selection of an appropriate area |  |  |  |
| Correctly placing the tool on the skin |  |  |  |
| Implementing the process |  |  |  |
| Proper cleaning of equipment |  |  |  |
| Ensuring hand hygiene |  |  |  |
| Saving saturation |  |  |  |