Appendix A

An optimized piecewise linear method was applied to the nonlinear force-displacement data from the FE simulation to discriminate hip fracture cases from controls.

We assumed the force-displacement curve to be bilinear. The intersection point of the two linear pieces was identified through optimization with a Rosenbrock function that minimizes the difference between the FE-calculated force-displacement data points and the fitted linear segments. The Rosenbrock function is presented in Eq. (4) in which x1 and x2 demonstrate the FE-calculated force-displacement data points and the fitted linear segments, respectively.

| fx=100 (x22-x1)2+(1-x1)2 | (4) |
| --- | --- |

The optimization was constrained by imposing: 1) the initial stiffness, according to the first converged FE solver increments; 2) the final force-displacement point calculated by the FE solver; 3) the area under the piecewise linear (i.e., mechanical energy) curve should equal the area under the nonlinear force-displacement results returned by the FE solver. Calculations were done through an in-house MATLAB code.

Appendix B

Fig. 5 demonstrates 3D-DXA-based finite element results for a randomly selected femur under lateral fall loading.

| (a) | (b) |
| --- | --- |

Fig. 5. 3D-DXA-based FE results for one randomly selected femur under side fall configuration at maximum reaction force: maximum principal plastic strain distribution (a) and maximum principal stress distribution (b)