# Appendix

Table A.1.  Waste heat temperatures in various processes with temperature levels and waste heat fluctuation characteristics.

| Industrial Process | Exhaust gas temperature (°C) | Waste heat carrier | Waste heat fluctuation time |
| --- | --- | --- | --- |
| Aluminum reverberatory furnace | 1100–1200 | Mass flow and temperature | Minutes – hours |
| Nickel refining furnace | 1370–1650 | Mass flow and temperature | Minutes – hours |
| Copper reverberatory furnace | 900–1090 | Mass flow and temperature | Minutes – hours |
| Glass oven without regenerator | 900–1300 | Mass flow and temperature | Minutes – hours |
| Basic oxygen furnace | 1200 | Mass flow and temperature | Minutes – hours |
| Iron- and steelmaking | 1450–1550 | Mass flow and temperature | Minutes – hours |
| Steel electric arc furnace | 1370–1650 | Mass flow and temperature | Minutes – hours |
| Glass melting furnace | 1300–1540 | Mass flow and temperature | Minutes – hours |
| Steel heating furnace | 930–1040 | Mass flow and temperature | Minutes – hours |
| Copper reverberatory furnace | 900–1090 | Mass flow and temperature | Minutes – hours |
| Glass oven without regenerator | 900–1300 | Mass flow and temperature | Minutes – hours |
| Iron cupola | 820–980 | Mass flow and temperature | Minutes – hours |
| Cooper refining furnace | 760–820 | Mass flow and temperature | Minutes – hours |
| Reheating furnace without regenerator | 700–1200 | temperature | Minutes – hours |
| Hydrogen plants | 650–980 | temperature | Minutes |
| Fume incinerators | 650–1430 | temperature | Minutes – hours |
| Coke oven | 650–1000 | temperature | Minutes – hours |
| Glass oven with regenerator | 600–800 | Mass flow | Minutes |
| Cement kiln | 450–620 | Mass flow | Minutes |
| Melting oven | 400–700 | temperature | Minutes |
| Gas turbine exhaust | 370–540 | temperature | Minutes |
| Reciprocating engine exhaust | 320–590 | temperature | Minutes |
| Reheating furnace with regenerator | 300–600 | Mass flow | Minutes |
| Blast furnace stoves | 250–300 | Mass flow | Minutes |
| Drying and baking ovens | 230–590 | temperature | Minutes |
| Steam boiler exhaust | 230–480 | temperature | Minutes |
| Steam boiler | 200–300 | temperature | Minutes |
| Coke oven | 190 | temperature | Minutes |
| Container glass melting | 160–200 | temperature | Minutes |
|  | 140–160 | temperature | Minutes |
| Flat glass melting | 160–200 | temperature | Minutes |
|  | 140–160 | temperature | Minutes |
| Ceramic kiln | 150–1000 | temperature | Minutes – hours |
| Drying, baking, and curing ovens | 90–230 | temperature | Minutes |

(a) Butane 					      (b) Butane

(a) Heptane					      (b) Heptane

(a) Hexane					      (b) Hexane

(a) Ipentane 					      (b) Ipentane

(a) Neopentane					      (b) Neopentane

(a) R134a					      (b) R134a

(a) R-245b					      (b) R-245b

(a) Toluene 					      (b) Toluene

Figure. A. 1. Optimal compromise solution on the Pareto frontiers of eight working fluid candidates (a) ORC and (b) RORC