# Appendix  A – Calculation of gain and phase margins

The transfer function for the linearized inverted-pendulum system around the zero-degree state (i.e. assuming sinθ≅θ for small values of θ around zero), in the frequency-domain (also known as Laplace transformed of the system equation) is shown in Eq.1. The transfer function for the PID controller comes in Eq.2. Finally, Eq.3 gives the transfer function of the open-loop (i.e. without considering the unity feedback) system (L(s)), which is the multiplication of the pendulum system and the controller, in a linear system scheme.

Pendulum (s) =  1(Js2-mBgh)							(1)

PID s=KP+KDs1+1Ns + KIse-τds					(2)

L(s) = Pendulum(s) * PID(s)						(3)

where s is the Laplace transform variable; J, mB, h, are the subject-specific system parameters (as described in Fig.1) and g = 9.81; KP, KD, KI, are the control parameters, τd is the time delay, and N is usually a very large number (here taken as N = 575.5), which plays as a low-pass filter to make the derivative term of the PID controller casual, in addition to limiting high-frequency noise. Considering L(s) in all ranges of frequency, that is considering L(jω) – taking Laplace variable as s = jω, with frequencies ω=-∞, + ∞ – brings us the frequency-response-function of the open-loop system (also called as Bode diagram), from which the GM and PM are calculated in Eq.4-7.

GM calculation:

GM (Eq. 4) is defined as the negative logarithmic magnitude of the system frequency-response (i.e. L(jω)) at the ‘phase crossover frequency (ωpc) (Eq. 5)’  [37]:

phase crossover frequency ωpc     ≜    where  ∠Ljωpc= -180°        	(4)

GM = -Ljωpc     	(in decibel (dB))					(5)

Where j=-1 is the unit imaginary number; operator ∠ gives the angular value of the complex number Ljωpc (the frequency-response of system at frequency ωpc); and the magnitude of Ljωpc is in decibel (dB), i.e. 20*log10(|Ljωpc|). The absolute value of GM (denoted by |GM|) is considered in this study.

PM calculation:

PM (Eq. 7) is defined as the difference between the angular value of the system frequency-response (∠L(jω)) at the ‘gain crossover frequency (ωgc) (Eq. 6)’, and value -180⁰ [37]:

gain crossover frequency ωgc     ≜    where  Ljωgc= 0   dB 	(6)

PM = ∠Ljωgc   +   180	(in degree)					(7)