# Appendix A – Geographic Seismic Hazard Deaggregation using Python

To translate standard Mw–R–ε outputs from EZ-FRISK into high-resolution geographic deaggregation maps, a complete Python-coded pipeline was developed. This workflow transforms tabular EZ-FRISK disaggregation results into spatial contribution maps using Python’s pandas, GeoPandas, and matplotlib libraries — eliminating the need for external GIS tools.

## Required Inputs

• .txt file exported from EZ-FRISK containing Mw–R–ε and contribution data
• CSV file with radial site coordinates (Distance, Latitude, Longitude)
• Note: Coordinates should be in geographic WGS84 (EPSG:4326) format

## Execution Steps and Code Modules

### Python Code Example

# Python Script for Geographic Seismic Hazard Deaggregation
# Author: [Name], Year: 2025
# Input: EZ-FRISK txt, radial points CSV
# No use of GIS or shapefiles required

import pandas as pd
import geopandas as gpd
import matplotlib.pyplot as plt
from shapely.geometry import Point

# Step 1: Read EZ-FRISK disaggregation output
ezfrisk_df = pd.read_csv('input/ezfrisk_output.txt', delim_whitespace=True)

# Step 2: Read radial site coordinates
radial_df = pd.read_csv('input/radial_points.csv')

# Step 3: Sum contributions across epsilon bins
epsilon_bins = ['eps<-1', '-1<eps<0', '0<eps<1', '1<eps<2', '2<eps']
ezfrisk_df['Contribution'] = ezfrisk_df[epsilon_bins].sum(axis=1)

# Step 4: Merge with radial data
merged_df = pd.merge(ezfrisk_df, radial_df, on='Distance', how='left')

# Step 5: Create GeoDataFrame
gdf = gpd.GeoDataFrame(
    merged_df,
    geometry=gpd.points_from_xy(merged_df['Longitude'], merged_df['Latitude']),
    crs='EPSG:4326'
)

# Step 6: Plotting
def plot_contribution_map(gdf, site_name='Site1', T='1.0s'):
    fig, ax = plt.subplots(figsize=(10, 10))
    gdf.plot(
        ax=ax,
        column='Magnitude',
        markersize=gdf['Contribution'] * 1e10,
        cmap='viridis',
        alpha=0.6,
        legend=True,
        edgecolor='k'
    )
    ax.plot(gdf.iloc[0]['Longitude'], gdf.iloc[0]['Latitude'], 'r*', markersize=15, label='Site Center')
    ax.set_title(f'Geographic Deaggregation Map – {site_name} – SA({T})')
    ax.axis('off')
    plt.legend()
    plt.tight_layout()
    plt.savefig(f'output/final_maps/{site_name}_sa_{T}.png')
    plt.show()

## Output

The output is a PNG-format map showing spatial distribution of seismic source contributions. Marker sizes reflect the magnitude of each contribution, while marker color indicates magnitude.