Appendix

| Research | Gas type | Oil sample | Core/ Model properties |  | °API | Inj. temp. | Inj. press. | Inj. rate | RF (%) | Asphaltene dep./ permeability reduction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fong et al. (1992) [62] | CO2 | Recombined oil (Stock tank + synthetic gas) | Composite Core-long (synthetic) | L: 61.28 cm D: 2.54 cm Porosity:6% Ko@ SWC:0.28 mD | 25 | 46.1 °C | 1520 psi (cte.) | 0.3 res. cc/hr (avg.) horizontal | 64.5 | At pressure as high as 3050 psi no asphaltene deposition was observed |
|  |  |  |  | L: 63.4 cm D: 2.54 cm Porosity: 49% Ko@ SWC:0.03 mD | 21 | 43.9 °C | 1875 psi (initial) | 0.3 res. cc/hr (cte.) vertical from top | 56 |  |
|  | Flue gas (80% CO2+20% N2) |  |  | L: 63.4 cm D: 2.54 cm Porosity:49% Ko@ SWC:0.03 mD | 21 | 43.9 °C | 2300 psi (initial) | 0.4 std. cc/min (cte.) vertical from top | 23 |  |
| Srivastava et al. (1999) [28] | Flue Gas: 15.6 mol % CO2, 84.4 mol % N2 | Dead oil Recombined live oil (dead oil + CH4 at 362.6 psi and 28°C) | Sandpack cores | Porosity: ~28–32 % Permeability: 0.5 to 2.4 mD | 14 | 28 °C | 363 to 392 psi | 1 cc/min horizontal | 42.1-51.1 | Not studied |
|  | Produced Gas: 14.9 mol % CO2, 85.1 mol % CH4 |  |  |  |  |  |  |  |  |  |
|  | CO2 |  |  |  |  |  |  |  |  |  |
| Dong and Huang (2002) [33] | -Flue Gas I: 84.4% N2, 15.6% CO2 -Flue Gas II: 80.8% N2, 5% O2, 14.2% CO2 -Flue Gas III: 75% N2, 25% CO2 | Dead oil | 2D physical model | Model size: 51.0 cm × 52.0 cm × 2.36 cm Porosity: ~36–41% Permeability: 0.9 to 9 Darcy | 14.1 | 28 °C | 362.5 psi | 1.71-25.58 cc/min horizontal | 13.4 | Not studied |
| Shokoya et al. (2004) [35] | Flue Gas A (100% N2) | Recombined oil GOR:122 (std vol/vol) asphaltene content: 0.16% | Slim Tube |  | 39.7 | 116 °C | 4007-6680 psi | 1.2 PV | 45.30-67.00 | Not studied |
|  | Flue Gas B (83%N2+16%CO2+ 1%CO) |  |  |  |  | 116 °C | 4043-6456 psi |  | 41.35-71.05 |  |
|  | Flue Gas C (69%N2+30%CO2+ 1%CO) |  |  |  |  | 116 °C | 4028-6532 psi |  | 47.40-74.20 |  |
| Shokoya et al. (2005) [63,64] | Nitrogen (100% N2) | Recombined Oil A: GOR: 104.79 (m3/m3) | Long sandstone core | L: 2.4384 m D: 5.08 cm Porosity: 21.53% Ko@ SWC: 327.2 mD (oil A) / 284.2 mD (oil B) | 39.7 | 116 °C | 27.47 MPa | 8.53 cc/hr | 35 | Not studied |
|  |  |  |  |  |  |  | 41.58 MPa |  | 50 |  |
|  | Flue Gas 1 (83%N2+16%CO2 +1%CO) | Recombined Oil A: |  |  | 39.7 | 116 °C | 27.71MPa |  | 35 |  |
|  |  | Recomb. Oil B: GOR:52.657(m3/m3) |  |  | 37.3 | 80.6 °C | 17.68 MPa |  | 40 |  |
|  | Flue Gas 2 (69%N2+30%CO2+ 1%CO) | Recombined Oil A |  |  | 39.7 | 116 °C | 27.82 MPa |  | 40 |  |
|  |  | Recombined Oil B |  |  | 37.3 | 80.6 °C | 17.64 |  | 48 |  |

Appendix- Table A.1: Overview of experimental study conducted on flue gas injection

| Research | Gas type | Oil sample | Core/ Model properties |  | °API | Inj. temp. | Inj. press. | Inj. rate | RF (%) | Asphaltene dep./ permeability reduction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mohsenzadeh et al. (2014) [65] | N2 | Dead oil | 3 Sets of 6 Core Series as Fractured Long Carbonate Core Annular space 0.65 cm around the core simulated the vertical fracture | L: 6*15 cm D: 8.7 cm Porosity: 16% Kmatrix: 5.5 mD | 15.8 | 80 °C | 750 psi | 1st step: 5 cc/min (producing from fractures until gas breakthrough) 2nd step: 1 cc/min (producing from matrixes) | Before GBT: 78.2 | Not studied |
|  |  |  |  |  |  |  |  |  | After GBT: 13.5 |  |
|  | CO2 |  |  | L: 6*15 cm D: 8.7 cm Porosity: 19% Kmatrix: 8 mD |  |  |  |  | Before GBT: 68.3 |  |
|  |  |  |  |  |  |  |  |  | After GBT: 14.8 |  |
|  | Flue Gas (69%N2+30%CO2+1%CO) |  |  | L: 6*15 cm D: 8.7 cm Porosity: 17% Kmatrix: 6.5 mD |  |  |  |  | Before GBT: 74.0 |  |
|  |  |  |  |  |  |  |  |  | After GBT: 9.8 |  |
| Li et al. (2017) [67] | Flue gas: 80 mol% N2, 20 mol% CO2 + n-Hexane + Steam | Dead oil | Sandpacks | Length: 60 cm Diameter: 2.54 cm Porosity: 35.85% - 36.31% Permeability: 2069 - 2133 mD | - | 200 °C | 435 psi | Steam flow: 1.2 to 2.0 cc/min Flue gas flow: 0.6 cc/min n-Hexane flow: 0.2 cc/min | 80.01 | Studied via SARA analysis of produced and residual oils and molecular weight of asphaltenes Result: Residual oil showed increased heavy components and less stable asphaltenes, indicating potential for asphaltene precipitation and deposition |
| Pang et al. (2018) [68] | Flue gas: 80% N2, 20% CO2 + Steam | Dead oil | 1D Sandpack | 60 cm length × 3.8 cm diameter; porosity: ~35–39%; permeability: ~2.2–2.6 µm² (2200–2600 mD) | - | 250 °C | 435 psi | - 1D tests: 5 cc/min - 3D tests: 25 cc/min (15 cc/min steam + 10 cc/min flue gas) | 49.5 | Not studied |
|  |  |  | 3D Model | Width: 40 cm, thickness: 30 cm; net pay simulated as 16 cm; porosity: 35%; permeability: 60,000 mD |  |  |  |  |  |  |
| Wu et al. (2018) [69] | Flue gas: (80% N2, 20% CO2) + Steam | Dead oil | 3D physical model | dimensions: 40 cm ×40 cm ×16 cm; Quartz sand is used as the porous medium | - | 250 °C | 435 psi | 25 cc/min (steam + flue gas) | 49.49 | Asphaltene content measured, decreased in produced oil, increased in residual oil |
| Tao et al. (2021) [70] | Flue gas: (80% N2, 20%CO2) + Steam | Dead oil | 3D cylindrical physical model | Size: 150 cm × 100 cm, Volume: 196.35 L Porosity: 34.4% Permeability: 2.7 Darcy | - | 350 °C | - | 16 cc/min steam + 4 cc/min flue gas | 5.7 (improvement) | Asphaltene content decreased in produced oil during all stages |

Appendix- Table A.1: Overview of experimental study conducted on flue gas injection (continued)

| Research | Gas type | Oil sample | Core/ Model properties |  | °API | Inj. temp. | Inj. press. |  | Inj. rate | RF (%) | Asphaltene dep./ permeability reduction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min and Zhang (2024) [71] | Flue gas: (80% N2, 20% CO2) + Steam | Dead oil | 2D physical model | 40 cm × 50 cm × 1 cm | - | 350 °C | 435 psi |  | Steam: 10 cc/min Flue gas: 10 cc/min | 68.8 | Not studied |
| Jalili et al. (2024) [4] | Flue Gas (A power plant’s exhaust): 82.47%N2+10.38% CO2+ 0.13%CO+ 6.14%O2 + reminder (SO2+ NO+ NO2) | Bottom hole live Oil: GOR 1113 (SCF/STB) | 1 long core (4 carbonate core series) | L:20.329 Cm D: 3.783 cm Porosity: 17% (avg.) K: 1.64 mD | 32.4 | 112 °C | 5000 psi |  | 0.05 cc/min (cte.) | 36 | %Asph. Dep: - |
|  |  |  |  |  |  |  |  |  |  |  | %Perm. Red: 4.425% |
|  | CO2 |  |  |  |  |  |  |  |  | 86 | %Asph. Dep: - |
|  |  |  |  |  |  |  |  |  |  |  | %Perm. Red: 14.832% |
| This study | Flue Gas: (75%N2+25%CO2) | Recombined Oil: GOR: 342.39 (SCF/STB) | 2 carbonate cores | F#1 | 21.4 | 65 °C | Immiscible | 4000 psi | Variable flow rate (Constant pressure) | 85.6 | %Asph. Dep:48.61 |
|  | Flue Gas: (75%N2+25%CO2) |  |  | F#1: L:5.121Cm D: 3.80 cm Porosity: 29.5% K: 1.5 mD |  | 110 °C |  | 3000 psi |  | 50.1 | %Asph. Dep: 35.21 |
|  |  |  |  |  |  |  |  |  |  |  | %Perm. Red: 3.10 |
|  |  |  |  |  |  |  |  | 4000 psi |  | 62.1 | %Asph. Dep: 41.12 |
|  |  |  |  |  |  |  |  |  |  |  | %Perm. Red: 8.27 |
|  |  |  |  |  |  |  |  | 6000 psi |  | 81.8 | %Asph. Dep: 50.41 |
|  |  |  |  |  |  |  |  |  |  |  | %Perm. Red: 15.04 |
|  | CO2 |  |  | F#1 |  | 110 °C | Miscible | 6000 psi (>MMP) |  | 90.0 | %Asph. Dep: 59.82 |
|  |  |  |  |  |  |  |  |  |  |  | %Perm. Red: 25.10 |
|  | CO2 |  |  | C#2: L: 4.885 Cm D: 3.84 cm Porosity:28.3% K: 3.6 mD |  |  | Immiscible | 3000 psi (<MMP) |  | 58.2 | %Asph. Dep: 20.16 |
|  |  |  |  |  |  |  |  |  |  |  | %Perm. Red: 27.78 |
|  |  |  |  |  |  |  | Immiscible | 4300 psi (<MMP) |  | 76.1 | %Asph. Dep: 52.41 |
|  |  |  |  |  |  |  |  |  |  |  | %Perm. Red: 27.22 |
|  |  |  |  |  |  |  | Miscible | 5000 psi (>MMP) |  | 81.8 | %Asph. Dep: 25.55 |
|  |  |  |  |  |  |  |  |  |  |  | %Perm. Red:23.16 |

Appendix- Table A.1: Overview of experimental study conducted on flue gas injection (continued)