Supplemental Results

Figure S1. Price changes in five wood product types with the introduction of planned bioenergy and bioproduct facilities (“mills”) in the U.S. Southeast. Date ranges refer to mill buildout scenarios, e.g., mills planned through the 2025-2028 period.

Supplemental Methods

- Historical regression baseline.

To estimate a historical baseline (Table S1), we fit a linear regression model to the most recent 10 years (2013–2022) of historic inventory data. Total inventory was aggregated annually across all wood product types and management categories. We used ordinary least squares to regress inventory against year, then projected the resulting trendline through 2044 to represent a historical baseline under a no-change scenario. Although the regression was performed on forest inventory carbon (measured in volume), a Pearson correlation test comparing inventory carbon to total carbon where data were available yielded a correlation coefficient of 0.98. This strong linear relationship supports the use of inventory carbon as a reliable proxy for total carbon in baseline projections.

- Planned biomass facilities.

Table S1. Planned bioenergy and bioproduct facilities in the U.S. Southeast included in the analysis, with projected biomass demand and estimated operational dates.

| Facility ID | Facility Name | Location | Product | Planned operational date | Green tons biomass/yr |
| --- | --- | --- | --- | --- | --- |
| 6 | Origin Materials | Geismar, LA | Bioplastics | 2025 | 2,000,0001 |
| 2 | Louisiana Green Fuels | Columbia, LA | SAF + CCS | 2026 | 2,000,0002 |
| 3 | Fidelis New Energy | Port Allen, LA | BECCS | 2026 | 800,0003 |
| 4 | USA Bioenergy | Bon Wier, TX | Biofuel plant | 2026 | 1,000,0004 |
| 7 | CO280 | Leesville, LA (centroid) | PP + CCS | 2027 | 300,0005 |
| 1 | SunGas Renewables / C2X | Alexandria, LA | Methanol | 2028 | 2,000,0006 |
| 8 | Orsted | Leesville, LA (centroid) | E-Methanol | 2030 | 500,0007 |
| 9 | Pathway | Leesville, LA (centroid) | SAF + CCS | 2030 | 2,000,0008 |

Note: CCS = Carbon Capture and Storage; SAF = Sustainable Aviation Fuel; PP = Pulp and Paper; BECCS = Bioenergy with Carbon Capture and Storage.

- Biomass Facility Carbon Footprint Analysis

We performed an attributional life cycle assessment (LCA) to estimate the carbon footprint, carbon removal potential, and substitution benefits of planned biomass conversion facilities across five technology pathways: biopower with carbon capture and storage (CCS), methanol with CCS, e-methanol, bioplastics, and pulp and paper with CCS. The analysis uses representative emission factors from peer-reviewed literature to model greenhouse gas (GHG) emissions and removals per tonne of biogenic carbon input or product (tC/tC).

The system boundary encompasses all major process stages from biomass harvesting and transport (assumed consistent across pathways), through pre-treatment, conversion (e.g., combustion, gasification, fermentation), product synthesis and upgrading, to CO2 capture, transport, and storage. Both attributional and consequential perspectives are considered, explicitly accounting for substitution benefits from displaced fossil-based products and permanent carbon storage via geologic sequestration.

Upstream emissions from biomass cultivation and land-use change, as well as embodied emissions from facility construction and decommissioning, product distribution, end-of-life use, waste management, and market-mediated effects, are excluded to focus on process-level impacts and ensure comparability across scenarios. The resulting carbon footprint for each pathway is reported as the net sum of direct and indirect GHG emissions and removals associated with the included life cycle stages, facilitating a comparative assessment of net climate impacts across biomass conversion technologies.

3.1 Process Emissions Assumptions Based on Pathway.

Transport emissions (all pathways)

Wood removal harvest emissions (roundwood production and roundwood harvest) from GREET model9. Wood removal transport emissions via Class 8 HHD diesel truck/trailer, 100-mile round trip10.

Wood processing emissions (all pathways)

Biomass chipping before production11.

Biopower + CCS

New build biopower plant. Biomass conversion to energy through combustion to produce electricity and heat.

Includes emissions from energy consumption (natural gas) from plant operations, pre-processing (drying), CO2 capture, compression, transportation, and storage. Electricity generation of 44.39 GWh/year. The plant has post-combustion absorption using monoethanolamine. CO2 storage is 70 km away in a saline aquifer, transported via pipeline11.

Methanol + CCS

Biomass conversion to methanol via gasification. Includes emissions from energy consumption from plant gasification operations, pre-processing (drying), syngas cleaning and conditioning, methanol synthesis, refining, and purification12.

E-methanol

Biomass is combusted to generate electricity, which is then used to produce green hydrogen via electrolysis, combined with captured CO2 to synthesize methanol. Results in a closed carbon loop using biogenic CO2 and renewable electricity. Assume the same biopower plant as above, but without transportation and storage - just capture11.

Includes emissions from pre-processing (drying),  electricity production for methanol, limestone extraction, limestone decomposition, waste water treatment, magnesium carbonate disposal, silica dioxide disposal, alumina disposal, and iron fraction disposal12.

Bioplastics

Biomass is used to produce bio-based polyethylene terephthalate (bio-PET), consisting of 30% bio-based monoethylene glycol (MEG, switchgrass feedstock) and 70% bio-based terephthalic acid (PTA, wood feedstock). Though MEG has a switchgrass feedstock rather than wood, both are lignocellulosic feedstocks that would undergo the same process. The only notable difference would be in the pretreatment step, wood may require a stronger acid or steam explosion, as there is a higher lignin content. Assume the difference in emissions is negligible13.

The Chen et al. (2016) paper14 includes emissions from biomass cultivation and harvesting, which our study omits. We deduct these emissions using assumptions from a switchgrass production LCA based in the Southeast US by Bergman et al. (2015)15 to ensure that the pathway aligns with others. The biomass input assumed by the paper is both switchgrass and forestry residues, but we deduct switchgrass production emissions as they are likely to be larger than forestry production emissions.

Includes emissions from biomass pre-treatment to disrupt lignin to access cellulose, and enzymatic hydrolysis for both MEG and PTA production. To produce MEG, the following process steps are fermentation to ethanol, catalytic dehydration to convert ethanol to ethylene, and hydration to convert ethylene to MEG. For PTA, the following process steps are fermentation to isobutanol, dehydration, oligomerization, and dehydrocyclization to convert isobutanol to paraxylene, and paraxylene oxidation to PTA. The two monomers, EG and PTA, are combined via polycondensation to form amorphous PET resin and upgraded to bottle-grade PET. Finally, bottle-grade PET is manufactured into bottles via injection blow molding16 17 18 19.

Includes emissions from plant energy use, operation, and bottle manufacturing. Carbon is captured through the production of MEG and PTA via biomass fermentation. Assumes that PET is 30% EG and 70% PTA by wt%.

P+P CCS

Retrofit pulp and paper plant, which converts biomass to paper industry products and also produces energy (heat and electricity). Assumes the same plant specifications as the biopower + CCS pathway above, but upstream emissions are calculated and net impacts are normalized based on the additional biomass needed per unit of CO2 captured from a P+P flue gas20.

Includes emissions from consumption (natural gas) from plant operations, CO2 capture, compression, transportation, and storage. The plant has post-combustion absorption using monoethanolamine. CO2 storage is 70 km away in a saline aquifer, transported via pipeline11.

SAF + CCS

An integrated alcohol-to-jet  SAF plant, colocated with a methanol production facility. Methanol is produced by converting biomass to syngas via gasification, syngas cleaning and conditioning, methanol synthesis, refining, and purification. Methanol is upgraded to SAF via dehydration and methanol-to-olefin oligomerization. While the ICACO paper uses ethanol as the feedstock, it is analogous to methanol as they have a similar chemical structure, and both undergo oligomerization. Methanol produces a mix of light olefins, aromatics, and paraffins, which is likely to require more separation than ethanol, which produces mostly ethylene. But for simplicity, we assume these additional emissions are negligible. Assumes an olefin selectivity of 95%. The jet energy share in total energy output is 75%12 21.

Includes emissions from energy consumption from plant gasification operations and upgrading methanol to SAF.

3.2  Substitution Benefits Assumptions Based on Pathway.

Biopower + CCS

Biopower-generated electricity displaces the average Louisiana grid electricity in 2023. Louisiana spans two eGRID subregions (SERC Mississippi Valley and SERC South), so the region with the lowest carbon intensity, SERC Mississippi Valley, is used to be conservative22.

Methanol + CCS

Methanol produced via biomass gasification displaces conventional methanol produced from natural gas via steam reforming in the US, which is combusted for energy. Methane Institute gives well-to-gate (cradle-to-gate) emissions, excluding combustion emissions. So combustion emissions are added23.

E-methanol

E-methanol produced via biomass gasification and electrolysis (green hydrogen production) displaces conventional methanol produced from natural gas via steam reforming in the US, which is combusted for energy. Methane Institute gives well-to-gate (cradle-to-gate) emissions, excluding combustion emissions. So combustion emissions are added23.

Bioplastics

Bio-based PET replaces conventional fossil-derived PET production. In the fossil case, PTA is made from p-xylene (derived from the catalytic reforming of petroleum naphtha), and MEG is made from ethylene (derived from steam cracking of petroleum-derived naphtha or natural gas liquids). PET is assumed to be incinerated for disposal14 24.

P+P CCS

Not applicable.

SAF + CCS

SAF replaces conventional petroleum jet fuel. International Civil Aviation Organization (ICAO) and the U.S. Renewable Fuels Standard (RFS) baseline value. Assumes that SAF has the same low heating value (LHV) as conventional jet fuel21.

3.3 Carbon Removal Potential Assumptions Based on Pathway.

Biopower + CCS

Gross CO2 removal potential (total CO2 available for capture and storage) from biomass, calculated using the energy and carbon balance of bone dry biomass. Assumes the carbon content of bone dry biomass is 50 wt%, that 1 tonne of bone dry biomass produces 18 GJ of energy, that the thermal efficiency of the process is 30%, and that the power requirement per tCO2 capture is around 1 MWh/tCO225 26.

Methanol + CCS

Gross CO2 removal potential (total CO2 available for capture and storage) from biomass, calculated using the carbon balance of bone dry biomass. Assumes the carbon content of bone dry biomass is 50 wt%, that 1 tonne of biomass produces 0.458 kmol of methanol via gasification, with the remaining carbon being captured and stored as CDR27.

E-methanol

Gross CO2 removal potential (total CO2 available for capture and storage) from biomass, calculated using the energy and carbon balance of bone dry biomass. For the bioenergy plant, it assumes the carbon content of bone dry biomass is 50 wt%, that 1 tonne of bone dry biomass produces 18 GJ of energy, that the thermal efficiency of the process is 30%, and that the power requirement per tCO2 capture is around 1 MWh/tCO2. Also assumes that 1.4 tCO2 is needed to produce 1 tonne of e-methanol. The remaining carbon is captured and stored as CDR25 28.

Bioplastics

Gross CO2 removal potential (total CO2 available for capture and storage) from biomass, calculated using the carbon balance of lignocellulosic ethanol production, which is an analogue for MEG and PTA inputs29.

P+P CCS

Gross CO2 removal potential (total CO2 available for capture and storage) from biomass, calculated using the energy and carbon balance of bone dry biomass. Assumes the carbon content of bone dry biomass is 50 wt%, that 1 tonne of bone dry biomass produces 18 GJ of energy, that the thermal efficiency of the process is 15%, and that the power requirement per tCO2 capture is around 1 MWh/tCO225 30.

SAF + CCS

Gross CO2 removal potential (total CO2 available for capture and storage) from biomass, calculated using the energy and carbon balance of bone dry biomass. Assumes the carbon content of bone dry biomass is 50 wt%, that 1 tonne of bone dry biomass produces 18 GJ of energy, that the thermal efficiency of the process is 15%, and that the power requirement per tCO2 capture is around 1 MWh/tCO225 30.

- Land Use and Resource Allocation Modeling.

Using facility locations and assumed resource demands, we apply the Land Use and Resource Allocation (LURA) model to project shifts in the spatial allocation of biomass in the case study region.

LURA is a price endogenous, partial equilibrium economic model of the US forest sector that captures detailed spatial dependencies between supply points (forest plots, with heterogeneous attributes) and demand points (forest product manufacturing facilities [mills], electricity generation units, and ports). The model has been used to develop spatially refined forest carbon projections31 32, evaluate regional implications of bioenergy developments33, and to explore timber merchantibility as a function of spatial dependencies and transportation logistics34.  The model has a rich supply-side representation representing more than 130,000 forest plots from the Forest Inventory and Analysis dataset (FIA). For each plot, LURA records a site class measure (a broad index of stand quality), aggregates forest tree species into 14 broad forest types, different condition classes, and age class (in five-year increments).

Once volume is collected for each plot and condition class, empirical growth curves are estimated for each Eco-Province using a standard von Bertalanffy functional form (for each forest type and site class combination). These growth curves are applied to each plot to simulate forest growth (net of mortality) at each forest plot in the absence of new removals. Latta et al. (2018) provides a detailed overview of the supply side of the model, including relevant structural equations31.

In this analysis, we optimize net economic welfare for the US forest industry, or the sum of consumer and producer surplus. We use product-specific demand elasticities pulled from the literature (or estimated directly) for 14 different product categories, including bioenergy products such as wood pellets, as well as 19 additional intermediate products that are inputs to final finished product processes (e.g., logs, wood chips, sawdust, etc.). National demand is then disaggregated to the mill level based on relative capacity shares such that each mill has an individual demand curve. National demand for different product categories including housing start projections,  grow over time based on income elasticities and macroeconomic growth projections from the U.S. Annual Energy Outlook 2023 report35. Latta et al. (2018) documents structural equations on the demand-side of the LURA model31.

LURA depicts detailed transportation networks by mapping road networks between plots and mills, and also between demand points (between mills and ports). Road networks and hauling cost parameters capture mill-to-mill interactions and the supply of sawmill residuals to meet pulpwood demand for traditional wood products or bioenergy. Road networks are converted to average travel time and distance and then average hauling costs based on projected diesel prices and assumed truck load capacities for different log or residual types (e.g., softwood pulp and saw logs, hardwood pulp and saw logs). Pokharel and Latta (2020) provide additional information on LURA transportation networks34.

For this analysis, we ran national-scale simulations with the LURA model under assumed business as usual market demand growth (following an AEO reference case scenario). Then, we included five additional bioenergy demand scenarios, adding new mills to the landscape at each point in time when we expect the facility to come online. Each facility was provided a geographic location and an exogenous demand requirement, with restrictions on feedstock type. For each new facility, we calculated hauling costs from plot to facility and between each facility and other demand points using the approach outlined in Pokharel and Latta (2020)34.

For each scenario, we saved feedstock sourcing results to map out plots individual plots and mills providing feedstock to each new bioenergy facility over time. We also compare this to the original regional sourcing map to assess changes in regional procurement zones for SRTS analysis, described in the next section.

- Sub-Regional Timber Supply (SRTS) Modeling.

Projections for the forest composition, landscape carbon, and market dynamics are made with the Sub-Regional Timber Supply model (SRTS) Excel Add-in v0.2.2. The goal of modeling was three-fold

- Integrate outputs from Land Use and Resource Allocation model (LURA) to determine the spatial boundaries of our analysis.

- Model the influence of staged, yearly pulpwood consumption capacity increases on market dynamics and forest inventory.

- Test the sensitivity of our analysis to different model specifications and spatial areas.

The data were derived from USDA Forest Service, Forest Inventory and Analysis (FIA) including plot-level data from the National Forest Inventory (volume, species, diameter, growth, removals) and mill consumption data from the National Resource Use Monitoring (NRUM) survey (previously the Timber Product Output (TPO) survey). This data provides standard inputs for the SRTS model. One particular note, we left out the NRUM data associated with Texas as their survey changed in 2020 and thus was unsuitable for our purposes.

Our data was structured in two regions, both defined by LURA. There was a core region, from which the new mills would be drawing feedstocks directly, and an outer region where existing mills, that had been drawing from the core region, were now getting their feed stocks. As documented in the SRTS User Guide36, the NRUM/TPO adjustments were applied at the beginning of our run to "true-up" removals. We used the same set of TPO adjustments in both regions. They are based on FIA Survey Unit level37 (see FIADB User Guide for Survey Unit definition) of adjustment factors weighted by inventory volume and averaged.

We set up our market with four products: Pine Pulpwood ranging from 5-10 inches in DBH, Pine Sawtimber at greater than 10 inches, Hardwood Pulpwood from 5 to 11 inches, and finally Hardwood Sawtimber at greater than 13 inches. A five-product specification, where an intermediate Pine Chip and Saw product was defined, was also tested. It was found to introduce error into the modeling and was ultimately abandoned and absorbed by the other pine products, which fit the existing forest and market dynamics better. For each of the four products we used the same, fairly inelastic, demand price elasticity of 0.5. The supply price elasticities were less elastic for the pulp markets at 0.3, and more elastic for the sawtimber markets at 0.5. Our inventory elasticities were unit elastic for pine and inelastic (0.25) for hardwoods.

Our projections were influenced by the opening of the new facilities and an assumption of constant demand in sawtimber markets. Our baseline scenario, the No Mills scenario, projects the current rate of removals (adjusted based on TPO data) for all 4 products and holds them constant to the end of the projection (2044). Subsequent scenarios layer in the new facilities by the year they enter the market. In all there are 6 scenarios ranging from No Mills to All Mills. This differs for the Two Region with Direct Change scenario, where we use a No Mills projection and then layer the additional demand into the market via the DC file. There are then 5 DC files ranging from the 2025 facilities to All Mills (more on this below).

We use three components of the SRTS model to make our projections: Demand mode, Harvest mode, and Demand mode with a Direct Change (DC) file. For convenience we describe them at a high level here, more detailed discussion of these approaches can be found in the SRTS User Guide.

- Demand mode: In Demand mode, the model uses supply and demand curves to identify the price at which removals for each in-scope region satisfy demand. Because supply and demand both react to price, one can expect the equilibrium market level removals to adjust based on the available inventory and demand shifts year to year.

- Harvest mode: In Harvest mode, the model still uses supply and demand curves, but rather than finding the market clearing prices, the demand curve is shifted along the supply curve until demand is met. This price then represents what would need to be paid by mills to satisfy the demand in the PRJ file. While not strictly a market equilibrium projection, it does reflect the shape of the supply and demand curve when calculating the new price.

- DC file: The DC file allows the user to specify where and when demand is changing. In a typical SRTS projection, demand is represented as the sum across all in-scope regions. The allocation of removals then follows the availability of supply at the region level. With the DC file, one can specify increases or reductions in demand in specific regions (and at specific times) to explore the short-run local effects of these changes.

We ran five different model specifications to test alternative assumptions about market scenarios and conditions:

- Core Region Demand Mode -- This model was intended to mimic local market conditions and price effects

- Core Region Demand Mode without Pine Sawtimber Residues -- This model projects local market conditions without a sawtimber residue market.

- Core Region Harvest Mode -- This model demonstrates what could happen if price is not relevant to market participants and instead demand is fully satisfied no matter the cost. It does not include a sawtimber residue market.

- Two Region Demand Mode -- This model treats the broader area as a frictionless market, where demand can freely reallocate based on inventory and price.

- Two Region Demand Mode with Direct Change -- In this model we still allow two regions to satisfy demand, but the new facilities' demand is placed in the core region. This simulates a world in which there's a trade barrier between the core and outer region, but prices are set holistically across the entire region.

Results are minimally post-processed to add labels and units to output.

Landscape carbon estimates are utilizing the new NSVB38 calculations and the revised allometric relationships that were made available in SRTS version 0.2.2. These new equations use a series of cluster level conversion factors and county level weights to build more geographically tuned estimates of above ground carbon. More details on the process can be found in the ISFRE 2025 Meeting Proceedings: Richard H Manner, The Sub-Regional Timber Supply Model (SRTS): A New Platform, New Carbon Calculations, and a Prospective on the Forest Sector in the Southeastern United States.

References

1.	Origin Materials Announces Louisiana State Bond Commission Approval of Financing for Origin 2. Business Wire https://www.businesswire.com/news/home/20230104005352/en/Origin-Materials-Announces-Louisiana-State-Bond-Commission-Approval-of-Financing-for-Origin-2 (2023).

2.	Americas, S. C. of. Strategic Biofuels and Sumitomo Corporation of Americas Announce Major Investment Commitment. PR Newswire https://www.prnewswire.com/news-releases/strategic-biofuels-and-sumitomo-corporation-of-americas-announce-major-investment-commitment-302056103.html (2024).

3.	Louisiana Economic Development. Proposed $1.35 Billion Investment in Southeast Louisiana Would Establish Large-Scale Renewable Natural Gas Plant and Green Hydrogen Facility. Louisiana Economic Development https://www.opportunitylouisiana.gov/news/proposed-1-35-billion-investment-in-southeast-louisiana-would-establish-large-scale-renewable-natural-gas-plant-and-green-hydrogen-facility (2024).

4.	BioEnergy, U. S. A. USA BioEnergy Announces First Advanced Biorefinery in Bon Wier, Texas After Receiving $150M in State and Local Incentives - USA BioEnergy. USA BioEnergy https://usabioenergy.com/usa-bioenergy-announces-first-advanced-biorefinery-in-bon-wier-texas-after-receiving-150m-in-state-and-local-incentives/ (2022).

5.	SLB and Aker Carbon Capture joint venture awarded FEED contract by CO280 for large-scale carbon removal project. SLB https://www.slb.com/news-and-insights/newsroom/press-release/2024/slb-and-aker-joint-venture-awarded-feed-contract-by-co280-for-large-scale-carbon-removal-project (2024).

6.	SunGas Renewables. SunGas Renewables Announces Beaver Lake Renewable Energy, a Green Methanol Facility in Central Louisiana. PR Newswire https://www.prnewswire.com/news-releases/sungas-renewables-announces-beaver-lake-renewable-energy-a-green-methanol-facility-in-central-louisiana-301886566.html (2023).

7.	Bellotte, C. Ørsted Selected by U.S. Department of Energy to Receive Industrial Decarbonization Funding. Orsted https://us.orsted.com/news-archive/2024/03/orsted-selected-by-us-department-of-energy-to-receive-industrial-decarbonization-funding (2024).

8.	Drax and Pathway Energy agree heads of terms on sustainable biomass deal to supply new US SAF plant. Drax Global https://www.drax.com/press_release/drax-and-pathway-energy-agree-heads-of-terms-on-sustainable-biomass-deal-to-supply-new-us-saf-plant/ (2024).

9.	GREET model. Argonne National Laboratoory https://greet.es.anl.gov (2023).

10.	U.S. EPA & NHTSA. Greenhouse Gas Emissions and Fuel Efficiency Standards for Medium- and Heavy-Duty Engines and Vehicles. (2016).

11.	Briones-Hidrovo, A., Copa Rey, J. R., Cláudia Dias, A., Tarelho, L. A. C. & Beauchet, S. Assessing a bio-energy system with carbon capture and storage (BECCS) through dynamic life cycle assessment and land-water-energy nexus. Energy Convers. Manag. 268, 116014 (2022).

12.	Galusnyak Stefan C., Petrescu Letitia, Chisalita Dora A., & Cormos Calin-Cristian. Life Cycle Assessment of Bio-methanol Derived from Various Raw-materials. Chem. Eng. Trans. 86, 667–672 (2021).

13.	te Molder, T. D. J., Kersten, S. R. A., Lange, J. P. & Ruiz, M. P. Ethylene Glycol from Lignocellulosic Biomass: Impact of Lignin on Catalytic Hydrogenolysis. Ind. Eng. Chem. Res. 60, 7043–7049 (2021).

14.	Chen, L., Pelton, R. E. O. & Smith, T. M. Comparative life cycle assessment of fossil and bio-based polyethylene terephthalate (PET) bottles. J. Clean. Prod. 137, 667–676 (2016).

15.	Bergman, R. D., Reed, D. L., Taylor, A. M., Harper, D. P. & Hodges, D. G. Cradle-To-Gate Life Cycle Assessment of Switchgrass Fuel Pellets Manufactured in the Southeastern United States. Wood Fiber Sci. 47, 147–159 (2015).

16.	Tibbetts, J. D., Russo, D., Lapkin, A. A. & Bull, S. D. Efficient Syntheses of Biobased Terephthalic Acid, p-Toluic Acid, and p-Methylacetophenone via One-Pot Catalytic Aerobic Oxidation of Monoterpene Derived Bio-p-cymene. ACS Sustain. Chem. Eng. 9, 8642–8652 (2021).

17.	Bioethanol – European Biomass Industry Association. https://www.eubia.org/cms/wiki-biomass/biofuels/bioethanol/.

18.	Solventis. Monoethylene glycol (MEG) from ethylene – production pathway and emissions. https://solventis.net/products/glycols/mono-ethylene-glycol/.

19.	Janlamool, J. & Jongsomjit, B. Catalytic Ethanol Dehydration to Ethylene over Nanocrystalline χ- and γ-Al2O3 Catalysts. J. Oleo Sci. 66, 1029–1039 (2017).

20.	Wang, N., Wang, D., Krook-Riekkola, A. & Ji, X. MEA-based CO2 capture: a study focuses on MEA concentrations and process parameters. Front. Energy Res. 11, (2023).

21.	ICAO. CORSIA Eligible Fuels – Life Cycle Assessment Methodology. (2024).

22.	U.S. EPA. eGRID2023 GHG emission factors. https://www.epa.gov/egrid/detailed-data (2022).

23.	Methanol Institute. Life-cycle emissions of conventional methanol. (2022).

24.	NIH. Life Cycle Environmental Impact of PET Water Bottles. Natl. Inst. Health (2022).

25.	Bauen, A., Berndes, G., Junginger, M., Londo, M. & Vuille, F. Bioenergy – a sustainable and reliable energy source: A review of status and prospects. Int. Energy Agency (2009).

26.	IRENA. Biomass for Heat and Power Technology Brief. Int. Renew. Energy Agency (2015).

27.	Puerari, F., Bosio, B. & Heyen, G. Energy Efficiency Optimisation in Different Plant Solutions for Methanol Production from Biomass Gasification. Chem. Eng. Trans. 37, 301–306 (2014).

28.	Capro, N. Lhyfe foresees 54kg green hydrogen per tonne requirement. EUROMETAL https://eurometal.net/lhyfe-foresees-54kg-green-hydrogen-per-tonne-requirement/ (2023).

29.	Lask, J. et al. Lignocellulosic ethanol production combined with CCS—A study of GHG reductions and potential environmental trade-offs. GCB Bioenergy 13, 336–347 (2021).

30.	Furszyfer Del Rio, D. D. et al. Decarbonizing the pulp and paper industry: A critical and systematic review of sociotechnical developments and policy options. Renew. Sustain. Energy Rev. 167, 112706 (2022).

31.	Latta, G. S., Baker, J. S. & Ohrel, S. A Land Use and Resource Allocation (LURA) modeling system for projecting localized forest CO2 effects of alternative macroeconomic futures. For. Policy Econ. 87, 35–48 (2018).

32.	Wade, C. M., Baker, J. S., Latta, G. & Ohrel, S. B. Evaluating Potential Sources of Aggregation Bias with a Structural Optimization Model of the U.S. Forest Sector. J. For. Econ. 34, 337–366 (2019).

33.	Visser, L., Latta, G., Pokharel, R., Hoefnagels, R. & Junginger, M. Exploring the limits to sustainable pellet production for international markets: The impact of increasing pellet production in the US Southeast on feedstock use, production cost and carbon sequestration in forest areas. GCB Bioenergy 14, 896–917 (2022).

34.	Pokharel, R. & Latta, G. S. A network analysis to identify forest merchantability limitations across the United States. For. Policy Econ. 116, 102181 (2020).

35.	EIA. Annual Energy Outlook 2025. U.S. Energy Information Administration https://www.eia.gov/outlooks/aeo/index.php.

36.	SRTS Model. Southern Forest Resource Assessment Consortium https://research.cnr.ncsu.edu/sofac/srts-model/.

37.	The Forest Inventory and Analysis Database User Guide (NFI) | US Forest Service Research and Development. https://research.fs.usda.gov/understory/forest-inventory-and-analysis-database-user-guide-nfi.

38.	Westfall, J. A. et al. A national-scale tree volume, biomass, and carbon modeling system for the United States. Gen Tech Rep WO-104 Wash. DC US Dep. Agric. For. Serv. 104, (2024).