|  | μa [cm-1] | μs [cm-1] | g [-] |
| --- | --- | --- | --- |
| Epidermis | 0.3700 | 156.1 | 0.8 |
| Dermis | 0.6900 | 156.1 | 0.8 |
| Subcutaneous tissue | 0.6900 | 156.1 | 0.8 |
| RBC core | 179.1 | 760.5 | 0.970 |
| Cell free layer (Case 1) | 0.0004 | 10.0 | 1.0 |
| Cell free layer (Case 2) | 537.3 | 760.5 | 0.970 |

Table1 Optical Properties [11]. The symbols μa, μs, and g represent the absorption coefficient, scattering coefficient, and anisotropy factor, respectively.

|  | ρ [g/cm3] | c [J/(g･℃)] | k [W/(cm･℃)] |
| --- | --- | --- | --- |
| Epidermis | 1.19 | 3.60 | 2.09×10-3 |
| Dermis | 1.12 | 3.22 | 3.00×10-3 |
| Subcutaneous tissue | 1.12 | 3.22 | 3.00×10-3 |
| RBC core | 1.00 | 3.84 | 4.92×10-3 |
| Cell free layer (Case 1) | 1.00 | 4.18 | 6.02×10-3 |
| Cell free layer (Case 2) | 1.00 | 3.84 | 4.92×10-3 |

Table2 Thermophysical Properties [10]. The symbols ρ, c​, and k represent the density, specific heat, and thermal conductivity, respectively.