## Supplementary material (SM)

## Data on waste generation

The amount of collected, generated and uncollected waste was estimated from the latest data of the National Information System on Sanitation (SNIS - Sistema Nacional de Informações sobre Saneamento 2020), collected through 2018. As Fundão and Guarapari, that did not send their data in 2018 and 2017, their data is from 2016 (SNIS 2019). The amount of organic waste processed by Organobom in 2020 was 5,193.23 t, which represents 0.749% of the total waste generated. The rest of gathered data is shown in Table 1 of the Manuscript and the results are in SM Table 1.

SM Table 1 – Results of data processing

|  | MSW collection | Collection coverage | MSW generation | Uncollected MSW | Source- -separated collection of recyclables | Commingled collection | Door-to- -door collection |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | t | % | t | t | t | t | % |
| Cariacica | 94,634.94 | 99,32 | 95,279.66 | 644.72 | 644.37 | 93.318,03 | 68.3% |
| Fundão | 4,242.73 | 84,45 | 5,023.75 | 781.02 | - | 4.242,73 | 100% |
| Guarapari | 62,038.71 | 94,00 | 65,998.08 | 3,959.37 | 523.46 | 60.857,36 | 96.2% |
| Serra | 134,112.72 | 100,0 | 134,112.72 | - | 455.41 | 133.170,43 | 99.3% |
| Viana | 15,706.48 | 91,74 | 17,120.52 | 1,414.04 | 196.38 | 15.343,70 | 100% |
| Vila Velha | 216,301.04 | 99,52 | 217,355.17 | 1,054.12 | 790.14 | 214.725,12 | 100% |
| Vitória | 158,598.49 | 100,0 | 158,598.49 | - | 3,110.41 | 153.064,33 | 100% |
| Total | 685,635.11 |  | 693,488.38 | 7,853.27 | 5,720.17 | 674.721,71 |  |
| % | 98.87% |  | 100% | 1.13% | 0.82% | 97,29% | ~94.8% |

The MSW collection was obtained by multiplying the population by the collection per capita and converted from kg per day to tons per year. The collection coverage is the ratio of citizens served by the collection service vs the total population of the municipality. This value was used to obtain the total amount of generated waste, by dividing the amount of collected waste by it. The uncollected waste was simply the difference between the total MSW generation and the MSW collection. And the percentage of door-to-door collection is the number of citizens served by this service type over the number of citizens that are benefited from any kind of collection.

## PLANARES targets

The National Plan for Solid Waste Management (PLANARES - Plano Nacional de Resíduos Sólidos) targets, on which the proposed scenarios were build are presented in SM Table 2.

SM Table 2 – Organic and recyclable waste diversion targets set by the Brazilian Ministry of the Environment for the years between 2015 and 2031.

|  | Year: | 2015 | 2019 | 2023 | 2027 | 2031 |
| --- | --- | --- | --- | --- | --- | --- |
| Biowaste diversions | Optimistic | 70 | 70 | 70 | 70 | 70 |
|  | Moderate | 35 | 45 | 55 | 65 | 70 |
|  | Pessimistic | 25 | 35 | 45 | 50 | 55 |
| Recyclable waste diversions | Optimistic | 70 | 70 | 70 | 70 | 70 |
|  | Moderate | 40 | 55 | 65 | 70 | 70 |
|  | Pessimistic | 30 | 37 | 42 | 45 | 50 |

Source: MMA (2012)

## Mass of collected waste for the different scenarios

In this section, the amounts of waste that were inputted in the life cycle assessment software are presented in SM Table 3 for the Baseline scenario.

For the proposed scenarios (both diesel and biomethane), the quantity of each fraction of waste is indicated in SM Table 4 for the Low Diversion (LD) scenarios and in SM Table 5 for High Diversion (HD) scenarios.

SM Table 3 – Inputted mass of collected waste in the for the different fractions of the Baseline scenario, for uncollected waste, commingled waste and the different categories of source-separated collection.

| Fraction name |  | Source-separated collection |  |  |  | Commingled collection |  |  |  | Uncollected |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | % |  | kg |  | % |  | kg |  | % |  | kg |  |
| Animal food waste |  | 1.57 |  | 0.926 |  | 97.29 |  | 57.25 |  |  |  | 0.666 |  |
| Batteries |  | 0.82 |  | 0.009 |  | 98.04 |  | 1.064 |  |  |  | 0.012 |  |
| Beverage cans (aluminum) |  | 0.82 |  | 0.009 |  | 98.04 |  | 1.115 |  |  |  | 0.013 |  |
| Brown glass |  | 0.82 |  | 0.125 |  | 98.04 |  | 14.86 |  |  |  | 0.172 |  |
| Clear glass |  | 0.82 |  | 0.625 |  | 98.04 |  | 74.31 |  |  |  | 0.858 |  |
| Diapers, sanitary towels, tampons |  | 0.82 |  | 0.106 |  | 98.04 |  | 12.54 |  |  |  | 0.145 |  |
| Dirty paper |  | 0.82 |  | 0.066 |  | 98.04 |  | 7.838 |  |  |  | 0.091 |  |
| Food cans (tinplate/steel) |  | 0.82 |  | 0.084 |  | 98.04 |  | 10.04 |  |  |  | 0.116 |  |
| Hard plastic |  | 0.82 |  | 0.170 |  | 98.04 |  | 20.18 |  |  |  | 0.233 |  |
| Juice cartons (carton/plastic/aluminum) |  | 0.82 |  | 0.090 |  | 98.04 |  | 10.76 |  |  |  | 0.124 |  |
| Magazines |  | 0.82 |  | 0.017 |  | 98.04 |  | 2.005 |  |  |  | 0.023 |  |
| Newsprints |  | 0.82 |  | 0.066 |  | 98.04 |  | 7.838 |  |  |  | 0.091 |  |
| Non-recyclable plastic |  | 0.00 |  | 0.000 |  | 98.87 |  | 29.40 |  | 1.13 |  | 0.337 |  |
| Office paper |  | 0.82 |  | 0.820 |  | 98.04 |  | 97.52 |  |  |  | 1.126 |  |
| Other clean cardboard |  | 0.82 |  | 0.842 |  | 98.04 |  | 100.1 |  |  |  | 1.156 |  |
| Other combustibles |  | 0.82 |  | 0.007 |  | 98.04 |  | 0.836 |  |  |  | 0.010 |  |
| Other non-combustibles |  | 0.00 |  | 0.000 |  | 98.87 |  | 31.43 |  |  |  | 0.360 |  |
| Plastic bottles |  | 0.82 |  | 0.031 |  | 98.04 |  | 3.656 |  |  |  | 0.042 |  |
| Plastic products (toys, hangers, pens) |  | 0.82 |  | 0.062 |  | 98.04 |  | 7.361 |  |  |  | 0.085 |  |
| Rubber |  | 0.82 |  | 0.019 |  | 98.04 |  | 2.281 |  |  |  | 0.026 |  |
| Shoes, leather |  | 0.82 |  | 0.016 |  | 98.04 |  | 1.900 |  |  |  | 0.022 |  |
| Soft plastic |  | 0.82 |  | 0.331 |  | 98.04 |  | 39.36 |  |  |  | 0.455 |  |
| Textiles |  | 0.82 |  | 0.000 |  | 98.87 |  | 20.85 |  |  |  | 0.239 |  |
| Vegetable food waste |  | 1.57 |  | 6.788 |  | 97.29 |  | 419.7 |  |  |  | 4.885 |  |
| Wood |  | 0.00 |  | 0.000 |  | 98.87 |  | 3.296 |  |  |  | 0.038 |  |
| TOTAL (1,000 kg) |  |  |  | 11.210 |  |  |  | 977.47 |  |  |  | 11.324 |  |
|  | Source-separated collection |  |  |  |  |  |  |  |  |  |  |  |  |
| Fraction name | DtD Collection |  |  |  | Bring Collection |  | Commercial Collection |  |  |  | 70/30 Blend Collection |  |  |
|  | % |  | kg |  | % | kg | % |  | kg |  | % |  | kg |
| Animal food waste |  |  | 0.098 |  |  | 0.042 |  |  | 0.177 |  |  |  | 0.609 |
| Batteries |  |  | 0.001 |  |  | 0.000 |  |  | 0.002 |  |  |  | 0.006 |
| Beverage cans (aluminum) |  |  | 0.001 |  |  | 0.000 |  |  | 0.002 |  |  |  | 0.006 |
| Brown glass |  |  | 0.013 |  |  | 0.006 |  |  | 0.024 |  |  |  | 0.082 |
| Clear glass |  |  | 0.066 |  |  | 0.029 |  |  | 0.119 |  |  |  | 0.411 |
| Diapers, sanitary towels, tampons |  |  | 0.011 |  |  | 0.005 |  |  | 0.020 |  |  |  | 0.069 |
| Dirty paper |  |  | 0.007 |  |  | 0.003 |  |  | 0.013 |  |  |  | 0.043 |
| Food cans (tinplate/steel) |  |  | 0.009 |  |  | 0.004 |  |  | 0.016 |  |  |  | 0.056 |
| Hard plastic | 10.54 |  | 0.018 |  | 4.56 | 0.008 | 19.09 |  | 0.032 |  | 65.81 |  | 0.112 |
| Juice cartons (carton/plastic/aluminum) |  |  | 0.010 |  |  | 0.004 |  |  | 0.017 |  |  |  | 0.060 |
| Magazines |  |  | 0.002 |  |  | 0.001 |  |  | 0.003 |  |  |  | 0.011 |
| Newsprints |  |  | 0.007 |  |  | 0.003 |  |  | 0.013 |  |  |  | 0.043 |
| Non-recyclable plastic |  |  | 0.000 |  |  | 0.000 |  |  | 0.000 |  |  |  | 0.000 |
| Office paper |  |  | 0.086 |  |  | 0.037 |  |  | 0.157 |  |  |  | 0.540 |
| Other clean cardboard |  |  | 0.089 |  |  | 0.038 |  |  | 0.161 |  |  |  | 0.554 |
| Other combustibles |  |  | 0.001 |  |  | 0.000 |  |  | 0.001 |  |  |  | 0.005 |
| Other non-combustibles |  |  | 0.000 |  |  | 0.000 |  |  | 0.000 |  |  |  | 0.000 |
| Plastic bottles |  |  | 0.003 |  |  | 0.001 |  |  | 0.006 |  |  |  | 0.020 |
| Plastic products (toys, hangers, pens) |  |  | 0.007 |  |  | 0.003 |  |  | 0.012 |  |  |  | 0.041 |
| Rubber |  |  | 0.002 |  |  | 0.001 |  |  | 0.004 |  |  |  | 0.013 |
| Shoes, leather | 10.54 |  | 0.002 |  | 4.56 | 0.001 | 19.09 |  | 0.003 |  | 65.81 |  | 0.011 |
| Soft plastic |  |  | 0.035 |  |  | 0.015 |  |  | 0.063 |  |  |  | 0.218 |
| Textiles |  |  | 0.000 |  |  | 0.000 |  |  | 0.000 |  |  |  | 0.000 |
| Vegetable food waste |  |  | 0.715 |  |  | 0.310 |  |  | 1.296 |  |  |  | 4.467 |
| Wood |  |  | 0.000 |  |  | 0.000 |  |  | 0.000 |  |  |  | 0.000 |
| TOTAL (11.210 kg) |  |  | 1.182 |  |  | 0.511 |  |  | 2.140 |  |  |  | 7.378 |

SM Table 4 – Inputted mass of collected waste in the for the different fractions in the proposed scenarios (LD) based on the Moderate/Pessimistic PLANARES waste diversion target for 2031.

| Fraction name | Source-separated Collection |  | Commingled collection |  |  |
| --- | --- | --- | --- | --- | --- |
|  | % | kg | % | kg |  |
| Animal food waste | 64.7% | 38.07 | 35.3% | 20.771 |  |
| Batteries | 0.0% | 0.00 | 100.0% | 1.085 |  |
| Beverage cans (aluminum) | 65.8% | 0.75 | 34.2% | 0.389 |  |
| Brown glass | 65.8% | 9.97 | 34.2% | 5.184 |  |
| Clear glass | 65.8% | 49.87 | 34.2% | 25.922 |  |
| Diapers, sanitary towels, tampons | 0.0% | 0.00 | 100.0% | 12.793 |  |
| Dirty paper | 0.0% | 0.00 | 100.0% | 7.995 |  |
| Food cans (tinplate/steel) | 65.8% | 6.74 | 34.2% | 3.501 |  |
| Hard plastic | 65.8% | 13.54 | 34.2% | 7.040 |  |
| Juice cartons (carton/plastic/aluminum) | 65.8% | 7.22 | 34.2% | 3.752 |  |
| Magazines | 65.8% | 1.35 | 34.2% | 0.699 |  |
| Newsprints | 65.8% | 5.26 | 34.2% | 2.734 |  |
| Non-recyclable plastic | 65.8% | 19.57 | 34.2% | 10.171 |  |
| Office paper | 65.8% | 65.45 | 34.2% | 34.019 |  |
| Other clean cardboard | 65.8% | 67.16 | 34.2% | 34.909 |  |
| Other combustibles | 0.0% | 0.00 | 100.0% | 0.853 |  |
| Other non-combustibles | 0.0% | 0.00 | 100.0% | 31.788 |  |
| Plastic bottles | 65.8% | 2.45 | 34.2% | 1.275 |  |
| Plastic products (toys, hangers, pens) | 65.8% | 4.94 | 34.2% | 2.568 |  |
| Rubber | 0.0% | 0.00 | 100.0% | 2.326 |  |
| Shoes, leather | 0.0% | 0.00 | 100.0% | 1.938 |  |
| Soft plastic | 65.8% | 26.42 | 34.2% | 13.730 |  |
| Textiles | 0.0% | 0.00 | 100.0% | 21.090 |  |
| Vegetable food waste | 64.7% | 279.09 | 35.3% | 152.271 |  |
| Wood | 64.7% | 2.16 | 35.3% | 1.177 |  |
| TOTAL |  | 600.02 |  | 399.98 |  |

Note: In these scenarios, it was considered that the population does not separate batteries, therefore throw them in the undifferentiated waste bin. Moreover, the efficiency of the sorting plants was considered to be 76% (lower-end) and 85% for the organic pretreatments.

SM Table 5 – Inputted mass of collected waste in the for the different fractions in the proposed scenarios (HD) based on the Optimistic PLANARES waste diversion target for 2031.

| Fraction name | Source-separated Collection |  | Commingled collection |  |  |
| --- | --- | --- | --- | --- | --- |
|  | % | kg | % | kg |  |
| Animal food waste | 82.40 | 48.486 | 17.6 | 10.356 |  |
| Batteries | 100 | 1.085 | 0.0 | 0.000 |  |
| Beverage cans (aluminum) | 73.70 | 0.838 | 26.3 | 0.299 |  |
| Brown glass | 73.70 | 11.172 | 26.3 | 3.987 |  |
| Clear glass | 73.70 | 55.861 | 26.3 | 19.934 |  |
| Diapers, sanitary towels, tampons | 0.00 | 0.000 | 100.0 | 12.793 |  |
| Dirty paper | 82.40 | 6.588 | 17.6 | 1.407 |  |
| Food cans (tinplate/steel) | 73.70 | 7.545 | 26.3 | 2.693 |  |
| Hard plastic | 73.70 | 15.171 | 26.3 | 5.414 |  |
| Juice cartons (carton/plastic/aluminum) | 73.70 | 8.085 | 26.3 | 2.885 |  |
| Magazines | 73.70 | 1.507 | 26.3 | 0.538 |  |
| Newsprints | 73.70 | 5.892 | 26.3 | 2.103 |  |
| Non-recyclable plastic | 0.00 | 0.000 | 100.0 | 29.739 |  |
| Office paper | 73.70 | 73.310 | 26.3 | 26.161 |  |
| Other clean cardboard | 73.70 | 75.229 | 26.3 | 26.846 |  |
| Other combustibles | 0.00 | 0.000 | 100.0 | 0.853 |  |
| Other non-combustibles | 0.00 | 0.000 | 100.0 | 31.788 |  |
| Plastic bottles | 73.70 | 2.748 | 26.3 | 0.981 |  |
| Plastic products (toys, hangers, pens) | 73.70 | 5.534 | 26.3 | 1.975 |  |
| Rubber | 0 | 0.000 | 100.0 | 2.326 |  |
| Shoes, leather | 0 | 0.000 | 100.0 | 1.938 |  |
| Soft plastic | 73.70 | 29.589 | 26.3 | 10.559 |  |
| Textiles | 0.00 | 0.000 | 100.0 | 21.090 |  |
| Vegetable food waste | 82.40 | 355.442 | 17.6 | 75.920 |  |
| Wood | 82.40 | 2.747 | 17.6 | 0.587 |  |
| TOTAL |  | 706.83 |  | 293.17 |  |

Note: In these scenarios, it was considered that the population fully separates batteries. Additionally, the efficiency of the sorting plants is higher (high-end) in this scenario, at 95%, and the same as in the previous scenarios for organic pretreatments (85%).

## Distance travelled by the waste transport vehicles

SM Table 6 presents the distances between the coordinates where the transfer stations and landfills are located. These values were used to calculate the transport consumptions that are a function of the distance travelled by the trucks that carry commingled waste. Similarly, for source-separated waste, the distances to the waste pickers organizations and respective coordinates are presented in SM Table 7.

SM Table 6 – Transport distances between transfer stations and landfills

| Municipality | Municipality centroid or TS coordinates | Distance from centroid* or TS to landfill | Landfill coordinates |
| --- | --- | --- | --- |
|  | (latitude, longitude) | (km) | (latitude, longitude) |
| Fundão* | -19.96829798 -40.35681096 | 32.5* | -19.843364, -40.247132 |
| Serra | -20.18547977560237 -40.206158105086395 | 26.0 | Cariacica landfill |
| Vitória | -20.456996073737578 -40.391257304852594 | 24.0 | Cariacica landfill |
| Vila Velha | -20.35665673400298 -40.32719617082109 | 19.3 | -20.453787212388203 -40.38349409968328 |
| Cariacica | -20.33596335403781 -40.38411503962181 | 16.3 | -20.242802213893064 -40.37432641809413 |
| Viana* | -20.40104195 -40.51276035 | 27.1* | Cariacica landfill |
| Guarapari | -20.612888855634967 -40.48068309084915 | 39.4 | Vila Velha landfill |
| AVERAGE |  | 29.46 |  |

Note: The municipalities indicated with an asterisk (*) send their waste directly to landfills instead of a transfer station, so the distance considered is from the centroid of the municipality to the landfill.

The municipality of Fundão used to send its waste to the landfill of Cariacica (ES 2019), but since 2017 changed the disposal to Aracruz, a neighbor municipality which has a closer landfill (SNIS 2019b).

SM Table 7 – Transport distances between transfer stations and waste pickers’ organizations

| Municipality | Distance from the closest TS-WPO | WPO coordinates |
| --- | --- | --- |
|  | (km) | (coordinates) |
| Fundão | * | - |
| Serra | 0 | -20.18547977560237, -40.206158105086395 |
|  | 19.8 | -20.125382138678194, -40.278620069143294 |
|  | 11 | -20.21004730017463, -40.27644529141424 |
| Vitória | 10 | -20.31679310351311, -40.35765503075301 |
|  | 5.1 | -20.30878431357461, -40.31297077338803 |
|  | 5.3 | -20.309689652023952, -40.313281303230674 |
| Vila Velha | 4.2 | -20.37137730270207, -40.32416407670781 |
| Cariacica | 13.7 | -20.255116701972653, -40.37635164773321 |
|  | 7.2 | -20.299914299337864, -40.38423688765959 |
| Viana | 1.2 | -20.368082475883668, -40.42120650821959 |
| Guarapari | 4.9 | -20.625683353398944, -40.47635794779673 |
| AVERAGE | 7.49 |  |

One of the waste pickers organizations in Serra is located in the same lot as the transfer station, therefore distance and fuel consumption of transport is negligible.

## Unit emissions

The emissions per distance unit that were found in literature for both diesel and biogas are presented in SM Table 8.

SM Table 8 – Unit emissions per km

|  | CO2 (kg/km) | CO (g/km) | NOx (g/km) | HC (g/km) | PM (g/km) |
| --- | --- | --- | --- | --- | --- |
| DIESEL |  |  |  |  |  |
| Farzaneh et al. (2009) | 2.8 | 17.1 | 17.1 | 0.6 | 0.060 |
| Fontaras et al. (2012) | 2.43 | 7.43 | 32.3 | 0.21 | 0.046 |
| Sandhu et al. (2014) | 2.467 | 6.467 | 14.95 | 3.150 | 0.182 |
| min./MAX. | 2.43/2.8 | 6.467/17.1 | 14.95/32.3 | 0.21/3.15 | 0.046/0.182 |
| BIOMETHANE |  |  |  |  |  |
| Fontaras et al. (2012) | 3.66 | 15.8 | 4.38 | 2.19 | 0.0114 |
| EEA (2009) | 0.00* | 1.49* | 5.17 | 1.26 | 0.0209 |
| Alberici et al. (2003); Hesterberg et al. (2008); López et al. (2009); ASM et al. (2002) | 1.70 | 6.32 | 1.11 | 1.79 | 0.0232 |
| Sandhu et al. (2020) | 2.7 | 18.7 | 0.8 | 3.8 | 0.0039 |
| min./MAX. | 1.70/3.66 | 6.32/18.7 | 0.8/5.17 | 1.26/3.8 | 0.0039/0.0232 |

Note: *Outlier excluded from average

## Intermediate results

With the calculated amount of waste (in kg) that is collected though each collection type (SM Tables SM Table 3 to SM Table 5) and the unit emissions per kg of collected waste (manuscript Table 5), the total emitted carbon dioxide and monoxide, nitrogen oxides, hydrocarbons and particulate material were computed (SM Table 9).

SM Table 9 – Total emitted pollutants during the collection phase in the Baseline and proposed scenarios, for Great Vitória Metropolitan Region.

|  | MSW | CO2 | CO | NOx | HC | PM2.5 |
| --- | --- | --- | --- | --- | --- | --- |
|  | kg |  |  |  |  |  |
|  | Baseline Scenario |  |  |  |  |  |
| DtD | 1.182 | 3.72E-02 | 1.50E-04 | 3.11E-04 | 1.91E-05 | 1.39E-06 |
| Bring | 0.511 | 6.34E-03 | 2.55E-05 | 5.30E-05 | 3.26E-06 | 2.37E-07 |
| Commercial | 2.140 | 6.74E-02 | 2.71E-04 | 5.63E-04 | 3.47E-05 | 2.52E-06 |
| 70/30 Blend | 7.378 | 1.90E-01 | 7.65E-04 | 1.59E-03 | 9.78E-05 | 7.11E-06 |
| Commingled | 977.47 | 9.64 | 3.88E-02 | 8.06E-02 | 4.96E-03 | 3.61E-04 |
| Transport total | 988.68 | 4.64 | 1.87E-02 | 3.88E-02 | 2.39E-03 | 1.74E-04 |
|  | Diesel Low Diversions |  |  |  |  |  |
| DtD | 600.02 | 18.89 | 7.61E-02 | 1.58E-01 | 9.72E-03 | 7.07E-04 |
| Bring |  | 7.442 | 3.00E-02 | 6.22E-02 | 3.83E-03 | 2.79E-04 |
| Transport to MRF |  | 0.918 | 3.70E-03 | 7.67E-03 | 4.72E-04 | 3.44E-05 |
| Commingled | 399.98 | 3.943 | 1.59E-02 | 3.30E-02 | 2.03E-03 | 1.48E-04 |
| Transport to landfill |  | 1.892 | 7.62E-03 | 1.58E-02 | 9.73E-04 | 7.08E-05 |
|  | Diesel High Diversions |  |  |  |  |  |
| DtD | 706.83 | 22.25 | 8.96E-02 | 1.86E-01 | 1.14E-02 | 8.33E-04 |
| Bring |  | 8.77 | 3.53E-02 | 7.33E-02 | 4.51E-03 | 3.28E-04 |
| Transport to MRF |  | 1.081 | 4.35E-03 | 9.04E-03 | 5.56E-04 | 4.05E-05 |
| Commingled | 293.17 | 2.89 | 1.16E-02 | 2.42E-02 | 1.49E-03 | 1.08E-04 |
| Transport to landfill |  | 1.387 | 5.58E-03 | 1.16E-02 | 7.13E-04 | 5.19E-05 |
|  | Biogas Low Diversions |  |  |  |  |  |
| DtD | 600.02 | 19.16 | 9.70E-02 | 2.04E-02 | 1.61E-02 | 1.06E-04 |
| Bring |  | 7.55 | 3.82E-02 | 8.03E-03 | 6.36E-03 | 4.17E-05 |
| Transport to MRF |  | 1.059 | 5.36E-03 | 1.13E-03 | 8.92E-04 | 5.85E-06 |
| Commingled | 399.98 | 4.00 | 2.02E-02 | 4.25E-03 | 3.37E-03 | 2.21E-05 |
| Transport to landfill |  | 2.183 | 1.10E-02 | 2.32E-03 | 1.84E-03 | 1.20E-05 |
|  | Biogas High Diversions |  |  |  |  |  |
| DtD | 706.83 | 22.58 | 1.14E-01 | 2.40E-02 | 1.90E-02 | 1.25E-04 |
| Bring |  | 8.89 | 4.50E-02 | 9.46E-03 | 7.49E-03 | 4.91E-05 |
| Transport to MRF |  | 1.248 | 6.31E-03 | 1.33E-03 | 1.05E-03 | 6.89E-06 |
| Commingled | 293.17 | 2.93 | 1.48E-02 | 3.12E-03 | 2.47E-03 | 1.62E-05 |
| Transport to landfill |  | 1.600 | 8.10E-03 | 1.70E-03 | 1.35E-03 | 8.83E-06 |

## Unit impacts

The unit impacts by kg of controlled pollutant that were used to calculate the total environmental impacts for each impact category are presented in SM Table 10 for diesel and SM Table 11 for biomethane.

SM Table 10 – Unit impacts per kg of pollutant emitted, using diesel.

| DIESEL |  | Carbon dioxide, fossil | Carbon monoxide, fossil | Nitrogen oxides | NMVOC, non-methane volatile organic compound | Particulates, < 2.5 µm |
| --- | --- | --- | --- | --- | --- | --- |
| GWP100 | (kg CO2-Eq) | 1 | 0 | 0 | 0 | 0 |
| PM | (kg PM2.5-eq) | 0 | 5.167E-04 | 7.778E-03 | 0 | 3.333 |
| POF | (kg NMVOC) | 0 | 0.04561 | 1 | 1 | 0 |
| TA | (mol H+ eq) | 0 | 0 | 0.74 | 0 | 0 |
| ET | (mol N eq) | 0 | 0 | 4.26 | 0 | 0 |
| EM | (kg N eq) | 0 | 0 | 0.389 | 0 | 0 |

Source: EC-JRC (2010)

SM Table 11 – Unit impacts per kg of pollutant emitted, using biomethane.

| BIOMETHANE |  | Carbon dioxide, NON- -FOSSIL | Carbon monoxide, NON-FOSSIL | Nitrogen oxides | METHANE | Particulates, < 2.5 µm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GWP100 | (kg CO2-Eq) | 0 | 0 | 0 | 25 |  | 0 |
| PM | (kg PM2.5-eq) | 0 | 5.167E-04 | 7.778E-03 | 0 |  | 3.333 |
| POF | (kg NMVOC) | 0 | 0.04561 | 1 | 0.0101 |  | 0 |
| TA | (mol H+ eq) | 0 | 0 | 0.74 | 0 |  | 0 |
| ET | (mol N eq) | 0 | 0 | 4.26 | 0 |  | 0 |
| EM | (kg N eq) | 0 | 0 | 0.389 | 0 |  | 0 |

Source: EC-JRC (2010)

- Results

To obtain the results, the mass of emitted pollutants (in kg, from SM Table 9) was multiplied by the emission factors (in SM Tables SM Table 10 and SM Table 11), the environmental impacts were calculated for each impact category and summed. Finally, the results were normalized, by multiplying by 1000 and dividing by the respective normalization factors (SM Table 12). To sum up, SM Table 13 and SM Table 14 show the results both non-normalized and normalized.

SM Table 12 – Recommended ILCD normalizing factors .

| Normalizing factors |  |
| --- | --- |
| ILCD2011, Climate change w/o LT; midpoint; GWP100; IPPC2007 kg CO2-Eq | 8400 |
| ILCD2011, Particulate matter w/o LT, from Humbert 2009, PM kgPM2.5-eq | 5.07 |
| ILCD2011, Photochemical ozone formation, human health w/o LT, POCP kg NMVOC | 40.6 |
| ILCD2011, Terrestrial acidification, Accumulated Exceedance mol H+ eq. | 55.5 |
| ILCD2011, Eutrophication Terrestrial, Accumulated Exceedance mol N eq. | 177 |
| ILCD2011, Eutrophication Marine w/o LT, ReCiPe2008 1.05 kg N eq. | 28.3 |

Source: Crenna et al. (2017)

SM Table 13 – Numerical results of the environmental impacts (non-normalized and normalized) for the Baseline scenario.

| Impacts | Commingled collection | Blend collection | Commercial collection | DtD collection | Bring collection | Total Transport |
| --- | --- | --- | --- | --- | --- | --- |
| MSW (kg) | 977.47 | 7.378 | 2.140 | 1.182 | 0.511 | 988.68 |
| GWP100 | 9.64 | 3.72E-02 | 6.34E-03 | 6.74E-02 | 1.90E-01 | 4.641 |
| PM | 1.85E-03 | 7.14E-06 | 1.22E-06 | 1.29E-05 | 3.65E-05 | 8.90E-04 |
| POF | 8.73E-02 | 3.37E-04 | 5.74E-05 | 6.10E-04 | 1.72E-03 | 4.20E-02 |
| TA | 5.96E-02 | 2.30E-04 | 3.92E-05 | 4.17E-04 | 1.18E-03 | 2.87E-02 |
| ET | 3.43E-01 | 1.32E-03 | 2.26E-04 | 2.40E-03 | 6.77E-03 | 0.165 |
| EM | 3.13E-02 | 1.21E-04 | 2.06E-05 | 2.19E-04 | 6.18E-04 | 1.51E-02 |
| Normalized impacts |  |  |  |  |  |  |
| GWP100 | 1.147 | 4.43E-03 | 7.55E-04 | 8.02E-03 | 2.26E-02 | 0.552 |
| PM | 0.365 | 1.41E-03 | 2.40E-04 | 2.55E-03 | 7.19E-03 | 0.176 |
| POF | 2.150 | 8.30E-03 | 1.41E-03 | 1.50E-02 | 4.24E-02 | 1.035 |
| TA | 1.074 | 4.15E-03 | 7.07E-04 | 7.51E-03 | 2.12E-02 | 0.517 |
| ET | 1.939 | 7.49E-03 | 1.28E-03 | 1.36E-02 | 3.82E-02 | 0.934 |
| EM | 1.107 | 4.28E-03 | 7.29E-04 | 7.74E-03 | 2.18E-02 | 0.533 |

SM Table 14 – Numerical results of the environmental impacts (non-normalized and normalized) for the proposed scenarios.

| Diesel – Low Diversions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Impacts |  |  |  | Normalized impacts |  |  |  |
|  | Commingled | DtD | Bring | Total Transport | Commingled | DtD | Bring | Total Transport |
| MSW (kg) | 399.98 | 600.02 |  | 1,000.00 |  |  |  |  |
| GWP100 | 3.943 | 18.89 | 7.442 | 3.98E-02 | 0.469 | 2.249 | 0.886 | 0.335 |
| PM | 7.57E-04 | 3.62E-03 | 1.43E-03 | 65.98 | 0.149 | 0.715 | 0.282 | 0.106 |
| POF | 3.57E-02 | 0.171 | 6.74E-02 | 8.239 | 0.880 | 4.215 | 1.660 | 0.627 |
| TA | 2.44E-02 | 0.117 | 4.60E-02 | 6.027 | 0.440 | 2.106 | 0.830 | 0.313 |
| ET | 0.140 | 0.673 | 0.265 | 1.890 | 0.793 | 3.801 | 1.498 | 0.565 |
| EM | 1.28E-02 | 6.14E-02 | 2.42E-02 | 11.82 | 0.453 | 2.171 | 0.855 | 0.323 |
| Diesel – High Diversions |  |  |  |  |  |  |  |  |
| MSW (kg) | 293.17 | 706.83 |  | 1,000.00 |  |  |  |  |
| GWP100 | 2.890 | 22.25 | 8.767 | 3.50E-02 | 0.344 | 2.649 | 1.044 | 0.294 |
| PM | 5.54E-04 | 4.27E-03 | 1.68E-03 | 57.95 | 0.109 | 0.842 | 0.332 | 9.34E-02 |
| POF | 2.62E-02 | 0.202 | 7.94E-02 | 7.237 | 0.645 | 4.965 | 1.956 | 0.551 |
| TA | 1.79E-02 | 0.138 | 5.42E-02 | 5.294 | 0.322 | 2.481 | 0.977 | 0.275 |
| ET | 1.03E-01 | 0.793 | 0.312 | 1.660 | 0.582 | 4.478 | 1.764 | 0.497 |
| EM | 9.40E-03 | 7.24E-02 | 2.85E-02 | 10.38 | 0.332 | 2.557 | 1.008 | 0.284 |
| Biomethane – Low Diversions |  |  |  |  |  |  |  |  |
| MSW (kg) | 399.98 | 600.02 |  | 1,000.00 |  |  |  |  |
| GWP100 | 8.42E-02 | 0.404 | 0.159 | 9.68E-04 | 0.010 | 4.80E-02 | 1.89E-02 | 8.13E-03 |
| PM | 1.17E-04 | 5.61E-04 | 2.21E-04 | 1.603 | 2.31E-02 | 0.111 | 4.36E-02 | 1.87E-02 |
| POF | 5.21E-03 | 2.50E-02 | 9.84E-03 | 0.200 | 0.128 | 0.615 | 0.242 | 0.104 |
| TA | 3.15E-03 | 1.51E-02 | 5.94E-03 | 0.146 | 5.67E-02 | 0.272 | 0.107 | 4.60E-02 |
| ET | 1.81E-02 | 8.68E-02 | 3.42E-02 | 4.59E-02 | 0.102 | 0.491 | 0.193 | 8.30E-02 |
| EM | 1.65E-03 | 7.93E-03 | 3.12E-03 | 0.287 | 5.85E-02 | 0.280 | 0.110 | 4.74E-02 |
| Biomethane – High Diversions |  |  |  |  |  |  |  |  |
| MSW (kg) | 293.17 | 706.83 |  | 1,000.00 |  |  |  |  |
| GWP100 | 6.17E-02 | 0.475 | 0.187 | 8.50E-04 | 7.35E-03 | 5.66E-02 | 2.23E-02 | 7.14E-03 |
| PM | 8.59E-05 | 6.61E-04 | 2.60E-04 | 1.408 | 1.69E-02 | 0.130 | 5.14E-02 | 1.64E-02 |
| POF | 3.82E-03 | 2.94E-02 | 1.16E-02 | 0.176 | 9.41E-02 | 0.724 | 0.285 | 9.14E-02 |
| TA | 2.31E-03 | 1.78E-02 | 7.00E-03 | 0.129 | 4.16E-02 | 0.320 | 0.126 | 4.04E-02 |
| ET | 1.33E-02 | 1.02E-01 | 4.03E-02 | 4.03E-02 | 7.50E-02 | 0.578 | 0.228 | 7.29E-02 |
| EM | 1.21E-03 | 9.34E-03 | 3.68E-03 | 0.252 | 4.29E-02 | 0.330 | 0.130 | 4.16E-02 |

- Sensitivity analysis results

The results of the sensitivity analysis, using the maximum and minimum values for the emissions found in the literature, patent in Item 5 of this SM are presented in SM Tables 15 and 16. SM Tables 17 and 18 show the sensitivity analysis made for the consumption values found in the literature and presented in Table 4 of the Manuscript.

SM Table 15 – Absolute deviations of the sensitivity analysis of the unit emissions for diesel scenarios

| Error | GWP100 |  | PM2.5 |  | POF |  | TA |  | ET |  | EM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline Collection | -0.06 | +0.11 | -0.17 | +0.29 | -0.74 | +1.24 | -0.34 | +0.56 | -0.61 | +1.01 | -0.35 | +0.58 |
| Baseline Transport | -2.92E-02 | +5.05E-02 | -7.78E-02 | +1.33E-01 | -3.47E-01 | +5.79E-01 | -1.57E-01 | +2.62E-01 | -2.83E-01 | +4.72E-01 | -1.62E-01 | +2.70E-01 |
| P/M Bring Collection | -0.07 | +0.12 | -0.19 | +0.33 | -0.85 | +1.42 | -0.38 | +0.64 | -0.69 | +1.16 | -0.40 | +0.66 |
| P/M Transport | -0.02 | +0.03 | -0.05 | +0.08 | -0.21 | +0.35 | -0.09 | +0.16 | -0.17 | +0.29 | -0.10 | +0.16 |
| O Bring Collection | -0.07 | +0.13 | -0.20 | +0.33 | -0.87 | +1.45 | -0.39 | +0.66 | -0.71 | +1.19 | -0.41 | +0.68 |
| O Transport | -0.02 | +0.03 | -0.04 | +0.07 | -0.18 | +0.31 | -0.08 | +0.14 | -0.15 | +0.25 | -0.09 | +0.14 |
| P/M DtD Collection | -0.14 | +0.25 | -0.38 | +0.66 | -1.71 | +2.85 | -0.77 | +1.29 | -1.39 | +2.32 | -0.80 | +1.33 |
| P/M Transport | -0.02 | +0.03 | -0.05 | +0.08 | -0.21 | +0.35 | -0.09 | +0.16 | -0.17 | +0.29 | -0.10 | +0.16 |
| O DtD Collection | -0.16 | +0.27 | -0.42 | +0.72 | -1.88 | +3.14 | -0.85 | +1.42 | -1.53 | +2.56 | -0.88 | +1.46 |
| O Transport | -0.02 | +0.03 | -0.04 | +0.07 | -0.18 | +0.31 | -0.08 | +0.14 | -0.15 | +0.25 | -0.09 | +0.14 |

SM Table 16 – Absolute deviations of the sensitivity analysis of the unit emissions for biomethane scenarios

| Error | GWP100 |  | PM2.5 |  | POF |  | TA |  | ET |  | EM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P/M Bring Collection | -0.01 | +0.03 | -0.05 | +0.06 | -0.27 | +0.36 | -0.11 | +0.17 | -0.20 | +0.31 | -0.12 | +0.18 |
| P/M Transport | -3.61E-03 | +5.61E-03 | -1.40E-02 | +1.15E-02 | -0.08 | +7.58E-02 | -3.33E-02 | +3.70E-02 | -6.02E-02 | +6.69E-02 | -3.44E-02 | +3.82E-02 |
| O Bring Collection | -0.01 | +0.03 | -0.05 | +0.06 | -0.27 | +0.37 | -0.12 | +0.18 | -0.21 | +0.32 | -0.12 | +0.18 |
| O Transport | -3.17E-03 | +4.93E-03 | -1.23E-02 | +1.01E-02 | -0.07 | +6.66E-02 | -2.93E-02 | +3.25E-02 | -5.29E-02 | +5.87E-02 | -3.02E-02 | +3.35E-02 |
| P/M DtD Collection | -0.02 | +0.05 | -0.10 | +0.11 | -0.53 | +0.72 | -0.23 | +0.35 | -0.41 | +0.62 | -0.23 | +0.36 |
| P/M Transport | -3.61E-03 | +5.61E-03 | -1.40E-02 | +1.15E-02 | -0.08 | +7.58E-02 | -3.33E-02 | +3.70E-02 | -6.02E-02 | +6.69E-02 | -3.44E-02 | +3.82E-02 |
| O DtD Collection | -0.02 | +0.06 | -0.10 | +0.12 | -0.59 | +0.79 | -0.25 | +0.38 | -0.45 | +0.69 | -0.26 | +0.39 |
| O Transport | -3.17E-03 | +4.93E-03 | -1.23E-02 | +1.01E-02 | -0.07 | +6.66E-02 | -2.93E-02 | +3.25E-02 | -5.29E-02 | +5.87E-02 | -3.02E-02 | +3.35E-02 |

SM Table 17– Absolute deviations of the sensitivity analysis of the unit consumptions for diesel scenarios

| Error | GWP100 |  | PM2.5 |  | POF |  | TA |  | ET |  | EM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline Collection | -0.333 | +0.327 | -0.106 | +0.104 | -0.624 | +0.612 | -0.312 | +0.306 | -0.563 | +0.552 | -0.322 | +0.315 |
| Baseline Transport | -1.10E-01 | +1.10E-01 | -3.51E-02 | +3.51E-02 | -2.07E-01 | +2.07E-01 | -1.03E-01 | +1.03E-01 | -1.87E-01 | +1.87E-01 | -1.07E-01 | +1.07E-01 |
| P/M Bring Collection | -0.16 | +0.16 | -0.05 | +0.05 | -0.31 | +0.30 | -0.15 | +0.15 | -0.28 | +0.27 | -0.16 | +0.15 |
| P/M Transport | -0.07 | +0.07 | -0.02 | +0.02 | -0.13 | +0.13 | -0.06 | +0.06 | -0.11 | +0.11 | -0.06 | +0.06 |
| O Bring Collection | -0.13 | +0.13 | -0.04 | +0.04 | -0.25 | +0.25 | -0.12 | +0.12 | -0.23 | +0.22 | -0.13 | +0.13 |
| O Transport | -0.06 | +0.06 | -0.02 | +0.02 | -0.11 | +0.11 | -0.06 | +0.06 | -0.10 | +0.10 | -0.06 | +0.06 |
| P/M DtD Collection | -0.91 | +1.08 | -0.29 | +0.34 | -1.70 | +2.03 | -0.85 | +1.01 | -1.54 | +1.83 | -0.88 | +1.05 |
| P/M Transport | -0.07 | +0.07 | -0.02 | +0.02 | -0.13 | +0.13 | -0.06 | +0.06 | -0.11 | +0.11 | -0.06 | +0.06 |
| O DtD Collection | -1.01 | +1.22 | -0.32 | +0.39 | -1.90 | +2.28 | -0.95 | +1.14 | -1.71 | +2.06 | -0.98 | +1.18 |
| O Transport | -0.06 | +0.06 | -0.02 | +0.02 | -0.11 | +0.11 | -0.06 | +0.06 | -0.10 | +0.10 | -0.06 | +0.06 |

SM Table 18 – Absolute deviations of the sensitivity analysis of the unit consumptions for biomethane scenarios

| Error | GWP100 |  | PM2.5 |  | POF |  | TA |  | ET |  | EM |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P/M Bring Collection | -3.66E-03 | +7.85E-03 | -8.44E-03 | +1.81E-02 | -4.69E-02 | +1.01E-01 | -2.07E-02 | +4.44E-02 | -3.74E-02 | +8.02E-02 | -2.14E-02 | +4.58E-02 |
| P/M Transport | -1.63E-03 | +1.63E-03 | -3.75E-03 | +3.75E-03 | -2.08E-02 | +2.08E-02 | -9.20E-03 | +9.20E-03 | -1.66E-02 | +1.66E-02 | -9.48E-03 | +9.48E-03 |
| O Bring Collection | -3.51E-03 | +7.25E-03 | -8.08E-03 | +1.67E-02 | -4.49E-02 | +9.28E-02 | -1.98E-02 | +4.10E-02 | -3.58E-02 | +7.41E-02 | -2.04E-02 | +4.23E-02 |
| O Transport | -1.37E-03 | +2.61E-03 | -3.16E-03 | +6.02E-03 | -1.75E-02 | +3.35E-02 | -7.75E-03 | +1.48E-02 | -1.40E-02 | +2.67E-02 | -7.99E-03 | +1.52E-02 |
| P/M DtD Collection | -1.41E-02 | +3.43E-02 | -3.25E-02 | +7.90E-02 | -1.81E-01 | +4.39E-01 | -7.98E-02 | +1.94E-01 | -1.44E-01 | +3.50E-01 | -8.22E-02 | +2.00E-01 |
| P/M Transport | -1.63E-03 | +1.63E-03 | -3.75E-03 | +3.75E-03 | -2.08E-02 | +2.08E-02 | -9.20E-03 | +9.20E-03 | -1.66E-02 | +1.66E-02 | -9.48E-03 | +9.48E-03 |
| O DtD Collection | -1.58E-02 | +3.84E-02 | -3.64E-02 | +8.85E-02 | -2.02E-01 | +4.92E-01 | -8.94E-02 | +2.17E-01 | -1.61E-01 | +3.92E-01 | -9.21E-02 | +2.24E-01 |
| O Transport | -1.37E-03 | +2.61E-03 | -3.16E-03 | +6.02E-03 | -1.75E-02 | +3.35E-02 | -7.75E-03 | +1.48E-02 | -1.40E-02 | +2.67E-02 | -7.99E-03 | +1.52E-02 |

## References

Alberici, F., Callera, G., Florio, S., 2003. Determinazione sperimentale delle emissioni allo scarico dimotori diesel di mezzi adibiti al transport publico ed al sevizio di raccolta RSU ed estensione relativa all uso di biodiesel (Experimental determination of the exhaust emissions of diesel engines of vehicles used for public transport and MSW collection service and extension relating to the use of biodiesel). ENITecnologie, Sala Motori ed Emissioni SAMO.

ASM, OMB, IVECO, 2002. Progetto metano, studio di settore per un azienda di igiene urbana. ASM Brescia, Brescia.

Crenna, E., Pant, R., Sala, S., Secchi, M., European Commission, Joint Research Centre, 2017. Global normalisation factors for the environmental footprint and Life Cycle Assessment.

EC-JRC - European Commission-Joint Research Centre, 2010. International Reference Life Cycle Data System (ILCD) Handbook :general guide for life cycle assessment : detailed guidance. Publications Office, LU.

EEA - European Environment Agency, 2009. COPERT emission factors.

ES - Espírito Santo, 2019. State Plan for Solid Waste Management - Diagnosis on MSW management in Espírito Santo (Plano Estadual de Resíduos Sólidos - Diagnóstico sobre a gestão dos resíduos sólidos urbanos no Espírito Santo).

Farzaneh, M., Zietsman, J., Lee, D.-W., 2009. Evaluation of In-Use Emissions from Refuse Trucks. Transportation Research Record 2123, 38–45. https://doi.org/10.3141/2123-05

Fontaras, G., Martini, G., Manfredi, U., Marotta, A., Krasenbrink, A., Maffioletti, F., Terenghi, R., Colombo, M., 2012. Assessment of on-road emissions of four Euro V diesel and CNG waste collection trucks for supporting air-quality improvement initiatives in the city of Milan. Science of The Total Environment 426, 65–72. https://doi.org/10.1016/j.scitotenv.2012.03.038

Hesterberg, T.W., Lapin, C.A., Bunn, W.B., 2008. A Comparison of Emissions from Vehicles Fueled with Diesel or Compressed Natural Gas. Environ. Sci. Technol. 42, 6437–6445. https://doi.org/10.1021/es071718i

López, J.M., Flores, N., Jiménez, F., Aparicio, F., 2009. Emissions Pollutant from Diesel, Biodiesel and Natural Gas Refuse Collection Vehicles in Urban Areas, in: Rauch, S., Morrison, G.M., Monzón, A. (Eds.), Highway and Urban Environment, Alliance for Global Sustainability Bookseries. Springer Netherlands, Dordrecht, pp. 141–148. https://doi.org/10.1007/978-90-481-3043-6_16

MMA - Ministério do Meio Ambiente (Brazilian Ministry of the Environment), 2012. National Plan for Solid Waste Management (Plano Nacional de Resíduos Sólidos).

Sandhu, G.S., Frey, H.C., Bartelt-Hunt, S., Jones, E., 2020. Real-world activity, fuel use, and emissions of heavy-duty compressed natural gas refuse trucks. Science of The Total Environment 143323. https://doi.org/10.1016/j.scitotenv.2020.143323

Sandhu, G.S., Frey, H.C., Bartelt-Hunt, S., Jones, E., 2014. In-use measurement of the activity, fuel use, and emissions of front-loader refuse trucks. Atmospheric Environment 92, 557–565. https://doi.org/10.1016/j.atmosenv.2014.04.036

SNIS - Sistema Nacional de Informações sobre Saneamento, 2020. Diagnóstico do manejo de Resíduos Sólidos Urbanos - 2019 [WWW Document]. Planos de Resíduos Sólidos. URL http://www.snis.gov.br/diagnostico-anual-residuos-solidos/diagnostico-do-manejo-de-residuos-solidos-urbanos-2019 (accessed 6.15.21).

SNIS - Sistema Nacional de Informações sobre Saneamento, 2019a. Diagnóstico do manejo de Resíduos Sólidos Urbanos - 2017  (re-published) [WWW Document]. Planos de Resíduos Sólidos. URL http://www.snis.gov.br/diagnostico-anual-residuos-solidos/diagnostico-rs-2017 (accessed 6.15.21).

SNIS - Sistema Nacional de Informações sobre Saneamento, 2019b. Diagnóstico do manejo de Resíduos Sólidos Urbanos - 2018 [WWW Document]. Planos de Resíduos Sólidos. URL http://www.snis.gov.br/diagnostico-anual-residuos-solidos/diagnostico-do-manejo-de-residuos-solidos-urbanos-2018 (accessed 6.15.21).