Table 1. Characteristics of the local Biological Oxygen Demand (BOD) data, including the corresponding departments where the studies were carried out, used to downscale BOD estimates provided by the DynQual 1.0 model for the province of Tucumán (Argentina).

| Reference | Objective of the study | Number of sites and coordinates (WSG84) | Sampling regime and coverage | Type(s) of environment | Summary statistics (rolling year mean ± standard deviation of BOD) |
| --- | --- | --- | --- | --- | --- |
| Isasmendi et al. (2007). Department: Tafí del Valle | Characterise the dynamics and water quality of mountainous streams | 7 sites. Blanquito: -65.70, -26.82 Churqui: -65.72, -26.83 Las Carreras: -65.73, -26.85 La Ovejeria: -65.73, -36.85 Tafi: -65.70, -26.88 El Mollar: -65.70, -26.93 Los Sosa: -65.67, -26.93 | Monthly BOD samples from October-2000 to October-2001. All sites sampled 13 months. | Relatively pristine mountainous streams and rivers with a few small populations nearby. | Blanquito: 0.70 ± 0.64 Churqui: 0.96 ± 0.47 Las Carreras: 0.67 ± 0.56 La Ovejeria: 1.02 ± 0.56 Tafi: 0.96 ± 0.44 El Mollar: 1.08 ± 0.55 Los Sosa: 0.73 ± 0.42 |
| Martínez De Marco et al. (2007) Departments: Trancas and Tafí Viejo | Evaluate the physical and chemical characteristics of rivers important for the province of Tucumán and assess the concentration of agrochemicals. | 4 sites. Salí 1: -65.21, -26.47 Salí 2: -65.19, -26.64 Vipos: -65.22, -26.48 Tapia: -65.24, -26.30 | BOD sampled from March-1998 to March-1999. Salí 1: 11 samples. Salí 2: 13 samples. Vipos: 9 samples. Tapia: 13 samples. | Valley surrounded by hill with forest, low-density human settlements, and extensive livestock activities. Main river of the province of Tucumán (Salí River) and sampling sites around the ‘El Cadillal’ reservoir, which provides water to the city of San Miguel de Tucumán. | Salí 1: 20.40 ± 32.7 Salí 2: 3.60 ± 5.40 Vipos: 14.00 ± 24.60 Tapia: 1.40 ± 2.20 |
| Powell (2011) Departments: Monteros, Famaillá, Lules, and Tafí Viejo | Evaluate the impact of industrial untreated wastewater discharge upon water quality. | 6 sites. There were 10 sites originally, but six of these were combined into two sites due to their closeness. Sites were originally assigned to three categories: 1) ref: denoting reference (control) sites; 2) vert: denoting sites receiving wastewater; and 3) abaj: denoting sites downstream of sites receiving wastewater. Sites Sref, Svert, and Sabaj were combined into one site. Sites Dref, Dvert, and Dabaj were combined into one site. Calimayo (Cref): -65.39, -26.93, Calimayo (Cvert): -65.36, -26.97 De La Cruz (Dref, Dvert, and Dabaj combined): -65.38, -27.03 Hollinado (Href): -65.47, -27.12 Hollinado (Hvert): -65.45, -27.13 Salí (Sref, Svert, and Sabaj combined): -65.16, -26.72 | Six BOD samples taken on March, April, August, September, and December 2008; and February 2009. | Variety of environments in the central plateaus of the province of Tucumán. Focussed on sampling both reference (unpolluted) and sites receiving raw wastewater from nearby industries. High levels of alteration and high population density. | Calimayo (Cref): 2.67 ± 1.75 Calimayo (Cvert): 486.0 ± 437.39 De La Cruz (Dref, Dvert, and Dabaj combined): 347.39 ± 577.55 Hollinado (Href): 4.50 ± 2.67 Hollinado (Hvert): 29.50 ± 60.58 Salí (Sref, Svert, and Sabaj combined): 4.22 ± 4.51 |

Table 2. Description and data sources for the six environmental and socio-economic covariates used to explore correlates of predicted mean annual Biological Oxygen Demand (BOD) in the 17 departments of the province of Tucumán (Argentina) in 1980, 1991, 2001, 2010, and 2019. Covariates were first estimated for each 5-km cells, and then aggregated at the department level following the procedures described in the table.

| Covariate | Source | Explanation/Rationale |
| --- | --- | --- |
| Leaf Area Index of High Vegetation: Mean of the monthly LAI of high vegetation in a year. Aggregation: mean of all cells inside a department. Transformation: squared. | Muñoz Sabater (2024) | LAI measures green leaf area, and it is a proxy for vegetation cover and composition. Vegetation affects water quality, and we expect a high score of LAI of high vegetation to negatively correlate with BOD since it serves as a proxy for high forest cover and high ecosystem integrity. |
| Mean total annual precipitation: Sum of the total monthly precipitation in a year. Aggregation: mean of all cells inside a department. Transformation: logarithm. | Muñoz Sabater (2024) | Precipitation contributes to water quality by dilluting pollutants. Therefore, we expect a positive relationship between the precipitation and the mean annual BOD. |
| Mean surface temperature: Mean of the monthly temperature in a year. Aggregation: mean of all cells inside a department. Transformation: squared. | Muñoz Sabater (2024) | Temperature influences the physical and bio-geochemical cycles determining water quality. We anticipate a positive relationship with mean annual BOD given that high temperatures lead to higher BOD. |
| Population density: Number of people / 100 km2 in each department. Aggregation: none since the values are already represented at the department level. Transformation: logarithm. | Instituto Nacional de Estadística y Censos de la República Argentina INDEC (2024) | Higher population densities will result in higher pressures to waterbodies and a higher likelihood of pollution through untreated wastewater discharge and open-air garbage dumps, amongst others. Additionally, it will indicate if higher numbers of people are exposed to low water quality levels. |
| Precarious housing: Proportion of the population of a department living in precarious conditions. Aggregation: none since the values are already represented at the department level. Transformation: arcsin. | Instituto Nacional de Estadística y Censos de la República Argentina INDEC (2024) | Precarious houses are more likely to lack basic sanitation and access to safe disposal of garbage. Therefore, we expect the percentage of precarious housing to correlate positively with mean annual BOD. Additionally, it will indicate the need for targeted measures to improve housing conditions to enhance water quality. |
| Density of industries: Number of industries / 100 km2 in each department. Aggregation: none since the values are already represented at the department level. Transformation: logarithm. | Instituto Geográfico Nacional (2025) | Industries are potential sources of water pollution. We expect a positive correlation with mean annual BOD. |

Table 3. Spearman’s partial correlations (ρ; maximum a posteriori probability estimate ± standard deviations, and 95% highest density intervals) between the predicted mean annual Biological Oxygen Demand (BOD) and six environmental and socio-economic weighted covariates in the 17 departments of the province of Tucumán (Argentina) in 1980, 1991, 2001, 2010, and 2019. Covariates were weighted according to the number of 5-km cells in each department. See Table 2 for descriptions of the covariates and the methods section for details on the methods.

| Covariate/Year | 1980 | 1991 | 2001 | 2010 | 2019 |
| --- | --- | --- | --- | --- | --- |
| Leaf Area Index of High Vegetation | 0.12 ± 0.19 (-0.27, 0.48) P(ρ ≠ 0) = 0.23 | 0.17 ± 0.20 (-0.26, 0.52) P(ρ ≠ 0) = 0.23 | 0.26 ± 0.20 (-0.15, 0.61) P(ρ ≠ 0) = 0.10 | 0.44 ± 0.24 (-0.18, 0.72) P(ρ ≠ 0) = 0.11 | 0.12 ± 0.26 (-0.40, 0.61) P(ρ ≠ 0) = 0.49 |
| Mean total annual precipitation | -0.13 ± 0.19 (-0.45, -0.31) P(ρ ≠ 0) = 0.31 | -0.07 ± 0.18 (-0.48, 0.27) P(ρ ≠ 0) = 0.33 | 0.02 ± 0.30 (-0.57, 0.50) P(ρ ≠ 0) = 0.45 | -0.06 ± 0.24 (-0.56, 0.38) P(ρ ≠ 0) = 0.40 | 0.05 ± 0.27 (-0.51, 0.51) P(ρ ≠ 0) = 0.49 |
| Mean surface temperature | 0.47 ± 0.18 (0.08, 0.76) P(ρ ≠ 0) = 0.02 | 0.64 ± 0.15 (0.28, 0.85) P(ρ ≠ 0) = 0.01 | 0.20 ± 0.27 (-0.30, 0.66) P(ρ ≠ 0) = 0.27 | 0.64 ± 0.19 (0.17, 0.86) P(ρ ≠ 0) = 0.01 | 0.71 ± 0.18 (0.23, 0.90) P(ρ ≠ 0) = 0.01 |
| Population density | -0.13 ± 0.24 (-0.49, 0.44) P(ρ ≠ 0) = 0.36 | -0.42 ± 0.25 (-0.73, -0.22) P(ρ ≠ 0) = 0.10 | -0.49 ± 0.12 (-0.70, -0.23) P(ρ ≠ 0) = 0.01 | -0.16 ± 0.30 (-0.59, 0.54) P(ρ ≠ 0) = 0.36 | 0.02 ± 0.30 (-0.54, 0.60) P(ρ ≠ 0) = 0.49 |
| Precarious housing | -0.18 ± 0.20 (-0.52, 0.25) P(ρ ≠ 0) = 0.25 | 0.13 ± 0.23 (-0.33, 0.57) P(ρ ≠ 0) = 0.30 | -0.13 ± 0.18 (-0.45, 0.23) P(ρ ≠ 0) = 0.23 | -0.37 ± 0.29 (-074, 0.35) P(ρ ≠ 0) = 0.18 | -0.40 ± 0.30 (-0.80, 0.30) P(ρ ≠ 0) = 0.18 |
| Density of industries | 0.67 ± 0.15 (0.34, 0.84) P(ρ ≠ 0) = 0.01 | 0.63 ± 0.17 (0.22, 0.82) P(ρ ≠ 0) = 0.02 | 0.39 ± 0.14 (0.10, 0.64) P(ρ ≠ 0) = 0.01 | 0.77 ± 0.18 (0.35, 0.92) P(ρ ≠ 0) = 0.01 | 0.75 ± 0.18 (0.32, 0.91) P(ρ ≠ 0) = 0.01 |