Confined Irradiation Vacuum Ultraviolet Abatement Technique for Perfluorinated Compounds

Yanei Xuea, Wenjun Suna,#, Wentao Lib, Guoqing Jianga, Ranlong Xua, Yuanna Zhanga

a School of Environment

Tsinghua University

Beijing, 100084, China

E-mail: wsun@tsinghua.edu.cn

b School of Science and Engineering

Shanghai Jiao Tong University

Shanghai, 200030, China

contents

Table 1. Available performance and conditions for reactions of PFASs in related VUV185 reactors.	2

Table 2. The structure of PFASs and BDE (bond dissociation energy) of C–C bonds in PFAS molecule.	3

Table 3. The kors and DeF% of PFASs by VUV_CI/CB with various confined height.	5

Table 4. The chain reaction between the various free radicals.	6

Figure S1. Time distributions of decay rate of 10 PFASs under VUV_DI, VUV_CI and VUV_CI/CB.	7

Fig. 2. the EPR spectra of FRs in the VUV_CI.	8

Fig. 5 Identified representative transfer products(TPs) of PFASs under VUV_CI/CB irradiation.	9

Text 1. Detailed Information on Materials and Methods	19

Text 2. Measurement of PFAS Parent Compound.	19

Text 3. Identification of TPs via non-target screening analysis.	19

Table 5. UPLC‒QTOF/MS analysis conditions for the identification of TPs.	20

References	21

Supplementary Information

## Table 1. Available performance and conditions for reactions of PFASs in related VUV185 reactors.

| PFAS | deC% (kros, min-1) | DeF% (VOF) | TOC% | Reaction conditions | Ref |
| --- | --- | --- | --- | --- | --- |
| PFOA | 62% (0.0018) | 27% | -- | pH 12, 480min, light sources(LS): 8.0 mW/cm2, closed quartz tubes, 40mL | [1] |
| PFOA | 99% (0.012) | 20% | -- | pH (original), 180min, SEN Light Corporation:20.73 mW/cm2, glass reactor, 1.5L | [2] |
| PFOA | 88% (0.0045) | 15% | -- | pH 10, 360min, Sankyo Electric Corporation, closed Pyrex glass reactor, 2L | [3] |
| PFOA | 80% (0.025) | 25% | -- | pH (original), 120min, Cnlight Corporation: 62–69 mW/cm2, tubular glass reactor,0.8L | [4] |
| PFOA | -- | 43.2% | -- | pH 5, 360min, Shanghai Jiguang Special Lighting Appliance Factory, China, cylindrical glass reactor, 250mL | [5] |
| PFOA | 61.7% (0.0558) | 17.1% | -- | pH 3.7, 120min, LS, tubular glass reactor, 800mL | [6] |
| C7 HFPO-TA | 58% (0.0021) | 27% | 30% | pH 12, 480min, LS: 8.0 mW/cm2, closed quartz tubes, 40mL | [1] |
| HFPO-DA | 40% (0.016) | 56% (40%) | -- | pH 6, 180min, Sankyo Electric Corporation:0.65mW/cm2, batch-type photoreactor, 1.2L | [7] |
| PFAS | 2%–100% (0-0.299) | 3–54% (0–26%) | 6–63% | pH(original), 180min, LS:0.095mW/cm2, stainless steel chamber photoreactor, dynamic irradiation, 0.237L | this article |
| PFAS | 100% (0.039–0.715) | 46−81% (4–15%) | 56−88% | pH(original), 80min, LS:0.095mW/cm2, stainless steel chamber photoreactor, CI, 0.237L | this article |
| PFAS | 100% (0.0738–0.914) | 100% (3.5–15%) | 87−100% | pH(original), 55min, LS: 0.13mW/cm2, VUV_CB,80 mL | this article |

## Table 2. The structure of PFASs and BDE (bond dissociation energy) of C–C bonds in PFAS molecule.

- PFOA

| Bond | BDE(kcal/mol) | Bond(left to right) | BDE(kcal/mol) |
| --- | --- | --- | --- |
| C–C 1 | 90.95004438 | C−F | 120.7730846 |
| C–C 2 | 86.87938701 | C−F | 113.1601333 |
| C–C 3 | 86.01342321 | C−F | 111.06927 |
| C–C 4 | 86.1759483 | C−F | 111.3503945 |
| C–C 5 | 86.85303159 | C−F | 110.6933915 |
| C–C 6 | 81.54115944 | C−F | 111.9195461 |
| C–C 7 | 87.8200245 | C−F | 106.6767 |

- GenX

| Bond | BDE(kcal/mol) | Bond(left to right) | BDE(kcal/mol) |
| --- | --- | --- | --- |
| C–C 1 | 90.29804382 | C−F | 121.3761218 |
| C–C 2 | 84.889112 | C−F | 111.5706505 |
| C–C 3 | 77.32740227 | C−F | 115.025093 |
| C–C 4 | 78.53810945 | C−F | 102.9430155 |
| C–C 5 | 88.90237690 | C−F | 123.2699469 |
| C–C 6 | 85.035892 |  |  |

- 6:2 FTS

| Bond | BDE(kcal/mol) | Bond(left to right) | BDE(kcal/mol) |
| --- | --- | --- | --- |
| C–C 1 | 90.50427831 | C−F | 120.43608612 |
| C–C 2 | 87.38421904 | C−F | 115.20951743 |
| C–C 3 | 83.01447206 | C−F | 107.59247012 |
| C–C 4 | 86.19985103 | C−F | 106.32857229 |
| C–C 5 | 86.893632 | C−F | 112.754209 |
| C–C 6 | 68.43205318 | C−F | 104.32188723 |
| C–C 7 | 71.724016 | C−H | 55.690352186 |
| C–S 8 | 70.24509445 | C−H | 57.903461 |

- PFOS

| Bond | BDE(kcal/mol) | Bond(left to right) | BDE(kcal/mol) |
| --- | --- | --- | --- |
| C–C 1 | 89.13465795 | C−F | 120.763672 |
| C–C 2 | 85.08470841 | C−F | 110.6419357 |
| C–C 3 | 84.26016027 | C−F | 109.3455 |
| C–C 4 | 84.56073756 | C−F | 110.6400532 |
| C–C 5 | 84.94665621 | C−F | 109.4383715 |
| C–C 6 | 84.64419639 | C−F | 111.1690441 |
| C–C 7 | 87.67130463 | C−F | 109.2865141 |
| C–S 8 | 64.73455911 | C−F | 112.2063181 |

## Table 3. The kors and DeF% of PFASs by VUV_CI/CB with various confined height.

| Confined height(mm) | 6:2FTS (kors/DeF%) | GenX (kors/DeF%) | PFOA (kors/DeF%) | PFOS (kors/DeF%) |
| --- | --- | --- | --- | --- |
| 4 | 0.384/67% | 0.113/71% | 0.085/69% | 0.012/38% |
| 5 | 0.531/73% | 0.192/78% | 0.108/81% | 0.033/55% |
| 6 | 0.609/82% | 0.236/89% | 0.122/90% | 0.0414/63% |
| 7 | 0.712/91% | 0.284/91% | 0.135/93% | 0.062/80% |
| 8 | 0.754/100% | 0.305/98% | 0.144/100% | 0.084/95% |
| 9 | 0.799/100% | 0.314/100% | 0.148/100% | 0.086/100% |
| 10 | 0.799/100% | 0.314/100% | 0.148/100% | 0.086/100% |
| 11 | 0.799/100% | 0.314/100% | 0.148/100% | 0.086/100% |
| 12 | 0.799/100% | 0.314/100% | 0.148/100% | 0.086/100% |
| 13 | 0.799/100% | 0.314/100% | 0.148/100% | 0.086/100% |
| 14 | 0.645/87% | 0.309/98% | 0.131/91% | 0.084/95% |
| 15 | 0.513/78% | 0.275/90% | 0.117/82% | 0.075/86% |
| 16 | 0.431/73% | 0.218/87% | 0.109/80% | 0.068/67% |
| 17 | 0.330/69% | 0.194/84% | 0.096/71% | 0.051/45% |
| 18 | 0.271/65% | 0.187/80% | 0.091/64% | 0.044/32% |
| 19 | 0.218/59% | 0.173/78% | 0.088/62% | 0.044/32% |
| 20 | 0.187/56% | 0.166/73% | 0.087/62% | 0.044/32% |
| 25 | 0.174/56% | 0.166/73% | 0.087/62% | 0.044/32% |
| 30 | 0.173/55% | 0.166/73% | 0.086/62% | 0.044/32% |
| 35 | 0.171/55% | 0.165/73% | 0.086/62% | 0.044/32% |
| 40 | 0.170/54% | 0.165/93% | 0.086/62% | 0.044/32% |

## Table 4. The chain reaction between the various free radicals.

|  | Reaction | Constant |
| --- | --- | --- |
| R1 | H2O+hv185nm→HO•+H• | Φ1=0.33 |
| R2 | H2O+hv185→HO•+H++eaq- | Φ2=0.045 |
| R3 | H2O2+hv185→2HO• | Φ3=0.5 |
| R4 | H2O2+hv254→2HO• | Φ4=0.5 |
| R5 | O2+H•→HO2• | k5=2.1×1010M-1s-1 |
| R6 | O2+eaq-→O2•- | k6=2.0×1010M-1s-1 |
| R7 | H2O2+HO•→HO2•+H2O | k7=2.7×107M-1s-1 |
| R8 | H2O2+HO2•→HO•+O2+H2O | k8=5.3×102M-1s-1 |
| R9 | H2O2+O2•-→HO•+O2+HO- | k9=1.6×101M-1s-1 |
| R10 | H2+HO•→H2O+H• | k10=6.0×107M-1s-1 |
| R11 | HO•+HO•→H2O2 | k11=4.0×109M-1s-1 |
| R12 | HO2•+HO2•→H2O2+O2 | k12=2.6×106M-1s-1 |
| R13 | H•+H•→H2 | k13=1.0×1010M-1s-1 |
| R14 | HO•+H•→H2O | k14=7.0×109M-1s-1 |
| R15 | HO•+O2•-→O2+HO- | k15=7.0×109M-1s-1 |
| R16 | HO•+HO2•→H2O+O2 | k16=6.6×109M-1s-1 |
| R17 | HO2•+O2•-→O2+HO2- | k17=9.7×107M-1s-1 |
| R18 | eaq-+HO•→HO- | k18=3.0×1010M-1s-1 |
| R19 | 2O2•-+2H2O→O2+H2O2+2HO- | k19=3.0×10-1M-1s-1 |
| R20 | eaq-+H•+H2O→H2+HO- | k20=2.5×1010M-1s-1 |
| R21 | HO•+HO2-→HO2•+HO- | k21=7.5×109M-1s-1 |
| R22 | HO•+HO-→O•-+H2O | k22=3.9×108M-1s-1 |
| R23 | H•+HO-→eaq-+H2O | k23=3.6×108M-1s-1 |
| R24 | H2O2+eaq-→HO•+HO- | k24=1.1×1010M-1s-1 |
| R25 | eaq-+H+→H• | k25=2.3×1010M-1s-1 |
| R26 | eaq-+H2O→H•+HO- | k26=1.9×101M-1s-1 |
| R27 | H•+HO2-→HO2•+HO- | k27=1.0×1010M-1s-1 |
| R28 | H•+H2O2→H2O+HO• | k28=9.0×107M-1s-1 |
| R29 | eaq-+HO2-+H2O→2HO-+HO• | k29=3.5×109M-1s-1 |
| R30 | H•+O2•-→HO2- | k30=2.0×1010M-1s-1 |
| R31 | H2O↔H++HO- | kH20=10-14 |
| R32 | H2O2↔H++HO2- | kH2O2=2.23×10-12 |

Figure S1. Time distributions of decay rate of 10 PFASs under VUV_DI, VUV_CI and VUV_CI/CB.

When CPFAS=10 mg L-1, the pH values of the samples were all at their initial values, and no buffering was performed.

Fig. 2. the EPR spectra of FRs in the VUV_CI.

EPR spectra of TEMPO trapped by a RFR with a reaction time(a) and (b), the OFR(·OH) response spectra of trapped by DMPO(c).

## Fig. 5 Identified representative transfer products(TPs) of PFASs under VUV_CI/CB irradiation.

Text 1. Detailed Information on Materials and Methods

The four PFCAs (PFPrA, PFHxA, PFOA, PFNA, ≥96%) and two PFECA (GenX and HFPO-TA, ≥96%) and 6:2 FTS (≥96%) were purchased from Macklin Reagent and ANPEL-TRACE Stard Technical Services Co., Ltd. (Shanghai, China). Three PFISA (LiTFSI, PFBS and PFOS) (≥96%), 4-Chlorobenzoic acid (pCBA, ≥99.0%) were obtained from Macklin Reagent Co., Ltd. Individual PFAS were dissolved in deionized water (produced by a Milli-Q system) as 10 mM stock solutions. All stock solutions were stored on benchtop at room temperature (20°C).

## Text 2. Measurement of PFAS Parent Compound.

The concentrations of individual PFAS during the reaction were measured using a high performance liquid chromatography coupled with a tripe-stage quadrupole mass spectrometer (HPLC-MS/MS, API3200; Applied Biosystems). The HPLC separation was performed with a HPLC BEH C18 column (2.1 mm × 50 mm, 1.7 μm) at 40 °C, A in-line filter column(2μm) and a Eclipse XDB-C18 protective column(4.6×12.5mm, 5μm ) should be placed before the column to filter out the micro-particle catalysts that may be present in the sample; In the detection process, the mobile phase needs to cross the mobile phase degassing device of high performance liquid chromatography, and the mobile phase was degassed by non-on-line degassing device; Mobile phase was a gradient composition of solvent A(ammonium acetate methanol solution, 10mM) and solvent B (ammonium acetate aqueous solution, 10 mM). The gradient expressed as the concentration of solvent A was as follows: 0−7.5 min, a linear increase from 60% to 100%; 7.5−11 min, held at 100% A; 11−16.5 min, a linear decrease from 100% to 75%; 16.5−17 min, hold at 60% A. The flow rate was 0.3 mL min−1. An electrospray negative ionization mode was used to identify the products in the liquid phase. The pressure of sheath gas (N2) was 0.4 MPa. The capillary potential was −4.5 kV. The source temperature was 120 °C, and the desolvation temperature was 450 °C. The analysis was carried out in multiple reaction monitoring (MRM) mode.

## Text 3. Identification of TPs via non-target screening analysis.

The non-targeted qualitative analysis data analyzed by LC-QTOF/MS went through the suspect screening process using UNIFI software (Waters, USA). Among the substances identified through the screening process, those that met all the following criteria were defined as TPs: (ⅰ) mass error < 10 ppm; (ⅱ) isotope match intensity RMS percent < 20%; and (ⅲ) peak area showing a continuous pattern over time, such as steady increase or a decrease after increase. After all, the confidence levels for the identified TPs were designated according to whether MSE fragmentation data was provided on UNIFI program.

## Table 5. UPLC‒QTOF/MS analysis conditions for the identification of TPs.

| Instrument |  | Parameter | Value |
| --- | --- | --- | --- |
| UPLC‒QTOF/MS | UPLC | Column | Waters, ACQUITY UPLC BEH C18 (2.1 × 100 mm, 1.7 µm) |
|  |  | Column temperature | 40 °C |
|  |  | Sample temperature | 10 °C |
|  |  | Injection volume | 10 µL |
|  |  | Flow rate | 0.3 mL/min |
|  |  | Mobile phase | (A) Deionized water with 2 mM ammonium acetate (B) Methanol with 2 mM ammonium acetate Time (min); Flow (mL/min); %A; %B; Curve. 0.0; 0.3; 100; 0; -. 0.5; 0.3; 80; 20; 6. 3.0; 0.3; 55; 45; 6. 6.0; 0.3; 20; 80; 6. 7.0; 0.3; 5; 95; 6. 8.0; 0.3; 5; 95; 6. 9.0; 0.3; 100; 0; 1. 10.0; 0.3; 100; 0; 1. |
|  | QTOF/MS | Ionization mode | ESI negative |
|  |  | Acquisition range | 50–1200 Da |
|  |  | Collision voltage | Low: 4 V / High: 20–70 V |
|  |  | Cone voltage | 40 V |
|  |  | Capillary voltage | 1.5 kV |
|  |  | Source temperature | 100 °C |
|  |  | Desolvation temperature | 250 °C |
|  |  | Cone gas flow | 0 L/h |
|  |  | Desolvation gas flow | 800 L/h |
|  |  | Nebuliser gas flow | 6.5 bar |

## References

(1)	Gu, M.; Liu, L.; Yu, G.; Huang, J. Deeper Defluorination and Mineralization of a Novel PFECA (C7 HFPO-TA) in Vacuum UV/Sulfite: Unique Mechanism of H/OCF 3 Exchange. Environ. Sci. Technol. 2023, 57 (40), 15288–15297.

(2)	Giri, R. R.; Ozaki, H.; Guo, X.; Takanami, R.; Taniguchi, S. Oxidative–Reductive Photodecomposition of Perfluorooctanoic Acid in Water. Int. J. Environ. Sci. Technol. 2014, 11 (5), 1277–1284.

(3)	Park, H.; Kim, T.; Kim, J.; Kim, M.-K.; Eom, S.; Choi, Y.; Zoh, K.-D. Reductive Degradation Mechanism of Perfluorooctanoic Acid (PFOA) during Vacuum Ultraviolet (VUV) Reactions Combining with Sulfite and Iodide. Chemosphere 2024, 348, 140759.

(4)	Cao, M. H.; Wang, B. B.; Yu, H. S.; Wang, L. L.; Yuan, S. H.; Chen, J. Photochemical Decomposition of Perfluorooctanoic Acid in Aqueous Periodate with VUV and UV Light Irradiation. J. Hazard Mater. 2010, 179 (1–3), 1143–1146.

(5)	Li, Y.; Lin, Q.; Li, C.; Weiguo, Z.; Xu, L.; Zhang, K. The Defluorination of Perfluorooctanoic Acid by Different Vacuum Ultraviolet Systems in the Solution. Water Environ. Res. 2021, 93 (3), 455–463.

(6)	Chen, J.; Zhang, P.; Liu, J. Photodegradation of Perfluorooctanoic Acid by 185 Nm Vacuum Ultraviolet Light. J Environ. Sci. 2007, 19 (4), 387–390.

(7)	Kim, J.; Kim, T.; Park, H.; Kim, M.-K.; Eom, S.; Choe, Y.; Choe, J. K.; Zoh, K.-D. Kinetics and Proposed Mechanisms of Hexafluoropropylene Oxide Dimer Acid (GenX) Degradation via Vacuum-UV (VUV) Photolysis and VUV/Sulfite Processes. J. Hazard Mater. 2024, 463, 132864.