Supplementary

### SMET model considerations

### Soil moisture sensor depth

The total amount of plant available water is the sum of the available water from all soil horizons occupied by the plant roots (Keller and Bliesner 1990), therefore for this method to work properly it is imperative that the soil moisture data be collected from the entire root zone. Typical root depths can be found in literature (e.g., FAO 56) but actual depth may vary from site to site and should therefore be locally checked(Keller and Bliesner 1990).

### Time scale

It is important to note that while the proposed SMET model calculates ETa on a daily basis, it does so only as an intermediate step to calculate cumulative values and it is not designed to work on a daily or even weekly time scale. The SMET model is shown to work on seasonal scale in this study, and although further investigation is required it is possible that it will work on a monthly scale. Additionally, it is important to note that since the algorithm calculates daily ETa values, the model cannot handle gaps that cannot be filled with reasonable estimates and therefore requires continuous soil moisture and ETr data to function properly.

### Potential challenges with drip irrigation

Due to the nature of drip irrigation scheduling being small but frequent, the SMET model might be skewed in favor of using the ETr term more often than the Δθ term because the days when Δθ>0 are more frequent than in a sprinkler irrigated field. This limitation can be mitigated by using a deep soil moisture sensor array that will not be overly sensitive to a small and shallow irrigation cycle, but further investigation is recommended.

### Potential challenges with flood irrigation

Flood irrigated systems can potentially incur in the opposite problem faced by drip irrigated systems, since the irrigation events are designed to saturate the root zone. Excessive deep percolation can introduce errors in the ETa estimation process due to large amounts of water moving through the profile, this was observed with a heavy irrigation event that occurred in the Vernal study site on June 26th, 2019, which created a large spike in the results before the outlier detection and correction algorithm was implemented. This issue can be mitigated by attempting to limit the saturation of the soil profile to the root zone and by spacing out the irrigation events to ensure that the soil is completely drained and well below field capacity before irrigation occurs.

Figure supplementary

Figure S1: Midnight soil moisture data for the 2019 and 2020 growing seasons in Vernal. The Left axis indicates the volumetric water content at the various soil sensor depth. The Right axis indicates the total soil water depth in the soil profile in mm.

Figure S2: Soil moisture timeseries for the East (top) and North (bottom) stations in the Modena study site for the 2021 growing season. The Left axis indicates the volumetric water content at the various soil sensor depth. The Right axis indicates the total soil water depth in the soil profile in mm.