Supplementary information

1. Isolate the cell-free, extracellular DNA (exDNA) by anion-exchange chromatography

(a)                                        (b)

Figure S1. The elution chromatograms of exDNA isolated and the control plasmid.

(a) Control chromatogram (λ=260 nm) with solution containing pure plasmid pMD20-T (9.15 ng µL-1, 1000 mL) and (b) Raw exDNA from an ACB filtered water sample (1000 mL) chromatogram (λ=260 nm). The recovery rate of the plasmid pMD20-T (control samples) was 92.49%.

- Calculation of pressure of inner wall and outer wall of the ACB

As shown in Fig. 1, UVC-SF is made up of an ACB cartridge and a build-in UVC-LED rod integrated at the central flow channel of the ACB. The size of the ACB filter cartridge is 248 mm×60 mm (Height × Outside diameter), and the Inside diameter is 30mm. The nominal filtration is 0.5 micron. If the inlet flow rate of the ACB filter is 0.5 L/min and flow direction is from inside to outside, the pressure on the inner wall of central flow channel of the ACB is 12.4 Pa under laminar flow conditions, which is 15.5 times higher than that of on the outside wall of the ACB with the flow direction from the outside to inside. The detailed calculation is described as follows:

2.1 Using the Hagen-Poiseulle equation for laminar flow in a cylindrical pipe:

Δ P = 8µLQ/πR4

where the Dynamic viscosity of water (µ) is 0.001 Pas; Flow channel length (L) is 0.248m; Flow rate (Q) is 0.5 L/min and 8.33 × 10-6 m3/s; the Radius (R) is 0.015m

Result: Δ P = 8 × 0.001 ×0.248 ×8.33 ×10-6 / π×(0.015)4 = 12.4 Pa

Thus, the pressure on the inner wall is 12.4 pa under laminar flow conditions.

2.2 The pressure on the outer wall of the central flow channel of the ACB cartridge (outside-to-inside, 0.5 L/mL)

For annular flow (outer diameter = 60mm, inner diameter =30mm), use the modified Hagen-Poiseulle equation: Δ P = 8µLQ/π( Ro4 - Ri4 )

Where the Outer radius (Ro) is 0.03m; Inner radius (Ri) is 0.015

Result:   Δ P = 8 × 0.001 ×0.248 ×8.33 ×10-6 / π×(0.034- 0.0154) = 0.8 Pa

Thus, the pressure on the outer wall is 0.8 Pa under laminar annular flow.

2.3 For the backwash experiment, the flow rate is 4 L/min, flow direction is from the outside to the inside, thus pressure on outer wall of the central flow channel is calculated as follows:

At 4 L/min, Flow transitions to turbulent (Re = 2,832). Use the Darcy-Weisbach equation:

ΔP = ƒ × Lρυ2/Dh2

Parameters: （ ρ:kg/m3）

Hydraulic diameter ( Dh ) =2 ( Ro - Ri ) = 0.03 m; Friction factor (ƒ) = 0.079 Re-0.25 = 0.0098;

Velocity (υ) = Q / Aannulus = 0.094 m/s

Result:  ΔP = 0.0098 × 0.248/0.03 × 1000×0.0942 / 2 = 34.7 Pa

Thus, the pressure on the outer wall is 34.7 Pa under turbulent flow.

2.4 For the backwash experiment, the flow rate is 4 L/min, flow direction is from the outside to the inside, thus pressure on inner wall of the central flow channel is calculated as follows:

Using Bernoulli’s principle and continuity equation:

P inner = P outer + 1/2ρ (υ2 outer - υ2 inner )

Parameters:

υ outer = 0.094 m/s (annular velocity)

P inner = P outer + 1/2ρ (υ2 outer - υ2 inner ) = 34.7 + 1/2x1000 × ( 0.0942 - 0.3142 ) = 79.9 Pa

Thus, the pressure on the inner wall is 79.9 Pa, reflecting energy conversion from velocity to pressure.

- Explicit Cost of the UVC-SF

We outsourced the UVC-LED rod, purchased PB1 ACB filter cartridge and assembled the UVC-SF system. Based on 5000 units assembled at a time, the explicit cost was described as follows:(1) UVC-LED rod: 160 yuan, $22.22; (2) 10-inch PB1 ACB filter cartridge: 70yuan, $9.72; (3) power supply and other parts: 76yuan, $10.56; Thus, total explicit cost were 22.22+9.72+10.56= $42.5 ≈ $40. Therefore, the explicit cost of the UVC-SF was about $40 per unit based on 5000 units assembled at a time.

- Counts of target bacteria treated by the ACB, UVC-LED, UVC-SF and Backwash

Table S1  Results of the treatment by ACB,UVC-LED, UVC-SF and Backwash (MPN/100mL)

|  | Inflow | ACB Outflow | UVC Outflow | UVC-SF Outflow | ACB IS | Backwash IS |
| --- | --- | --- | --- | --- | --- | --- |

____________________________________________________________________________

| ARBs TCF | 2284±448 | 1453±158 | 387.33±102 | 0.40±0.35 | 1234±533 | not detected |
| --- | --- | --- | --- | --- | --- | --- |
| LevR-TCF | 300±126 | 241±108 | 65.47±22.9 | 0 | 234.8±92.8 | not detected |
| AmpR-TCF | 890±333 | 760±272 | 295.67±95.8 | 1.2±0.84 | 513.9±173 | not detected |
| StrR-TCF | 363±114 | 291±67 | 98.90±22.3 | 0 | 253.7±99.8 | not detected |
| ARGs |  |  |  |  |  |  |
| TCF-DH5ɑ* | 148.5±57.7 | 108.3±8.6 | 27.6±9.8 | 13.3±3.0 | 301.9±86.1 | not detected |
| LevR-DH5ɑ | 15.8±3.8 | 13.32±5.7 | 3.99±1.7 | 0.32±1.5 | 27.10±8.6 | not detected |
| AmpR-DH5ɑ | 38.0±11.4 | 28.88±7.5 | 6.7±1.6 | 1.18±6.4 | 46.78±12.5 | not detected |
| StrR-DH5ɑ | 24.6±9.7 | 15.54±5.0 | 4.46±1.6 | 0.42±0.4 | 58.06±21.0 | not detected |

______________________________________________________________________________

* TCF-DH5 was the positive control.

- The Removal efficacy (RE%) and log reduction values (LRV) treated by the ACB, UVC-LED, UVC-SF and Backwash

Table S2   The RE %*and LRV**of the ACB, UVC-LED, UVC-SF and Backwash

|  | ACB |  | UVC-LED |  | UVC-SF |  | Backwash |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ARBs | RE acb% | LRV | RE uvc% | LRV | RE uvc-sf % | LRV | RE backwash % | LRV |
| TCF | 36.38% | 0.196 | 83.01% | 0.770 | 99.98% | 3.757 | 99.99% | 5.091 |
| StrR-TCF | 19.43% | 0.097 | 72.88% | 0.097 | 99.99% | 4.560 | 99.99% | 4.371 |
| AmpR-TCF | 14.60% | 0.068 | 66.80% | 0.479 | 99.86% | 2.870 | 99.99% | 4.711 |
| LevR-TCF | 19.67% | 0.093 | 78.04% | 0.658 | 99.99% | 4.477 | 99.99% | 4.404 |
| ARGs |  |  |  |  |  |  |  |  |
| TCF-DH5 | 27.07% | 0.137 | 81.61% | 0.731 | 91.01% | 1.048 | 99.99% | 4.480 |
| StrR-DH5 | 15.67% | 0.199 | 81.87% | 0.742 | 98.30% | 1.769 | 99.99% | 3.433 |
| AmpR-DH5 | 24.88% | 0.119 | 82.25% | 0.751 | 96.89% | 1.507 | 99.99% | 3.670 |
| LevR-DH5 | 36.83% | 0.074 | 74.76% | 0.598 | 97.94% | 1.688 | 99.99% | 3.764 |

* The Removal efficacy (RE %) was calculated as [C_in-C_out]/ C_in ×100%.

** The log reduction values (LRV) was calculated as log10[C_in/C_out