Supplementary Information

Red-emitted AIEgens-based lateral flow immunoassay for the visual and quantitative detection of aflatoxin M1 in milk

Wenbo Guo1, Qingyi Liu1, Junhua Yang1, Haijuan Zeng2, *, Juan Liu3, Lemei Zhu3, Dongxia Nie1, Zhihui Zhao1, Zheng Han1,*

1 Institute for Agro-food Standards and Testing Technology, Shanghai Key Laboratory of Protected Horticultural Technology, Shanghai Academy of Agricultural Sciences, Shanghai, China

2 The Biotechnology Research Institute, Shanghai Academy of Agricultural Sciences, Key Laboratory of Agricultural Genetics and Breeding, Shanghai Professional Technology Service Platform of Agricultural Biosafety Evaluation and Testing, Shanghai, China

3 School of Public Health, Academician Workstation, Changsha Medical University, Changsha, China

* To whom correspondence should be addressed.

E-mail address: zenghaijuan@saas.sh.cn; hanzheng@saas.sh.cn.

Contents

1. UHPLC-MS/MS analysis

2. Fig. S1 Visual inspection by the FIT and FIC changes with various volume of AIEgens-mAb

3. Fig. S2 Images of the AIEgens-LFIA with AFM1 and other mycotoxins under 365 nm UV excitation

4. Table S1 Detection of AFM1 by AIEgens-LFIA at different storage temperatures

5. Table S2 Comparison of the analysis results for the AFM1 in spiked samples by AIEgens-LFIA and UPLC-MS/MS.

6. Graphical Abstract

1. UHPLC-MS/MS analysis

The detection results of the developed aggregation-induced emission luminogens based lateral flow immunochromatographic assay (AIEgens-LFIA) were further confirmed by UPLC-MS/MS analysis. Chromatographic separation was performed on an ACQUITY UPLC BEH C18 column (2.1×100 mm, 1.7 µm). The mobile phase was consisted of methanol (A) and 5 mmol L−1 ammonium acetate (B). A linear gradient elution program was set as follows: initial 10% A; 1 min, 10% A; 5 min, 90% A; 6 min, 90% A; 6.5 min, 10% A; 8 min, 10% A. The flow rate was 0.4 mL min−1. The injection volume was 3 µL, and the column temperature was 35 °C.

Electrospray ionization was used in positive electrospray ionization mode (ESI+) with the following parameters: interface voltages of capillary, 2.5 kV; desolvation temperature, 500 ℃; and source temperature, 150 ℃. The gas flow rates were 7.0 bar for nebulizing gas and 1000 L h−1 for desolvation gas, respectively. Multiple reaction monitoring (MRM) mode was used for the quantification and the precursor/product transitions (m/z) were 329.1/273.1 and 329.1/229.2.

Fig. S1 Visual inspection by the FIT and FIC changes with various volume of AIEgens-mAb.

Fig. S2 Images of the AIEgens-LFIA with AFM1 and other mycotoxins under 365 nm UV excitation.

Table S1 Detection of AFM1 by AIEgens-LFIA at different storage temperatures.

| Periods of experimental (days) | 4 °C |  |  |  | 25 °C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FT a | FC | FT/FC |  | FT | FC | FT/FC |
| 0 | 6532 | 18624 | 0.35 |  | 8564 | 21546 | 0.40 |
| 1 | 7816 | 23017 | 0.34 |  | 6357 | 19654 | 0.32 |
| 3 | 6908 | 19354 | 0.36 |  | 7954 | 23516 | 0.34 |
| 7 | 5237 | 17008 | 0.31 |  | 5634 | 18645 | 0.30 |
| 15 | 8036 | 20347 | 0.39 |  | 6842 | 20019 | 0.34 |
| 30 | 7291 | 21368 | 0.34 |  | 8146 | 19658 | 0.41 |
| 60 | 5986 | 18624 | 0.32 |  | 6518 | 17359 | 0.38 |
| 90 | 7546 | 19534 | 0.39 |  | 8964 | 24618 | 0.36 |
| 180 | 8026 | 24038 | 0.33 |  | 9214 | 22835 | 0.40 |
| Mean ±SD b | 7042±970 | 20213±2246 | 0.35±0.03 |  | 7577±1273 | 20872±2406 | 0.36±0.04 |
| CV c（%） | 0.14 | 0.11 | 0.08 |  | 0.17 | 0.12 | 0.11 |

a FT represents the fluorescence intensity of the test line (AFM1, 0.5 ng/mL), and FC represents the fluorescence intensity of the control line (AFM1, 0.5 ng/mL). The FT/FC represents the detected results, and the storage temperatures are 4 °C and 25 °C.

b Standard deviation (SD) (n = 3).

c Coefficient of variation (CV).

Table S2 Comparison of the analysis results for the AFM1 in spiked samples by AIEgens-LFIA and UPLC-MS/MS.

| Spiked levels (ng/mL) | AIEgens-LFIA |  |  | UPLC-MS/MS |  |
| --- | --- | --- | --- | --- | --- |
|  | Mean (ng/mL) | Recovery±SD (%) |  | Mean (ng/mL) | Recovery±SD (%) |
| 0.1 | 0.098 | 97.53±13.73 |  | 0.107 | 107.37±14.52 |
| 0.5 | 0.512 | 102.47±8.44 |  | 0.482 | 96.47±8.44 |
| 1 | 1.041 | 104.13±6.87 |  | 1.021 | 102.07±5.19 |