Table 1. calculation of the uncertainty value of SPF of a non- extract Sintrong leave

| Uncertainty type | formula | calculation | result |
| --- | --- | --- | --- |
| - Uncertainty of weighing |  |  |  |
| - Resolution | μr= resolution/23 | μr= 0.0001/23 | 0.000029 |
| - Calibration | μcal= calibration3 | μcal= 0.153 | 0.00013 |
| - Combined μ | μw= μr2+ μcal2 | μc= 0.0000292+ 0.000132 | 0.00013 |
| - Uncertainty of volume |  |  |  |
| - Calibration | μcal= calibration3 | μcal= 0.043 | 0.023 |
| - Temperature effect | μT=V×T3×cp | μT=5×33×0.00021 | 0.0018 |
| - Combined μ | μv= μr2+ μcal2 | μc= 0.0232+0.00182 | 0.023 |
| - Uncertainty of spectrophotometer |  |  |  |
| - Resolution of spectrophotometer | μs= resolution/23 | μs= 0.001/23 | 0.00029 |
| - Uncertainty of repeatability | n = 10 | Data in S1 |  |
| - 100 ppm | μs= SDn | μs= 0.0026910 | 0.00085 |
| - 200 ppm | μs= SDn | μs= 0.0104710 | 0.00331 |
| - 300 ppm | μs= SDn | μs= 0.0345810 | 0.01093 |
| - 400 ppm | μs= SDn | μs= 0.0097710 | 0.00309 |
| - 500 ppm | μs= SDn | μs= 0.0108610 | 0.0034 |
| - 1000 ppm | μs= SDn | μs= 0.369410 | 0.1169 |
| - Combined uncertainty | μc= μw2+ μv2+ μs2+SD2 |  |  |
|  | μc= 0.000529+SD2 | μc= 0.000529+SD2 |  |
| - 100 ppm | μc= μw2+ μv2+ μs2+SD2 | μc= 0.000529+0.000852 | 0.0230 |
| - 200 ppm | μc= μw2+ μv2+ μs2+SD2 | μc= 0.000529+0.003312 | 0.0232 |
| - 300 ppm | μc= μw2+ μv2+ μs2+SD2 | μc= 0.000529+0.010932 | 0.0255 |
| - 400 ppm | μc= μw2+ μv2+ μs2+SD2 | μc= 0.000529+0.003092 | 0.0232 |
| - 500 ppm | μc= μw2+ μv2+ μs2+SD2 | μc= 0.000529+0.00342 | 0.0232 |
| - 1000 ppm | μc= μw2+ μv2+ μs2+SD2 | μc= 0.000529+0.11692 | 0.1191 |
| - Expanded uncertainty | μe=2× μc |  |  |
| - 100 ppm | μe=2× μc | μe=2×0.0230 | 0.046 |
| - 200 ppm | μe=2× μc | μe=2× 0.0232 | 0.0464 |
| - 300 ppm | μe=2× μc | μe=2× 0.0255 | 0.051 |
| - 400 ppm | μe=2× μc | μe=2× 0.0232 | 0.0464 |
| - 500 ppm | μe=2× μc | μe=2× 0.0232 | 0.0464 |
| - 1000 ppm | μe=2× μc | μe=2× 0.1191 | 0.2382 |

Note: μr= uncertainty of resolution, μcal = uncertainty of calibration, μT = uncertainty of temperature effect, μw= combined uncertainty of weighing, μv= combined uncertainty of volume, μs= combined uncertainty of spectrophotometer, μc=combined uncertainty, μe= expanded uncertainty, V = volume used, cp = coefficient of expansion, SD = standard deviation.