Supporting Information

Comparative Study of 52734-U-SupelMIP™ Solid-Phase and QuEChERS Extraction Methods for Monitoring 5-Nitroimidazoles in Albanian Honey

Kozeta Vaso*a, Ina Pashoa,b, Landi Dardhac, Esenina Strokad

aDepartment of Chemistry, University of Tirana, Tirana, Albania.

bNovartis Pharma Albania & Kosovo & North Macedonia, Tirana, Albania.

cDepartment of Chemical Sciences, Ariel University, Ari’el, Israel.

dDepartment of Mechanical and Industrial Engineering, University of Brescia, Brescia, Italy

*Corresponding author: Kozeta Vaso, kozeta.vaso@fshn.edu.al

Abbreviations:

Å – Angstrom (unit of length, used for pore size in chromatography)

ACS – American Chemical Society (for reagent grade)

ANOVA – Analysis of Variance

C18 – Octadecylsilane (used in chromatography)

CCD – Colony Collapse Disorder

CCα – Decision Limit

CCβ – Detection Capability

CE – Collision Energy

CIR – Commission Implementing Regulation

CITAC – Cooperation on International Traceability in Analytical Chemistry

DMSO – Dimethyl Sulfoxide

DMZ – Dimetridazole

EU – European Union

EU CIR 2021/808 – European Commission Implementing Regulation 2021/808

EURACHEM – European Network of Analytical Chemistry Laboratories

EURL – European Reference Laboratory

FAPAS – Food Analysis Performance Assessment Scheme

HCL – Highest Calibration Level

Hg – Mercury (used in vacuum/pressure units)

HRMS – High-Resolution Mass Spectrometry

IPZ – Ipronidazole

kPa – Kilopascal (pressure unit)

LCL – Lowest Calibration Level

LC-MS/MS – Liquid Chromatography-Tandem Mass Spectrometry

LOD – Limit of Detection

LOQ – Limit of Quantification

ME – Matrix Effect

MIPs – Molecularly Imprinted Polymers

MMPR – Minimum Method Performance Requirement

MNZ – Metronidazole

MS – Mass Spectrometry

MS1/MS2 – Mass Spectrometry Stages 1 and 2

MU – Measurement Uncertainty

NDZs – Nitroimidazoles

Orbitrap Exploris 480 – High-Resolution Mass Spectrometer

pKa – Acid Dissociation Constant

PMCID – PubMed Central Identifier

PMID – PubMed Identifier

QuEChERS – Quick, Easy, Cheap, Effective, Rugged, and Safe

R² – Coefficient of Determination (used in calibration curves)

RNZ – Ronidazole

RS – Resolution

RSD – Relative Standard Deviation

RSD% – Relative Standard Deviation Percentage

SE – Standard Error

SPE – Solid-Phase Extraction

SupelMIP™ SPE – SupelMIP™ Solid-Phase Extraction

TIC – Total Ion Chromatogram

Tukey Test – Tukey's Honest Significant Difference Test

Table S1. Eluent timing for the Agilent Technologies 6460 Triple Quadrupole LC/MS system

| Time (min) | 3.00 min | 7.00 min | 8.50 min | 11.00 min |
| --- | --- | --- | --- | --- |
| Eluent A (%) | 5.0 % | 5.0 % | 85.0 % | 85.0 % |
| Eluent B (%) | 95.0 % | 95.0 % | 15.0 % | 15.0 % |

Table S2. Accuracy, repeatability, within-lab reproducibility, and measurement uncertainty obtained for the studied extracted compounds in honey via 52734-U-SupelMIP columns.

| Compound | Spike level (μg/kg) | Trueness (%) | Repeatability |  |  | Within-laboratory reproducibility |  |  | MU (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mean (μg/kg) | SD | RSD (%) | Mean (μg/kg) | SD | RSD (%) |  |
| Metronidazole | 0.30 0.50 1.00 1.50 2.00 3.00 | 99.66 97.68 102.70 98.70 100.08 99.89 | 0.29 0.47 1.00 1.48 1.99 2.98 | 0.01 0.03 0.00 0.03 0.02 0.03 | 2.10 5.35 0.01 1.72 0.78 0.98 | 0.29 0.45 0.99 1.47 1.99 2.95 | 0.01 0.05 0.02 0.04 0.01 0.05 | 3.88 10.64 0.58 2.47 0.66 1.71 | 10.55 |
| Metronidazole-OH | 0.30 0.50 1.00 1.50 2.00 3.00 | 102.48 98.77 98.99 98.39 80.81 89.99 | 0.30 0.47 0.97 1.49 1.98 2.97 | 0.01 0.03 0.03 0.01 0.03 0.06 | 1.99 6.14 2.50 0.63 1.48 1.95 | 0.28 0.50 0.97 1.46 1.92 2.96 | 0.03 0.00 0.06 0.05 0.14 0.05 | 9.95 0.12 5.63 3.62 7.40 1.76 | 13.01 |
| Dimetridazole | 0.30 0.50 1.00 1.50 2.00 3.00 | 78.99 85.69 75.41 79.99 83.44 69.98 | 0.29 0.48 0.96 1.47 1.95 2.95 | 0.01 0.03 0.03 0.02 0.05 0.05 | 3.45 5.63 3.06 1.05 2.31 1.69 | 0.25 0.46 0.91 1.44 1.95 2.80 | 0.05 0.07 0.10 0.05 0.08 0.10 | 17.24 14.74 10.15 3.51 4.15 3.39 | 15.95 |
| HMMNI | 0.30 0.50 1.00 1.50 2.00 3.00 | 99.09 78.67 95.99 87.88 89.56 88.88 | 0.29 0.44 0.99 1.46 1.94 2.94 | 0.02 0.02 0.01 0.05 0.01 0.07 | 7.96 3.31 0.68 3.42 0.30 2.45 | 0.29 0.44 0.99 1.44 1.94 2.94 | 0.06 0.00 0.06 0.03 0.12 0.11 | 19.90 0.25 5.51 2.20 6.22 3.75 | 15.05 |
| Ipronidazole | 0.30 0.50 1.00 1.50 2.00 3.00 | 95.79 79.98 89.59 79.99 88.99 97.89 | 0.28 0.47 0.93 1.46 1.94 2.94 | 0.02 0.03 0.06 0.03 0.06 0.07 | 5.27 5.35 5.63 2.21 2.92 2.20 | 0.30 0.47 0.93 1.47 1.96 2.95 | 0.01 0.01 0.060.03 0.04 0.07 | 1.99 2.23 6.08 2.09 2.07 2.48 | 14.45 |
| Ronidazole | 0.30 0.50 1.00 1.50 2.00 3.00 | 101.11 87.38 95.65 97.77 99.04 87.93 | 0.30 0.48 0.93 1.48 1.96 2.85 | 0.00 0.03 0.05 0.03 0.06 0.26 | 0.00 6.02 5.20 2.30 3.26 8.80 | 0.28 0.48 0.96 1.49 1.97 2.96 | 0.03 0.02 0.06 0.02 0.05 0.08 | 9.95 4.52 6.35 1.58 2.37 2.54 | 13.99 |
| Tinidazole | 0.30 0.50 1.00 1.50 2.00 3.00 | 97.65 79.79 90.90 97.88 95.45 76.65 | 0.27 0.48 0.93 1.48 2.00 2.70 | 0.03 0.03 0.06 0.03 0.00 0.25 | 9.12 6.14 5.77 2.37 0.00 8.38 | 0.30 0.47 1.00 1.50 1.94 2.81 | 0.01 0.01 0.00 0.00 0.03 0.23 | 1.79 2.23 0.00 0.14 1.36 7.84 | 12.12 |

Table S3. FAPAS proficiency test results for 5-nitroimidazole analysis in blank honey samples.

| Analyte | Assigned Value (μg/kg) | This work results (μg/kg) | Z-score | Performance |
| --- | --- | --- | --- | --- |
| Metronidazole | 1.21 | 1.11 | -0.80 | Satisfactory |
| Metronidazole-OH | 0.77 | 0.90 | +1.04 | Satisfactory |
| Dimetridazole | 1.10 | 1.07 | -0.24 | Satisfactory |
| HMMNI | 0.50 | 0.49 | -0.08 | Satisfactory |
| Ipronidazole | 1.00 | 1.09 | +0.72 | Satisfactory |
| Ronidazole | 0.33 | 0.33 | 0.00 | Satisfactory |
| Tinidazole | 0.35 | 0.30 | -0.4 | Satisfactory |

Table S4. FAPAS proficiency test results for 5-nitroimidazole analysis in blank honey samples.

| Analyte | Assigned Value (μg/kg) | This work results (μg/kg) | Z-score | Performance |
| --- | --- | --- | --- | --- |
| Metronidazole | 1.21 | 1.05 | -1.28 | Satisfactory |
| Metronidazole-OH | 0.77 | 0.75 | -0.16 | Satisfactory |
| Dimetridazole | 1.10 | 1.15 | +0.40 | Satisfactory |
| HMMNI | 0.50 | 0.45 | -0.40 | Satisfactory |
| Ipronidazole | 1.00 | 0.97 | -0.24 | Satisfactory |
| Ronidazole | 0.33 | 0.40 | +0.56 | Satisfactory |
| Tinidazole | 0.35 | 0.31 | -0.32 | Satisfactory |

Table S5. The ion fragmentation of metronidazole and the metabolite metronidazole-OH from precursor ion to product ions through MS.

| Scan Segments: | tR (min) | Precursor ion: | MS1/MS2 Residue: | Product ion: | CE (V): |
| --- | --- | --- | --- | --- | --- |
| Metronidazole | ≃6.80 | 172 | Unit/Enh (6490) | 128 and 82 | 7 and 9 |
| Metronidazole-OH | ≃ 3.47 | 1.88 |  | 144 and 123 | 7 and 9 |
| Metronidazole - D3 | ≃6.80 | 175 | Unit/Enh (6490) | 131 | 9 |
| Metronidazole-D4 | ≃ 3.47 | 190 |  | 125 | 9 |

Table S6. The data values for all the positive samples regarding MNZ and MNZ-OH (extraction method: 52734-U-SupelMIP columns vs. QuEChERS salts).

| Sample | M11 | M12 | M6 |
| --- | --- | --- | --- |
| City | Përmet | Përmet | Leskovik |
| Analyte | MNZ | MNZ | MNZ-OH |
| Extraction method | 52734-U-SupelMIP columns |  |  |
| C (μg/kg) | 53 | 0.59 | 35 |
| SE (μg/kg) | ± 11.3 | ± 0.25 | ± 5.6 |
| Extraction method | QuEChERS salts |  |  |
| C (μg/kg) | 43 | 0.98 | 30 |
| SE (μg/kg) | ± 15.33 | ± 1.99 | ± 10.02 |

* SE (μg/kg) means the standard error measured within the same sample after testing it three times with the same detection principle.

Table S7. Descriptive Statistics between MNZ, DMZ and MNZ-OH variables for two different extraction methods.

|  | N | Missing | Mean | Standard Deviation | SE of mean | Lower 95% CI of Mean | Upper 95% CI of Mean |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MNZ | 7 | 0 | 0.90286 | 0.84263 | 0.31848 | 0.12356 | 1.68216 |
| MNZ-OH | 7 | 0 | 0.32071 | 0.28388 | 0.1073 | 0.05817 | 0.58326 |
| DMZ | 7 | 0 | 0.2 | 0.18824 | 0.07115 | 0.02591 | 0.37409 |
| MNZ | 7 | 0 | 0.90571 | 0.85011 | 0.32131 | 0.11949 | 1.69194 |
| MNZ-OH | 7 | 0 | 0.30071 | 0.27219 | 0.10288 | 0.04898 | 0.55245 |

| Quantiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mini-mum | 1st Quartile (Q1) | Median | 3rd Quartile (Q3) | Maxi-mum | Interquartile Range (Q3 - Q1) | Range (Maximum - Minimum) | Median Absolute Deviation | Robust Coefficient of Variation |
| MNZ | 0 | 0.18 | 0.74 | 1.54 | 2.36 | 1.36 | 2.36 | 0.56 | 1.12197 |
| MNZ-OH | 0 | 0.075 | 0.27 | 0.55 | 0.8 | 0.475 | 0.8 | 0.195 | 1.07077 |
| DMZ | 0 | 0.05 | 0.15 | 0.33 | 0.53 | 0.28 | 0.53 | 0.12 | 1.18608 |
| MNZ | 0 | 0.23 | 0.67 | 1.54 | 2.4 | 1.31 | 2.4 | 0.48 | 1.06216 |
| MNZ-OH | 0 | 0.075 | 0.2 | 0.55 | 0.75 | 0.475 | 0.75 | 0.18 | 1.33434 |

| Parameter Estimates |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Distribution | Parameter | Estimate | Lower 95% | Upper 95% | SD of Estimate |
| MNZ | Normal | Location mu | 0.90286 | 0.27864 | 1.52707 | 0.31848 |
|  |  | Scale sigma | 0.84263 | 0.46665 | 1.52154 | 0.25406 |
| MNZ-OH | Normal | Location mu | 0.32071 | 0.11042 | 0.53101 | 0.1073 |
|  |  | Scale sigma | 0.28388 | 0.15721 | 0.5126 | 0.08559 |
| DMZ | Normal | Location mu | 0.2 | 0.06055 | 0.33945 | 0.07115 |
|  |  | Scale sigma | 0.18824 | 0.10425 | 0.3399 | 0.05676 |
| MNZ | Normal | Location mu | 0.90571 | 0.27595 | 1.53548 | 0.32131 |
|  |  | Scale sigma | 0.85011 | 0.47079 | 1.53506 | 0.25632 |
| MNZ-OH | Normal | Location mu | 0.30071 | 0.09908 | 0.50235 | 0.10288 |
|  |  | Scale sigma | 0.27219 | 0.15074 | 0.49149 | 0.08207 |

(a)(b)

(c) (d)

(e)  (f)

(g)

Figure S1. The calibration curves for analytes (nitroimidazole extraction via 52734-U-SupelMIP columns): (a) metronidazole (MNZ), (b) Metronidazole-OH (MNZ-OH), (c) Dimetridazole (DMZ), (d) Hydroxy dimetridazole (HMMNI), (e) Ipronidazole, (f) Ronidazole, and (g) Tinidazole.

Figure S2. (a, b, c, d, e, f, and g) Extracted ion chromatograms for the investigated compounds, showing the absence of peaks in blank honey samples extracts (from both extraction methods) and (h) TIC chromatogram peaks of internal standards 5-nitroimidazoles at a concentration level of 1 μg/kg. (Analyzed using ChromeleonSoft).

(a)(b)

(c)(d)

(e)  (f)

(g)

Figure S3. The calibration curves for analytes (5-nitroimidazole extraction via QuEChERS salts): (a) metronidazole (MNZ), (b) Metronidazole-OH (MNZ-OH), (c) Dimetridazole (DMZ), (d) Hydroxy dimetridazole (HMMNI), (e) Ipronidazole, (f) Ronidazole, and (g) Tinidazole.

Figure S4. Total Ion Chromatography via LC-MS/MS after QuEChERS salt extraction process for the following samples out of the 24 testes samples: (a) MNZ peak for M11, (b) MNZ peak for M12 (tR(MNZ)= 6.80 min), and (c) MNZ-OH peak for M06(tR(MNZ-OH)= 3.47 min).

Figure S5. Total Ion Chromatography via LC-MS/MS after 52734-U-SupelMIP column extraction process for the following samples out of the 24 testes samples: (a) MNZ peak for M11, (b) MNZ peak for M12 (tR(MNZ)= 6.80 min), and (c) MNZ-OH peak for M06(tR(MNZ-OH)= 3.47 min).

Figure S6. Bar Normal Distribution chart for MNZ, DMZ and MNZ-OH variables for two different extraction methods.

Figure S7. (a) MNZ m/z values and (b) MNZ-OH m/z values and their precursor/product ions (control experiment) for the positive samples via Orbitrap Exploris 480 Mass Spectrometer (HRMS).