| Supplemental Table 2. MRI acquisition parameters using the Signa Artist 1.5T device |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T1WI | T2WI | 3D FLAIR | T2*WI |  | 3D MR hydrography (FIESTA) |  | 3D T1WI (SPGR) |
| Acuisition plane | 2D axial | 2D axial | 3D sagittal | 2D Axial |  | 3D Axial |  | 3D sagittal |
| TR (ms) | 600 | 4000 | 8000 | 850 |  | 5.3 |  | 2680 |
| TE (ms) | 10 | 100 | 120 | 25 |  | 2.2 |  | 3.2 |
| TI (ms) | NA | NA | 2160 | NA |  | NA |  | 875 |
| ETL | NA | 18 | 200 | NA |  | NA |  |  |
| Flip angle (degrees) | 73 | 90 | 90 | 25 |  | 60 |  | 12 |
| BW (kHz) | 62.5 | 62.5 | 41.67 | 12.5 |  | 111.11 |  | 31.25 |
| FOV (cm) | 24x19.2 | 24x19.2 | 25.6x23 | 24x19.2 |  | 20x20 |  | 25.6x23 |
| Matrix size | 288x256 | 416x320 | 256x256 | 288x224 |  | 288x288 |  | 256x256 |
| ST (mm) | 5 | 5 | 1.4 | 5 |  | 0.6 |  | 1 |
| Gap | 1 | 1 |  | 1 |  |  |  |  |
| NEX | 2 | 2 | 1 | 1 |  | 1 |  | 1 |
| AF | 2 | 2 | 4 | 2 |  | 1.5 |  | 2 |
| AT | 2min. 22sec. | 1min. 12sec. | 5min. 30sec. | 1min. 33sec. |  | 3min. 00sec. |  | 4min. 56sec. |
| Note. 3D MR hydrography, Three-dimensional magnetic resonance hydrography; FIESTA, Fast imaging employing steady-state acquisition; SPGR, Spoiled gradient recalled echo; TR, Repetition time; TE, Echo time; TI, Inversion time; ETL, Echo train length; BW, Band width; FOV, Field of view; ST, Slice thickness; NEX, Number of excitation; AF, Acceleration factor; AT, Acquisition time; NA, Not applicable. |  |  |  |  |  |  |  |  |