| Supplemental Table 1. MRI acquisition parameters using the Signa Premier 3T device |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T1WI | T2WI | 3D FLAIR | T2*WI |  | 3D MR hydrography (FIESTA) |  | 3D T1WI (SPGR) |
| Acuisition plane | 2D axial | 2D axial | 3D sagittal | 2D Axial |  | 3D Axial |  | 3D sagittal |
| TR (ms) | 600 | 4000 | 10000 | 610 |  | 4.8 |  | 2500 |
| TE (ms) | 9 | 90 | 120 | 20 |  | 1.9 |  | 2 |
| TI (ms) | NA | NA | 2540 | NA |  | NA |  | 1000 |
| ETL | NA | 19 | 220 | NA |  | NA |  | NA |
| Flip angle (degrees) | 73 | 90 | 90 | 20 |  | 60 |  | 8 |
| BW (kHz) | 83.3 | 83.3 | 41.67 | 41.67 |  | 111.11 |  | 62.5 |
| FOV (cm) | 24x19.2 | 24x19.2 | 24x24 | 24x19.2 |  | 18x18 |  | 24x24 |
| Matrix size | 320x256 | 512x320 | 256x256 | 384x288 |  | 352x352 |  | 256x256 |
| ST (mm) | 5 | 5 | 1.2 | 5 |  | 0.5 |  | 0.9 |
| Gap | 1 | 1 |  | 1 |  |  |  |  |
| NEX | 2 | 2 | 1 | 1 |  | 1 |  | 1 |
| AF | 2 | 2 | 6 | 2 |  | 2 |  | 2 |
| AT | 2min. 22sec. | 1min. 10sec. | 5min. 50sec. | 1min. 23sec. |  | 3min. 00sec. |  | 4min. 59sec. |
| Note. 3D MR hydrography, Three-dimensional magnetic resonance hydrography; FIESTA, Fast imaging employing steady-state acquisition; SPGR, Spoiled gradient recalled echo; TR, Repetition time; TE, Echo time; TI, Inversion time; ETL, Echo train length; BW, Band width; FOV, Field of view; ST, Slice thickness; NEX, Number of excitation; AF, Acceleration factor; AT, Acquisition time; NA, Not applicable. |  |  |  |  |  |  |  |  |