Supplementary Material

Supplementary Methods

Material S1：CT examination and image segmentation

All patients underwent contrast-enhanced CT within one month prior to nephrectomy. The CT scanning parameters are outlined below: tube voltage of 120 kV-140 kV; tube current of 250 mA-400 mA; and slice thickness of 1-5 mm. First, a flat-scan CT of the abdomen was performed (non-contrast phase (NCP)), and then contrast agent (1.5-2 ml/kg) was injected intravenously into the anterior elbow vein at an injection rate of 2.5-4 ml/s by means of a motorized syringe. The contrast agent (1.5-2 ml/kg) was then injected intravenously into the anterior elbow vein at an injection rate via a motorized syringe, and three contrast-enhanced CT scans were performed, which were acquired at different time points: i.e., 25-40 seconds after the corticomedullary phase (CMP), 50-90 seconds after the nephrographic phase (NCP), and 50-90 seconds after the medullary phase (NCP).The nephrographic phase (NP) was initiated after 50-90 seconds, and the excretory phase (EP) after 160-190 seconds. Due to the paucity of EP images in a substantial number of patients, NCP, CMP and NP images were selected for subsequent deep learning and radiomics analysis.

Material S2：Features selection and Radiomics

In order to assess the reproducibility of the radiomics features, 200 patients (from the entire study cohort) were randomly selected after one month, and the tumours were re-segmented using nnU-net, with the ROIs being calibrated by the same two radiologists who had calibrated the ROIs of the segmented tumours for the first time. Subsequently, the intragroup correlation coefficient (ICC) values for the analysis of intra- and inter-observer consistency were calculated based on the radiomic features extracted from the two segmentation ROIs. This was undertaken in order to assess consistency and reproducibility with respect to feature extraction. Features with ICC values greater than 0.85 were included in subsequent analyses. Inter-observer variation was defined as the difference between the results obtained by two or more observers performing the same ROI assay. Intra-observer variability refers to differences in measurements from one observer when more than one experiment is performed.

Material S3：Establishment of deep learning model

In the context of the CNN model, the batch size was configured to 32, with the SGD optimiser operating at a learning rate of 0.01. The Transformer model training involved the establishment of additional pertinent hyperparameters, which are outlined below: patch size: 64, hidden dimension: 1024, depth: 6, heads: 16, and MLP Hidden Dimension: 768. Stochastic online data augmentation (rotation, horizontal/vertical flipping) was applied during training. The training was performed on a workstation with the following specifications: Intel Core i7-14900 K 6.0 GHz CPU, 64GB DDR5 RAM, 2TB SSD, and NVIDIA RTX 4090D GPU with 24GB memory.

Supplementary Tables and Figures

Table S1 Tumor segmentation results requiring manual adjustment

|  | All patients (n=1499) | Training set (n=929) | Internal validation set (n=398) | External validation set (n=172) |
| --- | --- | --- | --- | --- |
| Without adjustment, n (%) | 1362 (90.86) | 847 (91.17) | 360 (90.45) | 155 (90.12) |
| With adjustment, n (%) | 137 (9.14) | 82 (8.83) | 38 (9.55) | 17 (9.88) |

Table S2 Types of extracted radiomics features

| Feature types | counts |
| --- | --- |
| First order statistics | 1080 |
| Gray-level co-occurrence matrix (GLCM) | 1320 |
| Gray-level dependence matrix (GLDM) | 840 |
| Gray-level run-length matrix (GLRLM) | 960 |
| Gray-level size-zone matrix (GLSZM) | 960 |
| Neighboring gray-tone difference matrix (NGTDM) | 300 |
| shape | 42 |

Table S3 Multivariable logistic regression results for the clinical variates

|  | Univariable |  |  | Multivariable |  |
| --- | --- | --- | --- | --- | --- |
| Variables | OR (95% CI) | p value |  | OR (95% CI) | p value |
| Sex | 0.402 (0.329-0.491) | ＜0.001 |  | 0.514 (0.393-0.673) | ＜0.001 |
| Age | 0.989 (0.987-0.991) | ＜0.001 |  | 0.958 (0.952-0.964) | ＜0.001 |
| Tumorsize | 0.996 (0.994-0.998) | ＜0.001 |  | 1.032 (1.024-1.040) | ＜0.001 |
| cTstage | 1.076 (0.991-1.169) | 0.142 |  | - | - |
| cNstage | 1.783 (1.162-2.737) | 0.026 |  | 1.727 (0.917-3.254) | 0.156 |
| cTNMstage | 0.840 (0.783-0.902) | ＜0.001 |  | 1.236 (0.944-1.619) | 0.197 |

Figure S1. LASSO feature selection results for radiomics models. (A-C) Rad-NCP model: LASSO coefficient paths (A), cross-validation curve (B), and selected feature coefficients (C). (D-F) Rad-CMP model: LASSO coefficient paths (D), cross-validation curve (E), and selected feature coefficients (F). (G-I) Rad-NP model: LASSO coefficient paths (G), cross-validation curve (H), and selected feature coefficients (I). (J-L) Rad-Triphasic model: LASSO coefficient paths (J), cross-validation curve (K), and selected feature coefficients (L). The Rad-Triphasic model incorporated 20 optimal features: 5 NCP features, 8 CMP features, and 7 NP features selected through LASSO regression with non-zero coefficients. The vertical dashed lines in coefficient paths indicate the optimal lambda value determined by cross-validation. LASSO, least absolute shrinkage and selection operator; Rad-NCP, radiomics non-contrast phase; Rad-CMP, radiomics corticomedullary phase; Rad-NP, radiomics nephrographic phase; MSE, mean squared error.