# III - SUPPLEMENTARY MATERIALS 
Methods

# Simulation

To confirm our hypotheses and compare the effects of carbidopa premedication on dynamic analysis in the same brain tumor of patients who underwent 18F-FDOPA PET both without and with premedication, we performed simulations. Simulated time activity curves of the carbidopa premedication status were computed, i.e. a simulated carbidopa-premedicated TAC for patients that were not premedicated.

This was performed using a compartmental model analysis. Dynamic PET images were reconstructed using 26 frames of 8 x 15 s, 2 x 30 s, 2 x 60 s, and 5 x 300 s [1]. Total blood TACs of patients having undergone 18F-FDOPA PET imaging without any premedication were extracted from VOIs placed into the internal carotid using initial dynamic frames to identify early vascular phases [2]. These blood TACs were then fitted to the peak using linear interpolation followed by a tri-exponential function after the peak [3]. Plasma 18F-FDOPA input functions were computed according to the methodology described by Wardak et al. [4].  The proportions of each of the 18F-labeled metabolites determined by Melega et al. [5] were used as they included the proportions for no premedication as well as for premedication with 100 mg of carbidopa (similarly to ours) (Supplemental Figure 1.A). The compartmental model used hereafter was a two-tissue compartmental model validated for compartmental analysis of 18F-FDOPA PET imaging in gliomas [4].

Raw TACs for healthy brain and tumors were extracted from the dynamic analysis of the patients that had undergone 18F-FDOPA PET imaging without any premedication. Compartmental models were fitted to these TACs using the input function to obtain the four rate constants (K1, k2, k3, k4) as well as the blood volume fraction (Vb) (Supplemental Figure 1.B). The carbidopa premedication status of healthy brain and tumor TACs were then simulated using the model rate constants and blood volume fractions obtained previously but using the plasma 18F-FDOPA input function corresponding to the carbidopa premedication (Supplemental Figure 1.C). For the simulation of the carbidopa premedication status of brain and tumor TACs, the four rate constants (K1, k2, k3, k4) were assumed to be unchanged compared to those calculated for the patients without any premedication, given that 18F-FDOPA K1 [6] and net influx rate constant Ki [7] are not affected by the premedication.

TTP and slope were extracted from healthy brain and tumor TACs for the patients as well as for the simulated TACs. A normalized version of these parameters was also extracted.

# References

1. Schiepers C, Chen W, Cloughesy T, Dahlbom M, Huang S-C. 18F-FDOPA Kinetics in Brain Tumors. J Nucl Med. 2007;48:1651–61.

2. Kratochwil C, Combs SE, Leotta K, Afshar-Oromieh A, Rieken S, Debus J, et al. Intra-individual comparison of 18F-FET and 18F-DOPA in PET imaging of recurrent brain tumors. Neuro-Oncology. 2014;16:434–40.

3. Zanotti-Fregonara P, Hirvonen J, Lyoo CH, Zoghbi SS, Rallis-Frutos D, Huestis MA, et al. Population-Based Input Function Modeling for [18F]FMPEP-d2, an Inverse Agonist Radioligand for Cannabinoid CB1 Receptors: Validation in Clinical Studies. PLOS ONE. 2013;8:7.

4. Wardak M, Schiepers C, Cloughesy TF, Dahlbom M, Phelps ME, Huang S-C. 18F-FLT and 18F-FDOPA PET Kinetics in Recurrent Brain Tumors. Eur J Nucl Med Mol Imaging. 2015;41:1199–209.

5. Melega WP, Hoffman JM, Luxen A, Nissenson CHK, Phelps ME, Barrio JR. The effects of carbidopa on the metabolism of 6-[18F]fluoro-l-dopa in rats, monkeys and humans. Life Sciences. 1990;47:149–57.

6. Huang S, Yu D, Grafton S, Melega WP, Hoffman JM. Kinetics and Modeling of L-6-esP]Pluoro-DOPA in Human Positron Emission Tomographic Studies. J Cereb Blood Flow Metab. 1991;11:898–913.

7. Hoffman JM, Meiega W, Hawk T, Grafton S. The Effectsof CarbidopaAdministration on 6-[18F]Fluoro-L-DOPAKinetics in Positron Emission Tomography. J Nucl Med. 1992;33:1472–7.

Supplemental Table. Brain tumor radiomic parameters extracted from patients without and with carbidopa premedication and after normalizing for healthy brain parameters with Tumor to normal Brain Ratios (TBR).

| Parameters | Without carbidopa premedication | With carbidopa premedication | Correla -tion coeffi -cient with SUVmean | p value | Mean relative difference in % (mean absolute difference) between patients without and with carbidopa premedication | Median relative difference in % (median absolute difference) between patients without and with carbidopa premedication | TBR Without carbidopa premedication | TBR With carbidopa premedication | TBR p value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Morphological features (pyradiomics) |  |  |  |  |  |  |  |  |  |
| Volume (mesh) | 19555,728 [3815,043; 28696,657] | 19904,328 [6750,762; 24252,304] | 0,38 | 0,622 | 1,8% (+348,6) | 36,8% (+4156,151) | - | - | - |
| Volume (counting) | 19883,864 [4047,951; 29061,235] | 20247,182 [7064,821; 24688,684] | 0,38 | 0,622 | 1,8% (+363,318) | 36,1% (+4162,516) | - | - | - |
| Surface area (mesh) | 4617,381 [1672,326; 6606,956] | 4817,326 [2465,068; 5691,911] | 0,35 | 0,634 | 4,3% (+199,945) | 10,3% (+364,02) | - | - | - |
| Surface to volume ratio | 0,399 [0,227; 0,495] | 0,341 [0,221; 0,436] | -0,42 | 0,969 | -14,6% (-0,058) | -3,1% (-0,01) | - | - | - |
| Sphericity | 0,706 [0,624; 0,787] | 0,682 [0,577; 0,775] | 0,05 | 0,746 | -3,4% (-0,024) | 3,8% (+0,027) | - | - | - |
| Maximum 3D diameter | 46,141 [29,732; 59,353] | 49,391 [41,857; 55,214] | 0,29 | 0,855 | 7% (+3,249) | 0,4% (+0,182) | - | - | - |
| Major axis length | 38,592 [24,433; 50,377] | 43,344 [33,651; 50,281] | 0,31 | 0,893 | 12,3% (+4,752) | 10,5% (+4,177) | - | - | - |
| Minor axis length | 27,998 [17,012; 39,139] | 31,613 [23,139; 38,207] | 0,38 | 0,9 | 12,9% (+3,614) | 16,5% (+4,383) | - | - | - |
| Least axis length | 21,013 [12,497; 28,247] | 21,936 [15,938; 27,383] | 0,37 | 0,587 | 4,4% (+0,922) | 13,1% (+2,547) | - | - | - |
| Elongation | 0,729 [0,628; 0,86] | 0,738 [0,665; 0,799] | 0,2 | 0,789 | 1,3% (+0,01) | -5,8% (-0,045) | - | - | - |
| Flatness | 0,55 [0,478; 0,667] | 0,509 [0,457; 0,603] | 0,08 | 0,267 | -7,4% (-0,041) | -4,3% (-0,024) | - | - | - |
| Local intensity features (in-house software) |  |  |  |  |  |  |  |  |  |
| Local intensity peak | 3,321 [1,975; 4,139] | 4,732 [3,56; 5,928] | 0,86 | 0,04 | 42,5% (+1,411) | 48,2% (+1,354) | 2,86 [1,784;3,476] | 2,732 [1,95;3,312] | 0,314 |
| Global intensity peak | 3,406 [2,03; 4,204] | 4,9 [3,61; 6,138] | 0,85 | 0,035 | 43,9% (+1,494) | 49,7% (+1,43) | 2,931 [1,845;3,495] | 2,83 [2,01;3,34] | 0,339 |
| Intensity-based statistical features (pyradiomics) |  |  |  |  |  |  |  |  |  |
| Mean intensity | 2,394 [1,925; 2,802] | 3,629 [3,135; 4,051] | 1 | < 0,001 | 51,6% (+1,235) | 48,7% (+1,159) | 2,051 [1,776;2,19] | 2,055 [1,854;2,148] | 0,45 |
| Intensity variance | 0,365 [0,024; 0,41] | 0,573 [0,101; 0,858] | 0,65 | 0,719 | 57,2% (+0,209) | 193,9% (+0,19) | 0,274 [0,017;0,31] | 0,218 [0,044;0,281] | 0,304 |
| Intensity skewness | 0,995 [0,65; 1,33] | 0,953 [0,788; 1,095] | 0,02 | 0,616 | -4,2% (-0,041) | -8,5% (-0,086) | 0,995 [0,65;1,33] | 0,953 [0,788;1,095] | 0,616 |
| (Excess) intensity kurtosis | 0,727 [-0,195; 1,387] | 0,336 [-0,127; 0,507] | -0,03 | 0,311 | -53,8% (-0,391) | -56,1% (-0,308) | 0,727 [-0,195;1,387] | 0,336 [-0,127;0,507] | 0,311 |
| Median intensity | 2,264 [1,896; 2,546] | 3,467 [2,954; 3,966] | 0,98 | < 0,001 | 53,2% (+1,203) | 49,4% (+1,137) | 1,938 [1,748;2,055] | 1,957 [1,802;2,075] | 0,579 |
| Minimum intensity | 1,877 [1,575; 2,134] | 2,868 [2,435; 3,352] | 0,83 | < 0,001 | 52,7% (+0,99) | 44,3% (+0,845) | 1,602 [1,6;1,601] | 1,601 [1,6;1,601] | 0,847 |
| 10th intensity percentile | 1,936 [1,685; 2,204] | 2,955 [2,566; 3,404] | 0,86 | < 0,001 | 52,6% (+1,019) | 44,8% (+0,895) | 1,653 [1,627;1,676] | 1,653 [1,629;1,675] | 0,452 |
| 90th intensity percentile | 3,074 [2,147; 3,631] | 4,585 [3,7; 5,44] | 0,94 | < 0,001 | 49,2% (+1,511) | 61% (+1,698) | 2,642 [1,972;3,089] | 2,628 [2,187;2,944] | 0,406 |
| Maximum intensity | 4,047 [2,548; 5,113] | 5,745 [4,067; 7,131] | 0,84 | 0,032 | 41,9% (+1,698) | 43,3% (+1,555) | 3,504 [2,144;4,255] | 3,328 [2,418;4,062] | 0,313 |
| Intensity interquartile range | 0,587 [0,223; 0,787] | 0,884 [0,451; 1,204] | 0,67 | 0,297 | 50,6% (+0,297) | 84,4% (+0,362) | 0,512 [0,189;0,699] | 0,528 [0,328;0,618] | 0,498 |
| Intensity range | 2,17 [0,684; 3,399] | 2,878 [1,356; 4,239] | 0,6 | 0,642 | 32,6% (+0,708) | 59,5% (+0,946) | 1,902 [0,543;2,654] | 1,727 [0,817;2,457] | 0,313 |
| Intensity mean absolute deviation | 0,364 [0,127; 0,505] | 0,517 [0,259; 0,74] | 0,64 | 0,446 | 42,1% (+0,153) | 79,6% (+0,196) | 0,316 [0,107;0,449] | 0,309 [0,173;0,406] | 0,394 |
| Intensity-based robust mean absolute deviation | 0,25 [0,094; 0,336] | 0,369 [0,186; 0,503] | 0,65 | 0,348 | 47,8% (+0,119) | 81,5% (+0,148) | 0,218 [0,079;0,296] | 0,221 [0,126;0,267] | 0,456 |
| Intensity-based energy | 8077,473 [731,18; 11772,415] | 16067,135 [2685,699; 16389,009] | 0,68 | 0,245 | 98,9% (+7989,662) | 166,2% (+7279,404) | 6169,39 [736,93;8000,318] | 6268,353 [1320,586;5923,271] | 0,602 |
| Root mean square intensity | 2,455 [1,935; 2,877] | 3,701 [3,151; 4,123] | 1 | < 0,001 | 50,7% (+1,246) | 55,5% (+1,33) | 2,105 [1,781;2,286] | 2,1 [1,866;2,211] | 0,419 |
| Intensity histogram features (pyradiomics) |  |  |  |  |  |  |  |  |  |
| Discretized intensity entropy | 3,35 [2,437; 4,188] | 4,027 [3,378; 4,774] | 0,66 | 0,293 | 20,2% (+0,677) | 26,1% (+0,875) | 3,04 [2,173;3,897] | 3,182 [2,687;3,849] | 0,589 |
| Discretized intensity uniformity | 0,157 [0,07; 0,21] | 0,093 [0,046; 0,113] | -0,69 | 0,274 | -40,6% (-0,064) | -43% (-0,049) | 0,208 [0,087;0,252] | 0,186 [0,089;0,186] | 0,672 |
| Grey level co-occurrence based features (pyradiomics) |  |  |  |  |  |  |  |  |  |
| Joint maximum | 0,071 [0,013; 0,088] | 0,033 [0,008; 0,028] | -0,71 | 0,333 | -53,1% (-0,038) | -64,5% (-0,024) | 0,111 [0,02;0,104] | 0,106 [0,02;0,065] | 0,571 |
| Joint average | 25,289 [20,141; 29,952] | 38,029 [32,532; 42,384] | 1 | < 0,001 | 50,4% (+12,74) | 51% (+12,724) | 21,766 [18,683;23,625] | 21,801 [19,698;23,009] | 0,448 |
| Joint variance | 38,384 [2,864; 43,91] | 59,455 [10,817; 86,722] | 0,64 | 0,729 | 54,9% (+21,071) | 148,8% (+16,66) | 28,668 [1,895;33,403] | 22,51 [4,753;31,218] | 0,293 |
| Joint entropy | 6,614 [5,124; 8,037] | 7,964 [6,807; 9,357] | 0,69 | 0,229 | 20,4% (+1,35) | 23,7% (+1,566) | 6,079 [4,629;7,851] | 6,347 [5,646;7,637] | 0,598 |
| Difference average | 2,749 [1,446; 3,481] | 4,098 [2,491; 5,506] | 0,7 | 0,092 | 49,1% (+1,349) | 87,1% (+1,929) | 2,349 [1,176;3,123] | 2,374 [1,656;3,124] | 0,513 |
| Difference variance | 8,138 [1,496; 9,556] | 14,893 [4,46; 22,252] | 0,69 | 0,207 | 83% (+6,755) | 146,7% (+6,248) | 6,143 [0,994;8,109] | 5,546 [2,064;7,533] | 0,414 |
| Difference entropy | 2,773 [2,188; 3,337] | 3,364 [2,878; 3,944] | 0,7 | 0,122 | 21,3% (+0,591) | 27,4% (+0,753) | 2,582 [1,94;3,2] | 2,644 [2,37;3,192] | 0,563 |
| Sum average | 50,578 [40,281; 59,904] | 76,057 [65,064; 84,768] | 1 | < 0,001 | 50,4% (+25,479) | 51% (+25,449) | 43,532 [37,367;47,25] | 43,601 [39,397;46,018] | 0,448 |
| Sum entropy | 4,297 [3,374; 5,318] | 5,042 [4,39; 5,874] | 0,66 | 0,3 | 17,4% (+0,746) | 19,9% (+0,86) | 4,023 [3,068;5,117] | 4,18 [3,717;4,981] | 0,603 |
| Angular second moment | 0,038 [0,006; 0,034] | 0,014 [0,002; 0,012] | -0,7 | 0,298 | -64,2% (-0,024) | -65,6% (-0,009) | 0,064 [0,007;0,05] | 0,062 [0,009;0,026] | 0,57 |
| Contrast | 18,611 [3,588; 22,888] | 35,613 [10,67; 52,917] | 0,69 | 0,135 | 91,4% (+17,002) | 211,6% (+19,236) | 13,951 [2,378;17,657] | 12,897 [4,83;17,293] | 0,455 |
| Dissimilarity | 2,749 [1,446; 3,481] | 4,098 [2,491; 5,506] | 0,7 | 0,092 | 49,1% (+1,349) | 87,1% (+1,929) | 2,349 [1,176;3,123] | 2,374 [1,656;3,124] | 0,513 |
| Inverse difference | 0,447 [0,347; 0,527] | 0,354 [0,27; 0,414] | -0,7 | 0,111 | -20,7% (-0,092) | -28,4% (-0,126) | 0,488 [0,376;0,574] | 0,476 [0,382;0,508] | 0,578 |
| Normalized inverse difference | 0,943 [0,934; 0,955] | 0,939 [0,929; 0,942] | -0,32 | 0,86 | -0,4% (-0,004) | -0,5% (-0,005) | 0,945 [0,935;0,956] | 0,942 [0,931;0,943] | 0,958 |
| Inverse difference moment | 0,38 [0,257; 0,471] | 0,273 [0,18; 0,334] | -0,71 | 0,12 | -28,1% (-0,107) | -38% (-0,142) | 0,427 [0,292;0,53] | 0,41 [0,298;0,448] | 0,612 |
| Normalized inverse difference moment | 0,993 [0,991; 0,996] | 0,992 [0,99; 0,993] | -0,34 | 0,911 | -0,1% (-0,001) | -0,1% (-0,001) | 0,993 [0,991;0,996] | 0,992 [0,99;0,993] | 0,976 |
| Inverse variance | 0,347 [0,252; 0,427] | 0,265 [0,19; 0,337] | -0,7 | 0,119 | -23,6% (-0,082) | -36% (-0,13) | 0,351 [0,283;0,448] | 0,33 [0,287;0,364] | 0,877 |
| Correlation | 0,504 [0,258; 0,748] | 0,564 [0,443; 0,721] | 0,4 | 0,875 | 11,9% (+0,06) | 1,4% (+0,008) | 0,496 [0,245;0,749] | 0,557 [0,436;0,72] | 0,859 |
| Autocorrelation | 704,275 [414,868; 909,01] | 1546,75 [1060,297; 1852,55] | 0,99 | < 0,001 | 119,6% (+842,475) | 143,7% (+899,568) | 511,825 [349,533;585,331] | 502,712 [389,283;550,573] | 0,4 |
| Cluster tendency | 134,925 [7,141; 153,555] | 202,207 [32,374; 304,516] | 0,63 | 0,832 | 49,9% (+67,282) | 165,1% (+57,759) | 100,723 [4,701;110,419] | 77,142 [12,034;105,569] | 0,28 |
| Cluster shade | 3521,348 [6,366; 1980,427] | 3618,007 [99,964; 5143,127] | 0,59 | 0,543 | 2,7% (+96,659) | 150,7% (+326,471) | 2136,409 [6,038;1160,87] | 947,036 [20,11;1097,266] | 0,193 |
| Cluster prominence | 291266,819 [131,49; 85341,388] | 296546,876 [2738,921; 288079,153] | 0,61 | 0,497 | 1,8% (+5280,057) | 342,8% (+19812,477) | 148533,26 [61,508;42900,073] | 52040,959 [385,659;40989,81] | 0,199 |
| Information correlation 1 | -0,082 [-0,117; -0,035] | -0,082 [-0,109; -0,055] | -0,34 | 0,552 | 0,1% (+0) | 3% (+0,003) | -0,084 [-0,123;-0,038] | -0,091 [-0,119;-0,063] | 0,809 |
| Information correlation 2 | 0,603 [0,352; 0,814] | 0,672 [0,553; 0,811] | 0,44 | 0,877 | 11,4% (+0,069) | 4,6% (+0,032) | 0,583 [0,342;0,814] | 0,636 [0,521;0,788] | 0,855 |
| Grey level run length based features (pyradiomics) |  |  |  |  |  |  |  |  |  |
| Short runs emphasis | 0,913 [0,898; 0,943] | 0,94 [0,931; 0,964] | 0,7 | 0,074 | 2,9% (+0,027) | 3,5% (+0,032) | 0,893 [0,866;0,925] | 0,887 [0,89;0,927] | 0,295 |
| Long runs emphasis | 1,433 [1,252; 1,541] | 1,288 [1,169; 1,312] | -0,68 | 0,079 | -10,1% (-0,145) | -11,2% (-0,158) | 1,557 [1,386;1,698] | 1,668 [1,378;1,553] | 0,143 |
| Low grey level run emphasis | 0,002 [0,001; 0,003] | 0,001 [0,001; 0,001] | -0,98 | < 0,001 | -56,2% (-0,00119) | -49,5% (-0,00086) | 0,0026 [0,00227;0,00303] | 0,00255 [0,0023;0,00283] | 0,56 |
| High grey level run emphasis | 667,425 [399,413; 852,888] | 1466,873 [1019,969; 1746,618] | 1 | < 0,001 | 119,8% (+799,448) | 139% (+832,077) | 486,658 [336,999;568,244] | 477,197 [369,014;518,004] | 0,398 |
| Short run low grey level emphasis | 0,002 [0,001; 0,002] | 0,001 [0,001; 0,001] | -0,97 | < 0,001 | -54,6% (-0,00105) | -47,7% (-0,00076) | 0,0023 [0,00208;0,00264] | 0,00223 [0,00212;0,00245] | 0,81 |
| Short run high grey level emphasis | 619,305 [364,109; 801,69] | 1388,529 [913,955; 1674,421] | 0,99 | < 0,001 | 124,2% (+769,225) | 150,7% (+826,016) | 443,66 [290,026;517,924] | 431,755 [330,21;484,13] | 0,371 |
| Long run low grey level emphasis | 0,003 [0,002; 0,004] | 0,001 [0,001; 0,001] | -0,98 | < 0,001 | -61,5% (-0,00195) | -55,2% (-0,00135) | 0,0042 [0,00304;0,00515] | 0,0045 [0,00323;0,00447] | 0,212 |
| Long run high grey level emphasis | 901,439 [592,763; 1073,944] | 1836,455 [1372,391; 2279,344] | 0,98 | < 0,001 | 103,7% (+935,016) | 103,9% (+912,072) | 705,592 [562,172;775,252] | 733,045 [585,448;846,737] | 0,93 |
| Grey level non-uniformity | 956,211 [336,192; 1225,027] | 704,603 [381,929; 955,102] | 0,15 | 0,28 | -26,3% (-251,608) | 6% (+38,535) | 1132,883 [361,291;1549,98] | 1119,703 [677,965;1493,348] | 0,761 |
| Normalized grey level non-uniformity | 0,153 [0,067; 0,211] | 0,091 [0,046; 0,113] | -0,68 | 0,275 | -40,4% (-0,062) | -43,6% (-0,049) | 0,202 [0,087;0,252] | 0,178 [0,087;0,183] | 0,709 |
| Run length non-uniformity | 9934,876 [1985,378; 16236,51] | 11323,826 [3832,499; 13270,741] | 0,43 | 0,768 | 14% (+1388,95) | 32,6% (+1968,027) | 9475,703 [1837,784;14936,156] | 10025,124 [3103,605;11700,03] | 0,627 |
| Normalized run length non-uniformity | 0,801 [0,765; 0,859] | 0,857 [0,834; 0,91] | 0,71 | 0,062 | 7% (+0,056) | 8,6% (+0,069) | 0,763 [0,706;0,823] | 0,757 [0,75;0,828] | 0,371 |
| Run percentage | 0,887 [0,862; 0,926] | 0,92 [0,911; 0,95] | 0,7 | 0,064 | 3,7% (+0,033) | 4,8% (+0,042) | 0,862 [0,823;0,897] | 0,853 [0,858;0,899] | 0,268 |
| Grey level variance | 37,036 [2,567; 41,559] | 57,474 [10,18; 85,898] | 0,64 | 0,739 | 55,2% (+20,437) | 183,9% (+18,611) | 27,794 [1,716;31,388] | 21,742 [4,264;28,785] | 0,297 |
| Run length variance | 0,152 [0,087; 0,193] | 0,101 [0,061; 0,112] | -0,65 | 0,074 | -33,5% (-0,051) | -28,7% (-0,036) | 0,197 [0,14;0,244] | 0,244 [0,134;0,194] | 0,114 |
| Run entropy | 3,917 [3,006; 4,772] | 4,455 [3,854; 5,081] | 0,62 | 0,412 | 13,7% (+0,538) | 12,9% (+0,529) | 3,712 [2,883;4,558] | 3,886 [3,311;4,39] | 0,706 |
| Grey level size zone based features (pyradiomics) |  |  |  |  |  |  |  |  |  |
| Small zone emphasis | 0,497 [0,43; 0,587] | 0,561 [0,489; 0,658] | 0,57 | 0,355 | 12,9% (+0,064) | 8,8% (+0,046) | 0.506 [0.39;0.55] | 0.49 [0.36;0.50] | 0.634 |
| Large zone emphasis | 434,516 [26,192; 246,014] | 65,513 [9,476; 59,153] | -0,59 | 0,332 | -84,9% (-369,003) | -69,1% (-52,862) | 562.738 [62.62;820.51] | 732.286 [88.41;541.77] | 0.554 |
| Low grey level emphasis | 0,002 [0,001; 0,003] | 0,001 [0,001; 0,001] | -0,98 | < 0,001 | -55,4% (-0,00112) | -47% (-0,00077) | 0.002 [0.002;0.003] | 0.002 [.002;0.003] | 0.484 |
| High grey level emphasis | 723,418 [409,849; 862,442] | 1511,306 [996,965; 1871,249] | 0,98 | < 0,001 | 108,9% (+787,888) | 136,8% (+871,143) | 0,433 [0,386;0,549] | 0,441 [0,361;0,503] | 0,634 |
| Small zone low grey level emphasis | 0,001 [0,001; 0,001] | 0 [0; 0,001] | -0,83 | < 0,001 | -49,4% (-0,00048) | -42,9% (-0,00035) | 987,199 [62,62;820,508] | 1308,393 [88,409;541,767] | 0,554 |
| Small zone high grey level emphasis | 393,079 [190,251; 480,538] | 898,586 [480,173; 1262,765] | 0,92 | < 0,001 | 128,6% (+505,507) | 180% (+560,877) | 0,00234 [0,00193;0,00278] | 0,00234 [0,00195;0,00263] | 0,484 |
| Large zone low grey level emphasis | 1,427 [0,039; 0,662] | 0,068 [0,006; 0,072] | -0,74 | 0,386 | -95,3% (-1,359) | -84,4% (-0,119) | 558,827 [371,432;632,795] | 525,449 [397,914;624,557] | 0,336 |
| Large zone high grey level emphasis | 166847,021 [17237,612; 108724,768] | 75776,413 [16703,992; 50415,652] | -0,3 | 0,565 | -54,6% (-91070,608) | -42% (-16146,086) | 0,00094 [0,00084;0,00106] | 0,00099 [0,00088;0,00111] | 0,738 |
| Grey level non-uniformity | 13,607 [5,616; 23,489] | 17,948 [9,998; 23,964] | 0,5 | 0,904 | 31,9% (+4,341) | 30,5% (+3,085) | 274,633 [146,496;349,564] | 248,829 [127,16;295,795] | 0,329 |
| Grey level non-uniformity normalized | 0,142 [0,054; 0,203] | 0,086 [0,038; 0,109] | -0,63 | 0,32 | -39,3% (-0,056) | -37,6% (-0,036) | 3,072 [0,169;2,321] | 4,161 [0,257;1,628] | 0,534 |
| Zone size non-uniformity | 89,604 [4,228; 142,194] | 173,591 [25,136; 165,397] | 0,62 | 0,519 | 93,7% (+83,987) | 173,3% (+53,014) | 326076,02 [32518,004;276018,051] | 423113,96 [30860,905;251122,786] | 0,57 |
| Zone size non-uniformity normalized | 0,281 [0,214; 0,34] | 0,317 [0,235; 0,405] | 0,47 | 0,422 | 12,9% (+0,036) | 14,3% (+0,039) | 10,535 [3,569;16,765] | 13,498 [4,545;14,703] | 0,855 |
| Zone percentage | 0,238 [0,128; 0,314] | 0,355 [0,237; 0,508] | 0,66 | 0,031 | 49,4% (+0,117) | 57,1% (+0,127) | 0,149 [0,059;0,198] | 0,153 [0,059;0,16] | 0,418 |
| Grey level variance | 43,334 [3,927; 52,855] | 59,313 [10,825; 86,903] | 0,6 | 0,925 | 36,9% (+15,979) | 101,2% (+15,681) | 64,609 [2,237;64,34] | 78,916 [4,03;47,421] | 0,748 |
| Zone size variance | 373,77 [16,242; 188,356] | 46,277 [5,141; 41,244] | -0,54 | 0,339 | -87,6% (-327,493) | -70,6% (-34,028) | 0,263 [0,192;0,318] | 0,227 [0,183;0,268] | 0,203 |
| Zone size entropy | 5,082 [4,011; 6,421] | 5,774 [5,279; 6,468] | 0,56 | 0,546 | 13,6% (+0,692) | 11% (+0,601) | 0,169 [0,077;0,235] | 0,162 [0,084;0,212] | 0,413 |
| Neighborhood grey tone difference based features (pyradiomics) |  |  |  |  |  |  |  |  |  |
| Coarseness | 0,045 [0,005; 0,023] | 0,014 [0,006; 0,015] | -0,39 | 0,507 | -70% (-0,032) | -27,5% (-0,004) | 0,049 [0,005;0,021] | 0,015 [0,006;0,014] | 0,67 |
| Contrast | 0,155 [0,082; 0,182] | 0,234 [0,132; 0,33] | 0,64 | 0,047 | 50,8% (+0,079) | 81,5% (+0,098) | 0,139 [0,076;0,167] | 0,132 [0,092;0,167] | 0,537 |
| Busyness | 0,315 [0,17; 0,352] | 0,185 [0,098; 0,278] | -0,47 | 0,057 | -41,4% (-0,131) | -32,8% (-0,081) | 0,444 [0,276;0,449] | 0,524 [0,341;0,601] | 0,608 |
| Complexity | 1125,952 [30,359; 1092,577] | 1890,278 [170,467; 2466,181] | 0,65 | 0,76 | 67,9% (+764,326) | 168,4% (+368,217) | 745,484 [15,304;668,505] | 500,767 [48,524;617,939] | 0,303 |
| Strength | 4,277 [0,765; 4,805] | 5,666 [1,402; 9,811] | 0,53 | 0,671 | 32,5% (+1,389) | 42,8% (+1,049) | 3,21 [0,576;3,034] | 2,023 [0,674;3,14] | 0,202 |
| Neighboring grey level dependence based on features (pyradiomics) |  |  |  |  |  |  |  |  |  |
| Low dependence emphasis | 0,203 [0,125; 0,27] | 0,3 [0,208; 0,425] | 0,69 | 0,03 | 48,3% (+0,098) | 54,7% (+0,102) | 0,151 [0,078;0,21] | 0,152 [0,096;0,196] | 0,483 |
| High dependence emphasis | 20,758 [10,646; 26,042] | 13,169 [6,94; 13,534] | -0,65 | 0,124 | -36,6% (-7,589) | -44% (-8,154) | 27,065 [17,067;32,962] | 33,693 [16,508;26,269] | 0,15 |
| Low grey level count emphasis | 0,002 [0,001; 0,003] | 0,001 [0,001; 0,001] | -0,98 | < 0,001 | -56,2% (-0,00119) | -49,8% (-0,00087) | 0,00261 [0,00229;0,00302] | 0,00256 [0,0023;0,00284] | 0,571 |
| High grey level count emphasis | 664,558 [392,818; 856,187] | 1464,499 [1022,647; 1740,326] | 1 | < 0,001 | 120,4% (+799,941) | 138,2% (+826,735) | 482,966 [337,004;545,259] | 475,056 [368,251;511,768] | 0,408 |
| Low dependence low grey level emphasis | 0 [0; 0] | 0 [0; 0] | -0,38 | 0,002 | -31,8% (-0,00012) | -29,8% (-0,00011) | 0,000328 [0,000224;0,000422] | 0,000332 [0,000258;0,000433] | 0,613 |
| Low dependence high grey level emphasis | 171,444 [48,494; 220,598] | 504,486 [202,387; 766,196] | 0,9 | < 0,001 | 194,3% (+333,041) | 202,1% (+250,804) | 100,238 [25,734;129,634] | 91,363 [36,598;123,875] | 0,378 |
| High dependence low grey level emphasis | 0,051 [0,018; 0,063] | 0,013 [0,004; 0,015] | -0,89 | 0,01 | -74,2% (-0,038) | -74,3% (-0,028) | 0,079 [0,038;0,101] | 0,1 [0,04;0,077] | 0,17 |
| High dependence high grey level emphasis | 10909,861 [7907,166; 12181,293] | 16808,059 [10054,771; 15041,978] | 0,33 | 0,008 | 54,1% (+5898,198) | 46,3% (+4270,643) | 10385,968 [7652,302;12187,589] | 12597,522 [6639,877;11178,635] | 0,131 |
| Grey level non-uniformity | 86,107 [29,489; 106,266] | 59,77 [33,945; 80,597] | 0,1 | 0,265 | -30,6% (-26,336) | 1,9% (+1,064) | 105,627 [32,878;136,044] | 104,136 [64,747;130,531] | 0,828 |
| Dependence count non-uniformity | 178,905 [35,249; 278,743] | 245,986 [72,116; 270,641] | 0,5 | 0,893 | 37,5% (+67,08) | 36,2% (+40,934) | 164,021 [32,026;240,871] | 183,13 [54,727;205,119] | 0,67 |
| Dependence count non-uniformity normalized | 0,181 [0,134; 0,214] | 0,219 [0,186; 0,259] | 0,59 | 0,065 | 21,2% (+0,038) | 18,9% (+0,034) | 0,173 [0,128;0,198] | 0,154 [0,143;0,179] | 0,398 |
| Grey level variance | 36,56 [2,586; 41,044] | 57,37 [10,173; 85,795] | 0,64 | 0,721 | 56,9% (+20,809) | 189,5% (+18,804) | 27,38 [1,664;31,034] | 21,698 [4,223;27,933] | 0,304 |
| Dependence count variance | 4,004 [2,122; 5,701] | 2,78 [1,496; 2,793] | -0,5 | 0,203 | -30,6% (-1,224) | -30,9% (-0,98) | 4,618 [2,58;6,356] | 5,838 [3,13;4,95] | 0,166 |
| Dependence count entropy | 5,668 [4,911; 6,688] | 6,191 [5,605; 6,624] | 0,53 | 0,642 | 9,2% (+0,523) | 7,9% (+0,472) | 5,575 [4,809;6,613] | 5,902 [5,349;6,338] | 0,911 |

p-value for the predictive value of carbidopa premedication in a linear regression model also including the histo-molecular diagnosis without and with Tumor to normal Brain ratios (TBR) (in bold. significant p-values). Absolute correlation coefficients in bold are >=0.4.

# Supplemental Figure 2.

Illustration of bin spread effect on matrices in which some textural features were significantly modified with carbidopa, but poorly correlated to SUVmean.

These four textural matrices are:

- Grey level co-occurrence matrix (GLCM) with modified parameters: Inverse difference normalized, Inverse variance;

- Grey level run length matrix (GLRLM) with modified parameter: Grey level non-uniformity;

- Grey level size zone matrix (GLSZM) with modified parameters: Small zone emphasis, Zone size non-uniformity normalized;

- Neighboring grey level dependence based matrix (NGLDM) with modified parameters: Dependence count non-uniformity normalized, and Dependence count variance.

These textural matrices were extracted from a static healthy brain image of a patient without carbidopa premedication and a similar static image was then generated to simulate the effect of carbidopa premedication. This simulation was performed by multiplying by 1.5 the voxel-values in the original static image (to reproduce the 50% increase in patients, reported in our current study).

To emphasize the bin spread effect of carbidopa premedication on these matrices, the differences between carbidopa premedication and no premedication were computed after correcting matrices for the bin shift (linked to the increase of SUV values).

pij represent the joint probability for the combination of grey levels i and j in neighboring voxels of the image.

rij correspond to the joint probability of the number of runs with grey level i and length j in the image.

sij represent the number of zones with gray level i and size j appearing in the image

sik is the joint probability for a voxel with grey level i and with k dependent on voxels appearing in the image.