Table 1: Indices used in this study during an assessment of heavy metal pollution for the environment, i.e., ecology, and human health

| Category | Assessment indices | Description | Formula | Classification of Level of Contaminations |
| --- | --- | --- | --- | --- |
| Pollution indices | Single Pollution Index (PI) for soil (Kowalska et al. 2018) | Evaluates the degree of top soil pollution | PI=CnGB; Where, Cn is the concentration of the heavy metal of interest in the soil sample and GB is the concentration of the heavy metal of interest in the control soil sample. | PI > 5: Very strong contamination; 5 > PI > 3: Strong contamination; 3 > PI >2: Moderate contamination; 2 > PI >1: Low contamination; PI < 1: No pollution. |
|  | Nemerow Pollution Load Index (PINemerow) for soil (Qingjie et al. 2008) | Determines the environment quality of the studied soil | PINemerow= PIavg2+ PImax2n; Where PIavg is the average value of single pollution index, PImax is the maximum values of single pollution index, and n is the number of studied heavy metals. | PINemerow < 0.7: Clean soil; 0.7 < PINemerow < 1: Warning limit of soil; 1 < PINemerow < 2: Slight soil pollution; 2 < PINemerow < 3: Moderate soil pollution; PINemerow > 3: Greater soil pollution |
|  | Metal Pollution Index (MPI) in vegetables (Usero et al. 1997) | Compares the overall heavy metal pollution status of the studied areas | MPImgkg=(Cf1×Cf2 ×Cf3…..×Cfn)1n; Where, Cf1 is the concentration of first metal, Cf2 is the concentration of second metal, Cf3 is the concentration of third metal, and Cfn is the concentration of “nth” metal (mg/kg) in the analyzed vegetables. | The sites with high metal concentration is highlighted. |
|  | Enrichment factor (EF) (Mizan et al. 2023) | Focuses on comparing the concentration of heavy metals in a sample to background levels, emphasizing anthropogenic influence | EF= Celement in sampleCelement in background; Where, EF represents the Enrichment Factor. Celements in sample is the concentration of the metal of interest in the sample. Celement in background is the concentration of the metal of interest in the geochemical background based upon Upper Continental Crust (UCC), as 67.30 mg/kg for Cr, 0.10 mg/kg for Cd, 21 mg/kg for Pb and 12.70 mg/kg for Mn (Taylor and McLennan 1995; Turekian and Wedepohl 1961). | EF < 1: No enrichment (normal background levels); 1 ≤ EF < 2: Minor enrichment; 2 ≤ EF < 5: Moderate enrichment; 5 ≤ EF < 20: Significant enrichment; EF ≥ 20: Very high enrichment. |
|  | Geo-accumulation index (Igeo) (Mizan et al. 2023) | Evaluates the extent of contamination relative to natural conditions, considering both natural and anthropogenic sources | Igeo=log2 ( Cn1.5 × Bn ); Where, Cn is the measured concentration of the heavy metal (𝑛) in the soil. 𝐵𝑛 is the measured concentration of the same metal (n) in the control soil. Factor 1.5 is concentration of possible variations in the background data due to lithogenic effects. | Igeo<0: Practically uncontaminated; 0≤Igeo≤1: Uncontaminated to moderately contaminated; 1<<Igeo≤2: Moderately contaminated; 2<Igeo≤3: Moderately to strongly contaminated; 3<<Igeo≤4: Strongly contaminated; 4<Igeo≤5: Strongly to extremely contaminated; Igeo>5: Extremely contaminated. |
|  | Plant Transfer factor (PTF) in vegetables (Cui et al. 2005) | For determining the uptake of metals from soil to plants | PTF = MvMs ; where Mv = concentration of heavy metals in the edible parts of examined vegetables and Ms = concentration of heavy metals in soil | PTF > 1: The plants are capable of accumulating metals and can be used for phytoremediation; PTF around 1: The plants are not influenced by the metals; PTF < 1: The plants are capable of excluding the metals during cellular uptakes |
| Ecological risk indices | Ecological risk factor (Eri) (Hakanson 1980) | Signifies the impact of environmental contamination on the ecology | Eri= Tri ∙ Cif; Cf= CiBi; Where, Tr = the toxic – response factor for the studied metals [Tr values for respective Cr, Cd, and Pb is 2, 30, and 5]; Cf = the contamination factor; | Eri < 40 : low potential ecological risk; 40 ≤ Eri < 80 : moderate potential ecological risk; 80 ≤ Eri < 160 : considerable potential ecological risk; 160 ≤ Eri < 320 : high potential ecological risk; Eri ≥ 320 : very high ecological risk at hand for the substance in question. |
|  | Index for ecological risk (RI) | Measures the overall ecological risk factors for the studied areas | RI= i=18Eri= i=18Tri∙ Cif; Where, Er is the potential ecological risk factor for the studied metals. | Ri < 150: low ecological risk for the lake/basin; 150 ≤ Ri <300: moderate ecological risk for the lake/basin; 300 ≤ Ri <600: considerable ecological risk for the lake/basin; Ri ≥ 600: very high ecological risk for the lake/basin. |
| Human Health risk indices | Chronic daily intake (CDI) of contaminants via various exposure pathways (Liu et al. 2013; Mundy and Bascietto 1989) | Estimates the potential health risks associated with long-term exposure to contaminants | CDIing= C × IRS × EF × EDBW × AT ×CF ; Where, CDIing is chronic daily intake of soil particles via ingestion, mg/kg-day; CDIderm= C × SA × AF × ABS × EF × EDBW × AT×CF ; Where, CDIderm is chronic daily intake via dermal absorption of trace metals in particles adhered to exposed skin, mg/kg-day; CDIdiet=Cvegetable×IRvegetable×EF×EDBW×AT; Where, CDIdiet is chronic daily intake via diet through the vegetables grown in pollutant soils, mg/kg-day; CDIinh= C × IRinh×EF×EDPEF×BW×AT; Where, CDIinh is chronic daily intake via inhalation of suspended particulates emitted from soil by air through mouth and nose, mg/m3 for non-cancer (µg/m3 for cancer). C is concentration of heavy metal in soil/vegetable; The definitions of other input parameters were included in supporting file (Table S1) and also in our previous study (Hossain et al. 2021). | Refer to Table S2 and Table S3 |
|  | Non-carcinogenic risk (Mundy and Bascietto 1989) | Evaluates the adverse health effects other than cancer | Non-cancer hazard quotient (HQ): HQ= CDIRfD; Where, RfD is reference dose for each metal. | HQ < 1: no adverse health risk; HQ > 1: potential serious human health risks |
|  |  |  | Total non-cancer hazard index (HI): HI= k=1nHQ= HQing+ HQinh+HQderm+HQdiet = CDIingRfDing+CDIinhRfCinh+CDIdermRfDderm+CDIdietRfDdiet = CDIingRfDing+CDIinhRfCinh+CDIdermRfDing×ABSGI+CDIdietRfDdiet ; Where, ABSGI is gastrointestinal absorption factor | HI > 1: extreme human health risks, and the rising tendency of HI increases the chances of non-carcinogenic risk occurrences accordingly |
|  | Carcinogenic risk (Mundy and Bascietto 1989; Liu et al. 2013) | Evaluates the possible cancer risks of human from carcinogen over the period of their lifetime exposure to that carcinogen. | As cancer risk (CR): Cancer risk CR=CDI × SF; Where, SF is the corresponding slope factor of carcinogens (mg/kg−1d−1)−1; As cumulative cancer risk: Total cancer risk =∑CR= CRing + CRderm + CRinh + CRdiet= CDIing ×SFing+CDIderm ×SFingABSGI+CDIinh× IUR +CDIdiet ×SFdiet; Where, IUR is chronic inhalation unit risk; SFing and SFdiet are chronic oral slope factor and SFderm(SFing/ABSGI) is chronic dermal slope factor. | 1.0E-06 < CR < 1.0E-04: low but acceptable risk level; CR < 1.0E−06: negligible risk of cancer (a probability of less than 1 in 1,000,000 of an individual developing cancer); CR ~ 1.0E−06: carcinogenic target risk (approximately 1 in 1,000,000); CR > 1.0E−04: Undesirable cancer risk (greater than 1 in 10,000) indicating higher probability of an individual developing cancer due to exposure to the contaminant and typically requires risk management actions to reduce exposure |