# Supplementary Information

## Supplementary Note 1: CO2 Geological Storage definitions

In this study we discuss the permanent storage of CO2 in deep geological formations, herein referred to as CO2 geological storage (CGS). This definition is selected to be explicitly independent of the source of CO2. According to IPCC (2022), Carbon Capture and Storage (CCS) is defined as the process in which CO2 from “industrial and energy-related sources is separated (captured), conditioned, compressed and transported to a storage location for long-term isolation from the atmosphere”. This definition implies mitigation of emissions and is separate from the other primary use of CGS, that is the sequestration of CO2 to facilitate Carbon Dioxide Removal. Here, CO2 is sourced from negative emissions technologies such as Direct Air Capture, or Bioenergy. While the consequences of deploying emissions-reductions technologies versus CDR technologies have vast climatic and economic outcomes, the storage component remains largely ambivalent of the source, hence our use of the generic CGS definition. In separating and analysing datasets from the IPCC AR6 database (Byers et al., 2022), we separate where CGS is the storage component for emissions mitigation technologies (fossil energy and industrial processes) from carbon dioxide removal technologies (Bioenergy and Direct Air Capture). We refer to these throughout the study as “CGS for mitigation” and “CGS for removal” respectively.

## Supplementary Note 2: Integrated Assessment Model regions

IAMs typically group countries with similar socio-economic and energy system characteristics. The six IAMs we studied each use a different approach to aggregate countries, and so they each vary in terms of (a) the number of regions (b) the countries that belong to each region and (c) the constraints assigned to each region (Supplementary Table 1; Supplementary Figure 1).

Supplementary Table 1: Lists of default regions per IAM. The regions are shown in map form in Supplementary Figure 1.

| IMAGE (n = 26) | COFFEE (n = 18) | WITCH (n = 17) | GCAM (n = 14) | REMIND (n = 12) | MESSAGE (n = 12) |
| --- | --- | --- | --- | --- | --- |
| Brazil Canada Central America Central Asia Central Europe China region Eastern Africa India Indonesia region Japan Korea region Mexico Middle East Northern Africa Oceania Rest of South America Rest of South Asia Rest of Southern Africa Russia region South Africa Southeastern Asia Turkey Ukraine region USA Western Africa Western Europe | AFR AUS BRA CAM CAN CAS CHN EEU IND JPN KOR MEA RAS RUS SAF SAM USA WEU | brazil canada china europe india indonesia jpnkor laca mena mexico oceania sasia seasia southafrica ssa te usa | Africa Australia_NZ Canada China Eastern Europe Former Soviet Union India Japan Korea Latin America Middle East Southeast Asia USA Western Europe | CAZ CHA EUR IND JPN LAM MEA NEU OAS REF SSA USA | R12_AFR R12_CHN R12_EEU R12_FSU R12_LAM R12_MEA R12_NAM R12_PAO R12_PAS R12_RCPA R12_SAS R12_WEU |

Supplementary Figure 1: Global maps highlighting the default regional aggregation schemes adopted by GCAM (14), IMAGE (26), MESSAGE (12), REMIND (12), WITCH (17) and COFFEE (18). The regions are listed in Supplementary Table 1.

The use of different region nomenclature across the IAMs introduced some challenges to analysing the conformance between boundaries. To address this, we utilised Geographical Information System analysis, particularly raster analysis function, through the following. We first mapped the regional schemes for each IAM (Supplementary Table 1) in shapefile format (Supplementary Figure 1). We then converted each shapefile into a coarse raster file (at 100,000 m cell size). For each raster file, we then extracted the cells belonging to the boundaries between regions, and these boundaries were assigned a value of 1. Finally, we summed the per-IAM boundary raster files to produce a count of overlapping regional boundaries (Supplementary Figure 2).

Supplementary Figure 2: Global map showing the number of IAMs that agree on a given regional boundary. We consider six IAMs (COFFEE, GCAM, IMAGE, MESSAGE, REMIND and WITCH) hence the maximum possible value is 6. If a single IAM utilises a boundary, the value is 1.

## Supplementary Note 3: Studies constraining global saline aquifer CGS potential

The high degrees of uncertainty associated with estimating global geological storage potential, particularly within saline aquifer formations, has led to the publication of many different values often utilising different methodologies and datasets. Here, we summarise the evolution of these estimates from 1990 to present.

Saline aquifers consist of porous and permeable rock formations that are water-bearing, but at sufficient depths below the subsurface for that water to be saline. When buoyant CO2 is injected into a saline aquifer it is generally (in the absence of significantly vertical heterogeneity) expected to rise to the top of the storage formation and migrate up-dip, displacing brine along the way. As this takes place, there are a variety of mechanisms by which the CO2 can be permanently trapped, including structural trapping at the crest of a topographical high in the storage formation, residual trapping whereby CO2 is trapped via surface tension at the boundary between grains in the storage formation, dissolution trapping where the CO2 dissolves into the native brine and mineral trapping where the CO2 precipitates as a solid.

Saline aquifers are the most widespread type of CGS formation globally, with their distribution closely linked to the distribution of sedimentary basins. Globally, there are 764 sedimentary basins, collectively spanning an estimated area of 1.2 x 108 km2 (Evenick, 2021). These basins vary significantly in size, from smaller examples like the Bokaro Rift Basin in India (624 km²) to expansive systems such as the West Siberian Intracratonic Basin in Russia, which spans over 2.6 × 10⁶ km² (Evenick, 2021).

Efforts to estimate the storage potential of saline aquifers on a global scale started in the early 1990s (Supplementary Table 2, Supplementary Figure 3), and capacity estimates utilised within the IMAGE and WITCH Integrated Assessment Models (IAMs) are derived from studies that adopt this methodology (Hendriks et al., 2004; IEAGHG, 2011). These studies adopt an ‘area-based’ methodology that is anchored on an assessment of the global distribution of sedimentary basins, while also applying various assumptions about geological factors (e.g., aquifer thickness, porosity) and fluid properties (e.g., CO2 density, dissolution rates) to estimate storage potential.

Over the course of the early 1990s to early 2000s, several publications sought to estimate global CO2 geological storage (CGS) potential following this approach. The concept of residual trapping and CO2 plume mobility was relatively nascent in the early 1990s and some studies at this time considered that CO2 could only be stored through dissolution within the native brine. Koide et al. (1992) assumed that 40 kg of CO2 could be stored for every cubic metre of porewater, and calculated that from the distribution of onshore basins, and a constraint that only 1 % of each basin might be appropriate for storage, the global CO2 capacity was 320 GtCO2, which while large, is significantly lower than many subsequent estimates. A similar approach was adopted by Bruant et al. (2002), but those authors did not include the 1 % constraint on suitable areas, leading to a large global storage capacity of 56,000 GtCO2, with an uncertainty range of between 10,000 GtCO2 and 200,000 GtCO2.

As the understanding of CO2 displacing water upon injection matured (van der Meer, 1992), a suite of studies emerged that, while still treating the distribution of sedimentary basins as a key variable in capacity estimates, expanded the methodology to incorporate additional geological properties of the aquifer, such as porosity and thickness, displacement trapping, the need for a structural trap, and storage efficiency (Hendriks, 1994; Hendriks & Blok, 1993; Hendriks & Blok, 1995). With the exception of sedimentary basin area, all these parameters were kept constant—a necessary but highly simplifying assumption for calculating global storage capacity. These authors found that if structural trapping is required, the global capacity in saline aquifers is around 200 GtCO2, but if this is removed and residual trapping is instead the main mechanism (at 5 % storage efficiency), capacity is several orders of magnitude greater (50,000 GtCO2) (Hendriks, 1994). The findings from some of these early studies would be presented in the IPCC’s Second Assessment Report (Watson et al., 1996).

The work of Hendriks et al. (2004), which focused on defining global CGS potential and regional cost curves, is widely cited by IAMs, but in terms of storage potential, provided a relatively incremental update to Hendriks (1994). The authors considered that structural trapping was essential and assumed that 1 % of a basin might contain a trap and that 2 % of a trap could be filled with CO2. This resulted in relatively conservative storage potential estimates (Supplementary Figure 3; Supplementary Table 2) when compared to other studies. They also presented a low, best and high case, with the best case of 240 GtCO2, similar to their 1994 study (Hendriks, 1994).

Kearns et al. (2017) advanced this methodology by comparing sedimentary basin area to refined CGS capacity estimates in the USA, the authors demonstrated that the initially weak correlation between these two parameters could be improved by introducing a measure of the total sedimentary thickness, essentially providing a more refined correlation between sedimentary volume and CGS capacity. In addition to this, those authors presented a practically accessible storage capacity, based on further refinement of regions appropriate for CGS, including exclusion of deepwater areas (> 300 m water depth), regions 200 miles from a landmass and polar regions (outside 66° latitude). They found that this resulted in a global CGS capacity of either 7910 GtCO2 or 55581 GtCO2 depending on the precise correlation used. While the low estimates quoted by Kearns et al. (2017) align approximately with other studies, the high estimates are substantially larger than others (Supplementary Figure 3). Following this, Gidden et al. (2025) adopted further geospatial constraints on storage potential, presenting a significantly reduced global storage potential of 1,550 GtCO2.

These studies sought to develop a methodology to estimate storage potential that prioritises consistency of method over accuracy. An alternative approach has been adopted by other studies, where global storage potential is compiled from existing country or region-specific sources. This approach was adopted by Dooley et al. (2005) for incorporation into GCAM, IEAGHG (2008) which would inform storage potentials incorporated into COFFEE and the OGCI (2005). The latter represents a comprehensive database of published storage potential data, robustly classified according to storage maturity criteria. By taking the sum of all saline aquifer entries in this database, we calculate a mid-case global storage potential of up to 13,488 GtCO2, which is closely aligned to the median of all storage potential estimates compiled for this work (Supplementary Figure 3).

Supplementary Table 2: Summary of key publications, in chronological order, that estimate the global geological storage potential within saline aquifers

| Reference | Geological storage potential (GtCO2) | Approach | Used by IAM? |
| --- | --- | --- | --- |
| Koide et al. (1992) | 319 | Area-based volumetric approach assuming trapping via dissolution only | No |
| Hendriks (1994) | 200 | Area-based volumetric approach assuming mobile trapping in structures only | No |
| Hendriks (1994) | 50,000 | Area-based volumetric approach assuming residual and mobile trapping without need for structures | No |
| Bruant et al. (2002) | 56,000 (range from 10,000 to 200,000) | Area-based volumetric approach assuming trapping via dissolution only | No |
| Hendriks et al. (2004) | 30 (low), 240 (best), 1,081 (high) | Area-based volumetric approach assuming mobile trapping in structures only | IMAGE, WITCH |
| Dooley et al. (2005) | 9,531 | Compilation of other literature sources | GCAM |
| IEAGHG (2008) | 11,760 | Compilation of other literature sources | COFFEE |
| Kearns et al. (2017) | 7,910 (low, practical), 12,211 (low, technical), 55,580 (high, practical), 85,803 (high, technical) | Sedimentary-based volumetric approach based on volume-to-capacity correlations developed from US data | MIT Integrated Global Systems Model (IGSM) |
| Oil and Gas Climate Initiative (OGCI) (2024) | 2,663 (low), 13,488 (medium), 22,720 (high) | Compilation of other literature sources, calculated in this study by summing all saline aquifer database entries | No |
| Gidden et al. (2025) | 11,314 (technical), 1,550 (practical) | Area-based volumetric approach | No |

Supplementary Figure 3: Estimates of global CO2 geological storage potential in saline aquifers against the year of publication. There is no significant trend in the estimates over time, with most estimates are below 20,000 GtCO2 and a median estimate of 10,423 GtCO2. Some large outliers also exist (up to 86,000 GtCO2). 1: Koide et al. (1992), 2: Hendriks (1994), 3: Bruant et al. (2002), 4: Hendriks et al. (2004), 5: Dooley et al. (2005), 6: IEAGHG (2008), 7: Kearns et al. (2017), 8: Oil and Gas Climate Initiative (OGCI) (2024), 9: Gidden et al. (2025)

## Supplementary Note 4: CO2 geological storage potential constraints

IAM’s adopt limits on the geological storage potential, and we compiled this data for the six of the nine IAMs that produced modelling results for IPCC Sixth Assessment Report. We compiled data for COFFEE, GCAM, IMAGE, MESSAGE, REMIND and WITCH (Supplementary Figure 4; Supplementary Tables 3-8). Limits are also adopted with POLES (Keramidas et al., 2018) but the specific data was not available, and no information on storage potential constraints could be ascertained for AIM and GEM.

Supplementary Figure 4: Global maps highlighting regional geological storage potential limits adopted by GCAM (7,178 GtCO2), IMAGE (5,805 GtCO2), MESSAGE (1,503 GtCO2), REMIND (3,929 GtCO2), WITCH (11,372 GtCO2) and COFFEE (13,010 GtCO2). Note that for IMAGE and WITCH, the “medium” and “best” estimates, respectively, are adopted. The corresponding data can be viewed in Supplementary Tables 3-8).

Supplementary Table 3: COFFEE (Rochedo, 2016)

| Region | EOR onshore (GtCO2) | EOR offshore (GtCO2) | Gas onshore (GtCO2) | Gas offshore (GtCO2) | Low-cost aquifer onshore (GtCO2) | Low-cost aquifer offshore (GtCO2) | High-cost aquifer onshore (GtCO2) | High-cost aquifer offshore (GtCO2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AFR | 5 | 12 | 20 | 30 | 94 | 94 | 111 | 111 |
| AUS | 0 | 1 | 0 | 16 | 70 | 630 | 4 | 36 |
| BRA | 0 | 6 | 0 | 23 | 87 | 780 | 113 | 1020 |
| CAM | 1 | 5 | 1 | 11 | 4 | 40 | 5 | 47 |
| CAN | 0 | 0 | 7 | 1 | 264 | 88 | 711 | 237 |
| CAS | 1 | 4 | 34 | 17 | 30 | 10 | 36 | 12 |
| CHN | 2 | 1 | 20 | 1 | 803 | 268 | 273 | 91 |
| EEU | 1 | 9 | 5 | 50 | 2 | 17 | 5 | 46 |
| IND | 0 | 0 | 2 | 2 | 75 | 75 | 75 | 75 |
| JAP | 0 | 0 | 0 | 0 | 15 | 15 | 36 | 36 |
| KOR | 0 | 0 | 0 | 0 | 4 | 4 | 5 | 5 |
| MEA | 27 | 18 | 114 | 29 | 158 | 53 | 187 | 62 |
| RAS | 0 | 1 | 13 | 31 | 56 | 56 | 66 | 66 |
| RUS | 9 | 6 | 44 | 128 | 481 | 481 | 569 | 569 |
| SAF | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 17 |
| SAM | 6 | 5 | 15 | 12 | 88 | 88 | 104 | 104 |
| USA | 3 | 3 | 62 | 5 | 426 | 142 | 1149 | 383 |
| WEU | 0 | 0 | 4 | 0 | 64 | 21 | 176 | 59 |

Supplementary Table 4: GCAM (after Dooley et al. (2005))

| Region | Coal Basins (GtCO2) | Depleted Oil Plays (GtCO2) | Gas Basins (GtCO2) | Saline aquifers (onshore) (GtCO2) | Saline aquifers (offshore) (GtCO2) |
| --- | --- | --- | --- | --- | --- |
| Africa | 0 | 3 | 9 | 78 | 0 |
| Australia_NZ | 0 | 12 | 0 | 265 | 152 |
| Canada | 1 | 14 | 0 | 38 | 0 |
| China | 9 | 4 | 4 | 1716 | 584 |
| Eastern Europe | 1 | 2 | 0 | 13 | 0 |
| Former Soviet Union | 0 | 16 | 256 | 0 | 0 |
| India | 0 | 1 | 14 | 43 | 41 |
| Japan | 0 | 3 | 0 | 41 | 66 |
| Korea | 0 | 0 | 0 | 0 | 0 |
| Latin America | 0 | 7.575 | 45 | 1562 | 0 |
| Middle East | 0 | 53 | 169 | 0 | 0 |
| Southeast Asia | 0 | 3.75 | 33 | 0 | 0 |
| USA | 59 | 107 | 0 | 1161 | 491 |
| Western Europe | 0 | 22 | 0 | 36 | 43 |

Supplementary Table 5: IMAGE (after (Hendriks et al., 2004))

| Estimate | Region | Remaining oil, onshore (GtCO2) | Remaining oil, offshore (GtCO2) | Remaining gas, onshore (GtCO2) | Remaining gas, offshore (GtCO2) | Depleted oil, onshore (GtCO2) | Depleted oil, offshore (GtCO2) | Depleted gas, onshore (GtCO2) | Depleted gas, offshore (GtCO2) | ECBM (GtCO2) | Aquifers 1 (GtCO2) | Aquifers 2 (GtCO2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | Brazil | 0 | 0 | 3 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| Medium | Brazil | 3 | 2 | 6 | 5 | 0 | 1 | 1 | 0 | 1 | 11 | 186 |
| High | Brazil | 20 | 19 | 17 | 21 | 1 | 1 | 1 | 0 | 5 | 50 | 279 |
| Low | Canada | 0 | 0 | 7 | 1 | 1 | 0 | 0 | 0 | 0 | 2 | 0 |
| Medium | Canada | 0 | 0 | 8 | 1 | 1 | 0 | 7 | 0 | 8 | 17 | 291 |
| High | Canada | 3 | 3 | 10 | 1 | 2 | 0 | 8 | 0 | 51 | 78 | 437 |
| Low | Central America | 0 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Medium | Central America | 0 | 0 | 1 | 4 | 0 | 0 | 0 | 0 | 0 | 4 | 61 |
| High | Central America | 2 | 3 | 2 | 12 | 0 | 1 | 0 | 0 | 0 | 16 | 92 |
| Low | Central Asia | 0 | 0 | 12 | 4 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| Medium | Central Asia | 4 | 1 | 21 | 12 | 1 | 0 | 10 | 0 | 1 | 6 | 96 |
| High | Central Asia | 25 | 4 | 55 | 48 | 1 | 1 | 17 | 1 | 4 | 26 | 144 |
| Low | Central Europe | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Medium | Central Europe | 1 | 0 | 4 | 0 | 0 | 0 | 3 | 0 | 1 | 3 | 57 |
| High | Central Europe | 5 | 0 | 7 | 0 | 1 | 0 | 4 | 0 | 4 | 15 | 85 |
| Low | China + | 0 | 0 | 4 | 0 | 1 | 0 | 0 | 0 | 0 | 2 | 0 |
| Medium | China + | 3 | 0 | 8 | 0 | 2 | 1 | 4 | 0 | 156 | 13 | 222 |
| High | China + | 23 | 3 | 24 | 1 | 2 | 2 | 8 | 0 | 829 | 59 | 333 |
| Low | Eastern Africa | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| Medium | Eastern Africa | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 5 | 92 |
| High | Eastern Africa | 0 | 1 | 1 | 4 | 0 | 0 | 0 | 0 | 0 | 25 | 139 |
| Low | India | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 0 |
| Medium | India | 0 | 0 | 3 | 1 | 0 | 1 | 2 | 0 | 5 | 25 | 300 |
| High | India | 1 | 2 | 7 | 4 | 0 | 2 | 4 | 0 | 8 | 60 | 337 |
| Low | Indonesia + | 0 | 0 | 1 | 8 | 0 | 1 | 0 | 1 | 0 | 0 | 0 |
| Medium | Indonesia + | 1 | 1 | 4 | 16 | 1 | 2 | 2 | 2 | 18 | 3 | 52 |
| High | Indonesia + | 3 | 6 | 9 | 31 | 1 | 4 | 4 | 2 | 105 | 14 | 78 |
| Low | Japan | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Medium | Japan | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 31 |
| High | Japan | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 8 | 47 |
| Low | Korea Region | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Medium | Korea Region | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 0 | 4 |
| High | Korea Region | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 12 | 1 | 7 |
| Low | Mexico | 0 | 0 | 0 | 3 | 0 | 2 | 0 | 1 | 0 | 0 | 0 |
| Medium | Mexico | 2 | 2 | 1 | 5 | 1 | 6 | 1 | 1 | 0 | 4 | 61 |
| High | Mexico | 12 | 18 | 2 | 15 | 1 | 10 | 1 | 1 | 0 | 16 | 92 |
| Low | Middle East | 51 | 1 | 92 | 70 | 8 | 3 | 0 | 1 | 0 | 1 | 0 |
| Medium | Middle East | 62 | 9 | 168 | 85 | 12 | 10 | 92 | 1 | 0 | 8 | 142 |
| High | Middle East | 405 | 61 | 372 | 116 | 16 | 18 | 168 | 1 | 0 | 38 | 213 |
| Low | North Africa | 0 | 0 | 14 | 1 | 1 | 1 | 0 | 0 | 0 | 2 | 0 |
| Medium | North Africa | 5 | 1 | 19 | 3 | 2 | 3 | 14 | 0 | 0 | 13 | 227 |
| High | North Africa | 24 | 6 | 43 | 10 | 2 | 5 | 19 | 0 | 0 | 61 | 341 |
| Low | Oceania | 0 | 0 | 0 | 7 | 0 | 1 | 0 | 0 | 0 | 4 | 0 |
| Medium | Oceania | 0 | 1 | 0 | 17 | 0 | 1 | 0 | 0 | 11 | 28 | 474 |
| High | Oceania | 0 | 5 | 1 | 40 | 0 | 3 | 0 | 1 | 54 | 126 | 711 |
| Low | Rest of South America | 0 | 0 | 6 | 2 | 2 | 2 | 0 | 0 | 0 | 1 | 0 |
| Medium | Rest of South America | 5 | 4 | 12 | 9 | 3 | 6 | 8 | 0 | 1 | 12 | 202 |
| High | Rest of South America | 34 | 33 | 32 | 40 | 4 | 10 | 16 | 1 | 7 | 54 | 303 |
| Low | Rest of South Asia | 0 | 0 | 3 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| Medium | Rest of South Asia | 0 | 0 | 7 | 3 | 0 | 0 | 2 | 0 | 1 | 8 | 134 |
| High | Rest of South Asia | 1 | 1 | 17 | 9 | 0 | 0 | 6 | 1 | 4 | 36 | 201 |
| Low | Rest of Southern America | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| Medium | Rest of Southern America | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 0 | 12 | 195 |
| High | Rest of Southern America | 1 | 11 | 0 | 4 | 0 | 1 | 0 | 0 | 3 | 52 | 292 |
| Low | Russia + | 1 | 0 | 58 | 20 | 4 | 2 | 0 | 2 | 0 | 3 | 0 |
| Medium | Russia + | 18 | 2 | 103 | 58 | 7 | 5 | 59 | 2 | 24 | 25 | 420 |
| High | Russia + | 106 | 15 | 270 | 234 | 9 | 8 | 105 | 4 | 142 | 112 | 629 |
| Low | South Africa | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Medium | South Africa | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 7 | 2 | 42 |
| High | South Africa | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 42 | 11 | 63 |
| Low | Southeastern Asia | 0 | 0 | 1 | 8 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| Medium | Southeastern Asia | 0 | 1 | 3 | 16 | 0 | 1 | 1 | 1 | 1 | 3 | 56 |
| High | Southeastern Asia | 3 | 5 | 9 | 30 | 0 | 2 | 3 | 2 | 9 | 15 | 84 |
| Low | Turkey | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Medium | Turkey | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 21 |
| High | Turkey | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 32 |
| Low | Ukraine + | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| Medium | Ukraine + | 0 | 0 | 2 | 1 | 0 | 0 | 2 | 0 | 1 | 9 | 146 |
| High | Ukraine + | 2 | 0 | 6 | 5 | 0 | 0 | 3 | 0 | 4 | 39 | 220 |
| Low | USA | 1 | 0 | 6 | 1 | 2 | 1 | 2 | 1 | 0 | 2 | 0 |
| Medium | USA | 6 | 0 | 8 | 1 | 4 | 3 | 6 | 1 | 32 | 17 | 291 |
| High | USA | 45 | 5 | 15 | 1 | 5 | 5 | 8 | 2 | 190 | 78 | 437 |
| Low | Western Africa | 0 | 0 | 1 | 5 | 0 | 3 | 0 | 0 | 0 | 2 | 0 |
| Medium | Western Africa | 2 | 6 | 3 | 12 | 0 | 8 | 1 | 49 | 0 | 15 | 255 |
| High | Western Africa | 18 | 67 | 7 | 29 | 0 | 14 | 3 | 1 | 1 | 68 | 383 |
| Low | Western Europe | 0 | 0 | 5 | 13 | 0 | 3 | 0 | 10 | 0 | 1 | 0 |
| Medium | Western Europe | 0 | 4 | 10 | 27 | 0 | 10 | 5 | 10 | 1 | 7 | 119 |
| High | Western Europe | 1 | 40 | 17 | 112 | 0 | 18 | 10 | 13 | 6 | 32 | 178 |

Supplementary Table 6: MESSAGE

| Region | Capacity (GtCO2) |
| --- | --- |
| R12_AFR | 225 |
| R12_CHN | 58 |
| R12_EEU | 7 |
| R12_FSU | 305 |
| R12_LAM | 205 |
| R12_MEA | 210 |
| R12_NAM | 216 |
| R12_PAO | 126 |
| R12_PAS | 77 |
| R12_RCPA | 14 |
| R12_SAS | 17 |
| R12_WEU | 43 |

Supplementary Table 7: REMIND

| Region | Capacity (GtCO2) |
| --- | --- |
| CAZ | 303 |
| CHA | 367 |
| EUR | 148 |
| IND | 183 |
| JPN | 69 |
| LAM | 550 |
| MEA | 353 |
| NEU | 26 |
| OAS | 183 |
| REF | 1056 |
| SSA | 232 |
| USA | 458 |

Supplementary Table 8: WITCH (after (Rottoli & Vinca, 2017))

|  | Best estimate (GtCO2) |  |  | High estimate (GtCO2) |  |  | Low estimate (GtCO2) |  |  | Single estimate (GtCO2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Region | aquifers | ECBM | OG | aquifers | ECBM | OG | aquifers | ECBM | OG | EOR |
| brazil | 63 | 0 | 10 | 249 | 1 | 37 | 17 |  | 3 | 1 |
| canada | 4565 | 11 | 40 | 7288 | 14 | 42 | 1887 | 9 | 6 | 6 |
| china | 215 | 138 | 31 | 938 | 744 | 79 | 31 |  | 12 | 5 |
| europe | 147 | 3 | 26 | 147 | 3 | 26 | 147 | 3 | 26 | 5 |
| india | 58 | 25 | 10 | 252 | 136 | 26 | 8 |  | 4 | 1 |
| indonesia | 6 | 9 | 22 | 25 | 47 | 56 | 1 |  | 9 | 2 |
| jpnkor | 50 | 4 | 1 | 219 | 23 | 1 | 7 |  | 0 | 0 |
| laca | 53 | 2 | 50 | 209 | 9 | 187 | 14 |  | 16 | 6 |
| mena | 223 | 0 | 468 | 658 | 0 | 1281 | 82 |  | 187 | 101 |
| mexico | 5 | 0 | 29 | 20 | 2 | 107 | 1 |  | 9 | 3 |
| oceania | 2 | 11 | 20 | 9 | 54 | 49 | 0 |  | 8 | 0 |
| sasia | 20 | 0 | 4 | 88 | 1 | 12 | 3 |  | 2 |  |
| seasia | 21 | 2 | 23 | 92 | 13 | 60 | 3 |  | 9 | 2 |
| southafrica | 7 | 8 | 1 | 21 | 44 | 2 | 3 |  | 0 | 0 |
| ssa | 358 | 0 | 54 | 1057 | 1 | 147 | 132 |  | 22 | 4 |
| te | 314 | 23 | 303 | 314 | 25 | 303 | 314 | 23 | 303 | 46 |
| usa | 3436 | 165 | 116 | 5486 | 215 | 124 | 1420 | 148 | 16 | 36 |

## Supplementary Note 5: Storage costs

The following tables detail the storage and transport costs embedded in WITCH, IMAGE, GCAM and COFFEE. The costs have been adjusted for inflation to 2024 values and converted to U.S dollars where necessary using the following conversions:

Inflationary conversions (using CPI indexes):

- $1 (2005) = $1.61 (2024)

- $1 (2015) = $1.32 (2024)

- €1 (2004) = €1.56 (2024)

Currency conversions (using annual average spot prices):

- €1 (2024) = $1.08 (2024)

Supplementary Table 9: Summary of storage costs utilised by the WITCH IAM (after Rottoli and Vinca (2017), ZEP (2011) and Rubin et al. (2015)).

| Storage type | Storage |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low ($/tCO2) |  |  | Best ($/tCO2) |  | High ($/tCO2) |  |
|  | Source (2015) | Inflation-adjusted (2024) |  | Source (2015) | Inflation -adjusted (2024) | Source (2015) | Inflation -adjusted (2024) |
| Oil and gas (EOR, onshore) | 1.0 |  | 1.3 | 4 | 5.3 | 9 | 11.9 |
| Oil and gas (no EOR, onshore) | 1.0 |  | 1.3 | 5 | 6.6 | 13 | 17.2 |
| Aquifer onshore | 3.0 |  | 4.0 | 6 | 8.0 | 15 | 19.9 |
| Oil and gas (EOR, offshore) | 3.0 |  | 4.0 | 8 | 10.6 | 12 | 15.9 |
| Oil and gas (no EOR, offshore) | 4.0 |  | 5.3 | 13 | 17.2 | 18 | 23.8 |
| Aquifer offshore | 8.0 |  | 10.6 | 18 | 23.8 | 26 | 34.4 |
| ECBM | 0 |  | 0 | 45 | 59.6 | 90 | 119.1 |

Supplementary Table 10: Average storage cost per type (Hendriks et al., 2004). While not explicitly stated in Hendriks et al (2004), we assume costs are representative of 2004. The table presents the source values and values converted to 2024 values and converted to U.S dollars for comparison with the other datasets.

| Storage type | Average cost (€/tCO2) |  |
| --- | --- | --- |
|  | Source (€ 2004) | Inflation-adjusted ($ 2024) |
| Aquifer onshore | 2.7 | 4.6 |
| Aquifer offshore | 7.3 | 12.3 |
| Natural gas field onshore | 1.6 | 2.7 |
| Natural gas field offshore | 5.7 | 9.6 |
| Empty oil field onshore | 1.6 | 2.7 |
| Empty oil field offshore | 5.7 | 9.6 |
| EOR onshore | 0 | 0 |
| EOR offshore | 3 | 5.1 |
| ECBM | 10 | 16.9 |

Supplementary Table 11: Summary of storage costs used by the GCAM IAM to construct cost-supply curves

| Grade | Fraction of total resource | Cost ($/tCO2) |  |
| --- | --- | --- | --- |
|  |  | Source ($ 2005) | Inflation -adjusted ($ 2024) |
| 1 | 0 - 0.005 | 0.1 | 0.2 |
| 2 | 0.005 - 0.105 | 5.0 | 8.0 |
| 3 | 0.105 - 0.705 | 10.0 | 16.1 |
| 4 | 0.705 - 1 | 96 | 154.2 |

Supplementary Table 12: Storage costs per type used by the COFFEE IAM (Rochedo, 2016).

| Storage type | Average cost ($/tCO2) |  |
| --- | --- | --- |
|  | Source ($ 2016) | Inflation-adjusted ($ 2024) |
| EOR onshore | 3.0 | 3.9 |
| EOR offshore | 6.0 | 7.8 |
| Gas fields onshore | 4.0 | 5.2 |
| Gas fields offshore | 10.0 | 13.1 |
| Low cost aquifer onshore | 5.0 | 6.5 |
| Low cost aquifer offshore | 7.0 | 9.1 |
| High cost aquifer onshore | 12.0 | 15.7 |
| High costs aquifer offshore | 14.0 | 18.3 |

## References

Bruant, R., Guswa, A., Celia, M., & Peters, C. (2002). Safe storage of CO2 in deep saline aquifers. ENVIRONMENTAL SCIENCE AND TECHNOLOGY-WASHINGTON DC-, 36(11), 240A-245A.

Byers, E., Krey, V., Kriegler, E., Riahi, K., Schaeffer, R., Kikstra, J., Lamboll, R., Nicholls, Z., Sandstad, M., Smith, C., van der Wijst, K., Al -Khourdajie, A., Lecocq, F., Portugal-Pereira, J., Saheb, Y., Stromman, A., Winkler, H., Auer, C., Brutschin, E.,…van Vuuren, D. (2022). AR6 Scenarios Database (Integrated Assessment Modeling Consortium & International Institute for Applied Systems Analysis. https://doi.org/10.5281/ZENODO.7197970

Dooley, J. J., Kim, S. H., Edmonds, J. A., Friedman, S. J., & Wise, M. A. (2005). A first-order global geological CO2-storage potential supply curve and its application in a global integrated assessment model. In E. S. Rubin, D. W. Keith, C. F. Gilboy, M. Wilson, T. Morris, J. Gale, & K. Thambimuthu (Eds.), Greenhouse Gas Control Technologies 7 (pp. 573-581). Elsevier Science Ltd. https://doi.org/https://doi.org/10.1016/B978-008044704-9/50058-6

Evenick, J. C. (2021). Glimpses into Earth's history using a revised global sedimentary basin map. Earth-Science Reviews, 215, 103564. https://doi.org/https://doi.org/10.1016/j.earscirev.2021.103564

Gidden, M., Joshi, S., Armitage, J., Christ, A.-B., Boettcher, M., Brutschin, E., Koberle, A., Schellnhuber, H. J., Schleussner, C.-F., Riahi, K., & Rogelj, J. (2025, 27 Apr–2 May 2025). A prudent planetary boundary for geological carbon storage EGU General Assembly 2025, Vienna, Austria.

Hendriks, C. (1994). Carbon Dioxide Removal from Coal-Fired Power Plants. Energy & Environment. https://doi.org/10.1007/978-94-011-0301-5

Hendriks, C., Graus, W., & Van Bergen, F. (2004). Global Carbon Dioxide Storage Potential and Costs. http://inis.iaea.org/search/search.aspx?orig_q=RN:35053050

Hendriks, C. A., & Blok, K. (1993). Underground storage of carbon dioxide. Energy Conversion and Management, 34(9), 949-957. https://doi.org/https://doi.org/10.1016/0196-8904(93)90041-8

Hendriks, C. A., & Blok, K. (1995). Underground storage of carbon dioxide. Energy Conversion and Management, 36(6), 539-542. https://doi.org/https://doi.org/10.1016/0196-8904(95)00062-I

IEAGHG. (2008). Aquifer Storage – Development Issues.

IEAGHG. (2011). Potential for Biomass and Carbon Dioxide Capture and Storage (2011/06).

IPCC. (2022). Annex I: Glossary. In R. van Diemen, J.B.R. Matthews, V. Möller, J.S. Fuglestvedt, V. Masson-Delmotte, C. Méndez, A. Reisinger, & S. Semeno (Eds.), Climate Change 2022: Mitigation of Climate Change. Contribution of Working Group III to the Sixth Assessment Report of the Intergovernmental Panel on Climate Change. Cambridge University Press. https://doi.org/10.1017/9781009157926.020

Kearns, J., Teletzke, G., Palmer, J., Thomann, H., Kheshgi, H., Chen, Y.-H. H., Paltsev, S., & Herzog, H. (2017). Developing a Consistent Database for Regional Geologic CO2 Storage Capacity Worldwide. Energy Procedia, 114, 4697-4709. https://doi.org/https://doi.org/10.1016/j.egypro.2017.03.1603

Keramidas, K., Kitous, A., Schade, B., Després, J., & Schmitz, A. (2018). POLES-JRC model documentation – 2018 update. Publications Office. https://doi.org/doi/10.2760/814959

Koide, H., Tazaki, Y., Noguchi, Y., Nakayama, S., Iijima, M., Ito, K., & Shindo, Y. (1992). Subterranean containment and long-term storage of carbon dioxide in unused aquifers and in depleted natural gas reservoirs. Energy Conversion and Management, 33(5), 619-626. https://doi.org/https://doi.org/10.1016/0196-8904(92)90064-4

Oil and Gas Climate Initiative (OGCI). (2024). CO2 Storage Resource Catalogue - Cycle 4 Report. https://www.ogci.com/wp-content/uploads/2024/08/CSRC_Cycle_4_Main-Report_August_2024.pdf

Rochedo, P. R. R. (2016). Development of a global integrated energy model to evaluate the Brazilian role in climate change mitigation scenarios Federal University of Rio de Janeiro]. Brazil.

Rottoli, M., & Vinca, A. (2017). The role of carbon capture and storage for climate stabilization: a numerical assessment Politecnico di Milano]. POLITesi - Archivio digitale delle tesi di laurea e di dottorato. https://www.politesi.polimi.it/handle/10589/133979

Rubin, E. S., Davison, J. E., & Herzog, H. J. (2015). The cost of CO2 capture and storage. International Journal of Greenhouse Gas Control, 40, 378-400. https://doi.org/https://doi.org/10.1016/j.ijggc.2015.05.018

van der Meer, L. G. H. (1992). Investigations regarding the storage of carbon dioxide in aquifers in the Netherlands. Energy Conversion and Management, 33(5), 611-618. https://doi.org/https://doi.org/10.1016/0196-8904(92)90063-3

Watson, R. T., Zinyowera, M. C., & Moss, R. H. (1996). Climate change 1995. Impacts, adaptations and mitigation of climate change: scientific-technical analyses.

ZEP. (2011). The costs of CO2 storage: post-demonstration CCS in the EU. https://zeroemissionsplatform.eu/document/the-costs-of-co2-storage/