Table 1

Experimental variables and levels of response surface methodology (RSM) for dual-electrode hybrid discharge

| Factors | Experiment levels |  |  |
| --- | --- | --- | --- |
|  | Low (-1) | Medium (0) | High (+1) |
| Time (min) | 10 | 20 | 30 |
| Voltage (V) | 125 | 175 | 225 |
| NaCl Content (%) | 0 | 0.021 | 0.042 |

Table 2

Experimental design and results of RSM under dual-electrode hybrid discharge. All data are expressed in the form of mean ± standard deviation.

| NO. | Factors |  |  | Response variables |  |  |  | Physicochemical property |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Voltage (V) | NaCl content (%) | Time (min) | NO2− (mg/L) | NO3− (mg/L) | H2O2 (mg/L) | O3 (mg/L) | pH | ΔT |
| 1 | 175 | 0.042 | 30 | 145.87±11.34 | 5.11±0.08 | 2.50±0.16 | 0.72±0.02 | 3.5±0.47 | 8.5±0.41 |
| 2 | 225 | 0.042 | 20 | 156.37±4.28 | 4.53±0.01 | 1.90±0.16 | 0.28±0.02 | 3.5±0.00 | 8.5±0.71 |
| 3 | 175 | 0.042 | 10 | 72.41±6.54 | 2.16±0.08 | 1.50±0.04 | 0.46±0.01 | 3.6±0.47 | 2.5±0.41 |
| 4 | 225 | 0.021 | 10 | 93.40±12.85 | 3.34±0.03 | 2.70±0.12 | 0.44±0.04 | 3.6±0.00 | 3.5±0.41 |
| 5 | 125 | 0.042 | 20 | 51.42±4.28 | 3.46±0.06 | 2.50±0.11 | 0.54±0.02 | 3.7±0.00 | 6.5±0.41 |
| 6 | 175 | 0 | 30 | 72.41±8.57 | 4.47±0.02 | 4.00±0.12 | 0.67±0.02 | 3.6±0.47 | 10.5±0.41 |
| 7 | 175 | 0 | 10 | 40.92±8.57 | 1.72±0.21 | 2.30±0.16 | 0.48±0.01 | 3.7±0.47 | 4.5±0.41 |
| 8 | 125 | 0.021 | 10 | 19.94±4.28 | 1.73±0.03 | 2.20±0.21 | 0.23±0.04 | 3.8±0.47 | 5.5±0.41 |
| 9 | 125 | 0 | 20 | 14.69±4.28 | 3.16±0.00 | 3.50±0.26 | 0.17±0.03 | 3.9±0.00 | 8.5±0.41 |
| 10 | 225 | 0 | 20 | 93.40±8.57 | 3.91±0.06 | 3.40±0.19 | 0.72±0.03 | 3.6±0.00 | 11.5±0.41 |
| 11 | 125 | 0.021 | 30 | 46.18±8.57 | 5.52±0.028 | 4.50±0.26 | 0.54±0.03 | 3.7±0.47 | 7.5±0.41 |
| 12 | 225 | 0.021 | 30 | 161.61±1.06 | 5.06±0.04 | 3.00±0.16 | 0.66±0.01 | 3.4±0.00 | 9.5±0.41 |
| 13 (Center point) | 175 | 0.021 | 20 | 172.11±3.80 | 4.81±0.14 | 1.93±0.15 | 0.74±0.04 | 3.3±0.00 | 7.5±0.41 |

Table 3

Response surface optimization results under dual-electrode hybrid discharge

|  | Voltage (V) | NaCl content (%) | Time (min) | NO2− (mg/L) | NO3− (mg/L) | H2O2 (mg/L) | O3 (mg/L) | Composite desirability (D value) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Estimated value | 171.094 | 0.018 | 30 | 143.03 | 5.52 | 3.22 | 0.08 | 0.82 |
| Experimental value | 171 | 0.019 | 30 | 172.11±12.85 | 5.11±0.07 | 3.25±0.41 | 0.81±0.06 | / |