Appendices

A. Tables of Diagnostic Rules (Rules 1–500)

This section includes supplementary material that supports the main content of the paper by providing detailed reference information for researchers, engineers, and operators utilizing the proposed rule-based expert system for ESP diagnostics.

1.	Motor Current Analysis (Table 13)

Table 13. Motor Current Analysis (Rules 1–60).

| Rule # | Observation | Possible Cause | Recommended Action |
| --- | --- | --- | --- |
| 1 | Sudden drop to zero | Power failure or motor trip | Check VFD logs and breaker status |
| 2 | Gradual increase over time | Mechanical loading or wear | Inspect pump shaft and impellers |
| 3 | Spikes every few minutes | Intermittent blockage | Check for gas lock or debris |
| 4 | Sustained high current | Pump overload due to high fluid viscosity | Verify fluid properties; adjust VFD settings |
| 5 | Erratic fluctuations | Electrical instability or loose connections | Inspect electrical connections and grounding |
| 6 | Periodic dips below normal | Gas interference or cavitation | Check gas separator efficiency; adjust pump speed |
| 7 | Constant low current | Underloaded pump or low flow | Verify well inflow and pump sizing |
| 8 | Sudden increase followed by trip | Motor overheating or short circuit | Check motor temperature sensors; inspect wiring |
| 9 | Gradual decrease over days | Scale buildup on motor or pump | Perform acid backwash or chemical treatment |
| 10 | High current with low intake pressure | Pump intake blockage | Inspect intake for debris or sand |
| 11 | Oscillations at fixed intervals | Cyclic reservoir inflow | Analyze reservoir pressure data; adjust choke settings |
| 12 | Current spikes during startup | High starting torque or seized pump | Check for mechanical binding; lubricate bearings |
| 13 | Steady current above rated value | Misaligned pump or motor | Realign pump and motor assembly |
| 14 | Intermittent zero current | Intermittent power supply failure | Check power cables and transformer stability |
| 15 | Current trending upward with high motor temperature | Scale deposition on motor housing | Perform acid backwash with extended soak time |
| 16 | Low current with high vibration | Broken pump shaft | Pull ESP for inspection; replace shaft if damaged |
| 17 | Sporadic current drops | Gas slugging in pump | Install or optimize gas separator |
| 18 | Current stable but below normal | Pump wear or impeller degradation | Inspect pump internals for wear |
| 19 | High current with no flow | Pump stuck or seized | Attempt to free pump; consider pulling ESP |
| 20 | Cyclic current peaks | Variable fluid density | Monitor fluid composition; adjust pump speed |
| 21 | Gradual current increase with low production | Sand production clogging pump | Implement sand control measures |
| 22 | Sharp current drop with no trip | Cable fault or partial disconnection | Inspect power cable for damage |
| 23 | Current spikes with high intake pressure | Pump intake restriction | Clean intake screen or perforations |
| 24 | Low current with high motor temperature | Insufficient cooling due to low flow | Increase drive frequency to boost fluid speed |
| 25 | Erratic current with normal production | Faulty motor controller | Test and recalibrate VFD |
| 26 | Sustained current below threshold | Pump operating below capacity | Resize pump or adjust VFD settings |
| 27 | Current spikes during shutdown | Backspin or fluid hammer | Install check valve or adjust shutdown procedure |
| 28 | High current with low pump efficiency | Worn pump stages | Replace worn pump components |
| 29 | Periodic current surges | Electrical surges or grid instability | Install surge protection or stabilize power supply |
| 30 | Current stable but high vibration | Misalignment or bearing failure | Check alignment and bearing condition |
| 31 | Sudden current increase with no production change | Motor insulation degradation | Test motor insulation resistance |
| 32 | Low current with high discharge pressure | Pump restriction or blockage | Inspect discharge line for obstructions |
| 33 | Current spikes with low intake pressure | Gas lock in pump | Vent gas or install gas handling device |
| 34 | Gradual current decrease with stable production | Motor efficiency loss | Test motor performance; consider replacement |
| 35 | High current during low flow | High fluid density or emulsion | Analyze fluid properties; adjust pump settings |
| 36 | Current drops with increasing motor temperature | Scale or paraffin buildup | Perform chemical treatment or hot oiling |
| 37 | Erratic current with high pump differential pressure | Partial pump blockage | Flush pump or perform acid job |
| 38 | Current above normal with no production increase | Overloaded motor | Reduce pump speed or check for mechanical issues |
| 39 | Low current with normal production | Oversized pump | Evaluate pump sizing and replace if necessary |
| 40 | Periodic current drops with stable pressure | Intermittent electrical fault | Inspect wiring and connectors |
| 41 | High current with reduced flow | Pump impeller damage | Pull ESP for impeller inspection |
| 42 | Current spikes with high motor load | Excessive mechanical friction | Check bearings and lubricate |
| 43 | Sustained high current with low intake pressure | Pump cavitation | Adjust pump speed or install gas separator |
| 44 | Current trending downward over weeks | Gradual pump wear | Schedule pump inspection and maintenance |
| 45 | Sharp current increase with vibration | Broken rotor bar | Test motor and replace if confirmed |
| 46 | Low current with high wellhead pressure | Downhole restriction | Investigate tubing or flowline blockages |
| 47 | Current spikes during flow changes | Unstable reservoir conditions | Stabilize inflow with choke adjustments |
| 48 | High current with normal motor temperature | High fluid viscosity | Adjust VFD to match fluid conditions |
| 49 | Current drops during startup | Power supply undervoltage | Check power supply and transformer capacity |
| 50 | Periodic high current with stable production | Electrical noise or interference | Install noise filters on power lines |
| 51 | Low current with low discharge pressure | Leaking pump seals | Inspect and replace pump seals |
| 52 | Current spikes with high gas content | Gas interference in pump | Optimize gas separator or adjust pump depth |
| 53 | Gradual current increase with stable flow | Motor bearing wear | Monitor vibration; replace bearings if needed |
| 54 | High current with erratic production | Pump stage wear or damage | Pull ESP for stage inspection |
| 55 | Current drops with increasing intake pressure | Pump efficiency loss | Evaluate pump performance; consider repair |
| 56 | Sustained low current with normal pressure | Motor underload | Check pump sizing and well inflow |
| 57 | Current spikes with low motor temperature | Electrical fault in motor | Perform motor diagnostic tests |
| 58 | High current with stable intake pressure | Pump overload or restriction | Inspect pump for blockages or wear |
| 59 | Erratic current with high vibration | Unbalanced pump or motor | Check balance and alignment |
| 60 | Current trending upward with no flow increase | Scale or sand accumulation | Perform chemical cleaning or sand control |

2.	Motor Temperature Analysis (Table 14)

Table 14. Motor Temperature Analysis (Rules 61–100).

| Rule # | Observation | Possible Cause | Recommended Action |
| --- | --- | --- | --- |
| 61 | Temperature rises with stable current | Cooling fluid flow issue | Check circulation, fluid level |
| 62 | High temperature with high current | Overload or insulation failure | Conduct insulation resistance test |
| 63 | Sudden temperature spike | Electrical short in motor | Inspect motor windings; test insulation |
| 64 | Gradual temperature increases over days | Scale or paraffin buildup | Perform chemical cleaning or hot oiling |
| 65 | High temperature with low current | Insufficient cooling due to low flow | Increase pump speed or check well inflow |
| 66 | Temperature oscillations | Unstable fluid flow or gas slugs | Optimize gas separator; stabilize flow |
| 67 | Sustained high temperature | Motor operating above rated capacity | Reduce pump speed or check motor sizing |
| 68 | Temperature rises during startup | High starting torque or friction | Inspect bearings and lubricate |
| 69 | Low temperature with high current | Sensor malfunction | Calibrate or replace temperature sensor |
| 70 | Temperature spikes with normal current | Localized hot spot in motor | Perform motor diagnostic tests |
| 71 | Gradual temperature decrease | Improved cooling or reduced load | Verify fluid flow and pump settings |
| 72 | High temperature with low intake pressure | Pump cavitation or gas lock | Install gas handling device; adjust speed |
| 73 | Temperature stable but above normal | Inadequate motor cooling design | Evaluate motor cooling system; consider redesign |
| 74 | Periodic temperature spikes | Intermittent electrical surges | Install surge protection; check power supply |
| 75 | Temperature rises with high vibration | Misalignment or bearing failure | Check alignment and bearing condition |
| 76 | Low temperature with low production | Underloaded motor | Resize pump or adjust VFD settings |
| 77 | Temperature spikes during shutdown | Fluid backflow or heat soak | Install check valve; adjust shutdown procedure |
| 78 | High temperature with normal flow | Motor insulation degradation | Test insulation resistance; consider motor replacement |
| 79 | Temperature drops with increasing current | Improved cooling efficiency | Monitor fluid flow; ensure stable operation |
| 80 | Erratic temperature readings | Faulty temperature sensor | Replace or recalibrate sensor |
| 81 | High temperature with high discharge pressure | Pump restriction or blockage | Inspect discharge line for obstructions |
| 82 | Temperature rises with stable production | Scale deposition on motor housing | Perform acid backwash or chemical treatment |
| 83 | Low temperature with high motor load | Sensor drift or calibration issue | Recalibrate temperature sensor |
| 84 | Temperature spikes with gas content increase | Gas interference reducing cooling | Optimize gas separator or adjust pump depth |
| 85 | Gradual temperature increases with low flow | Reduced cooling due to low fluid velocity | Increase drive frequency to boost flow |
| 86 | High temperature with erratic current | Electrical fault in motor | Perform motor electrical diagnostics |
| 87 | Temperature stable but below normal | Overcooled motor | Verify fluid flow rate; adjust if excessive |
| 88 | Temperature rises with low pump efficiency | Worn pump stages reducing flow | Inspect and replace worn pump components |
| 89 | Periodic temperature drops | Intermittent high fluid flow | Analyze well inflow; stabilize choke settings |
| 90 | High temperature with normal intake pressure | Motor overload or friction | Check for mechanical binding; lubricate |
| 91 | Temperature spikes with low discharge pressure | Leaking pump seals | Inspect and replace pump seals |
| 92 | Gradual temperature increases with vibration | Bearing wear or misalignment | Monitor vibration; replace bearings if needed |
| 93 | Low temperature with high wellhead pressure | Downhole restriction reducing load | Investigate tubing or flowline blockages |
| 94 | Temperature rises during flow changes | Unstable reservoir conditions | Stabilize inflow with choke adjustments |
| 95 | High temperature with high fluid viscosity | Reduced cooling efficiency | Adjust VFD to match fluid conditions |
| 96 | Temperature drops during startup | Cold fluid influx | Monitor startup procedure; ensure stability |
| 97 | High temperature with stable pressure | Inefficient motor cooling | Evaluate motor cooling design; consider upgrade |
| 98 | Temperature spikes with electrical noise | Power supply instability | Install noise filters or stabilize power |
| 99 | Gradual temperature decreases with low current | Pump underload | Check pump sizing and well inflow |
| 100 | Erratic temperature with normal production | Controller or sensor communication issue | Test communication lines; replace faulty components |

3.	Intake/Discharge Pressure Behavior (Table 15)

Table 15. Intake/Discharge Pressure Behavior (Rules 101–160).

| Rule # | Observation | Possible Cause | Recommended Action |
| --- | --- | --- | --- |
| 101 | Intake pressure drops gradually | Reservoir depletion | Evaluate inflow performance relationship (IPR) |
| 102 | Discharge pressure oscillates | VFD instability or gas interference | Analyze VFD settings; optimize gas separator |
| 103 | Sudden intake pressure drop | Pump intake blockage | Inspect intake for debris or sand |
| 104 | High discharge pressure with low flow | Downhole restriction | Check tubing or flowline for blockages |
| 105 | Intake pressure spikes periodically | Gas slugging | Install or optimize gas handling device |
| 106 | Discharge pressure drops to zero | Pump trip or power failure | Check VFD logs and breaker status |
| 107 | Gradual increase in intake pressure | Scale or paraffin buildup | Perform chemical cleaning or hot oiling |
| 108 | Low intake pressure with high current | Pump cavitation | Adjust pump speed or install gas separator |
| 109 | Discharge pressure stable but low | Worn pump stages | Inspect and replace worn pump components |
| 110 | Intake pressure oscillates with stable current | Reservoir inflow instability | Adjust choke settings to stabilize flow |
| 111 | High intake pressure with low discharge pressure | Pump seal failure | Inspect and replace pump seals |
| 112 | Sudden discharge pressure spike | Surface choke restriction | Inspect and adjust surface choke |
| 113 | Intake pressure below normal with high flow | Oversized pump | Evaluate pump sizing; consider resizing |
| 114 | Discharge pressure spikes during startup | Fluid hammer | Install check valve; adjust startup procedure |
| 115 | Gradual discharge pressure decrease | Pump wear or impeller degradation | Pull ESP for inspection; replace worn parts |
| 116 | Intake pressure stable with low production | Reservoir underperformance | Conduct well test; evaluate stimulation options |
| 117 | High discharge pressure with high current | Pump overload | Reduce pump speed or check fluid properties |
| 118 | Periodic intake pressure drops | Cyclic gas interference | Optimize gas separator; adjust pump depth |
| 119 | Discharge pressure erratic with normal intake | VFD or controller fault | Test and recalibrate VFD |
| 120 | Low discharge pressure with high vibration | Pump misalignment or imbalance | Check pump alignment and balance |
| 121 | Intake pressure rises with high motor temperature | Reduced fluid flow | Increase pump speed or check for blockages |
| 122 | Discharge pressure drops with high intake pressure | Pump stage failure | Inspect pump internals for damage |
| 123 | Intake pressure spikes with low current | Gas lock in pump | Vent gas or install gas handling device |
| 124 | High discharge pressure with low intake pressure | Tubing restriction | Inspect tubing for scale or obstructions |
| 125 | Intake pressure stable but below normal | Inadequate reservoir pressure | Evaluate reservoir; consider artificial lift optimization |
| 126 | Discharge pressure spikes with high gas content | Gas interference in pump | Optimize gas separator or adjust pump speed |
| 127 | Gradual intake pressure decreases with stable flow | Sand production clogging intake | Implement sand control measures |
| 128 | Discharge pressure oscillates with high current | Electrical instability | Check power supply and grounding |
| 129 | Low intake pressure with normal discharge | Pump intake restriction | Clean intake screen or perforations |
| 130 | Discharge pressure drops during shutdown | Backflow or check valve failure | Inspect and replace check valve |
| 131 | Intake pressure spikes with high vibration | Pump mechanical issue | Check for broken shaft or bearing failure |
| 132 | High discharge pressure with low production | Partial pump blockage | Flush pump or perform acid job |
| 133 | Intake pressure erratic with normal production | Faulty pressure sensor | Calibrate or replace intake pressure sensor |
| 134 | Discharge pressure stable but above normal | Surface flowline restriction | Inspect flowline for blockages or valve issues |
| 135 | Intake pressure drops with high motor load | Excessive friction or pump wear | Inspect pump for mechanical issues |
| 136 | Discharge pressure spikes with low flow | High fluid viscosity or emulsion | Analyze fluid properties; adjust pump settings |
| 137 | Intake pressure rises with low discharge pressure | Pump efficiency loss | Evaluate pump performance; consider repair |
| 138 | Discharge pressure drops with increasing current | Pump stage wear | Pull ESP for stage inspection |
| 139 | Intake pressure stable with high wellhead pressure | Downhole restriction | Investigate tubing or flowline blockages |
| 140 | Discharge pressure erratic with high motor temperature | Overloaded pump | Reduce pump speed; check for mechanical binding |
| 141 | Intake pressure spikes during flow changes | Unstable reservoir conditions | Stabilize inflow with choke adjustments |
| 142 | Low discharge pressure with normal intake pressure | Leaking pump seals | Inspect and replace pump seals |
| 143 | Intake pressure drops with increasing vibration | Pump intake damage | Pull ESP for intake inspection |
| 144 | High discharge pressure with stable current | Surface equipment restriction | Check surface valves and flowlines |
| 145 | Intake pressure oscillates with high gas content | Gas interference reducing pump efficiency | Optimize gas separator or adjust pump depth |
| 146 | Discharge pressure drops with low motor temperature | Cold fluid influx | Monitor fluid temperature; ensure stable operation |
| 147 | Intake pressure rises with low production | Reservoir skin damage | Consider acidizing or fracturing |
| 148 | Discharge pressure spikes with electrical noise | Power supply instability | Install noise filters or stabilize power |
| 149 | Low intake pressure with high discharge pressure | Pump restriction or blockage | Inspect pump for debris or scale |
| 150 | Discharge pressure stable with low flow | Underperforming pump | Evaluate pump sizing and performance |
| 151 | Intake pressure drops during startup | Insufficient reservoir inflow | Check well inflow; adjust startup procedure |
| 152 | High discharge pressure with high vibration | Unbalanced pump or motor | Check balance and alignment |
| 153 | Intake pressure erratic with stable discharge | Reservoir heterogeneity | Analyze reservoir data; adjust choke settings |
| 154 | Discharge pressure drops with high fluid viscosity | Reduced pump efficiency | Adjust VFD to match fluid conditions |
| 155 | Intake pressure spikes with low discharge pressure | Gas lock or pump cavitation | Vent gas or adjust pump speed |
| 156 | Discharge pressure oscillates with normal current | Surface flow instability | Check surface equipment; stabilize flow |
| 157 | Low intake pressure with normal production | Oversized pump or high drawdown | Resize pump or optimize drawdown |
| 158 | Discharge pressure spikes with high motor load | Pump overload or restriction | Inspect pump for blockages or wear |
| 159 | Intake pressure stable but high | Excessive reservoir pressure | Adjust choke to reduce intake pressure |
| 160 | Discharge pressure erratic with sensor drift | Faulty discharge pressure sensor | Calibrate or replace discharge pressure sensor |

4.	Vibration Analysis (Table 16)

Table 16. Vibration Analysis (Vx, Vy, Vz) (Rules 161–200).

| Rule # | Observation | Possible Cause | Recommended Action |
| --- | --- | --- | --- |
| 161 | Increase in Vz only | Vertical shaft misalignment | Conduct downhole alignment check |
| 162 | All three axes increase | Worn bearings or unbalanced shaft | Pull pump for inspection |
| 163 | Sudden spike in Vx | Lateral impact or debris contact | Inspect pump housing for damage |
| 164 | High Vy with stable Vx, Vz | Horizontal misalignment | Check pump and motor alignment |
| 165 | Periodic Vz spikes | Gas slugging causing pump vibration | Optimize gas separator or adjust pump depth |
| 166 | All axes vibrate with high current | Pump overload or mechanical binding | Reduce pump speed; inspect for blockages |
| 167 | Gradual increase in Vx over time | Impeller wear or imbalance | Pull ESP for impeller inspection |
| 168 | High Vz with high motor temperature | Bearing failure | Replace bearings; check lubrication |
| 169 | Erratic Vx, Vy with normal Vz | Loose pump or motor components | Tighten connections; inspect mounting |
| 170 | Sudden drop in all vibration axes | Sensor failure | Calibrate or replace vibration sensors |
| 171 | High Vx with low intake pressure | Pump cavitation | Adjust pump speed or install gas separator |
| 172 | Periodic Vy spikes | Cyclic reservoir inflow | Stabilize inflow with choke adjustments |
| 173 | High Vz with low discharge pressure | Broken pump shaft | Pull ESP for shaft inspection |
| 174 | All axes increase with high current | Unbalanced rotor or impeller | Balance rotor; inspect impeller condition |
| 175 | Gradual Vx, Vy increase | Scale or sand accumulation | Perform chemical cleaning or sand control |
| 176 | High Vz during startup | Excessive starting torque | Adjust VFD startup settings |
| 177 | Erratic Vx with high motor load | Worn couplings | Inspect and replace couplings |
| 178 | High Vy with normal current | Lateral pump instability | Check pump installation and alignment |
| 179 | Periodic Vz drops | Intermittent fluid flow | Analyze fluid flow; stabilize well conditions |
| 180 | High Vx, Vy with low Vz | Pump housing damage | Pull ESP for housing inspection |
| 181 | Vibration spikes with gas content increase | Gas interference causing imbalance | Optimize gas separator; adjust pump depth |
| 182 | All axes stable but above normal | Improper pump installation | Reinstall ESP with proper alignment |
| 183 | High Vz with low flow | Pump restriction or blockage | Inspect pump for debris or scale |
| 184 | Gradual Vx increase with high temperature | Bearing wear | Monitor bearing condition; replace if needed |
| 185 | High Vy during shutdown | Backspin or fluid hammer | Install check valve; adjust shutdown procedure |
| 186 | Erratic Vx, Vy, Vz with normal production | Faulty vibration sensor | Calibrate or replace vibration sensors |
| 187 | High Vz with high discharge pressure | Pump stage restriction | Inspect pump stages for blockages |
| 188 | Periodic Vx spikes with stable flow | Unbalanced impeller | Balance impeller or replace if damaged |
| 189 | High Vx with low motor temperature | Cold fluid influx causing imbalance | Monitor fluid temperature; stabilize operation |
| 190 | All axes increase with low intake pressure | Cavitation or gas lock | Vent gas or adjust pump speed |
| 191 | High Vy with high current | Motor misalignment | Realign motor and pump assembly |
| 192 | Gradual Vz increase with stable pressure | Shaft wear or deformation | Pull ESP for shaft inspection |
| 193 | High Vx with erratic current | Electrical fault causing vibration | Perform motor electrical diagnostics |
| 194 | Periodic Vy drops with high flow | Intermittent pump efficiency loss | Inspect pump for partial blockages |
| 195 | High Vz with high wellhead pressure | Downhole restriction | Investigate tubing or flowline blockages |
| 196 | All axes vibrate with low production | Pump underload or wear | Evaluate pump sizing; inspect for wear |
| 197 | High Vx during flow changes | Unstable reservoir conditions | Stabilize inflow with choke adjustments |
| 198 | High Vy with high fluid viscosity | Reduced pump efficiency | Adjust VFD to match fluid conditions |
| 199 | Erratic Vz with normal intake pressure | Mechanical looseness | Inspect pump and motor for loose components |
| 200 | All axes increase with high motor temperature | Overloaded or seized pump | Reduce pump speed; inspect for mechanical issues |

5.	Voltage, VFD Current, Electrical Faults (Table 17)

Table 17. Voltage, VFD Current, Electrical Faults (Rules 201–250).

| Rule # | Observation | Possible Cause | Recommended Action |
| --- | --- | --- | --- |
| 201 | Sudden voltage drops to zero | Power supply failure | Check surface power supply and breakers |
| 202 | High VFD current with stable voltage | Motor overload | Reduce pump speed; inspect for mechanical binding |
| 203 | Periodic voltage spikes | Electrical surges from grid | Install surge protection; check grid stability |
| 204 | Erratic VFD current with normal voltage | Faulty VFD controller | Test and recalibrate VFD |
| 205 | Low voltage with high motor temperature | Insufficient power supply | Check transformer capacity; upgrade if needed |
| 206 | High VFD current with low intake pressure | Pump cavitation or gas lock | Adjust pump speed or install gas separator |
| 207 | Voltage fluctuations with stable current | Loose electrical connections | Inspect and tighten all connections |
| 208 | Sudden VFD current spike followed by trip | Motor short circuit | Test motor insulation; inspect windings |
| 209 | Gradual increase in VFD current | Scale or sand accumulation | Perform chemical cleaning or sand control |
| 210 | Low VFD current with high discharge pressure | Pump restriction | Inspect pump and tubing for blockages |
| 211 | Voltage stable but below rated value | Undersized power cable | Evaluate cable sizing; replace if necessary |
| 212 | High VFD current with high vibration | Unbalanced pump or motor | Check alignment and balance |
| 213 | Periodic VFD current drops | Intermittent power supply | Check power cables and transformer stability |
| 214 | Voltage spikes with high motor load | Electrical noise interference | Install noise filters on power lines |
| 215 | Low VFD current with low production | Underloaded pump | Resize pump or adjust VFD settings |
| 216 | High voltage with normal VFD current | Overvoltage from power supply | Adjust transformer tap settings |
| 217 | Erratic voltage with high motor temperature | Grounding issue | Inspect grounding system; correct faults |
| 218 | Sudden VFD current drop | Cable fault or disconnection | Inspect power cable for damage |
| 219 | High VFD current during startup | High starting torque | Adjust VFD startup ramp settings |
| 220 | Voltage drops during flow changes | Unstable power supply | Stabilize power source; check grid capacity |
| 221 | Low VFD current with high intake pressure | Pump efficiency loss | Inspect pump for wear or damage |
| 222 | High voltage with erratic VFD current | VFD harmonic distortion | Install harmonic filters |
| 223 | Periodic voltage drops with stable production | Intermittent grid issues | Monitor grid performance; install stabilizer |
| 224 | High VFD current with low flow | High fluid viscosity or emulsion | Analyze fluid properties; adjust VFD settings |
| 225 | Voltage stable but high with low current | Oversized power supply | Evaluate power system design |
| 226 | Erratic VFD current with high gas content | Gas interference in pump | Optimize gas separator or adjust pump depth |
| 227 | Low voltage with high VFD current | Motor insulation degradation | Test insulation resistance; consider motor replacement |
| 228 | High VFD current with high wellhead pressure | Downhole restriction | Investigate tubing or flowline blockages |
| 229 | Voltage spikes during shutdown | Backspin or electrical feedback | Install check valve; adjust shutdown procedure |
| 230 | Low VFD current with normal voltage | Pump underload | Check pump sizing and well inflow |
| 231 | High voltage with low motor temperature | Cold fluid influx | Monitor fluid temperature; ensure stable operation |
| 232 | Erratic voltage with normal production | Faulty voltage sensor | Calibrate or replace voltage sensor |
| 233 | High VFD current with high motor temperature | Overloaded motor | Reduce pump speed; check for mechanical issues |
| 234 | Low voltage during startup | Insufficient transformer capacity | Upgrade transformer or adjust startup settings |
| 235 | Periodic VFD current spikes | Electrical surges or noise | Install surge protection or noise filters |
| 236 | High voltage with low discharge pressure | Leaking pump seals | Inspect and replace pump seals |
| 237 | Low VFD current with high vibration | Broken pump shaft | Pull ESP for shaft inspection |
| 238 | Voltage drops with increasing current | Power supply overload | Check power system capacity; upgrade if needed |
| 239 | High VFD current with stable intake pressure | Pump restriction or wear | Inspect pump for blockages or wear |
| 240 | Erratic VFD current with normal voltage | VFD communication fault | Test communication lines; replace faulty components |
| 241 | Low voltage with high discharge pressure | Surface equipment restriction | Check surface valves and flowlines |
| 242 | High VFD current during flow changes | Unstable reservoir conditions | Stabilize inflow with choke adjustments |
| 243 | Voltage spikes with high fluid viscosity | Reduced motor efficiency | Adjust VFD to match fluid conditions |
| 244 | Low VFD current with low motor temperature | Pump underperforming | Evaluate pump sizing and performance |
| 245 | High voltage with erratic current | Electrical fault in motor | Perform motor electrical diagnostics |
| 246 | Periodic voltage spikes with stable current | Grid instability | Install voltage stabilizer; monitor grid |
| 247 | Low VFD current with high gas content | Gas interference reducing load | Optimize gas separator or adjust pump depth |
| 248 | High VFD current with low intake pressure | Pump cavitation | Adjust pump speed or install gas separator |
| 249 | Voltage drops with high motor load | Undersized power supply | Evaluate and upgrade power system |
| 250 | Erratic voltage and VFD current | Controller or sensor drift | Test and recalibrate all electrical sensors |

6.	Derived Parameters & Combined Conditions (Table 18)

Table 18. Derived Parameters & Combined Conditions (Rules 251–300).

| Rule # | Observation | Possible Cause | Recommended Action |
| --- | --- | --- | --- |
| 251 | Low pump efficiency with high motor current | Worn pump stages | Inspect and replace worn pump components |
| 252 | High motor load with low intake pressure | Pump cavitation or gas lock | Adjust pump speed or install gas separator |
| 253 | Increased power consumption with stable production | Scale or paraffin buildup | Perform chemical cleaning or hot oiling |
| 254 | High motor temperature and high VFD current | Motor overload | Reduce pump speed; check for mechanical binding |
| 255 | Low pump efficiency with high vibration (all axes) | Unbalanced pump or impeller | Balance impeller; inspect pump alignment |
| 256 | High discharge pressure with low motor load | Oversized pump | Evaluate pump sizing; consider resizing |
| 257 | Erratic power consumption with voltage fluctuations | Unstable power supply | Stabilize power source; install voltage stabilizer |
| 258 | High motor temperature with low intake pressure | Insufficient cooling due to low flow | Increase pump speed or check well inflow |
| 259 | Low pump efficiency with high discharge pressure | Downhole restriction | Inspect tubing or flowline for blockages |
| 260 | High VFD current and high Vz vibration | Bearing failure or shaft misalignment | Pull ESP for bearing and shaft inspection |
| 261 | Sudden drop in pump efficiency with high current | Pump intake blockage | Inspect intake for debris or sand |
| 262 | High motor load with erratic VFD current | Electrical fault in motor | Perform motor electrical diagnostics |
| 263 | Low power consumption with high wellhead pressure | Pump underload or restriction | Check pump sizing; inspect flowlines |
| 264 | High motor temperature with periodic Vx spikes | Loose pump components | Tighten connections; inspect mounting |
| 265 | Low pump efficiency with high gas content | Gas interference reducing pump performance | Optimize gas separator or adjust pump depth |
| 266 | High power consumption with low discharge pressure | Leaking pump seals | Inspect and replace pump seals |
| 267 | High motor temperature and high intake pressure | Reduced fluid flow | Check for blockages; increase pump speed |
| 268 | Low pump efficiency with high VFD current | Pump stage wear or damage | Pull ESP for stage inspection |
| 269 | Erratic power consumption with stable voltage | Faulty VFD controller | Test and recalibrate VFD |
| 270 | High motor load with high Vz vibration | Broken pump shaft | Pull ESP for shaft inspection |
| 271 | Low pump efficiency with high fluid viscosity | Reduced pump performance | Adjust VFD to match fluid conditions |
| 272 | High power consumption with high motor temperature | Motor insulation degradation | Test insulation resistance; consider motor replacement |
| 273 | Low motor load with high intake pressure | Pump efficiency loss | Inspect pump for wear or damage |
| 274 | High VFD current with periodic voltage spikes | Electrical surges or grid instability | Install surge protection or noise filters |
| 275 | Low pump efficiency with high vibration (Vy) | Horizontal misalignment | Check pump and motor alignment |
| 276 | High power consumption during startup | High starting torque | Adjust VFD startup ramp settings |
| 277 | Low motor load with low discharge pressure | Pump underperforming | Evaluate pump sizing and performance |
| 278 | High motor temperature with erratic current | Electrical fault causing overheating | Perform motor electrical diagnostics |
| 279 | Low pump efficiency with high wellhead pressure | Downhole or surface restriction | Investigate tubing or flowline blockages |
| 280 | High VFD current with low motor temperature | Cold fluid influx reducing load | Monitor fluid temperature; ensure stable operation |
| 281 | High power consumption with high vibration (all axes) | Unbalanced rotor or impeller | Balance rotor; inspect impeller condition |
| 282 | Low pump efficiency with periodic Vz spikes | Gas slugging affecting pump | Optimize gas separator; adjust pump depth |
| 283 | High motor load with low voltage | Undersized power supply | Evaluate and upgrade power system |
| 284 | Erratic power consumption with high gas content | Gas interference in pump | Optimize gas separator or adjust pump depth |
| 285 | High motor temperature with high discharge pressure | Pump overload or restriction | Inspect pump for blockages or wear |
| 286 | Low pump efficiency with low intake pressure | Pump cavitation or gas lock | Vent gas or adjust pump speed |
| 287 | High power consumption with stable current | Scale or sand accumulation | Perform chemical cleaning or sand control |
| 288 | High motor load with high Vx vibration | Lateral pump instability | Check pump installation and alignment |
| 289 | Low pump efficiency with erratic VFD current | Faulty VFD or sensor drift | Test and recalibrate VFD and sensors |
| 290 | High motor temperature with low production | Insufficient cooling or pump wear | Inspect pump and increase fluid flow |
| 291 | High power consumption with high intake pressure | Pump restriction or inefficiency | Inspect pump for blockages or wear |
| 292 | Low motor load with high vibration (Vz) | Broken shaft or bearing failure | Pull ESP for inspection |
| 293 | High VFD current during flow changes | Unstable reservoir conditions | Stabilize inflow with choke adjustments |
| 294 | Low pump efficiency with high motor load | Worn pump components | Inspect and replace worn pump parts |
| 295 | High power consumption with low flow | High fluid viscosity or emulsion | Analyze fluid properties; adjust VFD settings |
| 296 | High motor temperature with voltage spikes | Electrical surges causing overheating | Install surge protection; stabilize power |
| 297 | Low pump efficiency with high Vz vibration | Vertical shaft misalignment | Conduct downhole alignment check |
| 298 | High motor load with low discharge pressure | Pump seal failure | Inspect and replace pump seals |
| 299 | Erratic power consumption with high wellhead pressure | Surface flow restriction | Check surface valves and flowlines |
| 300 | Low motor load with erratic sensor readings | Sensor or controller communication issue | Test communication lines; replace faulty components |

7.	Anomaly Detection via Statistical Methods (Table 19)

Table 19. Anomaly Detection via Statistical Methods (Rules 301–350).

| Rule # | Observation | Possible Cause | Recommended Action |
| --- | --- | --- | --- |
| 301 | Motor current exceeds 3σ above moving average | Motor overload or mechanical binding | Reduce pump speed; inspect for blockages |
| 302 | Motor temperature deviates >2σ from historical mean | Insufficient cooling or scale buildup | Check fluid flow; perform chemical cleaning |
| 303 | Intake pressure drops below 2σ of baseline | Reservoir depletion or intake blockage | Evaluate inflow performance; inspect intake |
| 304 | Discharge pressure shows >3σ oscillation amplitude | Gas interference or VFD instability | Optimize gas separator; recalibrate VFD |
| 305 | Vz vibration exceeds 3σ above normal | Vertical shaft misalignment | Conduct downhole alignment check |
| 306 | Power consumption spikes >2σ with stable production | Scale or paraffin accumulation | Perform chemical cleaning or hot oiling |
| 307 | Motor current shows cyclic pattern outside 2σ | Gas slugging or reservoir instability | Install gas handling device; adjust choke |
| 308 | Motor temperature trends upward >2σ over 24 hours | Reduced cooling or motor overload | Increase pump speed or check fluid flow |
| 309 | Intake pressure deviates below 3σ with high current | Pump cavitation | Adjust pump speed or install gas separator |
| 310 | Discharge pressure drops >2σ with normal current | Pump wear or seal failure | Inspect pump seals and stages |
| 311 | Vx vibration exceeds 3σ during startup | Excessive starting torque | Adjust VFD startup ramp settings |
| 312 | VFD current shows >3σ spikes periodically | Electrical surges or noise | Install surge protection or noise filters |
| 313 | Motor load exceeds 2σ above normal with low flow | Pump restriction or high viscosity | Inspect pump; analyze fluid properties |
| 314 | Pump efficiency drops below 2σ of baseline | Worn pump stages | Pull ESP for inspection; replace worn parts |
| 315 | Voltage deviates >3σ from rated value | Power supply instability | Stabilize power source; check transformer |
| 316 | High Vz vibration with motor temperature >2σ | Bearing failure | Replace bearings; check lubrication |
| 317 | Intake pressure shows cyclic drops >2σ | Cyclic reservoir inflow | Stabilize inflow with choke adjustments |
| 318 | Discharge pressure exceeds 3σ with low production | Downhole restriction | Inspect tubing or flowline for blockages |
| 319 | Motor current trends downward >2σ over days | Pump underload | Check pump sizing and well inflow |
| 320 | Vx, Vy, Vz vibrations all exceed 2σ | Unbalanced pump or motor | Balance rotor; inspect impeller condition |
| 321 | Power consumption deviates >3σ with high gas content | Gas interference in pump | Optimize gas separator or adjust pump depth |
| 322 | Motor temperature spikes >2σ with normal current | Sensor malfunction or hot spot | Calibrate sensor; perform motor diagnostics |
| 323 | Intake pressure drops >3σ with high vibration | Pump intake damage | Pull ESP for intake inspection |
| 324 | VFD current exceeds 3σ with high motor load | Motor insulation degradation | Test insulation resistance; consider replacement |
| 325 | Pump efficiency trends downward >2σ over weeks | Gradual pump wear | Schedule pump inspection and maintenance |
| 326 | Voltage shows periodic drops >2σ | Intermittent grid issues | Monitor grid; install voltage stabilizer |
| 327 | Motor temperature exceeds 3σ with low intake pressure | Insufficient cooling | Increase pump speed or check well inflow |
| 328 | Discharge pressure oscillates >2σ with high current | Electrical instability or pump restriction | Check power supply; inspect pump |
| 329 | Vx vibration spikes >3σ with high fluid viscosity | Reduced pump efficiency | Adjust VFD to match fluid conditions |
| 330 | Power consumption exceeds 2σ during flow changes | Unstable reservoir conditions | Stabilize inflow with choke adjustments |
| 331 | Motor current deviates >3σ with high wellhead pressure | Downhole restriction | Investigate tubing or flowline blockages |
| 332 | Pump efficiency drops >2σ with high motor temperature | Overloaded pump | Reduce pump speed; inspect for mechanical issues |
| 333 | VFD current shows erratic pattern >3σ | Faulty VFD or sensor drift | Test and recalibrate VFD and sensors |
| 334 | Intake pressure exceeds 3σ with low discharge pressure | Pump seal failure | Inspect and replace pump seals |
| 335 | Motor temperature trends upward >2σ with high Vz | Shaft misalignment or bearing wear | Conduct alignment check; replace bearings |
| 336 | Power consumption spikes >3σ with low flow | High fluid viscosity or emulsion | Analyze fluid properties; adjust VFD settings |
| 337 | Voltage deviates below 2σ with high VFD current | Undersized power supply | Evaluate and upgrade power system |
| 338 | Vx, Vy vibrations exceed 2σ with low motor load | Loose pump components | Tighten connections; inspect mounting |
| 339 | Pump efficiency drops >3σ with high gas content | Gas interference reducing performance | Optimize gas separator or adjust pump depth |
| 340 | Motor current spikes >2σ during startup | High starting torque or seized pump | Adjust VFD settings; inspect for binding |
| 341 | Discharge pressure deviates >3σ with low intake pressure | Pump restriction or blockage | Inspect pump for debris or scale |
| 342 | Motor temperature exceeds 2σ with erratic current | Electrical fault causing overheating | Perform motor electrical diagnostics |
| 343 | VFD current trends upward >2σ over days | Scale or sand accumulation | Perform chemical cleaning or sand control |
| 344 | Intake pressure shows >3σ oscillation with normal production | Faulty pressure sensor | Calibrate or replace intake pressure sensor |
| 345 | Pump efficiency drops >2σ with high vibration (all axes) | Unbalanced rotor or impeller | Balance rotor; inspect impeller condition |
| 346 | Power consumption exceeds 3σ with high motor temperature | Motor overload or insulation failure | Reduce pump speed; test insulation resistance |
| 347 | Voltage spikes >2σ with high VFD current | Grid instability or electrical noise | Install noise filters or stabilize power |
| 348 | Motor load exceeds 3σ with low discharge pressure | Pump seal failure or wear | Inspect and replace pump seals or components |
| 349 | Vz vibration deviates >2σ with high wellhead pressure | Downhole restriction | Investigate tubing or flowline blockages |
| 350 | Erratic power consumption with normal sensor readings | Controller communication issue | Test communication lines; replace faulty components |

8.	Machine Learning & Pattern Recognition Findings (Table 20)

Table 20. Machine Learning & Pattern Recognition Findings (Rules 351–400).

| Rule # | Observation | Possible Cause | Recommended Action |
| --- | --- | --- | --- |
| 351 | ML model detects anomalous motor current pattern | Motor overload or early insulation failure | Inspect motor; perform insulation resistance test |
| 352 | Clustering identifies high motor temperature outlier | Insufficient cooling or scale buildup | Check fluid flow; perform chemical cleaning |
| 353 | Regression model predicts intake pressure decline | Reservoir depletion | Evaluate inflow performance relationship (IPR) |
| 354 | Anomaly detection flags oscillating discharge pressure | Gas interference or VFD instability | Optimize gas separator; recalibrate VFD |
| 355 | Pattern recognition detects high Vz vibration cluster | Vertical shaft misalignment | Conduct downhole alignment check |
| 356 | ML model flags high power consumption anomaly | Scale or paraffin accumulation | Perform chemical cleaning or hot oiling |
| 357 | Classification model predicts cyclic current pattern | Gas slugging or reservoir instability | Install gas handling device; adjust choke |
| 358 | Neural network detects rising motor temperature trend | Reduced cooling or motor overload | Increase pump speed or check fluid flow |
| 359 | Anomaly detection flags low intake pressure with high current | Pump cavitation | Adjust pump speed or install gas separator |
| 360 | ML model identifies low pump efficiency pattern | Worn pump stages | Pull ESP for inspection; replace worn parts |
| 361 | Clustering detects Vx vibration spike during startup | Excessive starting torque | Adjust VFD startup ramp settings |
| 362 | Anomaly detection flags periodic VFD current spikes | Electrical surges or noise | Install surge protection or noise filters |
| 363 | Regression model predicts high motor load with low flow | Pump restriction or high viscosity | Inspect pump; analyze fluid properties |
| 364 | ML model flags pump efficiency drop outlier | Gradual pump wear | Schedule pump inspection and maintenance |
| 365 | Pattern recognition detects voltage deviation anomaly | Power supply instability | Stabilize power source; check transformer |
| 366 | Classification model identifies high Vz with high temperature | Bearing failure | Replace bearings; check lubrication |
| 367 | Anomaly detection flags cyclic intake pressure drops | Cyclic reservoir inflow | Stabilize inflow with choke adjustments |
| 368 | ML model predicts high discharge pressure with low production | Downhole restriction | Inspect tubing or flowline for blockages |
| 369 | Clustering identifies low motor current trend | Pump underload | Check pump sizing and well inflow |
| 370 | Neural network detects high vibration in all axes | Unbalanced pump or motor | Balance rotor; inspect impeller condition |
| 371 | Anomaly detection flags power consumption with high gas content | Gas interference in pump | Optimize gas separator or adjust pump depth |
| 372 | ML model identifies motor temperature spike with normal current | Sensor malfunction or hot spot | Calibrate sensor; perform motor diagnostics |
| 373 | Pattern recognition detects intake pressure drop with high vibration | Pump intake damage | Pull ESP for intake inspection |
| 374 | Classification model predicts high VFD current with high load | Motor insulation degradation | Test insulation resistance; consider replacement |
| 375 | Regression model flags pump efficiency declines over weeks | Gradual pump wear | Schedule pump inspection and maintenance |
| 376 | Anomaly detection identifies periodic voltage drops | Intermittent grid issues | Monitor grid; install voltage stabilizer |
| 377 | ML model flags high motor temperature with low intake pressure | Insufficient cooling | Increase pump speed or check well inflow |
| 378 | Clustering detects oscillating discharge pressure with high current | Electrical instability or pump restriction | Check power supply; inspect pump |
| 379 | Pattern recognition flags high Vx with high fluid viscosity | Reduced pump efficiency | Adjust VFD to match fluid conditions |
| 380 | Regression model predicts power consumption spike during flow changes | Unstable reservoir conditions | Stabilize inflow with choke adjustments |
| 381 | Anomaly detection flags high motor current with high wellhead pressure | Downhole restriction | Investigate tubing or flowline blockages |
| 382 | ML model identifies low pump efficiency with high motor temperature | Overloaded pump | Reduce pump speed; inspect for mechanical issues |
| 383 | Classification model detects erratic VFD current pattern | Faulty VFD or sensor drift | Test and recalibrate VFD and sensors |
| 384 | Neural network flags high intake pressure with low discharge | Pump seal failure | Inspect and replace pump seals |
| 385 | Anomaly detection identifies high motor temperature with high Vz | Shaft misalignment or bearing wear | Conduct alignment check; replace bearings |
| 386 | ML model predicts high power consumption with low flow | High fluid viscosity or emulsion | Analyze fluid properties; adjust VFD settings |
| 387 | Clustering detects low voltage with high VFD current | Undersized power supply | Evaluate and upgrade power system |
| 388 | Pattern recognition flags high Vx, Vy with low motor load | Loose pump components | Tighten connections; inspect mounting |
| 389 | Regression model predicts low pump efficiency with high gas content | Gas interference reducing performance | Optimize gas separator or adjust pump depth |
| 390 | Anomaly detection flags motor current spike during startup | High starting torque or seized pump | Adjust VFD settings; inspect for binding |
| 391 | ML model identifies high discharge pressure with low intake | Pump restriction or blockage | Inspect pump for debris or scale |
| 392 | Classification model detects high motor temperature with erratic current | Electrical fault causing overheating | Perform motor electrical diagnostics |
| 393 | Neural network flags rising VFD current trend over days | Scale or sand accumulation | Perform chemical cleaning or sand control |
| 394 | Anomaly detection identifies erratic intake pressure with normal production | Faulty pressure sensor | Calibrate or replace intake pressure sensor |
| 395 | ML model predicts low pump efficiency with high vibration | Unbalanced rotor or impeller | Balance rotor; inspect impeller condition |
| 396 | Clustering detects high power consumption with high motor temperature | Motor overload or insulation failure | Reduce pump speed; test insulation resistance |
| 397 | Pattern recognition flags voltage spikes with high VFD current | Grid instability or electrical noise | Install noise filters or stabilize power |
| 398 | Regression model predicts low pump efficiency with high motor load | Pump seal failure or wear | Inspect and replace pump seals or components |
| 399 | Anomaly detection flags high Vz with high wellhead pressure | Downhole restriction | Investigate tubing or flowline blockages |
| 400 | ML model identifies erratic power consumption with normal sensors | Controller communication issue | Test communication lines; replace faulty components |

9.	Expert Rules from Field Experience (Table 21)

Table 21. Expert Rules from Field Experience (Rules 401–450).

| Rule # | Observation | Possible Cause | Recommended Action |
| --- | --- | --- | --- |
| 401 | Motor current spikes after extended runtime | Gradual scale buildup on pump | Perform acid backwash; monitor fluid chemistry |
| 402 | Motor temperature rises with stable current in high-gas wells | Gas locking reducing cooling flow | Install high-capacity gas separator; adjust pump depth |
| 403 | Intake pressure drops with sand production spikes | Sand clogging pump intake | Implement sand control (e.g., gravel pack); clean intake |
| 404 | Discharge pressure drops after choke adjustments | Surface choke miscalibration | Recalibrate choke; verify surface flow settings |
| 405 | High Vz vibration after recent ESP installation | Improper pump alignment during setup | Pull ESP; re-align pump and motor |
| 406 | High VFD current with low flow in heavy oil wells | High fluid viscosity overloading pump | Adjust VFD speed; consider hot oil treatment |
| 407 | Motor temperature spikes post-shutdown | Heat soak from stagnant fluid | Adjust shutdown procedure; ensure check valve functionality |
| 408 | Erratic intake pressure in mature wells | Reservoir heterogeneity or skin damage | Conduct well test; consider stimulation (e.g., acidizing) |
| 409 | High motor current with low pump efficiency | Worn impellers from abrasive fluids | Pull ESP; replace worn impellers |
| 410 | Periodic voltage drops in remote fields | Weak grid infrastructure | Install voltage stabilizer; upgrade transformer capacity |
| 411 | High Vx vibration after pump restart | Debris lodged in pump housing | Flush pump; inspect for foreign material |
| 412 | Low discharge pressure with high wellhead pressure | Tubing leak or blockage | Pressure test tubing; inspect for leaks |
| 413 | Motor temperature rises with high water cut | Reduced cooling due to water-dominated flow | Increase pump speed; evaluate water handling |
| 414 | High VFD current after scale treatment | Incomplete scale removal | Extend chemical soak time; repeat treatment |
| 415 | Intake pressure spikes with high gas-oil ratio (GOR) | Gas interference in pump | Optimize gas separator; vent gas if possible |
| 416 | High power consumption with stable production in old wells | Gradual pump wear | Schedule ESP inspection; plan for replacement |
| 417 | Erratic VFD current in high-temperature reservoirs | VFD overheating or controller fault | Check VFD cooling system; recalibrate controller |
| 418 | Low pump efficiency with high motor load in sandy wells | Sand erosion of pump stages | Install sand-resistant pump; implement sand control |
| 419 | High Vz vibration with high motor temperature | Bearing seizure from overheating | Replace bearings; improve motor cooling |
| 420 | Motor current drops with stable intake pressure | Pump underload due to oversized design | Resize pump to match well inflow |
| 421 | Discharge pressure oscillates after VFD tuning | Incorrect VFD frequency settings | Re-tune VFD; verify optimal frequency |
| 422 | High Vx, Vy vibrations in deviated wells | Pump instability from well trajectory | Reinstall ESP with stabilizers; check alignment |
| 423 | Motor temperature spikes with low flow in viscous wells | Insufficient cooling from low fluid velocity | Increase pump speed; consider viscosity reducer |
| 424 | High VFD current with low voltage in rural fields | Undersized power cables | Replace cables with higher capacity |
| 425 | Intake pressure drops after workover | Incomplete well cleanup | Perform additional well flushing; verify perforations |
| 426 | High power consumption with high GOR | Gas interference reducing pump efficiency | Install gas handling device; adjust pump depth |
| 427 | Erratic motor current after power outages | Electrical surge damage to motor | Test motor insulation; install surge protection |
| 428 | Low discharge pressure with high vibration | Broken pump shaft | Pull ESP; inspect and replace shaft |
| 429 | High motor temperature with stable production | Inadequate motor cooling design | Evaluate motor cooling; consider redesign |
| 430 | Periodic VFD current spikes in high-sand wells | Sand causing intermittent blockages | Implement sand control; flush pump regularly |
| 431 | Intake pressure spikes with low motor load | Gas lock in pump | Vent gas; install gas separator |
| 432 | High Vz vibration after scale treatment | Residual scale causing imbalance | Repeat chemical treatment; inspect pump |
| 433 | Low pump efficiency with high wellhead pressure | Surface flowline restriction | Inspect surface valves and flowlines |
| 434 | Motor current spikes with high fluid viscosity | Pump overload from heavy oil | Adjust VFD speed; consider chemical thinner |
| 435 | High VFD current with erratic voltage | Grid instability or electrical noise | Install noise filters; stabilize power supply |
| 436 | Low intake pressure with high production | Oversized pump causing excessive drawdown | Resize pump; optimize drawdown settings |
| 437 | High motor temperature after restart | Fluid hammer from rapid startup | Adjust VFD ramp-up settings; install check valve |
| 438 | Discharge pressure drops with high motor load | Pump stage failure | Pull ESP; inspect pump stages |
| 439 | High Vx vibration with low flow | Debris or scale in pump housing | Flush pump; perform chemical cleaning |
| 440 | Erratic intake pressure with high gas content | Gas slugging affecting pump | Optimize gas separator; stabilize well conditions |
| 441 | High power consumption with low motor temperature | Cold fluid influx reducing load | Monitor fluid temperature; ensure stable operation |
| 442 | Motor current spikes with high vibration | Unbalanced impeller or rotor | Balance rotor; inspect impeller condition |
| 443 | Low discharge pressure in high-GOR wells | Gas interference reducing pump lift | Install gas handling device; adjust pump depth |
| 444 | High VFD current after well stimulation | Residual stimulation fluid affecting pump | Flush well; monitor fluid properties |
| 445 | Motor temperature rises with high sand production | Sand abrasion reducing cooling efficiency | Implement sand control; inspect motor housing |
| 446 | Periodic voltage spikes in offshore fields | Subsea cable degradation | Inspect and repair subsea power cables |
| 447 | Low pump efficiency with high discharge pressure | Partial pump blockage | Flush pump or perform acid job |
| 448 | High Vz vibration with low intake pressure | Pump cavitation from low inflow | Adjust pump speed; evaluate well inflow |
| 449 | Erratic motor current with normal sensors | Controller communication failure | Test communication lines; replace faulty components |
| 450 | High power consumption with high wellhead pressure | Surface choke or valve restriction | Adjust surface choke; inspect flowlines |

10.	Rule-based System for Real-Time Monitoring (Table 22)

Table 22. Rule-based System for Real-Time Monitoring (Rules 451–500).

| Rule # | Observation | Possible Cause | Recommended Action |
| --- | --- | --- | --- |
| 451 | Motor current exceeds threshold by 20% | Motor overload or blockage | Reduce VFD speed by 5%; inspect for blockages |
| 452 | Motor temperature rises 10°C above normal in 1 hour | Insufficient cooling | Increase pump speed by 10%; check fluid flow |
| 453 | Intake pressure drops below 80% of baseline | Pump intake blockage | Trigger intake flush; inspect for debris |
| 454 | Discharge pressure oscillates ±15% in 5 minutes | Gas interference or VFD instability | Optimize gas separator; check VFD settings |
| 455 | Vz vibration exceeds 2x normal amplitude | Vertical shaft misalignment | Alert operator; schedule alignment check |
| 456 | Power consumption spikes 15% with stable flow | Scale or paraffin buildup | Initiate chemical injection; monitor response |
| 457 | Motor current shows cyclic spikes every 10 minutes | Gas slugging | Vent gas; adjust gas separator settings |
| 458 | Motor temperature exceeds 120°C | Overheating from low flow | Reduce pump speed; check well inflow |
| 459 | Intake pressure below 50 psi with high current | Pump cavitation | Reduce pump speed by 10%; install gas separator |
| 460 | Discharge pressure drops below 70% of normal | Pump wear or seal failure | Alert operator; schedule pump inspection |
| 461 | Vx vibration spikes 3x normal during startup | High starting torque | Adjust VFD ramp-up time by 2 seconds |
| 462 | VFD current spikes 20% above normal | Electrical surge | Activate surge protection; check power supply |
| 463 | Motor load exceeds 90% with low flow | Pump restriction or high viscosity | Reduce pump speed; analyze fluid properties |
| 464 | Pump efficiency drops below 60% | Worn pump stages | Alert operator; plan ESP pull for inspection |
| 465 | Voltage drops below 90% of rated value | Power supply instability | Switch to backup power; check transformer |
| 466 | High Vz vibration with motor temperature >110°C | Bearing failure | Stop pump; replace bearings |
| 467 | Intake pressure drops cyclically every 15 minutes | Reservoir inflow instability | Adjust choke by 5%; monitor inflow |
| 468 | Discharge pressure exceeds 120% of normal | Downhole restriction | Inspect tubing; clear blockages |
| 469 | Motor current drops 20% below normal | Pump underload | Increase pump speed by 5%; check sizing |
| 470 | Vx, Vy, Vz vibrations all exceed 2x normal | Unbalanced pump or motor | Stop pump; balance rotor or inspect impeller |
| 471 | Power consumption spikes with high GOR | Gas interference | Adjust gas separator; reduce pump depth |
| 472 | Motor temperature spikes with normal current | Sensor fault or hot spot | Calibrate sensor; run motor diagnostics |
| 473 | Intake pressure drops with high Vx vibration | Pump intake damage | Stop pump; inspect intake |
| 474 | VFD current exceeds 25% above normal with high load | Motor insulation failure | Stop pump; test insulation resistance |
| 475 | Pump efficiency trends below 65% over 6 hours | Gradual pump wear | Alert operator; schedule maintenance |
| 476 | Voltage drops periodically by 10% | Intermittent grid issue | Activate voltage stabilizer; monitor grid |
| 477 | Motor temperature exceeds 115°C with low intake pressure | Insufficient cooling | Increase pump speed by 10%; check inflow |
| 478 | Discharge pressure oscillates with high current | Pump restriction or electrical fault | Inspect pump; check power supply |
| 479 | Vx vibration spikes with high fluid viscosity | Reduced pump efficiency | Adjust VFD speed; add viscosity reducer |
| 480 | Power consumption spikes during choke adjustments | Unstable reservoir conditions | Stabilize choke; monitor inflow |
| 481 | Motor current exceeds threshold with high wellhead pressure | Downhole restriction | Inspect tubing or flowline for blockages |
| 482 | Pump efficiency drops with motor temperature >110°C | Overloaded pump | Reduce pump speed by 10%; inspect |
| 483 | VFD current erratic with normal voltage | VFD controller fault | Recalibrate VFD; check communication lines |
| 484 | Intake pressure exceeds 130% of normal | Pump seal failure | Stop pump; inspect and replace seals |
| 485 | Motor temperature rises with high Vz vibration | Shaft misalignment or bearing wear | Stop pump; check alignment and bearings |
| 486 | Power consumption exceeds 20% above normal with low flow | High fluid viscosity or emulsion | Analyze fluid; adjust VFD settings |
| 487 | Voltage drops below 85% with high VFD current | Undersized power supply | Switch to backup power; upgrade system |
| 488 | Vx, Vy vibrations exceed threshold with low load | Loose pump components | Stop pump; tighten connections |
| 489 | Pump efficiency drops with high gas content | Gas interference | Adjust gas separator; monitor GOR |
| 490 | Motor current spikes 15% during startup | High starting torque | Extend VFD ramp-up time; inspect pump |
| 491 | Discharge pressure drops with low intake pressure | Pump restriction | Trigger pump flush; inspect for debris |
| 492 | Motor temperature exceeds 125°C with erratic current | Electrical fault | Stop pump; run electrical diagnostics |
| 493 | VFD current trends up 10% over 4 hours | Scale or sand accumulation | Initiate chemical injection; monitor |
| 494 | Intake pressure erratic with normal production | Faulty pressure sensor | Calibrate or replace intake sensor |
| 495 | Pump efficiency drops with high vibration (all axes) | Unbalanced rotor or impeller | Stop pump; balance rotor or inspect impeller |
| 496 | Power consumption exceeds threshold with high temperature | Motor overload or insulation issue | Reduce pump speed; test insulation |
| 497 | Voltage spikes 15% with high VFD current | Grid instability or noise | Activate noise filters; stabilize power |
| 498 | Pump efficiency drops with high motor load | Pump seal failure or wear | Stop pump; inspect seals or components |
| 499 | Vz vibration exceeds 3x normal with high wellhead pressure | Downhole restriction | Inspect tubing; clear blockages |
| 500 | Erratic power consumption with normal sensor readings | Controller communication failure | Check communication lines; replace faulty components |

B. Glossary of Terms

To enhance clarity and ensure consistent interpretation of the rule-based system, this appendix provides a glossary of technical terms used throughout the rule sets and diagnostic logic. It defines key parameters and concepts related to ESP monitoring (Table 23).

Table 23. Glossary of Terms for ESP Monitoring Data (Rules 1–500).

| Term | Definition |
| --- | --- |
| Acid Backwash | A chemical treatment process involving the injection of acid to dissolve scale or other deposits in the ESP system, restoring performance. |
| Anomaly Detection | The process of identifying unusual patterns or outliers in data that deviate from expected behavior, often using statistical or machine learning methods. |
| Bearing Failure | A mechanical failure in the bearings of the ESP motor or pump, often caused by wear, lack of lubrication, or overheating, leading to increased vibration or temperature. |
| Cavitation | The formation and collapse of vapor bubbles in the pump due to low intake pressure, causing reduced efficiency, vibration, and potential damage. |
| Check Valve | A valve installed in the production tubing to prevent backflow of fluid, reducing fluid hammer and backspin during pump shutdown. |
| Choke | A device used to control fluid flow rate at the surface or downhole, affecting pressure and production stability. |
| Clustering | A machine learning technique that groups similar data points based on patterns, used to identify anomalies or operational states in ESP data. |
| Controller Communication Failure | A disruption in data transmission between the ESP controller (e.g., VFD) and monitoring systems, leading to erratic or missing readings. |
| Discharge Pressure | The pressure measured at the outlet of the ESP, indicating the pump’s ability to lift fluid to the surface. |
| Downhole Restriction | A blockage or obstruction in the tubing, flowline, or pump, causing high discharge pressure or reduced flow. |
| Electrical Noise | Unwanted electrical signals or interference in the power supply, causing erratic voltage or current readings. |
| Electrical Surge | A sudden increase in voltage or current, potentially damaging ESP components or causing erratic performance. |
| Emulsion | A mixture of oil and water that increases fluid viscosity, reducing pump efficiency and increasing motor load. |
| Fluid Hammer | A pressure surge caused by sudden changes in fluid flow (e.g., during pump shutdown), potentially damaging the ESP system. |
| Gas Interference | The presence of free gas in the pump, reducing efficiency, causing cavitation, or leading to gas lock. |
| Gas Lock | A condition where gas accumulates in the pump, preventing fluid flow and causing low current, high vibration, or pump trips. |
| Gas-Oil Ratio (GOR) | The ratio of gas to oil in the produced fluid, affecting pump performance and cooling efficiency. |
| Gas Separator | A device installed with the ESP to separate gas from liquid before entering the pump, reducing gas interference. |
| Grid Instability | Fluctuations or interruptions in the electrical power supply, causing voltage or current anomalies in the ESP system. |
| Harmonic Distortion | Electrical noise caused by non-linear loads in the VFD, leading to erratic current or voltage readings. |
| Hot Oiling | A treatment process involving the injection of heated oil to dissolve paraffin deposits in the ESP system. |
| Impeller | A rotating component in the pump that imparts energy to the fluid, critical for lift; wear or imbalance reduces efficiency. |
| Inflow Performance Relationship (IPR) | A model describing the relationship between reservoir pressure and production rate, used to assess reservoir depletion or performance. |
| Insulation Resistance Test | A diagnostic test to measure the integrity of motor insulation, identifying potential electrical faults. |
| Intake Pressure | The pressure measured at the pump intake, reflecting reservoir inflow and pump suction conditions. |
| Machine Learning (ML) | Algorithms (e.g., PCA, XGBoost, neural networks) used to analyze patterns, predict failures, or detect anomalies in ESP data. |
| Motor Current | The electrical current drawn by the ESP motor, indicating load, efficiency, or potential faults. |
| Motor Load | The mechanical or electrical load on the ESP motor, often expressed as a percentage of rated capacity. |
| Motor Temperature | The temperature of the ESP motor, critical for assessing cooling efficiency and detecting overheating risks. |
| Moving Average | A statistical method to smooth data over time, used to establish baselines and detect deviations in ESP parameters. |
| Paraffin Buildup | The accumulation of waxy hydrocarbons in the ESP system, reducing flow and cooling efficiency. |
| Pattern Recognition | The use of algorithms to identify recurring patterns or anomalies in ESP data, often via machine learning techniques. |
| Power Consumption | The total electrical power used by the ESP system, derived from voltage and current, indicating efficiency or overload conditions. |
| Pump Efficiency | A derived parameter measuring the pump’s ability to convert electrical energy into fluid lift, reduced by wear, blockages, or gas interference. |
| Pump Underload | A condition where the pump operates below its designed capacity, often due to low inflow or oversized equipment. |
| Reservoir Depletion | A reduction in reservoir pressure or fluid volume, leading to lower intake pressure and reduced production. |
| Sand Control | Methods (e.g., gravel packs, screens) to prevent sand production from clogging or damaging the ESP system. |
| Scale Buildup | The accumulation of mineral deposits (e.g., calcium carbonate) in the pump or motor, reducing efficiency and increasing load. |
| Sensor Drift | Gradual deviation in sensor readings from actual values, leading to inaccurate data and potential misdiagnosis. |
| Shaft Misalignment | Improper alignment of the pump or motor shaft, causing increased vibration and wear. |
| Standard Deviation (σ) | A statistical measure of data variability, used to detect anomalies when parameters deviate from the mean (e.g., 2σ or 3σ thresholds). |
| Surface Choke | A valve at the surface to control fluid flow, affecting discharge pressure and production stability. |
| Tubing Restriction | A blockage or narrowing in the production tubing, increasing discharge pressure or reducing flow. |
| Variable Frequency Drive (VFD) | A device controlling the ESP motor speed by adjusting power frequency, critical for optimizing performance and detecting faults. |
| Vibration (Vx, Vy, Vz) | Mechanical oscillations in the X (lateral), Y (horizontal), and Z (vertical) axes, indicating misalignment, imbalance, or mechanical issues. |
| Voltage Stabilizer | A device to maintain consistent voltage supply, mitigating grid instability or surges. |
| Wellhead Pressure | The pressure at the surface wellhead, reflecting flowline conditions and potential restrictions. |
| Worn Pump Stages | Degradation of pump impellers or diffusers, reducing efficiency and increasing motor load. |