Supporting Information

For

Risk in the quality of aquatic ecosystems exposed to pesticides in a natural protected area of influence in western Mexico.

Cruz-Esquivel Ángela, Martínez-Rivera Luis Manuela, Sandoval-Pinto Elenab, Meza-Rodríguez Demetrioa, Peregrina-Lucano Alejandro Aaronc

a Department of Ecology and Natural Resources, South Coast University Center, University of Guadalajara, Autlan de Navarro, Jalisco, Mexico.

b Department of Cellular and Molecular Biology, University Center for Agricultural Biological Sciences, University of Guadalajara, Guadalajara, Jalisco

c Department of Pharmacobiology. University Center for Exact Sciences and Engineering, University of Guadalajara, Jalisco, Mexico

Table S1. Description of study sites

| Sites | Altitude | Coordinates |  | Description | Foto |
| --- | --- | --- | --- | --- | --- |
|  |  | Latitude | Longitude |  |  |
| S1 | 910 | 19º51'25.7'' | 104º17'22.8'' | Tropical deciduous forest, canyon-type landscapes. |  |
| S2 | 870 | 19º47'28.1'' | 104º13'45.7'' | Peri-urban area, beginning of agricultural extensions. |  |
| S3 | 866 | 19º45'27.6'' | 104º13'43.5'' | Valley area, sugar cane and corn crops. |  |
| S4 | 866 | 19º45'41.3'' | 104º12'56.7'' | Wastewater discharge channel of the municipality of Autlan. |  |
| S5 | 860 | 19º44'3'' | 104º10'40.8'' | Valley area, sugar cane and corn crops. |  |
| S6 | 860 | 19º44'41.7'' | 104º09'58.1'' | Wastewater discharge channel of the municipality of Grullo. |  |
| S7 | 858 | 19º42'40.6'' | 104º09'10.3'' | Valley area, sugar cane and corn crops. |  |
| S8 | 848 | 19º41'27.6'' | 104º08'19.6' | Sierra de Manantlan Biosphere Reserve Area, tropical deciduous forest |  |
| S9 | 848 | 19º41'24.9'' | 104º08'18' | Sierra de Manantlan Biosphere Reserve Area, tropical deciduous forest |  |
| S10 | 810 | 19º39'57.7'' | 104º05'9.7' | Sierra de Manantlan Biosphere Reserve Area, relicts of horticultural crops. |  |
| S11 | 692 | 19º36'2.9'' | 103º57'46.9' | Tropical deciduous forest, small relicts of agave plantations. |  |
| S12 | 701 | 19º36'41.7'' | 103º57'5' | Agave crops, influence of urban runoff. |  |
| S13 | 680 | 19º35'58.6'' | 103º57'27.5' | Tropical deciduous forest, small relicts of agave plantations. |  |

Chromatographic conditions:

For non-polar compounds: HPLC model 1200, Agilent Technologies brand, Zorbax Eclipse XDB C-18 chromatographic column, 2.1 x 5.0 mm, 3.5 micrometers. Mobile phase Water with 0.1% formic acid and 0.1% amino formate, Acetonitrile, gradient 60:40 to 0:100. Flow rate 0.5 mL/min, 3 minutes.

For polar compounds: HPLC model 1200, Agilent Technologies, Zorbax Eclipse XDB C-18 chromatographic column, 2.1 x 5.0 mm, 3.5 micrometers. Mobile phase 30:70 Water with 0.1% ammonium formate, 0.1% ammonium formate, acetonitrile. Flow rate 0.5 mL/min, 1 minute.

Table S2. Conditions for HPLC-MS/MS analysis

| Non-polar pesticides |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Phase | Polarity | Ion Precursor | Ion Product |
| Acetochlor | Formic acid: Ammonium formate | Positive | 270.1 | 148.4 |
|  |  |  |  | 224.2 |
| Ametrine |  | Positive | 228.1 | 186 |
|  |  |  |  | 96 |
| Atrazine |  | Positive | 216 | 132 |
| Carbendazim |  | Positive | 192.1 | 160 |
| Carbofuran |  | Positive | 222 | 123 |
| Diazinon |  | Positive | 305 | 153 |
| Dimethoate |  | Positive | 230 | 171 |
| Emamectin |  | Positive | 887.1 | 158.1 |
| Imazalil |  | Positive | 297 | 159 |
| L-Cyalothrin |  | Positive | 467.1 | 225.1 |
| Malathion |  | Positive | 331 | 99 |
| Meclizine |  | Positive | 391.2 | 201.1 |
| Methomyl |  | Positive | 163.1 | 106 |
|  |  |  |  | 65 |
|  |  |  |  | 88.1 |
| Metoxuron |  | Positive | 229.1 | 72.1 |
| Molinate |  | Positive | 188.1 | 55.1 |
| Parathion |  | Positive | 292 | 236 |
|  |  |  |  | 264 |
| Picloram |  | Positive | 240.9 | 194.9 |
|  |  |  |  | 222.9 |
| Pyraclostrobin |  | Positive | 388 | 163 |
| Thiabendazole |  | Positive | 202 | 131 |
| Polar Pesticides |  |  |  |  |
| 2,4-D | Ammonium formate | Negative | 218.9 | 160.9 |
|  |  |  | 220.9 | 162.9 |
| Glufosinate |  | Negative | 179.9 | 136 |
|  |  |  |  | 118.9 |
|  |  |  |  | 94.9 |
| Glifosate |  | Negative | 168.2 | 149.9 |
|  |  |  |  | 81.1 |
|  |  |  |  | 62.8 |

Table S3. Reference values on permissible thresholds for physicochemical variables

| Parameters | Aquatic life* | Reference |
| --- | --- | --- |
| Temperature (°C) | 8 a 31 | -- |
| pH | 6.5-8.5 | U.S. EPA (2024) |
| DO (%) | 58.45 | CE-CCA (1989) |
| DO (ppm) | 5.5 | CE-CCA (1989) |
| µS/cm/C. E | 300-500 | U.S. EPA (2011) |
| TDS (ppm) | 500 | CCME (2007) |
| Turbidity (NTU) | 25 | CCME (2007) |
| NO3-N (mg/L) | 3 | CE-CCA (1989), CCME (2007) |
| PO4 (mg/L) | 0.11 | CE-CCA (1989) |
| FCOL (100ml) | 1000 | CE-CCA (1989) |
| L-cyhalothrin (ng/ml) | 0.23 | PPDB, 2009 |
| Malathion (ng/ml) | 2 | PPDB, 2009 |
| Methomyl (ng/ml) | 1.6 | PPDB, 2009 |

Table S4. Spatial distribution of physicochemical variables in the study sites.

| Parameters |  | S1 | S2 | S3 | S4 | S5 | S6 | S7 | S8 | S9 | S10 | S11 | S12 | S13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature (°C) | Mediana | 24.7 | 23.7 | 27.4 | 24.2 | 26.1 | 28 | 26 | 26.7 | 24.7 | 26.2 | 24.6 | 25.6 | 24.8 |
|  | Min-Max | 18.2-33 | 18.8-30.6 | 21.1-30.5 | 16.6-33.6 | 21.7-32.1 | 22.3-34 | 22.4-29.6 | 23-31.3 | 19.9-31 | 21.6-29.8 | 20.6-29 | 21.9-28.5 | 19.8-29.2 |
|  | DAM | 4.5 | 2.4 | 3.1 | 1.5 | 2.3 | 1.9 | 2.1 | 2.7 | 2.6 | 2.3 | 1.3 | 2 | 1.9 |
|  | Test Dunn* | a | a | a | a | a | a | a | a | a | a | a | a | a |
| pH | Mediana | 8.51 | 7.7 | 7.6 | 7.8 | 7.67 | 7.72 | 7.76 | 7.9 | 8.67 | 8.24 | 8.45 | 8.28 | 8.34 |
|  | Min-Max | 7.6-9.3 | 7.2-8.2 | 7.3-8.1 | 7.3-8.3 | 7.2-8 | 7.4-8.3 | 7.3-8 | 7.6-8.2 | 7.8-9.2 | 7.8-8.6 | 8-8.8 | 7.9-8.6 | 7.1-8.8 |
|  | DAM | 0.45 | 0.28 | 0.28 | 0.22 | 0.26 | 0.23 | 0.21 | 0.24 | 0.32 | 0.24 | 0.26 | 0.22 | 0.25 |
|  | Test Dunn | b | a | a | a | a | a | a | a | b | b | b | b | b |
| DO (%) | Mediana | 102.5 | 75.3 | 57.3 | 20.6 | 29.3 | 27.4 | 28.9 | 72.4 | 98.7 | 91.7 | 101.8 | 86.3 | 96.7 |
|  | Min-Max | 52.1-198.6 | 39.1-88.9 | 6.1-93.6 | 0.3-87.1 | 15.6-78.6 | 4.5-175.4 | 13-68.1 | 38.8-91.9 | 72.3-124.5 | 38.5-130.4 | 27.3-118.9 | 28-122.4 | 54.2-116 |
|  | DAM | 18.2 | 12.3 | 12.3 | 10.7 | 8.08 | 6.6 | 9 | 9.6 | 9.6 | 9.3 | 10.2 | 12.4 | 16.4 |
|  | Test Dunn | d | bce | a | ab | a | a | a | bc | d | cde | d | cde | de |
| DO (ppm) | Mediana | 11.4 | 7.88 | 4.02 | 2.37 | 2.25 | 2.4 | 2.61 | 5.75 | 8.6 | 9.2 | 10.71 | 7.11 | 8 |
|  | Min-Max | 3.45-16.2 | 2.9-11.7 | 0.81-12.5 | 0.02-6.6 | 1.09-7.46 | 0.55-18.9 | 0.97-5.19 | 2.73-11.6 | 5.66-15.8 | 3.08-13.9 | 5.5-16.5 | 0.2-16.7 | 4.11-16 |
|  | DAM | 3.53 | 2.41 | 1.87 | 0.8 | 0.77 | 0.91 | 1.17 | 1.91 | 2.36 | 2.28 | 2.84 | 3.94 | 2.99 |
|  | Test Dunn | d | bd | a | ac | a | a | a | bc | d | bd | d | bd | bd |
| µS/cm/C. E | Mediana | 217 | 412 | 661 | 716 | 685 | 1023 | 701 | 682 | 109 | 557 | 647 | 684 | 608 |
|  | Min-Max | 37.5-455 | 38.2-496 | 42.7-831 | 44.2-1001 | 169.2-804 | 41.6-2000 | 44.8-827 | 42.8-804 | 35.3-190 | 41.5-740 | 42.9-870 | 85.2-1289 | 42.3-914 |
|  | DAM | 54 | 63 | 76 | 143 | 58 | 565 | 35 | 42 | 45 | 106 | 147 | 171 | 96 |
|  | Test Dunn | de | cde | ab | a | ab | b | ab | a | d | ace | a | ab | ac |
| TDS (ppm) | Mediana | 115 | 205 | 330 | 359 | 343 | 534 | 350 | 340 | 64 | 278 | 323 | 345 | 302 |
|  | Min-Max | 26.8-229 | 27-248 | 30.4-417 | 31.3-500 | 30.5-421 | 29.6-1000 | 31.1-413 | 30-401 | 25.2-699 | 29.5-370 | 30.4-435 | 60.1-644 | 23.8-457 |
|  | DAM | 33 | 33 | 37 | 70 | 28 | 279 | 19 | 23 | 19 | 56 | 74 | 85 | 68 |
|  | Test Dunn | d | cd | ab | a | a | b | ab | a | d | ac | a | ab | ac |
| Turbidity (NTU) | Mediana | 5.37 | 33 | 37.6 | 46.1 | 47.1 | 89.6 | 42 | 39.1 | 4.74 | 19.6 | 21.3 | 38.8 | 28.7 |
|  | Min-Max | 2.02-8 | 10.9-15 | 19-41 | 13.4-93 | 9.82-60 | 16.4-212 | 7.18-82 | 7.52-85 | 1.19-13 | 6.53-112 | 8.17-393 | 2.268-120 | 21-160 |
|  | DAM | 3.35 | 10.1 | 14.3 | 23 | 22.4 | 54.8 | 18.5 | 21 | 3.55 | 11.23 | 8.1 | 12.4 | 6.1 |
|  | Test Dunn | d | abc | abc | abc | abc | a | abc | abc | d | bc | b | ac | abc |
| NO3-N (mg/L) | Mediana | 2 | 3.4 | 3.3 | 4.4 | 3.2 | 3.8 | 3.2 | 4.1 | 1.65 | 4 | 3.4 | 2.5 | 3.3 |
|  | Min-Max | 0.2-11.6 | 2.4-6.8 | 1.2-11.4 | 1.6-15.6 | 0.8-7.7 | 1.7-12.1 | 0.9-11.9 | 0.9-7.6 | 0.4-3.1 | 1.1-9.8 | 2.1-11.5 | 0.9-7.7 | 1.6-11.4 |
|  | DAM | 0.9 | 0.7 | 0.7 | 2.1 | 0.8 | 1.8 | 1.4 | 1.2 | 0.6 | 1.3 | 0.7 | 1 | 1.1 |
|  | Test Dunn | bc | a | a | a | ab | a | ab | a | c | a | a | ab | ab |
| PO4 (mg/L) | Mediana | 0.39 | 0.67 | 0.63 | 2.33 | 1.99 | 1.82 | 1.86 | 1.61 | 0.2 | 1.26 | 1.17 | 0.63 | 1.11 |
|  | Min-Max | 0.14-1.23 | 0.37-1.54 | 0.05-12 | 0.14-12 | 1.35-4.8 | 0.07-39 | 1.37-15 | 1.07-21 | 0.04-0.55 | 0.65-5.9 | 0.04-2.15 | 0.016-27 | 0.09-22 |
|  | DAM | 0.17 | 0.11 | 0.27 | 1.96 | 0.5 | 0.72 | 0.34 | 0.33 | 0.14 | 0.22 | 0.13 | 0.34 | 0.39 |
|  | Test Dunn | ef | cef | ab | b | b | ab | ab | abc | f | abe | acd | cde | abcd |
| F. Coliforms (100ml) | Mediana | 300 | 700 | 900 | 925000 | 500000 | 530000 | 59000 | 23350 | 150 | 1200 | 300 | 2200 | 1750 |
|  | Min-Max | 100-7100 | 200-26800 | 100-48x10⁴ | 10000-88x10⁵ | 6700-36x10⁵ | 12800-395x10⁴ | 1700-118x10⁴ | 100-10x10⁵ | 100-28000 | 100-45x10⁴ | 100-12400 | 100-5x10⁴ | 100-26000 |
|  | DAM | 100 | 400 | 700 | 849100 | 360000 | 370000 | 52300 | 17700 | 50 | 1100 | 200 | 1000 | 1250 |
|  | Test Dunn | d | cd | cd | a | a | a | ab | bc | d | cd | d | bc | cd |

*Different letters indicate statistically significant differences

Table S5. Principal component analysis

|  | Comp.1 | Comp.2 |
| --- | --- | --- |
| Habitat | 0.12437755 | 0.74679342 |
| TEM | 0.09443582 | 0.10317857 |
| pH | -0.29058755 | -0.27225737 |
| DO | -0.47408819 | -0.17427917 |
| OS | -0.46234351 | -0.18598328 |
| EC | 0.38441755 | -0.38171436 |
| TDS | 0.41797761 | -0.33490026 |
| TUR | 0.27199461 | -0.1723505 |
| NO3N | 0.23703723 | -0.07069621 |

Fig S1. Principal component analysis of physicochemical variables.

Fig S2.  Linearity of predictors

Fig S3. Verification of statistical assumptions

References

Canadian Council of Ministers of the Environment, (2017). Canadian water quality guidelines for the protection of aquatic life: CCME Water Quality Index, User’s Manual – 2017 Update. In: Canadian environmental quality guidelines, 1999, Canadian Council of Ministers of the Environment, Winnipeg. p 1-23

del Agua CEDC CE-CCA-001/89 (1989) ACUERDO por el que se establecen los Criterios Ecológicos de Calidad del Aguas CE-CCA-001/89. Diario Oficial de la Federación. SEGOB.

Footprint PPDB, (2009). Creating tools for pesticide risk assessment and management in Europe. The Pesticide 0, 2(4): 6. Retrieved April 10, 2024, from https://sitem.herts.ac.uk/aeru/ppdb/

U.S. EPA, (2024). National Recommended Water Quality Criteria - Aquatic Life Criteria. Retrieved March 25, 2024, from https://www.epa.gov/wqc/national-recommended-water-quality-criteria-aquatic-life-criteria-table

U.S. EPA, (2011). A Field-Based Aquatic Life Benchmark For Conductivity In Central Appalachian Streams (Final Report). U.S. Environmental Protection Agency, Washington, DC, EPA/600/R-10/023F, 2011.