| Data | Purpose | Resolution / scale | Year | Source |
| --- | --- | --- | --- | --- |
| Landsat -9 | LULC | 30 mt | April, 2024 | (http://earthexplorer.usgs.gov/ ) |  |
| SRTM-DEM | Drainage morphometry | 30 mt | April, 2024 | (http://earthexplorer.usgs.gov/ ) |  |
| Toposheet | Delineation of watershed | 1: 2,50,000 | 1961 | Survey Of India (SOI), Kolkata |  |

Table 1.   Summary of datasets used in the study

Table 2. Geomorphometric parameters used in the study along with their formulas and scientific references, categorized under linear, areal, and relief attributes

| Geomorphometric units | Formula |  |  |  | References |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Linear parameters |  |  |  |  |  |  |  |
| Stream order (U) | Hierarchical rank Nu1+Nu2…. +Nun Lu1+Lu2……. + Lun Average of stream length of all orders |  | Higher stream orders are associated with increased erosion risk and lower groundwater potential (Rawat et al., 2021) Stream order and other parameters are used to prioritize sub watersheds for conservation measures (Barman et al., 2021). |  |  |  |  |
| Stream number (Nu) | Nu/Nu+1 Nu= total number of streams of ‘U’, Nu+1=number of streams of next higher order |  | Stream number (Nu) plays a crucial role in assessing soil erosion risk and groundwater potential in watersheds. Morphometric analysis, including stream number and other parameters, is essential for hydrological analysis and watershed management (Benzougagh et al., 2021). |  |  |  |  |
| Stream length (Lu) | Nu/Nu+1 Nu= total number of streams of ‘U’, Nu+1=number of streams of next higher order |  | Stream length plays a crucial role in watershed characteristics and nutrient export. In small watersheds, stream length is highly correlated with drainage area size and length to center of area (Gray, 1961). |  |  |  |  |
| Mean stream Length (Lsm) | Nu/Nu+1 Nu= total number of streams of ‘U’, Nu+1=number of streams of next higher order |  | The mean stream length (Lsm) plays a crucial role in watershed analysis and characterization. It is used to evaluate stream network development and can be derived from digital elevation models using GIS techniques (Jain et al., 2020). |  |  |  |  |
| Bifurcation Ratio (Rb) | Nu/Nu+1 Nu= total number of streams of ‘U’, Nu+1=number of streams of next higher order |  | Studies have shown that Rb values ranging from 3 to 4.66 indicate undistorted geologic structures and moderate erosion potential (Sinha et al., 2024). Lower Rb values suggest well-dissected watersheds with reduced flooding and erosion risks (W. R. Singh et al., 2021) |  |  |  |  |
| Areal Parameters |  |  |  |  |  |  |  |
| Drainage density (Dd) | Lu/A Lu=Stream length A =Area |  | Drainage density (Dd) is a crucial parameter for assessing soil erosion and watershed characteristics. Studies have shown a significant correlation between Dd and erosion rates in various watersheds (Moeini et al., 2015). Higher Dd values indicate increased susceptibility to surface erosion and soil degradation (Kumar et al., 2023) |  |  |  |  |
| Stream Frequency (Fs) | Nu/A Nu=Stream number A = area | Stream frequency (Fs) is identified as a crucial parameter influencing erosion and hydrological processes (Benzougagh et al., 2021). Higher Fs values indicate increased erosion risk and reduced groundwater potential (Rawat et al., 2021). |  |  |  |  |  |
| Drainage texture (Dt) | Nu/P Nu=Stream number P = perimeter | Drainage texture plays a crucial role in geologic interpretation and water management. It provides insights into underlying materials and structures, especially in areas of low relief (Howard, 1967) |  |  |  |  |  |
| Infiltration Number (If) | Dd*Fs Dd = Drainage density Fs = Stream Frequency | The Infiltration Number (If) plays a crucial role in watershed hydrology, influencing runoff estimation and water resource management. Infiltration rates exhibit significant spatial variability within watersheds, with parameters often following log-normal distribution (M. L. Sharma et al., 1980). |  |  |  |  |  |
| Circularity Ratio (Rc) | 4πA/P2 A = area P = perimeter | The circularity ratio (Rc) is a crucial morphometric parameter in watershed analysis, influencing flood hazard and erosion potential. Higher Rc values (>0.5) indicate circular basins with shorter flood peaks, while lower values (<0.5) suggest elongated basins with extended flood periods and greater erosion risk (M. C. Singh et al., 2023; Sudaryatno et al., 2024). |  |  |  |  |  |
| Length of overland flow (Lg) | 1/(2*Dd) Dd = Drainage density | The length of overland flow plays a crucial role in watershed hydrology and sediment transport. Longer slope lengths decrease runoff connectivity and increase spatial variability of infiltration (Ghahramani & Ishikawa, 2013). |  |  |  |  |  |
| Constant of channel maintenance (c) | 1/Dd Dd = Drainage density | The constant of channel maintenance (c) plays a crucial role in watershed geomorphology and sediment transport. It represents the minimum drainage area required to maintain a channel (Sakthivel et al., 2019) |  |  |  |  |  |
| Fineness ratio (Rfn) | Lar/p Lar = Stream Length Ratio P = Perimeter |  |  |  |  |  |  |
| Relief Parameters |  |  |  |  |  |  |  |
| Maximum elevation of the area (Me) | Extracted from DEM |  |  |  |  |  |  |
| Minimum elevation of the area (Mm) | Extracted from DEM |  |  |  |  |  |  |
| Basin relief(H) | Me – Mm Me = Maximum elevation of the area Mm = Minimum elevation of the area |  |  | High relief ratios and elongation ratios indicate increased erosion risk and elongated basin shapes, respectively (Elhag et al., 2017; Ganie et al., 2023). |  |  |  |
| Relative relief (Rr) | H/P H = Basin Relief P = Basin Parameter |  |  | Relief morphometry plays a crucial role in understanding watershed characteristics and their hydro geomorphological dynamics. Quantitative analysis of relief features, including relative relief, , provides insights into slope stability, erosion capacity, and landform development stages (Tajvir, 2023) |  |  |  |
| Dissection Index | H/ Me H = = Basin Relief M = Maximum elevation of the area |  |  | The Dissection Index (DI) is a crucial parameter for assessing terrain roughness and watershed characteristics. It is influenced by various geomorphic factors, with relative relief having the strongest impact, followed by average slope and absolute relief (Tajvir, 2023). |  |  |  |

Table 3.     Computation of morphometric parameters

| sub watersheds | Stream Length Ratio | Bifurcation Ratio | Fineness Ratio | Drainage Texture | Drainage Density | Stream Frequency |
| --- | --- | --- | --- | --- | --- | --- |
| 2A2B5k | 13.27 | 13 | 0.29 | 1.2 | 0.66 | 0.71 |
| 2A2B5b | 6.37 | 15.5 | 0.16 | 0.85 | 0.38 | 0.75 |
| 2A2B5j | 4.94 | 19.5 | 0.2 | 1.71 | 0.97 | 1.85 |
| 2A2B5h | 5.91 | 22 | 0.14 | 0.54 | 0.39 | 0.52 |
| 2A2B5m | 17.73 | 7.0 | 0.65 | 0.55 | 1.85 | 0.54 |
| sub watersheds | Relative Relief Ratio | Dissection Index | Infiltration Number | Constant of Channel Maintenance | Circularity Ratio | Average Length Of Overland Flow |
| 2A2B5k | 7.49 | 0.55 | 0.47 | 1.49 | 0.48 | 0.74 |
| 2A2B5b | 8.9 | 0.58 | 0.28 | 2.62 | 0.37 | 1.31 |
| 2A2B5j | 14.41 | 0.59 | 1.81 | 1.02 | 0.49 | 0.51 |
| 2A2B5h | 8.89 | 0.62 | 0.2 | 2.55 | 0.31 | 1.27 |
| 2A2B5m | 11.47 | 0.5 | 1.01 | 0.53 | 0.47 | 0.26 |
| sub watersheds | Hypsometric Integral |  |  |
| 2A2B5k | 0.42 |  |  |
| 2A2B5b | 0.13 |  |  |
| 2A2B5j | 0.099 |  |  |
| 2A2B5h | 0.17 |  |  |
| 2A2B5m | 0.67 |  |  |

Table 4. Computation of Inter-Correlation matrix of 13 selected geomorphic parameters

|  | Inter Correlation Matrix |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | SLR | BR | FR | DT | DD | SF | RRR | DI | IN | CCM | CR | ALOF | HI |
| SLR | — |  |  |  |  |  |  |  |  |  |  |  |  |
| BR | -0.92 | — |  |  |  |  |  |  |  |  |  |  |  |
| FR | 0.91 | -0.89 | — |  |  |  |  |  |  |  |  |  |  |
| DT | -0.36 | 0.27 | -0.33 | — |  |  |  |  |  |  |  |  |  |
| DD | 0.74 | -0.74 | 0.93 | -0.1 | — |  |  |  |  |  |  |  |  |
| SF | -0.49 | 0.37 | -0.29 | 0.9 | 0.02 | — |  |  |  |  |  |  |  |
| RRR | -0.16 | 0.06 | 0.19 | 0.49 | 0.52 | 0.78 | — |  |  |  |  |  |  |
| DI | -0.93 | 0.98 | -0.93 | 0.18 | -0.82 | 0.27 | -0.05 | — |  |  |  |  |  |
| IN | -0.02 | -0.05 | 0.25 | 0.68 | 0.56 | 0.83 | 0.9 | -0.19 | — |  |  |  |  |
| CCM | -0.63 | 0.61 | -0.78 | -0.29 | -0.9 | -0.31 | -0.59 | 0.73 | -0.75 | — |  |  |  |
| CR | 0.48 | -0.54 | 0.51 | 0.62 | 0.62 | 0.49 | 0.46 | -0.63 | 0.72 | -0.86 | — |  |  |
| ALOF | -0.64 | 0.61 | -0.78 | -0.28 | -0.9 | -0.31 | -0.59 | 0.73 | -0.74 | 1 | -0.86 | — |  |
| HI | 0.99 | -0.89 | 0.94 | -0.41 | 0.78 | -0.5 | -0.11 | -0.91 | 0 | -0.65 | 0.44 | -0.65 | — |

Table 5. Principal Component Unrotated and rotated loading Matrix

Morphometric Parameters; Unrotated Matrix; Rotated Matrix. Components; Components. 1; 2; 3; 1; 2; 3. SLR; -0.309; -0.238; -0.152; -0.418; -0.021; 0.006. BR; 0.31; 0.195; 0.19; 0.409; 0.016; 0.051. FR; -0.342; -0.139; 0.194; -0.271; 0.238; 0.209. DT; -0.004; 0.42; -0.513; 0.043; -0.028; -0.661. DD; -0.346; 0.02; 0.341; -0.139; 0.408; 0.224. SF; -0.001; 0.474; -0.063; 0.23; 0.238; -0.345. RRR; -0.133; 0.382; 0.502; 0.274; 0.566; 0.14. DI; 0.337; 0.141; 0.151; 0.388; -0.052; 0.057. IN; -0.183; 0.405; 0.169; 0.132; 0.436; -0.137. CCM; 0.339; -0.157; -0.024; 0.17; -0.316; 0.107. CR; -0.282; 0.223; -0.463; -0.26; 0.061; -0.522. ALOF; 0.34; -0.155; -0.024; 0.172; -0.315; 0.106. HI; -0.311; -0.237; -0.029; -0.377; 0.045; 0.1. Eigen value; 7.541; 4.41; 0.84; 7.541; 4.41; 0.84. % of total factor covariance; 58.005; 33.921; 6.461; 58.005; 33.921; 6.461. Cumulative % of total factor covariance; 58.005; 91.926; 98.386; 58.005; 91.926; 98.386.

Table 6. Selection of most influential parameter

Morphometric Parameters; Standard score for final cluster center. Cluster 1; Cluster 2; Cluster 3. SLR; -0.201; 1.445; -0.84. BR; 0.245; -1.437; 0.701. FR; -0.434; 1.72; -0.418. DT; -0.216; -0.85; 1.498. DD; -0.613; 1.642; 0.197. SF; -0.386; -0.602; 1.759. RRR; -0.658; 0.451; 1.524. DI; 0.337; -1.495; 0.484. IN; -0.653; 0.382; 1.577. CCM; 0.624; -1.201; -0.672. CR; -0.467; 0.576; 0.826. ALOF; 0.623; -1.204; -0.665. HI; -0.237; 1.528; -0.816.

Table 7.  Classification of Micro Watershed into Cluster

| Micro watersheds | Cluster | Priority Rank | Remarks |
| --- | --- | --- | --- |
| 2A2B5k | 3 | 1 | A typical scene in villages includes vast stretches of fallow ground and exposed rock formations. The elevation ranges from 350-600 meters above msl. Villages are in moderate to steep slope (> 7 ) resulting in shallow soil depth and susceptibility to erosion. High drainage density (> 2.5 km/ km sq.) and rapid permeability. Granite gneiss constitutes the primary geological makeup of most villages, whereas hornblede, ambhibolite, biotite gneiss are only sparsely distributed in a small percentage of villages. Soil in these areas typically have low to moderate available water capacity. |
| 2A2B5b | 1 | 3 | Most of the villages are situated within a gently sloping to very gently sloping plain (< 2), with an elevation ranging from 150-200 meters above msl and low drainage density (< 0.5 km/ km sq), high lineament density. Soils develop across undulating lowlands and valley fills. The soils exhibit considerable depth, moderately slow permeability maintaining a balanced soil-water- air relationship. |
| 2A2B5j | 1 | 3 |  |
| 2A2B5h | 1 | 3 |  |
| 2A2B5m | 2 | 2 | elevations spanning from 200-350 meters above msl, denotes their placement within areas of moderate slope (2 -7) and moderate drainage density (0.5 – 2.5 km/km sq). The soil found in pediments, undulating plateaus, and dissected highlands exhibit varying characteristics. Soils in these areas typically have low to medium available water capacity and face moderate to severe erosional challenges. |

Table 8.  Fuzzy Shannon Entropy Weights for Groundwater Potentiality Assessment

| Morphometric Parameters | Fuzzy Shannon Entropy Weights |
| --- | --- |
| SLR | 0.02845779 |
| BR | 0.02686594 |
| FR | 0.02751192 |
| DT | 0.03153297 |
| DD | 0.03931010 |
| SF | 0.05033305 |
| RRR | 0.06523466 |
| DI | 0.07459912 |
| IN | 0.08526698 |
| CCM | 0.11242149 |
| CR | 0.13186779 |
| ALOF | 0.15807540 |
| HI | 0.16852278 |

Table 9: Prioritization of Micro Watersheds using MCDM Models based on 13 Selected Morphometric Parameters, Evaluating Groundwater Potentiality for Sustainable Resource Management.

| Micro watersheds | VIKOR |  |  |  | TOPSIS |  |  |  | COPRAS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Si | Ri | Qi | Rank | SJ+ | SJ- | Pi | Rank | Pi | Ri | Qi | Rank |
| 2A2B5k | 0.541 | 0.085 | 0.264 | 1 | 0.154 | 0.115 | 0.427 | 2 | 0.151 | 0.049 | 0.194 | 2 |
| 2A2B5b | 0.572 | 0.159 | 0.831 | 4 | 0.176 | 0.111 | 0.387 | 4 | 0.145 | 0.041 | 0.188 | 3 |
| 2A2B5j | 0.474 | 0.168 | 0.500 | 2 | 0.185 | 0.110 | 0.372 | 5 | 0.131 | 0.073 | 0.163 | 5 |
| 2A2B5h | 0.601 | 0.147 | 0.873 | 5 | 0.166 | 0.110 | 0.399 | 3 | 0.143 | 0.038 | 0.186 | 4 |
| 2A2B5m | 0.537 | 0.158 | 0.685 | 3 | 0.130 | 0.144 | 0.526 | 1 | 0.158 | 0.066 | 0.198 | 1 |

Table 10. Fuzzy Shannon Entropy Weights for Soil Erosion Risk

| Morphometric Parameters | Fuzzy Shannon Entropy Weights |
| --- | --- |
| SLR | 0.05797529 |
| BR | 0.06843286 |
| FR | 0.08054471 |
| DT | 0.08734897 |
| DD | 0.10020312 |
| SF | 0.08484450 |
| RRR | 0.07314309 |
| DI | 0.07284309 |
| IN | 0.07314309 |
| CCM | 0.05799085 |
| CR | 0.05502927 |
| ALOF | 0.13401191 |
| HI | 0.05418926 |

Table 11: Micro-Watershed Prioritization Based on Soil Erosion Risk Using MCDM Models and 13 Selected Morphometric Parameters

| Micro watersheds | VIKOR |  |  |  | TOPSIS |  |  |  | COPRAS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Si | Ri | Qi | Rank | SJ+ | SJ- | Pi | Rank | Pi | Ri | Qi | Rank |
| 2A2B5k | 0.617 | 0.081 | 0.435 | 2 | 0.111 | 0.061 | 0.356 | 4 | 0.085 | 0.105 | 0.155 | 3 |
| 2A2B5b | 0.633 | 0.100 | 0.650 | 3 | 0.124 | 0.076 | 0.381 | 3 | 0.096 | 0.081 | 0.188 | 1 |
| 2A2B5j | 0.419 | 0.102 | 0.198 | 1 | 0.108 | 0.099 | 0.478 | 2 | 0.090 | 0.144 | 0.152 | 4 |
| 2A2B5h | 0.646 | 0.099 | 0.673 | 4 | 0.147 | 0.081 | 0.355 | 5 | 0.093 | 0.076 | 0.178 | 2 |
| 2A2B5m | 0.576 | 0.134 | 0.846 | 5 | 0.099 | 0.105 | 0.514 | 1 | 0.081 | 0.144 | 0.137 | 5 |

Table 12. Final prioritization and ranking

Micro watersheds; VIKOR. GWPZ rank; Soil Erosion rank; Sum; Priority rank. 2A2B5k; 1; 2; 3; 1. 2A2B5b; 4; 3; 7; 2. 2A2B5j; 2; 1; 3; 1. 2A2B5h; 5; 4; 9; 3. 2A2B5m; 3; 5; 8; 3. TOPSIS. 2A2B5k; 2; 4; 6; 2. 2A2B5b; 4; 3; 7; 3. 2A2B5j; 5; 2; 7; 3. 2A2B5h; 3; 5; 8; 3. 2A2B5m; 1; 1; 2; 1. COPRAS. 2A2B5k; 2; 3; 5; 2. 2A2B5b; 3; 1; 4; 1. 2A2B5j; 5; 4; 9; 3. 2A2B5h; 4; 2; 6; 2. 2A2B5m; 1; 5; 6; 2.

Table 13. Comparison of micro-watersheds priority rankings derivedfrom three MCDM models — VIKOR, TOPSIS and COPRAS —with the reference ranking and cluster classification ontained through K-means clustering

| MODEL | VIKOR | TOPSIS | COPRAS | REMARKS |
| --- | --- | --- | --- | --- |
| TP | 2 | 3 | 2 | TOPSIS model demonstrates superior performance |
| TN | 7 | 8 | 7 |  |
| FP | 3 | 2 | 3 |  |
| FN | 3 | 2 | 3 |  |
| RECALL | 0.4 | 0.6 | 0.4 |  |
| F1 SCORE | 0.4 | 0.6 | 0.4 |  |
| FPR | 0.3 | 0.2 | 0.3 |  |
| FNR | 0.6 | 0.4 | 0.6 |  |
| FDR | 0.6 | 0.4 | 0.6 |  |
| NPV | 0.7 | 0.8 | 0.7 |  |
| PRECISION | 0.4 | 0.6 | 0.4 |  |
| SPECIFITY | 0.7 | 0.8 | 0.7 |  |
| ACCURACY | 0.6 | 0.73 | 0.6 |  |

| Micro watersheds | VIKOR priority rank | TOPSIS priority rank | COPRAS priority rank | K means Cluster priority rank |
| --- | --- | --- | --- | --- |
| 2A2B5k | 1 | 2 | 2 | 1 |
| 2A2B5b | 2 | 3 | 1 | 3 |
| 2A2B5j | 1 | 3 | 3 | 3 |
| 2A2B5h | 3 | 3 | 2 | 3 |
| 2A2B5m | 3 | 1 | 2 | 2 |

Table 14. performance evaluation metrics of VIKOR, TOPSIS, and COPRAS models  in comparison with the reference classification derived from K-Means clustering,