Supporting Information

Sources of Environmentally Persistent Free Radicals in PM2.5 in Dalian: Important Role of Polycyclic Aromatic Hydrocarbons

Zhansheng Li,1 Hongxia Zhao*,1 Xintong Li,1 Tadiyose Girma Bekele1

1 Key Laboratory of Industrial Ecology and Environmental Engineering (Ministry of Education), School of Environmental Science and Technology, Dalian University of Technology, Linggong Road 2, Dalian 116023, China

*Corresponding author phone: +86-411-84707965; Fax: +86-411-84707965; Email: hxzhao@dlut.edu.cn.

Chemicals and regents. Targets standards including 15 PHAs congeners, including Acenaphthylene (ACY, 3-ring), Acenaphthene (ACE, 3-ring), Fluorene (FLU, 3-ring), Phenanthrene (PHE, 3-ring), Anthracene (ANT, 3-ring), Fluoranthene (FLA, 4-ring), Pyrene (PYR, 4-ring), Benzo[a]anthracene (BaA, 4-ring), Chrysene (CHR, 4-ring), Benzo[b]fluoranthene (BbF, 5-ring), Benzo[k]fluoranthene (BkF, 5-ring), Benzo[a]pyrene (BaP, 5-ring), Dibenz[a,h]anthracene (DahA, 5-ring), Indeno[1,2,3-c,d]pyrene (IcdP, 6-ring), Benzo[g,h,i]perlene (BghiP, 6-ring) and 5  surrogate standards NAP-d8, ACE-d10, PHE-d10, CHR-d12, and PYR-d12 were both purchased from Aladdin (Shanghai, China). 2-diphenyl-1-picrylhudrazyl (DPPH) standard was purchased from Aladdin (Shanghai, China). Purities of all the standards were higher than 95%. HPLC grade solvents, including acetone, dichloromethane, ethyl acetate, methanol, and n-hexane were provided by Sigma-Aldrich (St. Louis, MO, USA). Analytical grade chemicals, including anhydrous sodium sulfate, (Na2SO4) (60 mesh), were purchased from Sigma-Aldrich (St. Louis, MO, USA). Neutral silica gel (70-230 mesh) was purchased from Thermo Fisher Scientific Co. A Millipore Milli-Q system was used to purify water.

Fig. S1. EPR spectrums of EPFRs bounded to PM2.5 in Dalian

Fig. S2. Liner relationship between DI/N and number of spin for DPPH

EPR spectra of standard substance DPPH: the relationship between doubling integration of fisrt-order derivative and the concentration of spins.

N(spins)=2.33×1011x+1.165×1015(R2=0.953)

Fig. S3. EPR spectra of the PM2.5 sample before and after visible light

illumination (November 13-15,2018)

Table S1 LOD and LOQ of 15 kinds of PAHs for PM2.5 samples

| Congener | LOD (pg/m3) | MDL (pg/m3) |
| --- | --- | --- |
| ACY | 0.670 | 2.21 |
| ACE | 5.09 | 16.8 |
| FLU | 5.30 | 17.5 |
| PHE | 16.2 | 53.5 |
| ANT | 2.01 | 6.61 |
| FLA | 8.46 | 27.9 |
| PYR | 3.69 | 12.2 |
| BaA | 0.681 | 2.25 |
| CHR | 1.15 | 3.80 |
| BbF | 0.593 | 1.96 |
| BkF | 0.612 | 2.02 |
| BaP | 0.842 | 2.78 |
| InD | 0.327 | 1.08 |
| DiB | 0.500 | 1.65 |
| BghiP | 0.506 | 1.67 |

The limit of detection and the limit of quantification were defined as three and ten times of the ratio of single to noise.

Table S2 PAH concentrations in PM2.5 samples in Dalian

| Congener | PAHs concentrations in PM2.5 (ng/m3) |  |  |
| --- | --- | --- | --- |
|  | Range | Mean | Std. |
| ACY | n.d.-0.16 | 0.04 | 0.06 |
| ACE | n.d.-0.13 | 0.06 | 0.03 |
| FLU | 0.01-0.33 | 0.13 | 0.07 |
| PHE | 0.07-3.65 | 1.34 | 1.36 |
| ANT | n.d.-0.51 | 0.09 | 0.15 |
| FLA | 0.11-13.12 | 2.49 | 3.18 |
| PYR | 0.07-11.9 | 2.13 | 2.74 |
| BaA | 0.02-7.01 | 1.46 | 1.92 |
| CHR | 0.07-9.85 | 2.13 | 2.49 |
| BbF | 0.06-13.0 | 2.86 | 3.2 |
| BkF | n.d,-3.46 | 0.72 | 0.88 |
| BaP | n.d.-5.21 | 1.16 | 1.40 |
| InP | 0.03-4.52 | 1.06 | 1.25 |
| DahA | 0.01-1.95 | 0.25 | 0.36 |
| BghiP | 0.02-3.34 | 0.87 | 0.98 |
| Σ16PAHs | 1.18-72.64 | 16.3 | 19.34 |

n.d.: not detected

| Date | g | ΔHp-p | Concentration | Date | g | ΔHp-p | Concentration |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2018.11.15-11.17 | 2.00344 | 8.732 | 1.21×1015 | 2019.04.03-04.05 | 2.00265 | 6.412 | 3.85×1013 |
| 2018.11.17-11.19 | 2.00311 | 5.993 | 1.45×1015 | 2019.04.05-04.07 | 2.00309 | 6.325 | 4.91×1013 |
| 2018.11.28-11.30 | 2.00325 | 5.962 | 3.56×1015 | 2019.04.08-04.10 | 2.00295 | 5.865 | 2.829×1013 |
| 2018.12.03-12.05 | 2.00344 | 5.371 | 2.99×1014 | 2019.04.10-04.12 | 2.00293 | 5.923 | 2.99×1013 |
| 2018.12.12-12.14 | 2.00294 | 7.136 | 8.97×1015 | 2019.04.12-04.14 | 2.00322 | 8.602 | 8.93×1013 |
| 2018.12.14-12.16 | 2.00333 | 6.549 | 2.54×1015 | 2019.04.15-04.17 | 2.00293 | 5.372 | 3.85×1013 |
| 2018.12.17-12.19 | 2.00335 | 5.963 | 1.29×1015 | 2019.04.17-04.19 | 2.00240 | 5.923 | 3.52×1013 |
| 2018.12.19-12.21 | 2.00324 | 6.648 | 1.75×1015 | 2019.04.19-04.21 | 2.00280 | 6.825 | 1.91×1013 |
| 2018.12.23-12.25 | 2.00335 | 7.527 | 1.60×1015 | 2019.04.21-04.23 | 2.00263 | 6.159 | 2.68×1013 |
| 2018.12.28-12.30 | 2.00344 | 6.647 | 1.10×1015 | 2019.04.25-04.27 | 2.00300 | 6.852 | 2.02×1013 |
| 2019.01.02-.1.04 | 2.00327 | 6.647 | 5.91×1014 | 2019.04.28-04.30 | 2.00281 | 6.312 | 7.02×1013 |
| 2019.01.04-01.06 | 2.00393 | 6.57 | 2.63×1015 | 2019.04.30-05.02 | 2.00332 | 6.549 | 6.76×1013 |
| 2019.01.06-01.08 | 2.00290 | 6.158 | 4.71×1015 | 2019.05.03-05.05 | 2.00275 | 5.963 | 1.60×1013 |
| 2019.01.08-01.10 | 2.00328 | 7.038 | 2.03×1015 | 2019.05.05-05.07 | 2.00296 | 6.648 | 3.60×1013 |
| 2019.01.10-01.12 | 2.00326 | 6.156 | 1.31×1015 | 2019.05.07-05.09 | 2.00245 | 7.527 | 4.78×1013 |
| 2019.01.12-01.14 | 2.00344 | 6.863 | 5.91×1014 | 2019.05.10-05.12 | 2.00303 | 6.647 | 1.13×1013 |
| 2019.01.14-01.16 | 2.00293 | 6.584 | 2.83×1015 | 2019.05.13-05.15 | 2.00296 | 5.693 | 4.81×1013 |
| 2019.01.16-01.18 | 2.00291 | 7.82 | 6.69×1015 | 2019.05.17-05.19 | 2.00341 | 6.285 | 1.63×1013 |
| 2019.01.19-01.21 | 2.00325 | 7.038 | 1.12×1015 | 2019.05.19-05.21 | 2.00326 | 6.648 | 1.86×1013 |
| 2019.01.21-01.23 | 2.00316 | 7.038 | 4.01×1015 | 2019.05.22-05.24 | 2.00222 | 6.156 | 3.74×1013 |
| 2019.01.23-01.25 | 2.00387 | 6.82 | 4.51×1014 | 2019.05.24-05.26 | 2.00262 | 6.583 | 3.85×1013 |
| 2019.05.29-05.31 | 2.00302 | 5.868 | 2.13×1013 | 2019.05.27-05.29 | 2.00255 | 6.292 | 4.21×1013 |

Table S3 Concentrations(spins/m3), g factor and ΔHp-p(G) of EPFRs in PM2

| Table S4 person correlation coefficients of EPFR with PAH congeners and criteria pollutants. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PM2.5 | PM10 | SO2 | NO | O3 | CO | ACY | ACE | FLU | PHE | ANT | FLA | PYR | BaA | CHR | BbF | BkF | BaP | IND | DiB | BghiP | ∑PAHs |
| EPFRs | 0.080 | 0.015 | .602** | .453** | -.558** | .405** | .606** | 0.055 | 0.135 | .595** | 0.231 | .552** | .561** | .549** | .545** | .493** | .514** | .560** | .564** | .422** | .564** | .560** |
| **Statistically significant correlation at p < 0.01 level |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| *Statistically significant correlation at p< 0.05 level. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |