Supplementary Material

Figure 1. The intracranial compliance curve that shows the relationship between intracranial volume and intracranial pressure. In the regime of high compliance, an accurate estimation of PRx is unlikely, even though autoregulation is expected to be preserved.

Code in R for the computation of PRx

# Function PRx computes PRx with 80% overlap to avoid aliasing

# Function PRx0 computes PRx as it is described in literature

library(signal)

PRx <- function(x, y, fs = 100) {

# 80% overlap

window_size <- 10 * fs   # 10 seconds

step_size <- 2 * fs      # 2 seconds

num_windows <- 30

# Moving average filter (FIR with equal weights)

ma_filter <- rep(1 / window_size, window_size)

# Filter the signals

x_avg <- filter(ma_filter, 1, x)

y_avg <- filter(ma_filter, 1, y)

# downsample the signals

x_down<-x_avg[seq(window_size, length(x_avg), by = step_size)]

y_down<-y_avg[seq(window_size, length(y_avg), by = step_size)]

return(cor(x_down, y_down, method = "pearson"))

}

PRx0 <- function(x, y, fs = 100) {

# 0% overlap

step <- 10 * fs

# Reshape x and y into matrices with 30 columns (one per chunk)

x_mat <- matrix(x[1:(30 * step)], nrow = step)

y_mat <- matrix(y[1:(30 * step)], nrow = step)

# Compute 10 sec averages

xav <- colMeans(x_mat)

yav <- colMeans(y_mat)

return(cor(xav, yav, method = "pearson"))

}

Code in R to plot PRx as a function of the frequency or amplitude of a sinusoidal interference on maximally anticorrelated linear signals (calls functions PRx and PRx0)

# Determines and plots PRx as a function of the interference frequency and amplitude

library(ggplot2)

library(latex2exp)

library(signal)

fs<-100

time<-seq(from=1/fs, to=300, by=1/fs)

u<-time/300

sinus<-function(f){return(sin(2*pi*f*time)/sqrt(6))}

# Predefine frequency range

frequency <- seq(1/300, 0.4, by = 1/6000)

n_freq <- length(frequency)

# Predefine relative amplitude range

logamplitude<-seq(-1,2.5,by = 0.1)

n_loga <- length(logamplitude)

# Preallocate results vectors

prxbyf <- numeric(n_freq)

prx0byf <- numeric(n_freq)

prxbyA <- numeric(n_loga)

prx0byA <- numeric(n_loga)

prxbyB <- numeric(n_loga)

prx0byB <- numeric(n_loga)

# Precompute sinusoidal matrix: each column is sinus(2πft) for a frequency f

sin_matrix <- sin(2 * pi * outer(frequency, time)) /sqrt(6) # dimensions: [length(frequency) x length(time)]

# Compute PRx for each frequency

for (i in seq_along(frequency)) {

presin_f <- sin_matrix[i, ]

sin_f<-presin_f-mean(presin_f)

prx0byf[i] <- PRx0(u + 0.75*sin_f, 1 - u + 0.75*sin_f,fs)

prxbyf[i] <- PRx(u + 0.75*sin_f, 1 - u + 0.75*sin_f, fs)

}

# Plot the result by frequency

df<-data.frame(

x<-rep(frequency,2),

y<-c(prx0byf,prxbyf),

Tattr <- factor(rep(c("Aliasing", "No aliasing"), each = length(frequency)))

)

pf<-ggplot(df,aes(x=x, y=y, color = Tattr))+

geom_line(linewidth=0.5)+

scale_x_continuous(breaks=seq(0,0.4,0.05))+

scale_color_manual(values = c("Aliasing" = "tomato3", "No aliasing" = "navyblue")) +

labs(x="frequency [Hz]",

y="PRx",

color="Keys")+

theme(

legend.position = c(0.95, 0.95),

legend.justification = c(0.95, 0.95),

legend.background = element_rect(fill = "white", color = "black")  # Optional: make it more readable

)

# print (pf)

# Compute PRx for each amplitude

testfreq<-c(0.15,0.175)

k<-floor(6000*testfreq-19)

presin_A <- sin_matrix[k[1], ]

sin_A<-presin_A-mean(presin_A)

i<-1

for (A in logamplitude){

prx0byA[i] <- PRx0(u + 0.75*10**A*sin_A, 1 - u + 0.75*10**A*sin_A,fs)

prxbyA[i] <- PRx(u + 0.75*10**A*sin_A, 1 - u + 0.75*10**A*sin_A, fs)

i <- i+1

}

presin_B <- sin_matrix[k[2], ]

sin_B<-presin_B-mean(presin_B)

i<-1

for (A in logamplitude){

prx0byB[i] <- PRx0(u + 0.75*10**A*sin_B, 1 - u + 0.75*10**A*sin_B,fs)

prxbyB[i] <- PRx(u + 0.75*10**A*sin_B, 1 - u + 0.75*10**A*sin_B, fs)

i <- i+1

}

# Plot the result my amplitude

df<-data.frame(

x<-rep(logamplitude,4),

y<-c(prx0byA,prxbyA,prx0byB,prxbyB),

Tattr <- factor(rep(c("f=0.15 Hz, Aliasing", "f=0.15 Hz, No aliasing", "f=0.175 Hz, Aliasing", "f=0.175 Hz, No aliasing" ), each = length(logamplitude)))

)

pA<-ggplot(df,aes(x=x, y=y, color = Tattr))+

geom_line(linewidth=0.5)+

scale_x_continuous(breaks=10*logamplitude)+

scale_color_manual(values = c("f=0.15 Hz, Aliasing" = "tomato3", "f=0.15 Hz, No aliasing" = "navyblue", "f=0.175 Hz, Aliasing" = "darkolivegreen4", "f=0.175 Hz, No aliasing" = "pink2" )) +

# scale_linetype_manual(values = c("signal 1" = "solid", "signal 2" = "dotdash"))+

labs(x=TeX("$\\log(A/A_0)$"),

y="PRx",

#linetype="Keys"

color="Keys")+

theme(

legend.position = c(0.95, 0.1),

legend.justification = c(0.95, 0.05),

legend.background = element_rect(fill = "white", color = "black")  # Optional: make it more readable

)

# print(pA)