Table 1. Mass balance equations for four models

| Model | Governing Equations | Remarks |
| --- | --- | --- |
| PFM | dXdZ+ΩNTUod X-Y=0 | Continuous phase mass balance |
|  | dYdZ+NTUodX-Y=0 | Dispersed phase mass balance |
|  | Z=0→X0=Xin=1 | Boundary Conditions at top of column (z=0) |
|  | Z=1 →Y1=Yin=0 | Boundary Conditions at bottem of column (z=H) |
| BFM | 1+α Xn-1-1+2α Xn+α Xn+1- ΩNTUodNXn-Yn=0 | Continuous phase mass balance |
|  | 1+β Yn+1-1+2β Yn+β Yn-1+NTUodN (Xn-Yn)=0 | Dispersed phase mass balance |
|  | Z=0 → X0+α(X0-X1)=1Y0=Y0=Y1 | Boundary Conditions at top of column (z=0) |
|  | Z=1 → XN+1=XN+1=XN YN+1-βYN-YN+1=0 | Boundary Conditions at bottem of column (z=H) |
| ADM | dXdZ-1Pecd2XdZ2+ΩNTUodX-Y=0 | Continuous phase mass balance |
|  | dYdZ+1Ped d2YdZ2+NTUodX-Y=0 | Dispersed phase mass balance |
|  | Z=0 → UcEc 1-X0=-dXdZ0 dYdZ0=0 → Y0=Yout | Boundary Conditions at top of column (z=0) |
|  | Z=1 → dXdZ1=0→ X1=XoutUdEd Y1=-dXdZ1 | Boundary Conditions at bottem of column (z=H) |
| FMM | dXdZ-1Pec d2XdZ2+Ωi=1N NTUod,i X-Yi=0 | Continuous phase mass balance |
|  | dYidZ+NTUod,igi X-Yi=0 (i=1, 2,…N) | Dispersed phase mass balance |
|  | Z=0 → UcEc (1-X0)=-dXdZ0 Yi0=Yiout i=1, 2,…N | Boundary Conditions at top of column (z=0) |
|  | Z=1 → dXdZ1=0 → X1=Xout Yi1=Yin=0 i=1, 2,…N | Boundary Conditions at bottem of column (z=H) |
|  | gi=fi uij=1Nfj uj | Dynamic drop size distribution |
|  | ui=did43 Uslip-Uc1-ϕ | Drop velocity |
|  | d43=i=1Nni di4j=1Nnj dj3 |  |
|  | fi=Pi3i=1NPi3 | Volumetric drop size distribution |
|  | ai= 6 ϕidi | Drop specific surface area |
|  | ϕi=ϕ fi | Drop holdup |