Archives of Environmental Contamination and Toxicology

# Supporting Information for

Sorption of polycyclic musks on soil components of different aggregate sizes: The effect of organic-mineral interactions

Qingwei Bu1*, Hongmei Cao1, Dongkui Wu1,2, Ming Zhou1

1 School of Chemical & Environmental Engineering, China University of Mining & Technology-Beijing, Beijing 100083, P.R. China

2 State Key Laboratory of Pollution Control & Resource Reuse, School of the Environment, Nanjing University, Nanjing 210023, China

* Corresponding author

Address correspondence to Qingwei Bu, School of Chemical & Environmental Engineering, China University of Mining & Technology-Beijing, Beijing 100083, P.R. China. E-mail: qingwei.bu@cumtb.edu.cn

Number of pages: 6

Number of tables: 5

Number of figures: 3

Mathematical manipulation of sorption data. All sorption data were fitted to the logarithmic form of the Freundlich equation:

Qe=KF×CeN	(1)

Where Qe is the amount of solute adsorbed per unit mass of adsorbent (mg·kg-1); Ce is the solution concentration (mg·L-1) after the equilibrium time; and KF (Freundlich coefficient) and N are empirical constants related to sorption.

The sorption partition coefficient (Kd, L·g−1) and the organic carbon (OC) content normalized partition coefficient (Koc, L·g−1) were also analyzed from:

Kd=QeCe	(2)

Koc=Kdfoc	(3)

where foc is the OC content of SOM (mg·g-1)

Calculation of contributions of different aggregate fractions to PCMs sorption by soils. The contribution (α) of each size fraction to the total sorption of butachlor in soils was calculated based on the proportions of butachlor absorbed by each soil fraction.

α=Qi,fWfQi,b	(4)

where Qi,f is the amount of sorption in the i aggregate fractions (mg kg-1), Wf is the percentage by weight of the fraction in bulk soil (%) and Qi,b is the amount of sorption in the bulk soil sorption (mg kg-1).

Calculation of overlay ∆Koc value. The ∆Koc of each size fraction to the total sorption of butachlor in soils was calculated based on the proportions of butachlor absorbed by each soil fraction.

∆Kocoverlay=∆Koci,fWf∆Koci,b	(5)

where ∆Koci,f is the amount of Koc change before and after HF treatment in the i aggregate fraction (g·L-1), Wf is the percentage by weight of the fraction in bulk soil (%) and ∆Koc i,b is the amount of Koc change before and after HF treatment in the bulk soils (g·L-1).

Table S1 Classification of soils.

| Soil No. | Soil type (Chinese classification) | Province | Textural class | pH |
| --- | --- | --- | --- | --- |
| Soil 1 | Yellow earths | Hunan | Silty clay loam | 7.05 |
| Soil 2 | Latosol | Guangdong | Clay | 7.71 |
| Soil 3 | Red earths | Jiangxi | Silt loam | 4.78 |
| Soil 4 | Desert soil | Gansu | Silt loam | 7.16 |
| Soil 5 | Red earths | Zhejiang | Sandy loam | 4.28 |
| Soil 6 | Black soils | Jilin | Silty clay loam | 7.35 |

Table S2 Procedures for aggregate size fractionation of soils

| Step | Size | Procedures |
| --- | --- | --- |
| 1 | 80 mesh 200 μm | About 50 g of soil was suspended in 250 mL of deionized water with 5-mm-diameter glass beads and the aggregates ruptured by mechanical shaking for 16 h. and then sonicated for 2 min in a 200 W ultrasound device (approximately 60 J·mL-1) |
| 2 | 50-200 μm | The suspended material passing the 300-mesh sieve and then rinse with deionized water repeatedly until clear and dry at 50℃. |
| 3 | 5-50 μm | The suspended material from previous step with 500 ml deionized water were further dispersed by sonication for 12.5 min and stirred for 1 min to mix the suspension distribution. The settling time was recorded when the agitation stopped. Fine sands were then separated by sedimentation under gravity. |
| 4 | 1-5 μm | The suspension collected in the previous step was transferred to a 100 mL centrifuge tube, centrifuged at 1500 r·min-1 for 6 mins (20℃), and dried at 50℃. |
| 5 | < 1 μm | The supernatant obtained in the previous step was transferred to 100 mL centrifuge tube, centrifuged at 5000 r·min-1 for 1 min (20℃), and then put into the oven for drying at 50℃. |

Table S3 Freundlich parameters for HHCB and AHTN sorption on soil with the highest and lowest organic carbon (OC) content based on the Freundlich equation

| Soil No. | HHCB |  |  | AHTN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KF | N | R2 | KF | N | R2 |
| Soil-H | 98.87** | 0.265** | 0.894 | 85.52** | 0.287** | 0.902 |
| Soil-L | 75.24** | 0.250** | 0.892 | 59.15** | 0.273** | 0.902 |

**: Correlation is significant at p < 0.01.

Soil-H and Soil-L represented soil with the highest and lowest organic carbon (OC) content.

Table S4 The OC content of SOM (foc) in different particle-size fractions and bulk soil

| Soil No. | foc / mg·g-1 |  |  |  |  | The sum percentage of foc in different aggregate fractions （%） |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bulk soil | 50-200 μm | 5-50 μm | 1-5 μm | <1 μm |  |
| Soil 1 | 5.12 | 5.03 | 5.16 | 5.94 | 6.85 | 100.6 |
| Soil 2 | 9.33 | 8.15 | 12.4 | 7.63 | 6.85 | 95.8 |
| Soil 3 | 6.75 | 15.02 | 3.54 | 5.19 | 9.44 | 83.5 |
| Soil 4 | 8.94 | 13.9 | 5.15 | 15.4 | 13.8 | 98.4 |
| Soil 5 | 15.0 | 10.4 | 11.7 | 23.8 | 22.5 | 101.8 |
| Soil 6 | 17.8 | 9.95 | 19.2 | 34.4 | 35.5 | 89.4 |

Table S5 The OC content of SOM (foc) in different particle-size fractions and bulk soil after HF treatment

| Soil No. | foc / mg·g-1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Bulk soil | 50-200 μm | 5-50 μm | 1-5 μm | <1 μm |
| Soil 1 | 22.45 | 22.62 | 22.82 | 12.06 | 43.69 |
| Soil 2 | 23.57 | 14.74 | 25.85 | 205.94 | 24.26 |
| Soil 3 | 19.89 | 16.95 | 25.04 | 45.25 | 65.54 |
| Soil 4 | 27.59 | 44.99 | 18.32 | 44.42 | 30.19 |
| Soil 5 | 46.00 | 14.62 | 51.60 | 22.09 | 230.81 |
| Soil 6 | 61.93 | 42.83 | 0.86 | 187.50 | 188.41 |

Fig.S1. Particle size distribution

Fig.S2. ∆ Koc versus ∆ foc for all bulk soils and different aggregate fractions

Fig.S3. The Koc variation of AHTN before and after HF treatment among selected bulk soils and their different aggregate fractions