Supplementary Material and Methods

Determination of M. abscessus complex

All material samples suspected of mycobacterial contamination in the Juntendo university hospital were cultured in a mycobacteria growth indicator tube (MGIT; Becton Dickinson, USA) broth and incubated at 37°C in the BACTEC MGIT 960 (Becton Dickinson, USA) instrument with ambient air. MGIT positive tubes were classiﬁed as M. abscessus based on the results of DNA–DNA hybridization (DDH) analysis (DDH Mycobacterium “Kyokuto” kit; Kyokuto Pharmaceutical Industrial, Japan) or matrix-assisted laser-desorption/ionization time-of-flight mass spectrometry (MALDI-TOF MS). Detected species were reconfirmed as three subspecies of M. abscessus complex by sequencing the 16S rRNA, rpoB, hsp65, and erm genes [1,2]. All strains of M. abscessus were cultured on BD trypticase soy agar II with 5% sheep blood (Blood agar; Nippon Becton-Dickinson and Company, Japan) at 35°C for approximately 4 to 6 days to observe colony morphology and purity and then used for species identification based on multi-locus sequence analysis.

PCR amplification, DNA sequencing, and MALDI-TOF MS analysis

DNA was extracted from cultured colonies using the DNeasy UltraClean Microbial Kit (QIAGEN, Germany), and PCR was conducted using Ex Taq DNA polymerase, hot-start version (Takara, Japan) according to the manufacturer’s instructions. The gene-specific primer pairs used for PCR analysis are listed in table S1; these primers were used in previous studies [3,4]. The sequencing PCR products were purified with the BigDye XTerminator purification kit (Life Technologies, USA), and samples were loaded on the ABI Prism 3130 Genetic Analyzer (Thermo Fisher Scientific, USA). The DNA sequencing results were analyzed using a BLAST search to identify sequence similarity between samples and the three species of M. abscessus complex.

MALDI-TOF MS analysis was performed based on previously described methods [5]. Colonies of M. abscessus complex on blood agar were scratched with a needle, and particles on the needle surface were diluted in 50 μL 80% trifluoroacetic acid. After incubation for 15 minutes at room temperature, the solution was added to 150 μL distilled water and 200 μL 100% acetonitrile, followed by a centrifugation step (16,200 × g, 2 min). One microliter of the cleared supernatant containing the bacterial extract was transferred onto a MALDI target plate (Bruker Daltonik, Germany). We overlaid dried spots with MALDI matrix (10 mg/mL α-cyano-4-hydroxy cinnamic acid [α-HCCA] in 50% acetonitrile:2.5% trifluoroacetic acid) (Bruker Daltonik, Germany). After drying the matrix, we conducted MALDI-TOF MS analysis with a Microflex LT/SH benchtop mass spectrometer (Bruker Daltonik, Germany) equipped with a 60-Hz nitrogen laser. We had optimized parameter settings (ion source 1 [IS1], 20 kV; IS2, 18.2 kV; lens, 6.85 kV; detector gain, 2854 V; gating, none) for the mass range between 2,000 and 20,000 Da. We achieved spectra in the positive linear mode with the maximum laser frequency. An external standard (bacterial test standard [BTS]) (Bruker Daltonik, Germany) was applied for instrument calibration. Data evaluation was performed by visually comparing spectra to search for peak shifts using flexAnalysis 3.4 (Bruker Daltonik, Germany).

Antimicrobial susceptibility testing

Susceptibility testing was performed according to Clinical and Laboratory Standard Institute (CLSI) guideline M24-A2 [6]. MIC determinations and synergy testing were performed by the checkerboard method, using frozen broth microdilution plates (Eiken Chemical Co., Ltd., Japan). The ranges of antibiotic concentrations tested were as follows: arbekacin (ABK) 1 to 8 μg/mL, amikacin (AMK) 1 to 64 μg/mL, imipenem (IPM) 2 to 32 μg/mL, and sitafloxacin (STFX) 0.12 to 32 μg/mL. MICs of each antimicrobial agent were determined by broth microdilution methods as recommended by the CLSI. The panels were prepared with a 96-channel dispenser and stored at -80°C until use. Sitafloxacin were dispensed alone in the first row, and arbekacin, amikacin, imipenem were dispensed in the first column. Each well was inoculated with a concentration of 1 × 105 colony-forming units (CFU)/mL. The MICs were determined after 7 days of incubation at 35°C.

Reference

1.	Kim BJ, Yi SY, Shim TS, et al. Discovery of a novel hsp65 genotype within Mycobacterium massiliense associated with the rough colony morphology. PLoS One. 2012;7(6):e38420.

2.	Yoshida S, Tsuyuguchi K, Suzuki K, et al. Further isolation of Mycobacterium abscessus subsp. abscessus and subsp. bolletii in different regions of Japan and susceptibility of these isolates to antimicrobial agents. Int J Antimicrob Agents. 2013;42(3):226-231.

3.	Nakanaga K, Sekizuka T, Fukano H, et al. Discrimination of Mycobacterium abscessus subsp. massiliense from Mycobacterium abscessus subsp. abscessus in clinical isolates by multiplex PCR. J Clin Microbiol. 2014;52(1):251-259.

4.	Brown-Elliott BA, Vasireddy S, Vasireddy R, et al. Utility of sequencing the erm(41) gene in isolates of Mycobacterium abscessus subsp. abscessus with low and intermediate clarithromycin MICs. J Clin Microbiol. 2015;53(4):1211-1215.

5.	Sparbier K, Lange C, Jung J, Wieser A, Schubert S, Kostrzewa M. MALDI biotyper-based rapid resistance detection by stable-isotope labeling. J Clin Microbiol. 2013;51(11):3741-3748.

6.	Woods GL, Brown-Elliott BA, Conville PS, et al. In: nd, ed. Susceptibility Testing of Mycobacteria, Nocardiae, and Other Aerobic Actinomycetes. Wayne (PA)2011.

Supplementary Table

Supplementary Table 1: Forward and backward primers used for PCR

| Target | Sequence |
| --- | --- |
| 16S rRNA | Forward, 5’-AGA GTT TGA TCM TGG CTC AG-3’ Reverse, 5’-TAC GGT TAC CTT GTT ACG AC-3’ |
| rpoB | Forward, 5’-GAG GGT CAG ACC ACG ATG AC -3’ Reverse, 5’-AGC CGA TCA GAC CGA TGT T-3’ |
| hsp65 | Forward, 5’ACC AAC GAT GGT GTG TCC AT -3’ Reverse, 5’ CTT GTC GAA CCG CAT ACC CT-3’ |
| erm | Forward, 5’-GAC CGG GCC TTC GTG AT -3’ Reverse, 5’-GAC TTC CCC GCA CCG ATT CC-3’ |

Supplementary Table 2: The changes of median MIC of sitafloxacin and arbekacin between monotherapy and combination therapy.

| Species | Median MIC of STFX (monotherapy) | Median MIC of STFX (combination therapy) | p value | Median MIC of ABK (monotherapy) | Median MIC of ABK (combination therapy) | p value |
| --- | --- | --- | --- | --- | --- | --- |
| M.abscessus complex | 2 | 0.25 | 0.028* | 4 | 2 | <0.001** |
| N=34† |  |  |  |  |  |  |
| M. abscessus subsp. massiliense | 2 | 2.25 | 0.42 | 4 | 3 | 0.002** |
| N=22 |  |  |  |  |  |  |
| M. abscessus subsp. abscessus | 2 | 0.12 | 0.031* | 4 | 2 | 0.008* |
| N=11 |  |  |  |  |  |  |

†Including M. abscessus subsp. boletii (n=1)

*p value <0.05, ** p value <0.01

Abbreviations: STFX, sitafloxacin; ABK, arbekacin.