Table 1: The General Characterization of the Used Crude Oil (QARUN)

| Experimental | Method | Result |
| --- | --- | --- |
| Density @ 15.56°C Specific gravity API gravity @ 60 °F | ASTM D-4052 | 0.8933 0.8941 26.75 |
| Kinematic viscosity, cSt, @ 40o C | ASTM D-445 | 11.71 |
| Asphaltene content, wt.% | IP-143 | 5.34 |
| Wax content, wt. % | UOP-64 | 14.58 |
| Pour point, °C | ASTM D-97 | 9 |
| Flash point, °C | ASTM D-93 | ˂30 |
| Water content, vol. % | ASTM E-203 | 0.1 |

Table 2: Surface Active and Thermodynamic Properties of the Prepared Nonionic and Anionic Surfactants at 25oC

| Surfactant |  | γ (mN m-1) | CMC x 103 (mol dm-3) | πcmc (mN m-1) | Γmax x 1010 (mol cm-2) | Amin (Ao2 molecule-1) | ∆ G mic (kJ mol-1) | ∆Gads (kJ mol-1) | pC20 (mol dm-3) | C20 x 105 (mol dm-3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Non-Ionic | ## NMAE-10 | 32 | 2.48 | 36 | 2.25 | 73.82 | -14.86 | -20.38 | 4.34 | 4.54 |
|  | NMAE-12 | 27 | 1.84 | 41 | 1.79 | 92.81 | -15.60 | -23.37 | 4.86 | 1.37 |
|  | NMAE-14 | 30 | 2.24 | 38 | 1.89 | 87.67 | -15.11 | -22.45 | 4.69 | 2.04 |
| ## Anionic | ## AMAES-10 | 25 | 2.03 | 43 | 1.68 | 98.60 | -15.35 | -21.81 | 5.47 | 0.34 |
|  | AMAES-12 | 20 | 1.66 | 48 | 1.48 | 112.34 | -15.85 | -24.63 | 6.43 | 0.04 |
|  | AMAES-14 | 23 | 1.84 | 45 | 1.58 | 104.89 | -15.60 | -23.33 | 6.08 | 0.08 |

Table 3: Surface Tension, Interfacial Tension and Contact Angle of the Prepared Nonionic and Anionic Surfactants at CMC and 25 oC

| Surfactant |  | γ, mN m-1 | IFT, mN m-1 | Contact Angle (o) |
| --- | --- | --- | --- | --- |
| Blank crude oil |  | NA | 26 | 130.07 |
| ## Non-Ionic | ## NMAE-10 | 32 | 0.9 | 64.94 |
|  | NMAE-12 | 27 | 0.4 | 52.57 |
|  | NMAE-14 | 30 | 0.7 | 55.54 |
| ## Anionic | ## AMAES-10 | 25 | 0.1 | 47.34 |
|  | AMAES-12 | 20 | 0.06 | 40.34 |
|  | AMAES-14 | 23 | 0.08 | 44.38 |

Table 4: Surface Tension, Interfacial Tension and Contact Angle of the Prepared Nonionic and Anionic Surfactants at CMC and 50 ⁰C

| Surfactant |  | γ, mN m-1 | IFT x 102, mN m-1 | Contact Angle (o) |
| --- | --- | --- | --- | --- |
| ## Non-Ionic | ## NMAE-10 | 29.6 | 0.09 | 26 |
|  | NMAE-12 | 24.5 | 0.04 | 21 |
|  | NMAE-14 | 27.5 | 0.07 | 24 |
| ## Anionic | ## AMAES-10 | 21.5 | 0.08 | 16 |
|  | AMAES-12 | 16.3 | 0.01 | 9 |
|  | AMAES-14 | 19.2 | 0.05 | 13 |

Table 5: Work of Adhesion, Surface Free Energy and Spreading Coefficient at the CMC of the Prepared Surfactants

| Surfactant |  | Work Adhesion, Wa (mJ/m2) | Surface Free Energy "Interfacial Free Energy", S (mJ/m2) | Spreading Coefficient, SC (mJ/m2) |
| --- | --- | --- | --- | --- |
| ## Non-Ionic | ## NMAE-10 | 45.55 | 14.45 | -18.45 |
|  | NMAE-12 | 43.41 | 17.67 | -10.59 |
|  | NMAE-14 | 46.97 | 16.81 | -13.03 |
| ## Anionic | ## AMAES-10 | 41.94 | 17.02 | -8.06 |
|  | AMAES-12 | 35.24 | 16.50 | -4.76 |
|  | AMAES-14 | 39.44 | 15.24 | -6.56 |

| Surfactant | RF2ry |  | RF3ry |  | Residual Oil |  | Total Recovery Factor, RFTotal ( RF2ry + RF3ry) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ml | % | ml | % | ml | % | ml | % |
| ## AMAES-10 | 58.5 | 45 | 48 | 36.92 | 23.5 | 18.07 | 106.5 | 81.92 |
| AMAES-12 | 57 | 43.84 | 63 | 48.46 | 10 | 7.69 | 120 | 92.30 |
| AMAES-14 | 58.5 | 54 | 52 | 40 | 24 | 18.46 | 106 | 81.53 |

Table 6: Recovery Factor (RF) for the 2ry and 3ry Flooding at 50 oC and Salinity equals 50x103 ppm

N.B.: OOIP = 130 ml