Sources

— Bonus dataset/source you can cite in Methods if you used it —
• W. Kirchgässner, O. Wallscheid, and J. Böcker, “Electric Motor Temperature (PMSM) Dataset,” Kaggle, 2021, doi:10.34740/KAGGLE/DSV/2161054. Kaggle+1

Appendix A: Scheduling Parameters and Limits

Nominal controller settings

Kp0 = 0.8;   Ki0 = 0.2;   Kd0 = 0.05;    λ0 = 0.8;   μ0 = 0.6

Scheduling law weights (used unless stated otherwise)

αp = 0.35;   αi = 0.35;   αd = 0.25;     βλ = 0.15;  βμ = 0.15;   γ = 0.25

Risk filter time constant: τf = 50 ms;   Deadband: ε = 0.02

Scheduling horizon: H = 5 s;             Margin band: Δ = 10 °C

Clamps (ensure safe ranges)

Kp ∈ [0.50·Kp0, 1.05·Kp0]   →  [0.40, 0.84]

Ki ∈ [0.40·Ki0, 1.05·Ki0]   →  [0.08, 0.21]

Kd ∈ [0.50·Kd0, 1.10·Kd0]   →  [0.025, 0.055]

λ  ∈ [0.70, 0.95]

μ  ∈ [0.55, 0.90]

Thermal limits (component-wise Tsafe)

Stator winding (Tw): 160 °C

Rotor magnet (Tpm): 120 °C

Stator tooth / yoke (Ttooth/Tyoke): per motor datasheet

Soft actuator cap

τmax(t) = τ0 · (1 − γ r)   (or iq,max(t) cap in current-limited drives); τ0 is the nominal torque limit.

Notes

• Parameters chosen by light grid/GA search on validation sessions; tune per motor if needed.

• All errors reported in rpm unless stated otherwise.