Supplementary Material

1. List of 13 Tidal Constituents data and their Influence

Table 1 Tidal Constituent

Sr. No.; Constituent; Type; Primary Astronomical Influence. 1; M2; Principal lunar semi-diurnal; Moon's gravitational pull. 2; S2; Principal solar semi-diurnal; Sun's gravitational pull. 3; N2; Larger lunar elliptic semi-diurnal; Lunar elliptical orbit variation. 4; K2; Lunisolar semi-diurnal; Combined lunar/solar declination. 5; K1; Lunisolar diurnal; Earth's rotation & celestial declination. 6; O1; Principal lunar diurnal; Lunar declination effects. 7; P1; Principal solar diurnal; Solar declination effects. 8; Q1; Larger lunar elliptic diurnal; Lunar perigee influence. 9; 2N2; Lunar elliptical semi-diurnal; Second-order lunar elliptic component. 10; L2; Smaller lunar elliptic semi-diurnal; Lunar nodal modulation. 11; MU2; Lunar-solar semi-diurnal; Shallow-water interaction (M2+O1). 12; NU2; Lunar elliptic semi-diurnal; Lunar nodal cycle variation. 13; T2; Solar elliptic semi-diurnal; Solar radiation pressure effects.

2. River Discharge for Naraj and Jagatpur Station

Table 2 River Discharge

Date and time; Naraj (m3/sec); Jagatpur (m3/sec). 02/05/2019 0800; 168.29; 61.50. 03/05/2019 0800; 182.78; 66.50. 04/05/2019 0800; 299.49; 82.50. 05/05/2019 0800; 385.80; 91.28.

3. MODIS-Derived LULC Data and Assigned Mannings N value

|  | Class; Manning's n Coefficient; Surface Roughness Length. Evergreen broad leaf forest; 0.12; 0.5. Deciduous broad leaf forest; 0.10; 0.5. Mixed forest; 0.10; 0.35. Shrublands; 0.05; 0.03. Grasslands; 0.05; 0.10. Permanent wetlands; 0.05; 0.30. Croplands; 0.04; 0.30. Urban and built-up areas; 0.03; 0.50. Natural Vegetation; 0.035; 0.055. Barren; 0.30; 0.02. Water bodies; 0.0035; 0.0001. |
| --- | --- |
| Figure 1 MODIS LULC Map Year 2018 | Table 3 Manning’s ‘n Coefficients and Surface Roughness Length for Land Cover Classes |

4.  Sentinel-2 Classified LULC and Assigned Mannings N Value

|  | Class; Manning's n; Impervious Percentage (%). Dense Deciduous Forest; 0.1; 5. Open Deciduous Forest; 0.15; 3. Cropland; 0.04; 2. Water; 0.02; 0. Wetland; 0.05; 0. Built-up; 0.1; 85. Wasteland; 0.025; 10. |
| --- | --- |
| Figure 2 Sentinel 2 LULC Map Year 2018 | Table 4 Manning’s ‘n Coefficients for Land Cover Classes of Sentinel 2 |

| (a) | (b) |
| --- | --- |
| Figure 3 Simulated (a) Atmospheric Pressure and (b) Wind Speed Distribution in Optimized model |  |

5. Simulated Atmospheric Pressure and Wind Speed for the ADCIRC Domain Region