Supplementary Information

1. Details of the numerical simulations

We have performed finite element simulations to get more insights on the mechanism controlling the ion-exchange micropumps. The simulations were done using the software COMSOL Multiphysics v4.3. The studies were performed by solving the stationary coupled governing equations:

-ε∇2φ=ρe 		                                    (S.1)

∇∙v=0     	                                    	(S.2)

η∇2 v=∇p+ ρe∇φ      	                  		(S.3)

v∙∇Ci=∇ ∙Di∇Ci+ziFμi∇φCi.      	  	(S.4)

Equation 1 stands for Poisson’s equation for electrostatics, relating the local charge density  ρe=iFziCi  with the electrostatic potential φ, where ε (=εrε0) represents the permittivity of the liquid, F is the Faraday’s constant and Ci and zi are the molar concentration and the valence of each ionic species, respectively, denoted by the subscript i. Equations 2 and 3 are the stationary Stokes’ equations describing the motion of an incompressible fluid at low Reynolds numbers, where v , p and η are the fluid velocity,  pressure and viscosity, respectively. Finally, Eq. 4 is the stationary Nernst-Planck’s equation for mass transport, where Di is the diffusion coefficient, and μi the mobility of the different species. The mobility is connected to the diffusion coefficient through Einstein´s relation, Di = μi RT, where R is the ideal gas constant and T is the absolute temperature.

To avoid charge accumulation and to maintain steady-state conditions, we imposed in the simulations a local current exchange constraint at the Nafion interface, jH+=-jM+, i.e., the rate of protons released by the Nafion into the solution must be the same as the rate of uptaken salt cation (M+). This requirement is implemented in COMSOL simulations through the boundary conditions at the Nafion surface. Water equilibrium dissociation reaction is considered in the bulk liquid55.

In the simulations, four different charged species were considered: protons, hydroxide ions, and the monovalent LiCl salt. The concentration of LiCl used in the simulations was Cd=1.0×10-1mol/m3.

Figure S1. Scheme of the layout of the micropump systems used in the simulations. a) Radial Nafion pump. b) Micropump array of SiO2/Nafion/Al2O3 stripes designed to achieve unidirectional pumping.

The following boundary conditions were defined: φ(z=h)=0 and φ(z=0)= ζm, where h=300 μm is the height of the experimental cell and m represents  the zeta potential of the different surfaces. For the fluid velocity, stick boundary conditions were imposed on the substrate and all cell boundaries. For the concentrations of the different species, the bulk values were imposed at the upper boundary h. The proper boundary conditions to be set for ion exchange at the Nafion interface are not well known. We performed simulations using two different sets of boundary conditions: by assuming that Nafion is a perfect drain for the salt cation, i.e., cM+z=0=0; or treating the ion exchange as a first order reaction characterized by a exchange rate jM+z=0=-kexcM+z=0, where kex is the exchange rate constant. All results reported have been obtained considering the ion exchange as a first order reaction with an exchange rate constant kex=10-4 m/s.

Two different micropump geometries, shown in Fig. S1, have been simulated. The first one, shown in Fig. S1a, resembles the radial Nafion micropump and consist of a 50 µm Nafion radius disc surrounded by a ring of deactivated Nafion of 200 µm width. Taking advantage of the radial symmetry of this configuration, a 2D axisymmetric domain was implemented. The simulation domain was discretized using an extremely fine mesh with 155250 triangular elements that was refined using 100 boundary layers near the surface. The values used for the zeta potential of the Nafion (Nafion = -73 mV), and deactivated Nafion (DN = -37 mV) regions were the same as the ones measured experimentally by streaming potential/current.

The second simulated system resembles the repeating structure of alternating SiO2/Nafion/Al2O3 stripes designed to achieve unidirectional pumping. The geometrical layout of the simulation is illustrated in Fig. S1b. Given the symmetry of the problem, a 2D section of the real system was implemented as simulation domain, which was discretized using an extremely fine mesh with 903462 triangular elements that was refined using also 100 boundary layers near the surface. Five repeating structures of the basic SiO2/Nafion/Al2O3 pumping unit were simulated flanked by 100 µm of SiO2 at the beginning and end of the structure. The values used for the zeta potential of the Nafion (Nafion = -73 mV), SiO2 (SiO2 = -66 mV), and Al2O3 (Al2O3 = +17 mV) stripes were those measured experimentally by streaming potential/current.

A mesh density convergence study was performed to guarantee that discretization errors in the numerical solution were below 5% in all cases. Only stationary solutions were evaluated corresponding to steady state conditions.

Table S1 collects the parameters used to make the simulations, as well as the dimensions of the 2D models detailed in the work.

| Parameter | Description | Value |
| --- | --- | --- |
| DH+ | Diffusion coefficient of H+ | 9.3×10-9m2/s |
| DOH- | Diffusion coefficient of OH- | 5.3×10-9m2/s |
| DLi+ | Diffusion coefficient of Li+ | 1.03×10-9m2/s |
| DCl- | Diffusion coefficient of Cl- | 1.0×10-9m2/s |
| T | Temperature | 298K |
| RNaf | Radius of Nafion disc | 50 µm |
| wNaf | Width of Nafion stripes | 25 µm |
| wAl2O3 | Width of Al2O3 stripes | 30 µm |
| wSiO2 | Width of SiO2 stripes | 25 µm |
| Cd | Bulk salt concentration | 1.0×10-1.mol/m3 |
| Cb | Water pH (bulk) | 5.7 |
| Nafion | Zeta potential Nafion | -73 mV |
| DN | Zeta potential of deactivated Nafion | -37 mV |
| Al2O3 | Zeta potential Al2O3 | 17 mV |
| SiO2 | Zeta potential SiO2 | -66 mV |
| h | Cell height | 300 µm |
| εr | Relative Permittivity | 78. |
| Ρ | Density | 103kg/m3 |
|  | Dynamic Viscosity | 10-3Pa·s |

Table S1. Reference parameters used in the simulations

1.1 Radial Nafion pump

Figure S2 summarizes the results for the proton concentration, the electric field and the streamlines of the fluid flow obtained in the simulation of the radial Nafion pump. The simulations show that the exchange of protons by Li+ ions taking place at the Nafion disc generates a concentration gradient and an electric field pointing radially towards the Nafion disc. The tangential component of this electric field acting on the positive mobile counterions accumulated on the negatively-charged deactivated Nafion surface, drags the fluid towards the Nafion disc generating convection rolls.

Figure S2. Cross-section of the radial Nafion pump showing the values of the proton concentration (color scale), electric field lines (in red) and fluid streamlines (in white) obtained in the simulation.

Figure S3 shows the value of the radial component of the electric field as a function of the radial distance, measured from the center of the Nafion disc, obtained at 2 µm above the surface. The radial component of the electric field is negative (i.e. pointing towards the disc center) and shows a strong peak at the disc edge. Simulations made using a uniform value of the zeta potential of  Nafion = -73 mV show that the tangential electric field mainly depends on the ion-exchange and not on the zeta potential gradients or the value of the zeta potential in the surrounding ring. Simulations made suppressing the ion-exchange (i.e. setting kex=0) show that the radial electric field is practically zero and accordingly there is no fluid pumping.

Figure S3. Radial component of the electric field as a function of the radial distance for a pump made of a Nafion disc surrounded by deactivated Nafion under different conditions: i) with ion exchange and using the experimental zeta potentials of Nafion and deactivated Nafion; ii) with ion exchange but using a uniform zeta potential (Nafion) on the whole surface; iii) without ion-exchange and using the experimental zeta potentials of Nafion and deactivated Nafion.

1.2 Simulations of the SiO2/Nafion/Al2O3 micropump array

Figure S4 shows the value of the tangential component of the electric field along the micropump strip array measured at 3 µm above the surface. The origin of the X coordinate has been placed at the start of the first SiO2/Nafion/Al2O3 repeating unit. The electric field is positive at the SiO2/Nafion boundary, pointing towards Nafion. The electric field is negative at the Nafion/Al2O3 border indicating that is also pointing towards Nafion and therefore promoting the fluid flow from Nafion towards the region of positive zeta potential provided by Al2O3.

Figure S4. Simulated electric field profile along the micropump array at a z height of 3 µm and for a constant ion exchange rate of 1. 10-5.

2. Pumping performance and regeneration

Figure S5. a) Radial velocity of polystyrene tracer particles as a function of salt concentration. b) Comparison between the radial velocity as a function of the radial distance obtained with a fresh Nafion pump (black line and symbols) and with a reused pump (red line and symbols) regenerated after immersion in 1.10-2M HCl for 6 hours. The velocities are very similar, verifying pump reusability.

3. Captions of the Movies

Movie 1. Motion of amidine functionalized polystyrene tracers. The movie was taken at Z distances close to the surface of the micropump system. A radial fluid flow drags the particles inwards the Nafion disc which has not so much contrast to optically visualize it in presence of the liquid.

Movie 2. Motion of amidine functionalized polystyrene tracers taken at high Z distances from the surface of the micropump system. Now the radial fluid flow lifts up the particles at the Nafion disc and drags them outwards the Nafion disc.

Movie 3. Motion of polystyrene tracer particles at the micropump array composed of alternating strips of deactivated Nafion/Nafion/Al2O3. The movie has been speeded up (x 4).

Movie 4. Motion of polystyrene tracer particles at the micropump array composed of alternating strips of deactivated SiO2/Nafion/Al2O3. The movie has been speeded up (x4).