Supplementary material to:

Activated sludge treatment for high ammonium nitrogen wastewater: performance and microbial adaptation with synthetic media

Kinga Marek1*, Katarzyna Pawęska1, Aleksandra Bawiec1

1Institute of Environmental Engineering, Wrocław University of Environmental and Life Sciences, 50-375 Wrocław, Poland, 24 Grunwaldzki Square, (kinga.marek@upwr.edu.pl*Corresponding author, katarzyna.paweska@upwr.edu.pl, aleksandra.bawiec@upwr.edu.pl)

Figure S1. Single cylinder along with its equipment.

Table S1. Synthetic wastewater composition under influent conditions used in the experiment.

| Parameter | Average value |
| --- | --- |
| pH [-] | 7.8 |
| NH4+ [mg/L] | 202.0 (I group) 453.0 (II group) |
| NO3- [mg/L] | 0.4 |
| NO2- [mg/L] | 4.43 |
| PO43- [mg/L] | 6.67 |
| TOC [ppm] | 190.0 |
| C:N [-] | 1.4 (I group) 0.45 (II group) |

Table S2. Analyzed wastewater and sludge parameters with frequency and measurement methods.

| Parameter | Methodology | Measurement frequency |
| --- | --- | --- |
| dissolved oxygen | luminescent quenching | controlled in each phase |
| pH | potentiometric measurement | controlled after each phase of the cycle |
| NH4+ | spectrophotometric analysis | beginning and end of each cycle |
| NO3- |  |  |
| NO2- |  |  |
| PO43- |  |  |
| NH4+ |  |  |
| TOC | persulfate oxidation |  |
| BOD5 | manometric measurement | initial and end of the cycle |
| total solids (TS) | gravimetric analysis | end of the experiment |
| sludge volume index (SVI | settling test |  |
| sludge/biofilm growth | weight measurement |  |

Table S3. Statistical significance of differences between cylinders with and without filling.

| Group | Data Sets | Normality Test (Shapiro-Wilk) | Statistic (W) | p-value | Comparison Test | Statistic | p-value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Group 1 | C-1 | Shapiro-Wilk | 0.85 | 0.02 | Mann-Whitney U | 112.5 | 0.03 |
|  | G1 | Shapiro-Wilk | 0.75 | <0.01 |  |  |  |
| Group 2 | C-2 | Shapiro-Wilk | 0.96 | 0.33 | t-Student test | 2.10 | 0.04 |
|  | G2 | Shapiro-Wilk | 0.94 | 0.20 |  |  |  |

Table S4. Details of relative abundance data for bacterial families (family level) in activated sludge samples providing for Figure 3.

| Type | Relative abundance [%] |  |  |  |
| --- | --- | --- | --- | --- |
|  | C-1 | G1 | C-2 | G2 |
| Rhodanobacteraceae | 1.30 | 2.80 | 15.90 | 26.10 |
| Pirellulaceae | 4.50 | 5.70 | 1.90 | 4.30 |
| Rhodobacteraceae | 2.80 | 3.80 | 3.20 | 5.70 |
| Comamonadaceae | 1.70 | 2.50 | 3.00 | 3.50 |
| Phycisphaeraceae | 4.00 | 2.50 | 1.30 | 1.80 |
| Chitinophagaceae | 1.10 | 0.70 | 2.30 | 3.50 |
| Gemmataceae | 4.50 | 1.60 | 0.60 | 1.60 |
| SC-I-84 | 3.90 | 1.20 | 2.70 | 0.60 |
| Rhodocyclaceae | 1.40 | 2.00 | 0.70 | 1.80 |
| Proteobacteria uncultured | 2.30 | 1.60 | 0.80 | 1.50 |
| S-70 | 0.60 | 3.70 | 0.30 | 0.30 |
| Rhizobiaceae | 2.40 | 1.00 | 1.50 | 1.30 |
| Xanthomonadaceae | 0.30 | 0.60 | 3.50 | 1.70 |
| Xanthobacteraceae | 0.90 | 1.20 | 0.60 | 1.90 |
| Nitrosomonadaceae | 1.00 | 1.90 | 0.40 | 1.20 |
| Acetobacteraceae | 0.70 | 0.80 | 1.30 | 1.70 |
| SAR324_clade(Marine_group_B);SAR324_clade(Marine_group_B) | 1.00 | 1.90 | 0.40 | 0.80 |
| Bryobacteraceae | 1.20 | 0.90 | 0.70 | 1.50 |
| OM190 | 1.40 | 1.40 | 1.10 | 0.70 |
| Microscilaceae | 1.60 | 1.30 | 0.90 | 0.90 |
| Caldilineaceae | 1.50 | 1.50 | 0.40 | 1.40 |
| Sphingomonadaceae | 0.40 | 0.30 | 1.50 | 2.10 |
| Gemmatimonadaceae | 1.10 | 1.60 | 0.00 | 1.10 |
| Blastocetallaceae | 0.20 | 1.20 | 1.10 | 1.20 |
| CCM11a | 0.80 | 2.40 | 0.00 | 0.20 |
| other/ remaining taxa (243) | 57.40 | 53.90 | 53.90 | 31.60 |

Table S5. Details of relative abundance data for bacterial genera (genus level) in activated sludge samples providing for Figure 4.

| Type | Relative abundance [%] |  |  |  |
| --- | --- | --- | --- | --- |
|  | C-1 | G1 | C-2 | G2 |
| Rhodanobacter | 0.00 | 0.00 | 17.80 | 25.10 |
| Proteobacteria uncultured | 6.10 | 6.20 | 2.70 | 3.50 |
| Planctomycetota uncultured | 4.60 | 7.80 | 1.20 | 3.70 |
| Chloroflexi uncultured | 2.50 | 2.60 | 0.60 | 0.80 |
| Bacteroidota uncultured | 2.60 | 1.40 | 1.30 | 1.20 |
| Pirellula | 1.30 | 2.00 | 0.90 | 1.60 |
| SC-I-84 | 3.90 | 1.20 | 0.00 | 0.60 |
| S-70 | 0.60 | 3.70 | 0.10 | 0.30 |
| Rubelimicrobium | 1.00 | 1.60 | 0.80 | 1.30 |
| SM1A02 | 2.90 | 1.10 | 0.50 | 0.90 |
| SAR324_clade(Marine_group_B);SAR324_clade(Marine_group_B) | 1.00 | 1.90 | 0.40 | 0.80 |
| Bryobacter | 1.20 | 1.20 | 0.70 | 1.60 |
| OM190 | 1.40 | 1.40 | 1.10 | 0.70 |
| Pir4_lineage | 1.40 | 1.20 | 0.80 | 0.70 |
| Gemmatimonadota uncultured | 1.10 | 1.70 | 0.00 | 0.80 |
| CCM11a | 0.80 | 2.40 | 0.00 | 0.20 |
| Thauera | 0.50 | 1.80 | 0.00 | 1.30 |
| Desulfobacterota uncultured | 0.90 | 1.00 | 0.00 | 1.10 |
| Paracoccus | 0.80 | 0.50 | 0.70 | 1.20 |
| OLB14 | 1.60 | 1.10 | 0.00 | 0.70 |
| Dokdonella | 0.20 | 0.80 | 0.80 | 0.90 |
| Thermomonas | 0.00 | 0.00 | 0.90 | 1.90 |
| Nitrosomonas | 1.00 | 1.70 | 0.00 | 0.35 |
| Nitrospira | 0.80 | 0.90 | 0.00 | 0.50 |
| DS-100 | 0.20 | 1.10 | 0.00 | 1.20 |
| other/ remaining taxa (437) | 61.60 | 53.70 | 68.70 | 48.35 |