Supplementary data S2

Component verification through HPLC

1. Materials and methods

1.1. Chemicals and reagents

HPLC-grade methanol, acetonitrile, and water were purchased from J.T. Baker Inc. (Phillipsburg, USA). Trifluoroacetic acid (TFA) was purchased from Sigma-Aldrich (St Louis, MO, USA). Benzoic acid and chlorogenic acid were purchased from ChemFace (Wuhan, Hubei, China).

1.2. Preparation of standard solution and sample solution

Benzoic acid and chlorogenic acid were accurately weighed and dissolved in the methanol at the concentrations of 1000 g/mL to make a standard solution. Sample (CeBiNES-02) was diluted with the HPLC-grade methanol and then filtered through a 0.2 m syringe filter (BioFACTTM, Korea) prior to HPLC injection.

1.3. Chromatography and mass spectrometry

Qualitative analysis was performed using Agilent 1200 series (Agilent Technologies, Palo Alto, CA, USA) and data was acquired using ChemStation (Agilent Technologies, Palo Alto, CA, USA). A Capcell Pak Mg II C18 column (4.6 mm × 250 mm, 5 μm; Shiseido, Tokyo, Japan) was used for separation of analytes in the column oven at 30 °C. The flow rate was set at 1 mL/min and the injection volume was set at 10 μL. Gradient elution system was applied with the mobile phase consisted of water containing 0.1% trifluoroacetic acid (A) and acetonitrile (B): 10–10% (B) over 0–2 min, 10–70% (B) over 10–45 min, held for 3 min, and then re–equilibrated to 10% until the end of analysis. The detection wavelengths were set at UV 230 and 310 nm.

2. Results

2.1. Identification of benzoic acid and chlorogenic acid in the sample

The peak of chlorogenic acid was detected in the sample (tR: 10.64 min) (Fig. S4A and S4B) and that of benzoic acid peak was detected in the sample (tR: 20.04 min) (Fig. S4C and S4D), which were identified using the retention time and the UV spectrum of each standard compound.

Figure S1. Chromatograms of CeBiNES-02. Chromatograms of chlorogenic acid solution (A), the sample solution (B) detected at the UV 230 nm, and chromatograms of benzoic acid solution (C) and the sample (D) detected at the UV 310 nm.

Component verification results through GC

Table S2. Predicted Components of CeBiNES

| No. | Name | Amount (μg/kg) |
| --- | --- | --- |
| 1 | Hexanal, 2-ethyl- | 1,020 |
| 2 | Ethanol, 2-(2-ethoxyethoxy)- | 112 |
| 3 | Ethanol, 2-(2-butoxyethoxy)- | 576 |
| 4 | 6-Octen-1-ol, 3,7-dimethyl- | 496 |
| 5 | Benzenepropanoic acid, ethyl ester | 64.6 |
| 6 | Propanoic acid, 2-methyl-2,2-dimethyl-1-(2-hydroxy-1-methylethyl)propyl ester | 139 |
| 7 | Propanoic acid, 2-methyl-3-hydroxy-2,4,4-trimethylpentyl ester | 173 |
| 8 | 3’,5’-Dimethoxyacetophenone | 244 |
| 9 | Phenol, 2,4-bis(1,1-dimethylethyl)- | 117 |
| 10 | Dodecanoic acid | 119 |
| 11 | Diethyl phthalate | 350 |
| 12 | Propanoic acid, 2-methyl-1-(1,1-dimethylethyl)-2-methyl-1 3-propanediyl ester | 303 |
| 13 | Hexanedionic acid, bis(-methylpropyl) ester | 102 |
| 14 | Tetradecanoic acid | 287 |
| 15 | Phthalic acid, cyclohexylmethyl butyl ester | 131 |
| 16 | 7-Hexadecene | 152 |
| 17 | 7,9-di-tert-butyl-1-oxaspiro(4,5)deca-6,9-diene-2,8-dione | 231 |
| 18 | cis-9-Hexadecenoic acid | 120 |
| 19 | Oleanitrile | 231 |
| 20 | cis-Vaccenic acid | 1,440 |
| 21 | 9-Octadecenamide | 32,700 |
| 22 | Octadecanoic acid | 148 |
| 23 | Hexadecanamide | 849 |
| 24 | Hexanediotic acid bis(2-ethylhexyl) ester | 236 |

The table above is excerpted from the summary page of the report (A-19-0572-11) published by The Korea Institute of Materials Convergence Technology (Korea Institute of Footwear and Leather Technology).

Figure S2. The components of CeBiNES This analysis was performed on August 27, 2019 by The Korea Institute of Materials Convergence Technology (A-19-0572-11).