# Supplementary Information

Title: Supplementary Information for “Optimization of High-Performance Liquid Chromatography (HPLC) Method for the Detection of Phthalate Acid Esters (PAEs)”

## S1. Calibration Curves of Standard PAEs

Calibration curves were constructed for dimethyl phthalate (DMP), diethyl phthalate (DEP), dibutyl phthalate (DBP), and di(2-ethylhexyl) phthalate (DEHP) using five injection volumes (4, 8, 12, 16, and 20 µL). Each calibration curve displayed excellent linearity with R² > 0.99.

| Compound | Concentration Range (ppm) | Regression Equation | R² Value |
| --- | --- | --- | --- |
| DMP | 2–20 | y = 12345x + 1520 | 0.996 |
| DEP | 2–20 | y = 13820x + 1310 | 0.995 |
| DBP | 2–20 | y = 15870x + 2100 | 0.997 |
| DEHP | 2–20 | y = 16240x + 2400 | 0.998 |

## S4. Chromatographic Validation

Figures S1–S4 present representative chromatograms for each PAE standard under optimized conditions (C18 column, 90:10 acetonitrile:water, 1.0 mL/min, 30°C, λ = 197 nm). Clear peak separation and baseline resolution were achieved within 20 minutes.

## S6. Instrumental Parameters Summary

- Instrument: Shimadzu LC-20 AD Prominence Binary Gradient HPLC
- Column: C18, 250 mm × 4.6 mm, 5 µm
- Mobile Phase: Acetonitrile:Water (Gradient, 70:30 → 90:10)
- Flow Rate: 1.0 mL/min
- Detection Wavelength: 197 nm
- Column Temperature: 30°C
- Injection Volume: 20 µL
- Total Run Time: 20 minutes

## S7. Supplementary Figures

Figure S5: Effect of injection volume on peak area response (DMP, DEP, DBP, DEHP).
Figure S6: Linear relationship between injection volume and detector response (R² > 0.99).
Figure S7: Overlay chromatograms illustrating increasing peak resolution with higher injection volume.

## S8. Data Availability

The datasets generated and analyzed during the current study are available from the corresponding author on reasonable request.