Supplementary material

- Supplementary Figures

Figure SM1. General geology of the area (1:200,000) (modified after  Mengesha et al.,1996)

Figure SM2.  Landslide inventory map of the study area

Figure SM3. Relative relief map

Figure SM4. Elevation map

Figure SM5. Land use land cover map

| Figure SM6. Seismic risk map of Ethiopia 100-year return period, 0.99 probability (Modified after Laike Mariam Asfaw, 1986) | Figure SM7. Relationship between the intensity of earthquake and ground acceleration (Hays, 1980) |
| --- | --- |

- Supplementary Tables

Table SM1. Distribution of maximum SSEP ratings assigned to various intrinsic and external causative parameters (Raghuvanshi et al., 2014).

| Distribution of maximum SSEP ratings assigned to various intrinsic and external parameters |  |  |
| --- | --- | --- |
| Slope Stability Susceptibility Evaluation Parameter |  | Maximum Rating |
| Intrinsic Parameters |  |  |
| Slope Geometry | Relative Relief | 1 |
|  | Slope Morphometry | 2 |
| Slope Material (rock and soil type) |  | 1 |
| Structural Discontinuities |  | 2.5 |
| Land use and landcover |  | 1.5 |
| Groundwater |  | 2 |
| External Triggering Parameters |  |  |
| Seismicity |  | 2 |
| Rainfall |  | 1.5 |
| Manmade Activities |  | 1.5 |
| Total |  | 15 |

Table SM2. Slope stability vulnerability evaluation parameter rating for Relative relief (Raghuvanshi et al. 2014).

| Relative Relief |  |  |
| --- | --- | --- |
| Class | Value Range | Ratings |
| Very High | >300m | 1 |
| High | 201-300m | 0.8 |
| Medium | 101-200m | 0.6 |
| Moderate | 51-100m | 0.2 |
| Low | <50m | 0.1 |

Table SM3. Estimation of slope angle (Anbalagan, 1992)

| a) Estimation of Slope Angle |  |  |
| --- | --- | --- |
| Number of contour lines over one cm length (1: 50,000) | Slope Angle |  |
| >25 | >450 |  |
| 19-25 | 360-450 |  |
| 13-18 | 260-350 |  |
| 8-12 | 160-250 |  |
| <7 | <150 |  |
| Slope stability susceptibility evaluation parameter rating for slope morphometry. |  |  |
| b) Slope Morphometry |  |  |
| Class | Value Range | Rating |
| Escarpment/cliff | >450 | 2 |
| Steep slope | 360-450 | 1.7 |
| Moderately steep slope | 260-350 | 1 |
| Gentle slope | 160-250 | 0.6 |
| Very gentle slope | <150 | 0.3 |

Table SM4. Slope materials – Rock classes and corresponding ratings (Hoek and Brown, 1997)

| Slope material |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| i. Rock type |  |  |  |  |  |
|  | class |  |  | Description | Rating |
| Grade I | Very weak rock |  |  | Strength: 1–5 MPa Field estimate of strength: Crumbles under firm blows with the point of a geological hammer, can be peeled by a pocket knife. Examples: Highly weathered or altered rock | 1.0 |
|  | Weak rock |  |  | Strength: 5–25 MPa Field estimate of strength: Can be peeled with a pocket knife with difficulty, shallow indentation made by a firm blow with the point of a geological hammer. Examples: Chalk, rock salt, potash | 0.8 |
| Grade II | Medium-strong rock |  |  | Strength: 25–50 MPa Field estimate of strength: Cannot be scraped or peeled with a pocket knife, the specimen can be fractured with a single blow from a geological hammer. Examples: Claystone, coal, concrete, schist | 0.5 |
|  | Strong rock |  |  | Strength: 50–100 MPa Field estimate of strength: Specimen needs more than one geological hammer blow to break. Examples: Limestone, marble, phyllite, sandstone, schist, shale | 0.4 |
| Grade III | Very strong rock |  |  | Strength: 100–250 MPa Field estimate of strength: Specimen requires several blows of a geological hammer to fracture it. Examples: Amphibolite, sandstone, basalt, gabbro, gneiss, granodiorite, limestone, marble, rhyolite, tuff | 0.3 |
|  | Extremely strong |  |  | Strength: >250 MPa Field estimate of strength: Specimen can only be chipped with a geological hammer. Examples: Fresh basalt, chert, diabase, gneiss, granite, quartzite | 0.1 |
| The adjustment factor for weathering grade for rock mass |  |  |  |  |  |
| Weathering grade |  |  | Description |  | Rating |
| Fresh |  |  | No visible sign of weathering of rock material; probably minor slight discoloration on large discontinuity surfaces |  | 1.0 |
| Slightly weathered |  |  | Discoloration shows weathering of the rock material and discontinuity surfaces |  | 1.2 |
| Moderately weathered |  |  | Less than 35% of the rock material is decomposed and/or disintegrated into the soil. Fresh or discolored rock is present either as a continuous framework as a core stone |  | 1.5 |
| Highly weathered |  |  | More than 35% of the rock material is decomposed and/or disintegrated into the soil. Fresh or discolored rock is present either as a continuous framework as a core stone |  | 1.65 |
| Extremely weathered |  |  | All the rock material is decomposed and/or disintegrated into the soil. The original mass structure is still largely intact |  | 1.8 |
| Residual soil |  |  | All the rock material is converted to soils, the mass structure and material fabrics are destroyed |  | 2.0 |
| ii, Soil type |  |  |  |  |  |
| class |  | Description |  |  | Rating |
| Collapsible soil |  | The loose mix of granular material containing primarily sand and silt. Mainly non-cohesive material that can be suffused |  |  | 1.0 |
| Alluvial deposits |  | Well graded material containing a mix of cobble/pebbles, sand, and silt in a matrix of clay |  |  | 0.8 |
| Residual expansive soils |  | Mainly containing clay. Exhibits swelling when saturated and typical shrinkage cracks when dry |  |  | 0.6 |
| Poorly graded colluvial material |  | Poorly graded rounded to sub-rounded rock fragments/ cobble/ pebbles mixed in a matrix of fine-grained sand or silt |  |  | 0.5 |
| Fluvio-glacial deposits |  | Well graded angular rock fragments mixed in a matrix of clayey material. The partial interlocking of angular rock fragments |  |  | 0.4 |
| Well graded colluvial material |  | Well graded angular to sub-angular rock fragments mixed in a matrix of clayey material. The partial or well interlocking of angular rock fragments |  |  | 0.3 |
| Residual soils |  | Well compacted residual deposits of soil mainly comprising silt and clay |  |  | 0.2 |

Table SM5. Slope Stability Susceptibility Evaluation Parameter (SSEP) rating system for structural discontinuity (Raghuvanshi et al., 2014).

| (i) The parallelism between discontinuities dip direction and slopes |  |  |  |  |  |  | ii) Relationship between dip of discontinuity and inclination of the slope |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| aj - as) or (ai - as) | Rating | Planer Mode of Failure (aj - as) Wedge Mode of Failure (ai - as) |  |  |  |  | βj - βs) or (βi - βs) |  | Rating | Planer Mode of Failure (βj - βs) Wedge Mode of Failure (βi - βs) |  |  |  |  |
| 00 | 0.5 | aj - Discontinuity dip direction |  |  |  |  | < (-100) |  | 0.5 | βj - Dip of Discontinuity plane |  |  |  |  |
| 10-50 | 0.4 | ai - Direction of line of intersection of two wedge forming discontinuities |  |  |  |  | 00 - (- 100) |  | 0.4 |  |  |  |  |  |
| 60-100 | 0.3 |  |  |  |  |  | 00 |  | 0.2 | βi - Plunge of the line of intersection of two wedge forming discontinuities |  |  |  |  |
| 110-150 | 0.25 |  |  |  |  |  | 100 - 00 |  | 0.15 |  |  |  |  |  |
| 160-200 | 0.2 | as - Direction of Slope inclination |  |  |  |  | >100 |  | 0.1 | βs - Direction of Slope inclination |  |  |  |  |
| >200 | 0.1 |  |  |  |  |  | (iii) The dip of discontinuity or plunge of the line of the intersection of wedge forming planes |  |  |  |  |  |  |  |
| (iv) Soil cover depth |  |  |  |  |  |  | (βi) or (βs) |  |  |  |  |  | Rating |  |
| Depth |  |  |  | Rating |  |  | 450 |  |  |  |  |  | 0.5 |  |
| >20m |  |  |  | 2.5 |  |  | 450-350 |  |  |  |  |  | 0.4 |  |
| 20-15m |  |  |  | 1.8 |  |  | 340-300 |  |  |  |  |  | 0.25 |  |
| 14-10m |  |  |  | 1.4 |  |  | 290-200 |  |  |  |  |  | 0.15 |  |
| 9-5m |  |  |  | 1 |  |  | <200 |  |  |  |  |  | 0.1 |  |
| <5m |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |
| (v) Structural discontinuities and rock mass condition*** |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Condition |  |  |  |  |  |  |  |  |  |  |  | Rating |  |  |
| Disintegrated – poorly interlocked heavily broken rock mass with a mixture of angular and rounded rock pieces. |  |  |  |  |  |  |  |  |  |  |  | 0.25 |  |  |
| Blocky/ Disturbed – folded and/or faulted with angular blocks formed by many intersecting discontinuity sets. |  |  |  |  |  |  |  |  |  |  |  | 0.2 |  |  |
| Very Blocky – interlocked, partially disturbed rock mass with multifaceted angular blocks formed by four or more discontinuity sets. |  |  |  |  |  |  |  |  |  |  |  | 0.1 |  |  |
| Blocky – very well interlocked undisturbed rock mass consisting of cubical blocks formed by three orthogonal discontinuity sets. |  |  |  |  |  |  |  |  |  |  |  | 0.05 |  |  |
| (vi) Characteristics of structural discontinuity |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Continuity |  |  | Rating |  | Separation | Rating |  | Roughness |  | Rating | Infilling |  |  | Rating |
| <1m |  |  | 0.02 |  | None | 0.02 |  | Very rough |  | 0.02 | None |  |  | 0.02 |
| 1-2m |  |  | 0.04 |  | < 0.1mm | 0.05 |  | Rough |  | 0.03 | Hard filling < 5mm |  |  | 0.04 |
| 3-9m |  |  | 0.07 |  | 0.1 - 0.9mm | 0.07 |  | Slightly rough |  | 0.07 | Hard filling > 5mm |  |  | 0.07 |
| 10-20m |  |  | 0.1 |  | 1 - 5mm | 0.12 |  | Smooth |  | 0.1 | Soft filling < 5mm |  |  | 0.12 |
| >20m |  |  | 0.15 |  | > 5mm | 0.15 |  | Slickensided |  | 0.15 | Soft filling > 5mm |  |  | 0.15 |
| Weathering |  |  | Rating |  | (Hoek and Bray 1997) |  |  |  |  |  |  |  |  |  |
| Unweathered |  |  | 0.02 |  |  |  |  |  |  |  |  |  |  |  |
| Slightly weathered |  |  | 0.05 |  |  |  |  |  |  |  |  |  |  |  |
| Moderately weathered |  |  | 0.07 |  |  |  |  |  |  |  |  |  |  |  |
| Highly weathered |  |  | 0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Decomposed |  |  | 0.15 |  |  |  |  |  |  |  |  |  |  |  |

Table SM6. Slope Stability Susceptibility Evaluation Parameter (SSEP) rating system for land use and land cover (Raghuvanshi et al., 2014).

| (e) Land-use and Land-cover |  |
| --- | --- |
| Class / Description | Rating |
| Barren land - No vegetation, land not used for any activity | 1.5 |
| Sparsely vegetated - Very thin scattered vegetation in the form of wild grass, bushes, scrub, and random | 1.2 |
| Moderately vegetated area - Moderately covered vegetation land in the form of wild grass, scrub, and trees | 0.75 |
| Thickly vegetated forest area - Dense forest area with thick vegetation cover. | 0.4 |
| Cultivated land with a populated area | 0.4 |

Table SM7. Slope Stability Susceptibility Evaluation Parameter (SSEP) rating system for groundwater (Raghuvanshi et al., 2014).

| (f) Ground Water |  |
| --- | --- |
| Surface traces of groundwater | Ratings |
| Flowing – the presence of spring on the slope face | 2 |
| Dripping – dripping of water through structural discontinuities. | 1.5 |
| Wet – watermarks on the rock surface, some droplets along structural discontinuities, algal growth in shadow areas. | 1 |
| Damp – Moist conditions on rock face surface, moss, and algal growth in shadow areas. | 0.6 |
| Dry – rock face surface is dry, no water traces along structural discontinuity surfaces | 0 |

Table SM8. Slope Stability Susceptibility Evaluation Parameter (SSEP) rating system for eternal Parameters (Raghuvanshi et al., 2014).

| (a) Rain Fall |  |  |  | (b) Seismicity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (i) Mean annual rainfall (mm) |  |  |  | Ground acceleration |  | Rating |
| Class | Mean annual rainfall (mm) |  | Rating | 1 - 0.5g |  | 2.0 |
| Very High | > 1500 |  | 0.75 | 0.5 - 0.1g |  | 1.5 |
| High | 1101 - 1500 |  | 0.60 | 0.1 - 0.05g |  | 1.0 |
| Moderate | 701 -1100 |  | 0.30 | 0.05 - 0.01g |  | 0.8 |
| Low | 300 - 700 |  | 0.20 | 0.01 - 0.005g |  | 0.4 |
| Very low | < 300 |  | 0.10 | < 0.005g |  | 0.4 |
| (ii) Rainfall induced manifestations on the slope |  |  |  |  |  |  |
| Surface Traces |  |  |  |  |  | Rating |
| Slope Toe erosion – The toe of the slope is eroded by streamflow, Surface material overhangs, and prone to failure. |  |  |  |  |  | 0.25 |
| Streambank erosion – Stream water has undercut the banks and made the sides of the slope hang. |  |  |  |  |  | 0.15 |
| Gully erosion over the slope face |  |  |  |  |  | 0.10 |
| No Rain induced manifestation on the slope |  |  |  |  |  | 0.00 |
| (iii) Slope Material |  |  |  |  |  |  |
| Description |  |  |  |  |  | Rating |
| Soil mass – Rainfall will saturate soil mass and pore water pressures will develop. |  |  |  |  |  | 0.25 |
| Disintegrated – heavily broken rock mass. Much of the rainwater will seep into the rock mass and will recharge groundwater considerably. |  |  |  |  |  | 0.2 |
| Blocky/ Disturbed – folded and/or faulted with angular blocks formed by many intersecting discontinuity sets. |  |  |  |  |  | 0.15 |
| Very Blocky – Partially disturbed rock mass having four or more discontinuity sets. |  |  |  |  |  | 0.1 |
| Blocky – Undisturbed rock mass consisting of cubical blocks formed by three orthogonal discontinuity sets. |  |  |  |  |  | 0.05 |
| (iv) Discontinuity Orientation with respect to slope |  |  |  |  |  |  |
| Soil Mass |  |  |  |  |  | 0.25 |
| More than one discontinuity dipping into the hill or the bedding joint/discontinuity dipping into the hill. |  |  |  |  |  | 0.25 |
| At least one discontinuity dipping into the hill. Which may or may not be a bedding joint. |  |  |  |  |  | 0.12 |
| None of the discontinuities dips into the hill. |  |  |  |  |  | 0.00 |
| The rainfall adjustment factor for slope morphometry # |  |  |  |  |  |  |
| Class |  | Value Range |  |  | Adjustment factor |  |
| Escarpment/cliff |  | > 450 |  |  | 0.1 |  |
| Steep slope |  | 350 - 450 |  |  | 0.3 |  |
| Moderately steep slope |  | 250 - 350 |  |  | 0.5 |  |
| Gentle slope |  | 150 - 250 |  |  | 0.8 |  |
| Very gentle slope |  | < 150 |  |  | 1.0 |  |
| # Rainfall adjustment factor for slope morphometry has to be multiplied with the Rainfall rating after summing up (i, ii, iii, and iv). |  |  |  |  |  |  |

Table SM9. Manmade activities and related ratings (Raghuvanshi et al., 2014).

| Description | Rating |
| --- | --- |
| (a) Developmental activities |  |
| Steep soil mass cut (slope > 45_). Slope overhangs | 1.25 |
| Moderate steep soil mass cut (slope 35–45_) | 1.0 |
| Soil mass cut into the gentle slope (slope < 35_) | 0.75 |
| Steep rock mass cut (slope > 45_). Slope overhangs | 1.0 |
| Moderate steep rock mass cut (slope 35–45_) | 0.75 |
| Rock mass cut into the gentle slope (slope < 35_) | 0.5 |
| Description | Adjustment Factor a |
| The adjustment factor for developmental activities a |  |
| Slope toe supported by retaining structure | 0.15 |
| Proper drainage system provided | 0.1 |
| Slope face dressing into terrace design | 0.1 |
| Excavated material not dumped over the downslope | 0.1 |
| Description | Rating |
| (b) Cultivation activities |  |
| Sparsely cultivated land with a small populated area | 0.1 |
| Moderately cultivated land with a medium populated area | 0.15 |
| Densely cultivated land with a largely populated area | 0.25 |
| Irrigation practice | rating |
| The adjustment factor for irrigation b |  |
| Rainfed irrigation | 0.75 |
| Unplanned irrigation | 0.5 |
| Channelized planned | 1.0 |
| a. The adjustment factor for developmental activities has to be subtracted from the rating of development activity. b. Multiply adjustment factor for irrigation with ratings of cultivated land. |  |

Table SM10. Seismicity and corresponding ratings (Raghuvanshi et al., 2014).

| Ground acceleration a | Rating |
| --- | --- |
| 1.0–0.5g | 2.0 |
| 0.5–0.1g | 1.5 |
| 0.1–0.05g | 1.0 |
| 0.05–0.01g | 0.8 |
| 0.01–0.005g | 0.4 |
| <0.005g | 0.2 |
| - For ground acceleration estimation refer to Hays's (1980) relation and seismic intensity map of the area. |  |