Sustainable Production and Consumption

Sustainable carotenoid production using amylaceous agro-industrial byproducts: Process efficiency and environmental assessment

Thércia Rocha Balbino*a, Salvador Sánchez-Muñoza, Stephanie Custódio Inácioa, Gabriele Campelo Almeidaa, Ana Cláudia Diasb, Júlio César Santosc, Jorge Fernando Brandão Pereirad, Silvio Silvério da Silvaa

Laboratory of Bioprocesses and Sustainable Products. Department of Biotechnology, Engineering School of Lorena, University of São Paulo (EEL-USP), 12.602.810. Lorena, SP, Brazil

- E-mail: thr.balbino@gmail.com

Centre for Environmental and Marine Studies (CESAM), Department of Environment and Planning, University of Aveiro, Campus Universitário de Santiago, 3810-193 Aveiro, Portugal

Laboratory of Biopolymers, Bioreactors, and Process Simulation. Department of Biotechnology, Engineering School of Lorena, University of São Paulo (EEL-USP), 12.602.810. Lorena, SP, Brazil

University of Coimbra, CERES, Department of Chemical Engineering, 3030-790 Coimbra, Portugal

The calculation formulas of total carotenoids concentration

The specific concentration of total carotenoids (mgcarotenoids/gcells) and total carotenoids per growth medium volume (mgcarotenoids/Lgrowth medium) were calculated using the equations (S1) and (S2).

The specific concentration of total carotenoids (mgcarotenoids/gcells)	          	                (S1)

= mgcarotenoidsLsolvent × solvent volume (L)gextracted cells

Total carotenoids per growth medium volume (mgcarotenoids/Lgrowth medium)   	     (S2)

=mgcarotenoidsgcells× final cells concentration gcellsLgrowth medium

Figure S1. Variation in cellular biomass (represented as the natural logarithm of Optical Density measured at 600nm – ln OD600nm) during 72h cultivation of Rhodotorula mucilaginosa in standardized hydrolysates: corn bran hydrolysate (CBH), soybean bran hydrolysate (SBH), rice bran hydrolysate (RBH), wheat bran hydrolysate (WBH).

Table S1. Global inventory of data by functional unit (1mg of carotenoids).

|  | Unit | CBH | SBH | RBH | WBH |
| --- | --- | --- | --- | --- | --- |
| Input |  |  |  |  |  |
| Dilute-acid Hydrolysis |  |  |  |  |  |
| Bran | g | 2.640 | 7.870 | 4.208 | 3.288 |
| H2SO4 | mL | 0.325 | 0.969 | 0.518 | 0.405 |
| Water | mL | 15.606 | 46.521 | 24.877 | 19.437 |
| Electricity | Kwh | 0.001 | 0.002 | 0.001 | 0.001 |
| Filtration |  |  |  |  |  |
| Electricity | Kwh | 0.002 | 0.005 | 0.003 | 0.002 |
| Preparation of standardized hydrolysates |  |  |  |  |  |
| Water | mL | 81. 611 | 0.000 | 13.061 | 32.313 |
| Cell activation |  |  |  |  |  |
| Glucose | g | 2.929 | 1.252 | 1.071 | 1.516 |
| Peptone | g | 0.488 | 0.209 | 0.178 | 0.253 |
| Yeast Extract | g | 0.293 | 0.125 | 0.107 | 0.152 |
| Malt Extract | g | 0.293 | 0.125 | 0.107 | 0.152 |
| Electricity | Kwh | 0.005 | 0.002 | 0.002 | 0.002 |
| Cell concentration (centrifugation) |  |  |  |  |  |
| Electricity | Kwh | 0.000 | 0.000 | 0.000 | 0.000 |
| Peptone | g | 0.001 | 0.000 | 0.000 | 0.000 |
| Water | mL | 0.781 | 0.334 | 0.286 | 0.404 |
| pH adjustment of standardized hydrolysate |  |  |  |  |  |
| NaOH | g | 0.166 | 0.495 | 0.265 | 0.207 |
| Fermentation |  |  |  |  |  |
| Electricity | Kwh | 0.202 | 0.086 | 0.074 | 0.104 |
| Output |  |  |  |  |  |
| Filtration |  |  |  |  |  |
| Hydrolysate losses | mL | 2.276 | 6.784 | 3.628 | 2.834 |
| Fermentation |  |  |  |  |  |
| Carotenoids | mg | 1.000 | 1.000 | 1.000 | 1.000 |