## Appendix A. Seismic Interferometry

The seismic interferometry method (Claerbout, 1968) was used to calculate the shear wave velocity of the building. The technique comes with an assumption that the building behaves as a pure-shear beam (Snieder & Şafak, 2006), and it is widely used to determine the velocity profile (Guéguen et al., 2019; Picozzi et al., 2011; Rahmani & Todorovska, 2021; Skłodowska et al., 2021, 2023). In our case, we use the station located on the top floor as a reference station to apply the seismic interferometry to create a wave propagation model for the building. In this method, it is assumed that the time delay of the impulse response of the building can provide the velocity between given stations, in our case, the entire vertical profile of the building. The deconvolution is applied by following the equation below:

ht=FT-1Y(f)max⁡(Xf,wl×max(|Xf|)×X(f)|Xf|							(A1)

In which wl defines the water level that is used for regularisation (Clayton & Wiggins, 1976), Y(f), and X(f) are the Fourier transform of the bottom and top floors, respectively, and FT-1 is the inverse Fourier transform operation. Up- and down-going waves refer to casual and acasual parts of the deconvolved signals. The time delay (Δt) is estimated by picking the maxima to determine the shear wave velocity (β) of the building with the following equation:

β=HΔt												(A2)

where H represents the distance between the two sensors. To estimate the velocity of the building, deconvolved signals are stacked to decrease the inter-event variability of the frequency content.