Supplementary Methods

Three-Dimensional Modeling and Fabrication

The fabrication process of the External Ventricular Drainage (EVD) training model begins with the development of a three-dimensional (3D) virtual model, comprising the external morphology of the brain parenchyma and the internal ventricular system. The model is exported as stereolithography (STL) files and processed using a slicing program (FlashPrint, FlashForge). Each component is printed separately using a fused deposition modeling (FDM) 3D printer (Guider2) with acrylonitrile butadiene styrene (ABS) material. Following the printing process, residual support structures are removed, and the surface of each component is polished via vapor treatment to ensure smoothness and accuracy.

Silicone Molding and Structural Assembly

In the subsequent stage, a double layer of silicone is applied to the outer surfaces of both the brain parenchyma mold and the ventricular model. The internal ventricular structure is carefully positioned within the parenchyma mold, and transparent silicone is injected to replicate the mechanical properties of cerebral tissue. Upon complete curing of the silicone, the external mold is removed, resulting in a transparent brain model with the embedded ventricular structure preserved within.

Cavity Formation and Model Finalization

The internal ABS ventricular model is chemically dissolved using an acetone-based solution, thereby creating a hollow cavity within the silicone brain model. This cavity is designed to be filled with colored fluid during training procedures, simulating cerebrospinal fluid dynamics. The completed brain model is integrated with a gypsum-based skull structure and includes designated inlets and outlets for fluid injection and drainage. Finally, the model is mounted on a surgical positioning device, enabling adjustable angulation for realistic simulation of neurosurgical interventions.

Supplementary material: Questionnaires

Below are the questionnaires used for the EVD simulation. Korean versions were used in the actual simulation.

Survey for efficacy of 3D-printed brain model for the simulation of external ventricular drainage.

For residents

- Please indicate which year of residency training you are in:

  - Resident: _______ year

- Have you ever participated in education course using 3D printed models?

  - Yes

  - No

- Have you ever independently performed EVD before participating in education course using 3D printed brain model?

  - Yes

  - No

- Did the education using a 3D printed brain model help you in learning the skull positioning required for EVD?

| Strongly disagree | Disagree | Neutral | Agree | Strongly agree |
| --- | --- | --- | --- | --- |

- Did the education program help you in learning important cranial burr hole locations in current procedure?

| Strongly disagree | Disagree | Neutral | Agree | Strongly agree |
| --- | --- | --- | --- | --- |

- Did the education using a 3D printed brain model help you in setting up the trajectory for EVD?

| Strongly disagree | Disagree | Neutral | Agree | Strongly agree |
| --- | --- | --- | --- | --- |

- Do you think that repetitive training using 3D printed brain models can improved your proficiency in performing EVD?

| Strongly disagree | Disagree | Neutral | Agree | Strongly agree |
| --- | --- | --- | --- | --- |

- Do you think that education using a 3D printed brain model is helpful for patient safety when performing the actual EVD procedure?

| Strongly disagree | Disagree | Neutral | Agree | Strongly agree |
| --- | --- | --- | --- | --- |

- Please indicate your level of satisfaction with the current education using a 3D printed brain model.

| Very unsatisfied | Unsatisfied | Neutral | Satisfied | Very satisfied |
| --- | --- | --- | --- | --- |

- Please indicate the number of simulations appropriate for the education program using a 3D printed brain model.

| 1 time | 2 times | 3 times | 4 times | 5 times |
| --- | --- | --- | --- | --- |

- Please freely let us know any improvements or suggestions for the current 3D printing brain model.

Survey for efficacy of 3D-printed brain model for the simulation of external ventricular drainage.

Faculty

1. How similar is the skull of the 3D printed brain model to the actual human skull?

| Very different | different | Neutral | Similar | Very similar |
| --- | --- | --- | --- | --- |

2.  How similar is the ventricular structure of the 3D printed brain model to the actual human brain?

| Very different | different | Neutral | Similar | Very similar |
| --- | --- | --- | --- | --- |

3. How similar is the drilling of a 3D printed brain model compared to the actual human skull?

| Very different | different | Neutral | Similar | Very similar |
| --- | --- | --- | --- | --- |

4. How similar is puncturing the ventricle of a 3d printed brain model compared to the actual human brain?

| Very different | different | Neutral | Similar | Very similar |
| --- | --- | --- | --- | --- |

5. Do you think that EVD simulation using a 3D printed brain model is helpful for resident education?

| Strongly disagree | Disagree | Neutral | Agree | Strongly agree |
| --- | --- | --- | --- | --- |

6. Do you think that repetitive simulations using a 3D printed brain model are helpful in teaching skills required for resident education?

| Strongly disagree | Disagree | Neutral | Agree | Strongly agree |
| --- | --- | --- | --- | --- |

7. Please freely let us know any improvements or suggestions for the current 3D printing brain model.

Assessment sheet for efficacy of 3D-printed brain model for the simulation of external ventricular drainage.

Faculty

Scenario

a) Kocher’s point

b) Frazier’s point

| Location of burr hole | Marking |
| --- | --- |
| < 5mm from correct location |  |
| Between 5 and 10mm from correct location |  |
| > 10mm from correct location |  |

| Time taken for ventricular puncture (seconds) |  |
| --- | --- |

| Number of attempts for ventricular puncture |  |
| --- | --- |

Kakarla grading system for accuracy of EVD placement

| Location of EVD catheter tip | Accuracy of placement | Grade |
| --- | --- | --- |
| Ipsilateral frontal horn including tip of third ventricle | Optimal / adequate | 1 |
| Contralateral frontal horn of lateral ventricle / corpus callosum / interhemispheric fissure | Suboptimal (shallow in noneloquent tissue) | 2 |
| Brainstem / cerebellum / internal capsule / basal ganglia / thalamus / occipital cortex / basal cisterns | Suboptimal in eloquent tissue | 3 |

| Overall proficiency (1st simulation) | Marking |
| --- | --- |
| Advanced |  |
| Intermediate |  |
| Pre-intermediate |  |
| Beginner |  |

| Overall proficiency (2nd simulation) | Marking |
| --- | --- |
| Advanced |  |
| Intermediate |  |
| Pre-intermediate |  |
| Beginner |  |