SUPPORTING INFORMATION FOR:

Savvidou, G., Johnsson, F., Ljunggren, M., Liu, Q., Tasseven, U., & Zachariadis, T. Technological advances can halve the demand for minor metals in future wind and solar photovoltaic expansion. Journal of Industrial Ecology.

This supporting information provides details that supplement the introduction, methods, and results chapters of the main manuscript. For the introduction chapter, it offers background information. For the methods chapter, it provides detailed information on data inputs, assumptions, and data sources. For the results chapter, it presents results in a more disaggregated manner.

Table of Contents

1. Historical and future installed capacity	2

2. Characteristics of minor metals	3

3. Wind and solar PV technology layers	4

4. Examples of commercial or near-commercial CE applications	4

5. Market shares	5

6. Lifespan	6

7. Metal intensities	7

8. Substitution	13

9. Results for disaggregated wind power demands	15

10. Sensitivity analysis	17

References	19

# 1. Historical and future installed capacity

Cumulative stock data for grid-connected solar PV were obtained from the Swedish Energy Agency (Lindahl et al., 2020) [see Figure S1.2]. For wind power, annual inflow (power capacity) data from the Swedish wind turbine registry (Lansstyrelsen, 2022) were used. Savvidou and Johnsson (2023) conducted an inflow-driven MFA to estimate cumulative stock wind capacity used in this study (Figure S1.1). Future MFA stocks are based on solar PV, onshore, and offshore wind capacities from the RE pathway developed by SvK (Svenska Kraftnät, 2021) [see Figure 2 in main manuscript], which projects electricity demand to rise from ~140 TWh today to nearly 300 TWh by 2050. By then, electricity generation is dominated by these technologies.

Figure S1.1: Cumulative installed capacities of onshore and offshore wind power between 1996 and 2021. Source: (Lansstyrelsen, 2022; Savvidou & Johnsson, 2023)

Figure S1.2: Cumulative installed capacities of solar PV off-grid and grid-connected (distributed and centralized) between 1996 and 2021. Source: (Lindahl et al., 2020)

# 2. Characteristics of minor metals

Table S1.1: Key demand and supply characteristics for minor metals used in wind and solar PV technologies.

| Minor metal | Sub-technology | Share of sub-technology production in relation to world production | Primary metal with which the by-product metal is recovered | Share of largest supplier in 2022 | EoL recycling rate of byproduct metala | Substitution potential of byproduct metalb | EU import reliance (2016-2020)c | Top EU supplier for the byproduct metal (2016-2020)c | EU's share of world byproduct metal productionc,d | Byproduct (B) or primary metal (P) EU reservesc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nd | Wind turbines with REE-containing PM | 75%-80% of PM global production (China) (Nassar et al., 2016) | Iron, other REE, tin (cassiterite), titanium (ilmenite) | 71%d (China) | ~0% | Medium | 100% | China (80%) | 0% | 0.5% (B) |
| Pr |  |  |  | 71%d (China) | ~0% | Medium | 100% | China (80%) | 0% |  |
| Dy |  |  |  | 68%d (China) | ~0% | Medium | 100% | Primarily China | 0% |  |
| Tb |  |  |  | 79%d (China) | ~0% | Medium | 100% | China (64%) | 0% |  |
| Ag | c-Si | 90% of the semiconductor wafers used to produce solar cells (China) (Bordoff & O’Sullivan Meghan, 2022) | Lead-zinc, copper, gold | 24% (Mexico) | ~0% | Medium | 5% | Poland (62%) | 8% | 16% (B) |
| Cd | CdTe |  | Zinc | 42% (China) | 77% | Low | 8% | The Netherlands (24%) | 8% | 0.03 (B)e |
| Te |  |  | Copper | 53% (China) | 77% | Low | 0% | Canada (27%) | 12% | 3.1% (P) |
| Ge | a-Si |  | Zinc, coal fly ash | 74% (China) | ~0% | Low | 42% | China (45%) | 5% | No significant reserves |
| In | CIGS |  | Zinc | 59% (China) | ~0% | Low | 11% | France (38%) | 7% | No significant reserves |
| Ga |  |  | Bauxite, zinc | 98% (China) | ~0% | Low | 98% | China (69%) | 0% | No significant reserves |
| Se |  |  | Copper | 41% (China) | ~0% | Low | 2% | Denmark (34%) | 20% | 4% (B) |

a End-of-life recycling rate (the proportion of a metal in waste flows that is recycled from solar PV and wind technologies). It represents the recyclability of metals within each technology. For instance, although silver’s total EoL-RR recycling rate amounts to 35% (Graedel et al., 2022), there is virtually no recycling happens of silver from PV panels. Source: (Dominish et al., 2019). EU specific sources: Carrara et al. (2020), Ciacci et al. (2019), Gregoir and van Acker (2022).

b Based on authors own assessment. For more details, see Supplementary Materials section 7.

c Source: Raw Materials Information System, European Commission, https://rmis.jrc.ec.europa.eu/rmp/

d Of primary or refined metal.

e Estimated based on mine production levels from mines of rare earth oxides (USGS, 2023) and the average REE content in different regions (Wang et al., 2024).

fDerived based on: 1) Primary metal (zinc) EU reserve: 10.8%; and 2) average cadmium content of typical zinc ores is about 0.03%. Source: (USGS, 2023)

# 3. Wind and solar PV technology layers

Table S1.2: Technology, sub-technology, component, and material layers of wind and solar PV technologies used for the minor metals included in the present analysis.

| Technology layer | Sub-technology layer | Component layer | Material layer |
| --- | --- | --- | --- |
| Wind (onshore & offshore) | Direct-drive permanent magnet synchronous generators (DD-PMSG) Gearbox permanent magnet synchronous generators (GB-PMSG) | REE permanent magnets (PM) | PM: Neodymium (Nd), Praseodymium (Pr), Dysprosium (Dy), Terbium (Tb) |
| Solar PV (grid-connected [distributed & centralized]) | Crystalline silicon (c-Si) Amorphous silicon (a-Si) Cadmium telluride (CdTe) Copper indium gallium diselenide (CIGS) | - | c-Si: Silver (Ag) a-Si: Germanium (Ge) CdTe: Cadmium (Cd), Tellurium (Te) CIGS: Indium (In), Gallium (Ga), Selenium (Se) |

# 4. Examples of commercial or near-commercial CE applications

Table S1.3: Technology design-related examples of commercial or near-commercial CE strategies in industry.

| CE strategy | Wind power | Solar power |
| --- | --- | --- |
| Longer design lifespan | Vestas latest offshore V236-15MW wind plant has a design lifetime of 30 years and depending on the conditions of operation may exceed 35 years (Allekotte & Garrett, 2024). | Maxeon provides a 40-year solar panel warranty in their solar panels (Fischer, 2022; Maxeon, 2023). |
| Material intensity reduction | Toyota has developed a new motor magnet that cuts neodymium use by 50%. The design also eliminates the need for other rare earth elements such as terbium and dysprosium (Tangemann, 2020). | The metal intensity of silver in c-Si solar cells has experienced a 40%–50% reduction over the past decade (IEA, 2022). |
| Substitution | GreenSpur Renewables has pioneered the development of the world's first permanent magnet synchronous generator (PMSG), which utilizes ferrite instead of REE (Snieckus, 2019). Tesla announced its upcoming generation of permanent magnet synchronous reluctance motors (PMSRM) for traction applications, specifying that these would not incorporate rare earth PM (Adamas Intelligence, 2023). The EcoSwing H2020 project achieved a significant milestone in wind turbine generator technology by designing, constructing, and testing the world's first full-size superconducting generator for a wind turbine. | Silver-free crystalline silicon (c-Si) panels are approaching the stage of commercial availability (Fraunhofer, 2022). Oxford PV re developing perovskite-silicon tandem cells that stack a perovskite layer on top of a silicon cell. As of 2024, they are scaling up production at their facility in Brandenburg, Germany, with commercial modules expected soon (Jowett, 2024). The start-up Microquanta Semiconductor is advancing efforts to bring all-perovskite modules to market for specialty applications (Shaw, 2025). |

# 5. Market shares

With regards to solar PV power, the 1st generation sub-technology c-Si currently has over 95% of the market share on the global level (Carrara et al., 2020). It has historically been the dominant sub-technology following its emergence at the beginning of the century, mainly due to its higher efficiencies compared to thin films (Jean et al., 2015).

In our analysis, the current market shares (95.4% for c-Si, 0.3% for a-Si, 2.4% for CdTe, and 1.9% for CIGS), as presented by Carrara et al. (2020), are applied for 2021 and all historical years in both distributed and centralized grid-connected applications. For the scenario period, the Medium Demand Scenario (MDS) provided by Carrara et al. (2020) is applied, according to which, c-Si decreases slightly to 90% market share by 2050, and thin-film technologies make up the remaining 10%, while maintaining their current sub-technology composition. These values are linearly interpolated between 2021 and 2050 in both distributed and centralized grid-connected applications.

For wind power, the prevalent drivetrain technologies that contain rare earth elements (REE) in permanent magnets (PM) are concerned: DD-PMSG and GB-PMSG. Despite the high costs and high metal intensities of PM, they are often favored over alternatives due to their higher levels of efficiency and performance (Rabe et al., 2017). Direct-drive systems that employ PM generators lack the prone-to-failure gearbox component, resulting in a lighter and more-compact turbine design (Carrara et al., 2020). In comparison to the gearbox system, direct-drive PM generators are associated with higher levels of reliability and lower maintenance needs (Nassar et al., 2016), making them particularly attractive for offshore applications and larger plants (Giurco et al., 2019; Pavel et al., 2017).

Currently, the share of DD-PMSG and GB-PMSG for onshore wind turbines in the EU is at about 30% (Carrara et al., 2020), while the offshore sector is dominated by PM drivetrain sub-technologies. Similar to the case of solar PV, the Medium Demand Scenario (MDS) provided by Carrara et al. (2020) is applied to the market shares of PM drivetrains throughout the scenario period (Figure S1.3). According to this, the PM market share in the onshore sector increases from 30% in 2018 to 59% in 2050 (Figure S1.3a), while for the offshore sector, there is a decrease to 68% in 2050 (Figure S1.3b). In both the onshore and offshore sectors, the largest changes occur in this decade, whereas after 2030 changes in market shares occur more slowly.

Figure S1.3: Market shares for the DD-PMSG and GB-PMSG sub-technologies (PM-containing drivetrains) over time for the period of 2018–2050 for a. onshore wind turbines and b. offshore wind turbines.

Note: The category of ‘Other’ technologies includes Direct Drive Electrically Excited Synchronous Generator (DD-EESG), Gearbox Double-Fed Induction Generator (GB-DFIG), Gearbox Squirrel Cage Induction Generator (GB-SCIG), and Direct Drive High-Temperature Superconductors (DD-HTS).

# 6. Lifespan

The solar power industry is still relatively young and relatively few solar panels have been decommissioned to date. Therefore, it is not easy to formulate with confidence assumptions regarding the relevant technology lifespans with a good degree of confidence. The wind turbine industry, especially the offshore wind industry, is also in its infancy, with a limited number of decommissioned wind turbines. The vast majority of the global offshore wind capacity is only up to a decade old (IEA, 2018), which means that the experience of the lifespans of offshore turbines is very limited. However, there is evidence that the offshore wind power is designed with longer lifespans than onshore wind power (Liang et al., 2022). Both the solar and wind power industries are also very dynamic, with great advances in the designs of their technologies over the past decade.

Based on a recent systematic review (Liang et al., 2022), the lifespan of onshore wind used in the literature is in the range of 20–25 years, with a mean of 21.7 years. For offshore wind turbines, the lifespan range is 20–30 years, with a mean of 23.6 years. The lifespan of solar PV used in the literature ranges is in the range of 20–40 years, with a mean of 26.3 years for the sub-technologies in focus.

For the historical period and the future period of the Baseline scenario, these mean lifespan values are used and retained throughout the scenario period. For the Longer design lifespan scenario, which assumes a lower replacement rate, the lifespans are assumed to reach 30 and 35 years for onshore and offshore wind turbines, respectively, by 2050, in agreement with JRC's Low Demand Scenario (Carrara et al., 2020). For solar PV, the mean lifespan increases to 40 years in 2050, in agreement with the max lifespan recorded by Liang et al. (2022). In all cases, the mean lifespans increase linearly between 2021 and 2050.

All the technologies are assumed to follow a 2-parameter Weibull distribution with standard deviation of 5 years. Figure S1.4 depicts the expected lifespan distribution used for the Baseline scenario throughout the scenario period and the 2050 lifespan distribution used for the Longer design lifespan scenario.

Figure S1.4: Expected lifespan Weibull distributions for all technologies in the Baseline (throughout the scenario period) and Longer design lifespan (in 2050) scenarios.

# 7. Metal intensities

Solar PV

Liang et al. (2022) have conducted a review of material intensities in studies in the literature. Values for the material intensity (MI) per material were collected from the literature for the current case, and for 2030 and 2050. Material efficiency improvements occur over time. For example, the intensity of silver in c-Si solar cells has experienced a 40%–50% reduction over the past decade, which is one of the reasons cited for the exponential increase in solar PV deployment (IEA, 2022). Therefore, for the current case, we filtered the data provided by Liang et al. (2022) to include the material intensity data from the most-recent publications (published between 2018 and 2020). The studies in the literature also provide estimates of the MI values in future years. We collected these for 2030 and 2050. For each material, the average values were used. In Tables S1.4–7, we provide the data for the current case, and Tables S1.8–11 lists the data for 2030 and 2050 cases, including the primary sources. Figure S1.5 provides a visualization of the MI in the three cases, as well as the average values used per case. In the Baseline scenario, the current MI values were kept constant throughout the scenario period, while for the MI reduction scenario the values were linearly interpolated between the current (2021), 2030, and 2050 values.

Table S1.4: Silver intensities (kg/MW) in the c-Si  sub-technology, as reported in the literature published between 2018 and 2020.

| Sub-technology | Material | Intensity (kg/MW) | Source |
| --- | --- | --- | --- |
| c-Si | Silver Ag | 21.9 | (W. Li & Adachi, 2019) |
|  |  | 54.8 | (W. Li & Adachi, 2019) |
|  |  | 10.3 | (W. Li & Adachi, 2019) |
|  |  | 54.8 | (W. Li & Adachi, 2019) |
|  |  | 4.1 | (W. Li & Adachi, 2019) |
|  |  | 54.8 | (W. Li & Adachi, 2019) |
|  |  | 80.0 | (Elshkaki & Shen, 2019) |
|  |  | 36.0 | (Watari et al., 2019) |
|  |  | 30.0 | (Watari et al., 2019) |
|  |  | 20.0 | (Carrara et al., 2020) |

Table S1.5: Germanium intensities (kg/MW) in the a-Si  sub-technology, as reported in the literature published between 2018 and 2020.

| Sub-technology | Material | Intensity (kg/MW) | Source |
| --- | --- | --- | --- |
| a-Si | Germanium Ge | 14.8 | (Valero et al., 2018) |
|  |  | 73.0 | (Elshkaki & Shen, 2019) |
|  |  | 48.0 | (Carrara et al., 2020) |

Table S1.6: Cadmium and tellurium intensities (kg/MW) in the CdTe  sub-technology, as reported in the literature published between 2018 and 2020.

| Sub-technology | Material | Intensity (kg/MW) | Source |
| --- | --- | --- | --- |
| CdTe | Cadmium Cd | 65.2 | (Valero et al., 2018) |
|  |  | 85.0 | (Elshkaki & Shen, 2019) |
|  |  | 244.0 | (Watari et al., 2019) |
|  |  | 87.0 | (Watari et al., 2019) |
|  |  | 53.0 | (Zhou et al., 2020) |
|  |  | 35.0 | (Carrara et al., 2020) |
|  |  | 50.0 | (Carrara et al., 2020) |
|  |  | 85.0 | (Carrara et al., 2020) |
|  | Tellurium Te | 70.0 | (Månberger & Stenqvist, 2018) |
|  |  | 65.4 | (Valero et al., 2018) |
|  |  | 97.5 | (Elshkaki & Shen, 2019) |
|  |  | 92.0 | (Watari et al., 2019) |
|  |  | 41.0 | (Zhou et al., 2020) |
|  |  | 35.0 | (Carrara et al., 2020) |
|  |  | 95.0 | (Carrara et al., 2020) |
|  |  | 52.0 | (Carrara et al., 2020) |

Table S1.7: Gallium, indium and selenium intensities (kg/MW) in the CIGS  sub-technology, as reported in the literature published between 2018 and 2020.

| Sub-technology | Material | Intensity (kg/MW) | Source |
| --- | --- | --- | --- |
| CIGS | Gallium Ga | 4.0 | (Månberger & Stenqvist, 2018) |
|  |  | 7.5 | (Elshkaki & Shen, 2019) |
|  |  | 19.0 | (Watari et al., 2019) |
|  |  | 3.0 | (Zhou et al., 2020) |
|  |  | 3.0 | (Carrara et al., 2020) |
|  |  | 4.0 | (Carrara et al., 2020) |
|  |  | 7.0 | (Carrara et al., 2020) |
|  | Indium In | 13.0 | (Månberger & Stenqvist, 2018) |
|  |  | 13.0 | (Wang et al., 2019) |
|  |  | 23.0 | (Elshkaki & Shen, 2019) |
|  |  | 31.0 | (Watari et al., 2019) |
|  |  | 12.0 | (Zhou et al., 2020) |
|  |  | 10.0 | (Carrara et al., 2020) |
|  |  | 15.0 | (Carrara et al., 2020) |
|  |  | 27.0 | (Carrara et al., 2020) |
|  | Selenium Se | 45.0 | (Elshkaki & Shen, 2019) |
|  |  | 61.0 | (Watari et al., 2019) |
|  |  | 26.0 | (Zhou et al., 2020) |
|  |  | 22.0 | (Carrara et al., 2020) |
|  |  | 35.0 | (Carrara et al., 2020) |
|  |  | 60.0 | (Carrara et al., 2020) |

Table S1.8: Silver intensities (kg/MW) in the c-Si  sub-technology, as reported in the  literature published between 2018 and 2020 reporting on data for the years 2030 and 2050.

| Sub-technology | Material | 2030 Intensity (tons/GW) | 2050 Intensity (tons/GW) | Source |
| --- | --- | --- | --- | --- |
| c-Si | Silver Ag | 10 | 1 | (Elshkaki & Shen, 2019) |
|  |  | 4 | 1 | (Carrara et al., 2020) |
|  |  | 6 | 2 | (Carrara et al., 2020) |
|  |  | 11 | 5 | (Carrara et al., 2020) |

Table S1.9: Germanium intensities (kg/MW) in the a-Si sub-technology, as reported in the literature published between 2018 and 2020 reporting on data for the years 2030 and 2050.

| Sub-technology | Material | 2030 Intensity (tons/GW) | 2050 Intensity (tons/GW) | Source |
| --- | --- | --- | --- | --- |
| a-Si | Germanium Ge | 35 | 14 | (Elshkaki & Shen, 2019) |
|  |  | 22 | 10 | (Carrara et al., 2020) |
|  |  | 27 | 15 | (Carrara et al., 2020) |
|  |  | 32 | 20 | (Carrara et al., 2020) |

Table S1.10: Cadmium and tellurium intensities (kg/MW) in the CdTe  sub-technology, as reported in the literature published between 2018 and 2020 reporting on data for the years 2030 and 2050.

| Sub-technology | Material | 2030 Intensity (tons/GW) | 2050 Intensity (tons/GW) | Source |
| --- | --- | --- | --- | --- |
| CdTe | Cadmium Cd | 31 | 12 | (Elshkaki & Shen, 2019) |
|  |  | 30 | 17 | (Zhou et al., 2020) |
|  |  | 20 | 10 | (Carrara et al., 2020) |
|  |  | 27 | 12 | (Carrara et al., 2020) |
|  |  | 60 | 35 | (Carrara et al., 2020) |
|  | Tellurium Te | 35 | 14 | (Elshkaki & Shen, 2019) |
|  |  | 29 | 20 | (Zhou et al., 2020) |
|  |  | 20 | 11 | (Carrara et al., 2020) |
|  |  | 27 | 15 | (Carrara et al., 2020) |
|  |  | 70 | 40 | (Carrara et al., 2020) |

Table S1.11: Gallium, indium and selenium intensities (kg/MW) in the CIGS  sub-technology, as reported in the literature published between 2018 and 2020 reporting on data for the years 2030 and 2050.

| Sub-technology | Material | 2030 Intensity (tons/GW) | 2050 Intensity (tons/GW) | Source |
| --- | --- | --- | --- | --- |
| CIGS | Indium In | 11 | 6 | (Elshkaki & Shen, 2019) |
|  |  | 9 | 7 | (Zhou et al., 2020) |
|  |  | 8 | 5 | (Carrara et al., 2020) |
|  |  | 10 | 6 | (Carrara et al., 2020) |
|  |  | 17 | 10 | (Carrara et al., 2020) |
|  | Gallium Ga | 3 | 1 | (Elshkaki & Shen, 2019) |
|  |  | 2.5 | 2 | (Zhou et al., 2020) |
|  |  | 2 | 1 | (Carrara et al., 2020) |
|  |  | 2.5 | 1.5 | (Carrara et al., 2020) |
|  |  | 4.5 | 2.5 | (Carrara et al., 2020) |
|  | Selenium Se | 23 | 11 | (Elshkaki & Shen, 2019) |
|  |  | 19 | 14 | (Zhou et al., 2020) |
|  |  | 17 | 9 | (Carrara et al., 2020) |
|  |  | 20 | 12 | (Carrara et al., 2020) |
|  |  | 40 | 20 | (Carrara et al., 2020) |

Figure S1.5: Intensities (kg/MW) per sub-technology and material for solar PV in the current case, and for years 2030 and 2050.

Wind power plants

For wind, we conducted our own literature review for material intensity in the current case, and 2050. Tables S1.12–19 summarize the current values for each material under DD-PMSG and GB-PMSG respectively.

Table S1.12: Neodymium intensities [kg/MW] in neodymium-iron-boron (NdFeB) permanent magnets in dirent-drive systems (DD-PMSG) reported in previous studies.

| Sub-technology | Material | NdFeB Magnet Kg/MW | Kg/MW | %w | Source |
| --- | --- | --- | --- | --- | --- |
| DD-PMSG | Neodymium Nd | 650 | 175.5 | 27% | (Imholte et al., 2018) |
|  |  | 650 | 175.5 | 27% | (J. Li et al., 2020) |
|  |  | 650 | 150.8 | 23% | (Kumari et al., 2018) |
|  |  | 616 | 198 | 32% | (Hoenderdaal et al., 2013) |
|  |  | 450 | 183 | 41% | (Elshkaki & Shen, 2019) |

Table S1.13: Praseodymium intensities [kg/MW] in neodymium-iron-boron (NdFeB) permanent magnets in dirent-drive systems (DD-PMSG) reported in previous studies.

| Sub-technology | Material | NdFeB Magnet Kg/MW | Kg/MW | %w | Source |
| --- | --- | --- | --- | --- | --- |
| DD-PMSG | Praseodymium Pr | 650 | 13 | 2% | (Imholte et al., 2018) |
|  |  | 650 | 43.875 | 7% | (J. Li et al., 2020) |
|  |  | 650 | 36.4 | 6% | (Kumari et al., 2018) |
|  |  | 450 | 48 | 11% | (Elshkaki & Shen, 2019) |

Table S1.14: Dysprosium intensities [kg/MW] in neodymium-iron-boron (NdFeB) permanent magnets in dirent-drive systems (DD-PMSG) reported in previous studies.

| Sub-technology | Material | NdFeB Magnet Kg/MW | Kg/MW | %w | Source |
| --- | --- | --- | --- | --- | --- |
| DD-PMSG | Dysprosium Dy | 650 | 19.5 | 3% | (Imholte et al., 2018) |
|  |  | 650 | 29.25 | 5% | (J. Li et al., 2020) |
|  |  | 650 | 27.3 | 4% | (Kumari et al., 2018) |
|  |  | 616 | 27.7 | 5% | (Hoenderdaal et al., 2013) |
|  |  | 450 | 29 | 6% | (Elshkaki & Shen, 2019) |

Table S1.15: Terbium intensities [kg/MW] in neodymium-iron-boron (NdFeB) permanent magnets in dirent-drive systems (DD-PMSG) reported in previous studies.

| Sub-technology | Material | NdFeB Magnet Kg/MW | Kg/MW | %w | Source |
| --- | --- | --- | --- | --- | --- |
| DD-PMSG | Terbium Tb | 650 | 6.5 | 1% | (Pavel et al., 2017) |
|  |  | 450 | 1.3 | 0.3% | (Elshkaki & Shen, 2019) |
|  |  | 650 | 7 | 1% | (Carrara et al., 2020) |
|  |  | 650 | 8 | 1% | (Leader et al. 2019) |

Table S1.16: Neodymium intensities [kg/MW] in neodymium-iron-boron (NdFeB) permanent magnets in gearbox systems (GB-PMSG) reported in previous studies.

| Sub-technology | Material | NdFeB Magnet Kg/MW | Kg/MW | %w | Source |
| --- | --- | --- | --- | --- | --- |
| GB-PMSG | Neodymium Nd | 120 | 27.84 | 23% | (Kumari et al., 2018) |
|  |  | 160 | 43.2 | 27% | (Imholte et al., 2018) |
|  |  | 120 | 32.4 | 27% | (J. Li et al., 2020) |

Table S1.17: Praseodymium intensities [kg/MW] in neodymium-iron-boron (NdFeB) permanent magnets in gearbox systems (GB-PMSG) reported in previous studies.

| Sub-technology | Material | NdFeB Magnet Kg/MW | Kg/MW | %w | Source |
| --- | --- | --- | --- | --- | --- |
| GB-PMSG | Praseodymium Pr | 160 | 3.2 | 2% | (Imholte et al., 2018) |
|  |  | 120 | 8.1 | 7% | (J. Li et al., 2020) |
|  |  | 160 | 8.96 | 6% | (Kumari et al., 2018) |

Table S1.18: Dysprosium intensities [kg/MW] in neodymium-iron-boron (NdFeB) permanent magnets in gearbox systems (GB-PMSG) reported in previous studies.

| Sub-technology | Material | NdFeB Magnet Kg/MW | Kg/MW | %w | Source |
| --- | --- | --- | --- | --- | --- |
| GB-PMSG | Dysprosium Dy | 160 | 4.8 | 3% | (Imholte et al., 2018) |
|  |  | 120 | 5.4 | 5% | (J. Li et al., 2020) |
|  |  | 120 | 5.04 | 4% | (Kumari et al., 2018) |

Table S1.19: Terbium intensities [kg/MW] in neodymium-iron-boron (NdFeB) permanent magnets in gearbox systems (GB-PMSG) reported in previous studies.

| Sub-technology | Material | NdFeB Magnet Kg/MW | Kg/MW | %w | Source |
| --- | --- | --- | --- | --- | --- |
| GB-PMSG | Terbium Tb | 160 | 1 | 1% | (Carrara et al., 2020) |
|  |  | 160 | 1.6 | 1% | (Pavel et al., 2017) |

For the future, we assumed a 30% reduction in MI for the LREE (Nd and Pr) by 2050, in accordance with Fishman and Graedel (2019) and Wang et al. (2024), and a 50% reduction in MI for the HREE (Dy and Tb) in accordance to Wang et al. (2024) and Gregoir and van Acker (2022).

A higher MI reduction for the HREE is assumed considering the recent developments in relation to their reduction (JRC, 2016; Schulze & Buchert, 2016), as well the anticipated further reductions (Rabe et al., 2017). There are several examples of industrial applications involving such material reductions. Lower levels of LREE per installed capacity  are used in Vestas’ EnVentus model, and the use of HREE has been entirely eliminated (Vestas, 2019, p. 2). Siemens Gamesa Renewable Energy has notably decreased the Dy content of their wind turbines to well under 1% (compared to the market average of 3%–6% in Tables S1.14 and S1.18). Meanwhile, Goldwind has completely eliminated the use of Dy in certain generators, and in other designs it has reduced its content to below 1% (Wind Power Monthly, 2018).

Similar to the approach used for solar PV, the current MI values were kept constant throughout the scenario period in the Baseline scenario, while for the MI reduction scenario the values were linearly interpolated between the current (2021) and 2050 values. Figure S1.6 provides a visualization of the MI in the current and 2050 cases, as well as the average values used per case.

Figure S1.6: Intensities (kg/MW) per sub-technology and material for wind power in the current year, and in 2050.

# 8. Substitution

We conducted a review of the scientific literature and industrial applications for substitution options used for wind and solar PV technologies.

For wind technologies, substitution options are available for the NdFeB permanent magnets used in wind turbines, both at the component and material levels. On the material level, substitution possibilities for the four REE used in PM exist. For example, Pr can replace Nd without compromising PM performance. Neodymium can replace Dy, which is associated with significantly higher costs than Nd (JRC, 2016)). However, in our analysis, we focus on substitution options that do not involve substituting one of the four REE with a different REE. Substitutions using platinum group metals, cerium (Ce), and copper (Cu) have been mentioned in the literature as a material substitution option for REEs. Investigations are underway to identify new magnetic alloy substitutes for Dy. An innovative approach involves using an alloy that combines Ce and cobalt, allowing for the substitution of Ce for Dy while retaining the desired magnetic characteristics, which could represent an attractive alternative in the future (Huber & Steininger, 2022). Furthermore, as mentioned above, certain manufacturers have redesigned their generators to eliminate completely the use of HREE in their generators.

From the component substitution perspective, ferrite magnets can be used in place of REE-containing permanent magnets. GreenSpur Renewables has pioneered the development of the world's first permanent magnet synchronous generator (PMSG), which utilizes ferrite instead of REE (Snieckus, 2019). Ferrite is not subject to the same supply chain constraints or market monopolies, potentially allowing for reduced capital expenditures, which is particularly advantageous for larger electric generators. PMSG has, however, been associated with a 3% reduction in efficiency (Church & Crawford, 2018). During the Investor Day event held on March 1, 2023, Tesla announced its upcoming generation of permanent magnet synchronous reluctance motors (PMSRM) for traction applications, specifying that these would not incorporate rare earth PM, which led to speculation that the REE-containing PM would be replaced by ferrite magnets (Adamas Intelligence, 2023).

In addition to substitutions on the component and material levels, there is a potential for substitution on the sub-technology level with the high-temperature superconducting (HTS) technology. The EcoSwing H2020 project achieved a significant milestone in wind turbine generator technology by designing, constructing, and testing the world's first full-size superconducting generator for a wind turbine. This generator, rated at 3.6 MW, was a groundbreaking step in demonstrating the viability of the HTS technology for operational wind converters. The project managed to integrate this technology into a modern wind turbine, and it proved that the superconducting drive train could be cost-competitive with both state-of-the-art, direct-drive PM generators and conventional, geared drive trains when produced in series.

In our analysis, we assume a 50% substitution rate by 2050 in the Substitution scenario, in line with recent assumptions made in the literature in relation to REE-free PM (Wang et al., 2024) and HTS (Carrara et al., 2020).

For solar PV, substitution options are available at the material and sub-technology levels. However, most of the options are theoretically possible but not yet commercially available. On the material level, recent advancements suggest that silver-free crystalline silicon (c-Si) panels are approaching the stage of commercial availability. The silver content can be replaced entirely by Cu, through an electroplating process (Fraunhofer, 2022). The literature also mentions that the silver content can be replaced entirely by nickel-copper plating, which has the potential to lower the cell resistance and increase the efficiency (Kim & Lee, 2013; Rehman & Lee, 2014). For the amorphous silicon (a-Si) sub-technology, Ge can theoretically be replaced by a carbon nanomaterial (G, CNT) (Arvidsson & Sandén, 2017). There is currently no broadly adopted industrial application for replacing cadmium and tellurium in cadmium telluride (CdTe) solar panels that does not compromise their efficiency and cost. However, on the sub-technology level, in the thin-film solar cell market, a-Si and copper-indium-gallium-selenide (CIGS) PV cells are competing with cadmium telluride (CdTe). For CIGS, carbon nanomaterials (G, CNT) are mentioned in the literature as theoretical substitutes for indium. Otherwise, for CIGS, sub-technology level substitution is possible, with a-Si and CdTe PV cells. Finally, ongoing research is exploring a novel thin-film technology using perovskite materials as a possible alternative for 2nd generation thin films.

In our analysis, we focus on substitution options that do not involve substituting one existing sub-technology for another. Therefore, we assume a 20% substitution rate by 2050 in the Substitution scenario, taking into account the 15% penetration of perovskite solar cells in the IEA’s distributed solar PV scenario for high perovskite penetration in 2040 (IEA, 2022) and given that the potential of silver-free c-Si panels is approaching the stage of commercial availability.

# 9. Results for disaggregated wind power demands

Figure S1.7: Minor metal demands in ton per year for each metal and scenario in onshore wind power disaggregated per sub-technology (DD-PMSG and GB-PMSG).

Figure S1.8: Minor metal demands in ton per year for each metal and scenario in offshore wind power disaggregated per sub-technology (DD-PMSG and GB-PMSG).

# 10. Sensitivity analysis

We assess the impacts of two key factors: market shares and metal intensities through sensitivity analysis. In particular, we assess two alternative scenarios: one with reduced MS for thin-film solar and for REE-containing PM wind sub-technologies and lower MI, and another with increased shares of these sub-technologies and higher MI. Table S1.20 provides the parameter settings for the sensitivity analysis. Tables S1.21 and S1.22 show the sensitivity analysis results for thenario SI and SII respectively.

Table S1.20: Parameter settings for sensitivity analysis

|  | Market shares (MS) | Metal intensities (MI) |
| --- | --- | --- |
| Baseline | JRC’s Medium Demand Scenario (MDS) Source: Carrara et al. (2020) See Figure S1.3 | See Figure S1.5 and Figure S1.6 |
| SI | JRC’s Low Demand Scenario (LDS) Source: Carrara et al. (2020) See Figure S1.9 | 10% lower than the Baseline scenario |
| SII | JRC’s High Demand Scenario (HDS) Source: Carrara et al. (2020) See Figure S1.10 | 10% higher than the Baseline scenario |

Table S1.21: Sensitivity analysis for the cumulative demands (inflows) for minor metals (in tons) for the period 2022-2050 in scenario SI. The percentage change from the original scenarios is presented in brackets.

| Technology | Sub-technology | Minor metal | Baseline | Lifespan expansion | MI reduction | Substitution | Composite |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Wind (onshore & offshore) | DD-PMSG & GB-PMSG | Nd | 3254 (-43%) | 3116 (-43%) | 2946 (-37%) | 2271 (-42%) | 2007 (-36%) |
|  |  | Pr | 649 (-43%) | 621 (-43%) | 587 (-37%) | 453 (-42%) | 400 (-36%) |
|  |  | Dy | 488 (-43%) | 467 (-43%) | 375 (-37%) | 341 (-42%) | 260 (-36%) |
|  |  | Tb | 108 (-42%) | 104 (-43%) | 83 (-36%) | 74 (-42%) | 58 (-35%) |
| Solar PV (distributed & centralized) | c-Si | Ag | 844 (-2%) | 811 (-1%) | 197 (6%) | 826 (9%) | 179 (7%) |
|  | a-Si | Ge | 13 (-87%) | 12 (-87%) | 7 (-77%) | 13 (-85%) | 7 (-75%) |
|  | CdTe | Cd | 10 (-87%) | 10 (-87%) | 7 (-80%) | 10 (-85%) | 6 (-79%) |
|  |  | Te | 4 (-71%) | 4 (-71%) | 3 (-58%) | 2 (-82%) | 3 (-53%) |
|  | CIGS | In | 2 (-90%) | 2 (-90%) | 1 (-88%) | 2 (-88%) | 1 (-87%) |
|  |  | Ga | 1 (-89%) | 1 (-89%) | 0 (-89%) | 1 (-88%) | 0 (-86%) |
|  |  | Se | 5 (-90%) | 5 (-90%) | 2 (-88%) | 5 (-88%) | 2 (-87%) |

Table S1.22: Sensitivity analysis for the cumulative demands (inflows) for minor metals (in

tons) for the period 2022-2050 in scenario SII. The percentage change from the original scenarios is presented in brackets.

| Technology | Sub-technology | Minor metal | Baseline | Lifespan expansion | MI reduction | Substitution | Composite |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Wind (onshore & offshore) | DD-PMSG & GB-PMSG | Nd | 9026 (58%) | 8661 (58%) | 6655 (43%) | 5346 (36%) | 4496 (43%) |
|  |  | Pr | 1803 (58%) | 1730 (58%) | 1329 (43%) | 1069 (36%) | 898 (43%) |
|  |  | Dy | 1355 (58%) | 1300 (58%) | 844 (43%) | 804 (36%) | 581 (43%) |
|  |  | Tb | 295 (57%) | 283 (56%) | 184 (42%) | 173 (34%) | 127 (42%) |
| Solar PV (distributed & centralized) | c-Si | Ag | 816 (-5%) | 780 (-5%) | 168 (-9%) | 657 (-13%) | 153 (-9%) |
|  | a-Si | Ge | 239 (139%) | 232 (142%) | 67 (109%) | 188 (116%) | 58 (108%) |
|  | CdTe | Cd | 186 (141%) | 181 (141%) | 71 (110%) | 147 (115%) | 62 (105%) |
|  |  | Te | 38 (215%) | 37 (204%) | 18 (151%) | 30 (173%) | 15 (155%) |
|  | CIGS | In | 48 (141%) | 47 (148%) | 22 (116%) | 38 (122%) | 18 (131%) |
|  |  | Ga | 18 (160%) | 18 (153%) | 6 (86%) | 14 (137%) | 5 (140%) |
|  |  | Se | 111 (142%) | 108 (147%) | 45 (124%) | 87 (123%) | 38 (126%) |

Figure S1.9: Market shares for the DD-PMSG and GB-PMSG sub-technologies (PM-containing drivetrains) over time for the period of 2018–2050 for a. onshore wind turbines and b. offshore wind turbines in JRC’s Low Demand Scenario (LDS). Note: The category of ‘Other’ technologies includes Direct Drive Electrically Excited Synchronous Generator (DD-EESG), Gearbox Double-Fed Induction Generator (GB-DFIG), Gearbox Squirrel Cage Induction Generator (GB-SCIG), and Direct Drive High-Temperature Superconductors (DD-HTS).

Figure S1.10: Market shares for the DD-PMSG and GB-PMSG sub-technologies (PM-containing drivetrains) over time for the period of 2018–2050 for a. onshore wind turbines and b. offshore wind turbines in JRC’s High Demand Scenario (HDS). Note: The category of ‘Other’ technologies includes Direct Drive Electrically Excited Synchronous Generator (DD-EESG), Gearbox Double-Fed Induction Generator (GB-DFIG), Gearbox Squirrel Cage Induction Generator (GB-SCIG), and Direct Drive High-Temperature Superconductors (DD-HTS).

# References

Adamas Intelligence. (2023). Implications: Tesla Announces Next Generation Rare-Earth-Free PMSM. https://www.adamasintel.com/tesla-rare-earth-free-motor/

Allekotte, L., & Garrett, P. (2024). Life Cycle Assessment of electricity production from an offshore V236.15 MW wind plant. Vestas. https://www.vestas.com/en/sustainability/environment/lifecycle-assessments

Arvidsson, R., & Sandén, B. A. (2017). Carbon nanomaterials as potential substitutes for scarce metals. Journal of Cleaner Production, 156, 253–261. https://doi.org/10.1016/j.jclepro.2017.04.048

Bordoff, J., & O’Sullivan Meghan, L. (2022). Green upheaval: The new geopolitics of energy (Foreign Aff. 101, 68).

Carrara, S., Alves Dias, P., Plazzotta, B., & Pavel, C. (2020). Raw materials demand for wind and solar PV technologies in the transition towards a decarbonised energy system. Publications Office. https://data.europa.eu/doi/10.2760/160859

Church, C., & Crawford, A. (2018). Green Conflict Minerals: The fuels of conflict in the transition to a low-carbon economy. https://madi.africa/wp-content/uploads/2022/10/IISD-green-conflict-minerals.pdf

Ciacci, L., Vassura, I., Cao, Z., Liu, G., & Passarini, F. (2019). Recovering the “new twin”: Analysis of secondary neodymium sources and recycling potentials in Europe. Resources, Conservation and Recycling, 142, 143–152. https://doi.org/10.1016/j.resconrec.2018.11.024

Dominish, E., Teske, S., & Florin, N. (2019). Responsible minerals sourcing for renewable energy (Report Prepared for Earthworks by the Institute for Sustainable Futures, University of Technology Sydney.).

Elshkaki, A., & Shen, L. (2019). Energy-material nexus: The impacts of national and international energy scenarios on critical metals use in China up to 2050 and their global implications. Energy, 180, 903–917. https://doi.org/10.1016/j.energy.2019.05.156

Fischer, A. (2022). Maxeon 40-year solar panel warranty available in select markets. https://pv-magazine-usa.com/2022/02/04/maxeon-40-year-solar-panel-warranty-available-in-select-markets/?utm_source=chatgpt.com

Fishman, T., & Graedel, T. E. (2019). Impact of the establishment of US offshore wind power on neodymium flows. Nature Sustainability, 2(4), 332–338. https://doi.org/10.1038/s41893-019-0252-z

Fraunhofer. (2022). Out with the silver, in with the copper: A new boost for solar cell production. https://www.fraunhofer.de/en/press/research-news/2022/september-2022/out-with-the-silver-in-with-the-copper-a-new-boost-for-solar-cells.html

Giurco, D., Dominish, E., Florin, N., Watari, T., & McLellan, B. (2019). Requirements for Minerals and Metals for 100% Renewable Scenarios. In S. Teske (Ed.), Achieving the Paris Climate Agreement Goals (pp. 437–457). Springer International Publishing. https://doi.org/10.1007/978-3-030-05843-2_11

Graedel, T. E., Reck, B. K., & Miatto, A. (2022). Alloy information helps prioritize material criticality lists. Nature Communications, 13(1), 150. https://doi.org/10.1038/s41467-021-27829-w

Gregoir, L., & van Acker, K. (2022, April 25). Metals for Clean Energy: Pathways to solving Europe’s raw materials challenge. Eurometaux. https://eurometaux.eu/media/rqocjybv/metals-for-clean-energy-final.pdf

Hoenderdaal, S., Tercero Espinoza, L., Marscheider-Weidemann, F., & Graus, W. (2013). Can a dysprosium shortage threaten green energy technologies? Energy, 49, 344–355. https://doi.org/10.1016/j.energy.2012.10.043

Huber, S. T., & Steininger, K. W. (2022). Critical sustainability issues in the production of wind and solar electricity generation as well as storage facilities and possible solutions. Journal of Cleaner Production, 339, 130720. https://doi.org/10.1016/j.jclepro.2022.130720

IEA. (2018). Offshore Energy Outlook (World Energy Outlook Series). Organisation for Economic Cooperation and Development and IEA.

IEA. (2022). The Role of Critical Minerals in Clean Energy Transitions (World Energy Outlook Special Report) [International Energy Agency]. https://www.iea.org/reports/the-role-of-critical-minerals-in-clean-energy-transitions

Imholte, D. D., Nguyen, R. T., Vedantam, A., Brown, M., Iyer, A., Smith, B. J., Collins, J. W., Anderson, C. G., & O’Kelley, B. (2018). An assessment of U.S. rare earth availability for supporting U.S. wind energy growth targets. Energy Policy, 113, 294–305. https://doi.org/10.1016/j.enpol.2017.11.001

Jean, J., Brown, P. R., Jaffe, R. L., Buonassisi, T., & Bulović, V. (2015). Pathways for solar photovoltaics. Energy & Environmental Science, 8(4), 1200–1219. https://doi.org/10.1039/C4EE04073B

Jowett, P. (2024). Oxford PV starts commercial distribution of perovskite solar modules. https://www.pv-magazine.com/2024/09/05/oxford-pv-starts-commercial-distribution-of-perovskite-solar-modules/?utm_source=chatgpt.com

JRC. (2016). Wind Energy Status Report: 2016 Edition. https://publications.jrc.ec.europa.eu/repository/handle/JRC105720

Kim, D. H., & Lee, S. H. (2013). Investigation on plated Ni/Cu contact for mono-crystalline silicon solar cells. Electronic Materials Letters, 9(5), 677–681. https://doi.org/10.1007/s13391-013-2169-4

Kumari, A., Sinha, M. K., Pramanik, S., & Sahu, S. K. (2018). Recovery of rare earths from spent NdFeB magnets of wind turbine: Leaching and kinetic aspects. Waste Management, 75, 486–498. https://doi.org/10.1016/j.wasman.2018.01.033

Lansstyrelsen. (2022). Vindbrukskollen [Master Data Register of Wind Turbines]. https://vbk.lansstyrelsen.se/

Leader, A., Gaustad, G., & Babbitt, C. (2019). The effect of critical material prices on the competitiveness of clean energy technologies. Materials for Renewable and Sustainable Energy, 8(2), 8. https://doi.org/10.1007/s40243-019-0146-z

Li, J., Peng, K., Wang, P., Zhang, N., Feng, K., Guan, D., Meng, J., Wei, W., & Yang, Q. (2020). Critical Rare-Earth Elements Mismatch Global Wind-Power Ambitions. One Earth, 3(1), 116–125. https://doi.org/10.1016/j.oneear.2020.06.009

Li, W., & Adachi, T. (2019). Evaluation of long‐term silver supply shortage for c‐Si PV under different technological scenarios. Natural Resource Modeling, 32(1), e12176. https://doi.org/10.1111/nrm.12176

Liang, Y., Kleijn, R., Tukker, A., & van der Voet, E. (2022). Material requirements for low-carbon energy technologies: A quantitative review. Renewable and Sustainable Energy Reviews, 161, 112334. https://doi.org/10.1016/j.rser.2022.112334

Lindahl, J., Oller Westerberg, A., & Vanky, K. (2020). National Survey Report of PV Power Applications in Sweden. https://iea-pvps.org/wp-content/uploads/2021/10/National-Survey-Report-of-PV-Power-Applications-in-Sweden-2020.pdf

Månberger, A., & Stenqvist, B. (2018). Global metal flows in the renewable energy transition: Exploring the effects of substitutes, technological mix and development. Energy Policy, 119, 226–241. https://doi.org/10.1016/j.enpol.2018.04.056

Maxeon. (2023). Maxeon Sustainability Report 2023. https://static1.squarespace.com/static/6740dc6b95218e0ecdaa68f6/t/67d9cec2984b8b0176515624/1742327527732/BRAND-227_ESG+Report+2023_JUN24+1.pdf

Nassar, N. T., Wilburn, D. R., & Goonan, T. G. (2016). Byproduct metal requirements for U.S. wind and solar photovoltaic electricity generation up to the year 2040 under various Clean Power Plan scenarios. Applied Energy, 183, 1209–1226. https://doi.org/10.1016/j.apenergy.2016.08.062

Pavel, C. C., Lacal-Arántegui, R., Marmier, A., Schüler, D., Tzimas, E., Buchert, M., Jenseit, W., & Blagoeva, D. (2017). Substitution strategies for reducing the use of rare earths in wind turbines. Resources Policy, 52, 349–357. https://doi.org/10.1016/j.resourpol.2017.04.010

Rabe, W., Kostka, G., & Smith Stegen, K. (2017). China’s supply of critical raw materials: Risks for Europe’s solar and wind industries? Energy Policy, 101, 692–699. https://doi.org/10.1016/j.enpol.2016.09.019

Rehman, A., & Lee, S. (2014). Review of the Potential of the Ni/Cu Plating Technique for Crystalline Silicon Solar Cells. Materials, 7(2), 1318–1341. https://doi.org/10.3390/ma7021318

Savvidou, G., & Johnsson, F. (2023). Material Requirements, Circularity Potential and Embodied Emissions Associated with Wind Energy. Sustainable Production and Consumption, 40, 471–487. https://doi.org/10.1016/j.spc.2023.07.012

Schulze, R., & Buchert, M. (2016). Estimates of global REE recycling potentials from NdFeB magnet material. Resources, Conservation and Recycling, 113, 12–27. https://doi.org/10.1016/j.resconrec.2016.05.004

Shaw, V. (2025). Microquanta developing perovskite solar modules for BIPV applications. https://www.pv-magazine.com/2025/02/17/microquanta-developing-perovskite-solar-modules-for-bipv-applications/?utm_source=chatgpt.com

Snieckus, D. (2019). Offshore Wind Turbine 20MW Generator Ready “Within Three Years”’. https://www.rechargenews.com/wind/offshore-wind-turbine-20mw-generator-ready-within-three-years/2-1-711845

Svenska Kraftnät. (2021). Långsiktig marknadsanalys 2021. Scenarier för elsystemets utveckling fram till 2050 (No. Svk 2019/3305). https://www.svk.se/siteassets/om-oss/rapporter/2021/langsiktig-marknadsanalys-2021.pdf

Tangemann, C. (2020). Developing electric motors less dependable on rare earth magnets. https://www.automotive-iq.com/electrics-electronics/articles/developing-electric-motors-less-dependable-on-rare-earth-magnets

USGS. (2023). Mineral commodity summaries 2023. https://doi.org/10.3133/mcs2023

Valero, A., Valero, A., Calvo, G., & Ortego, A. (2018). Material bottlenecks in the future development of green technologies. Renewable and Sustainable Energy Reviews, 93, 178–200. https://doi.org/10.1016/j.rser.2018.05.041

Vestas. (2019). Sustainability report 2019. https://www.vestas.com/en/sustainability/reports-and-ratings

Wang, P., Chen, L.-Y., Ge, J.-P., Cai, W., & Chen, W.-Q. (2019). Incorporating critical material cycles into metal-energy nexus of China’s 2050 renewable transition. Applied Energy, 253, 113612. https://doi.org/10.1016/j.apenergy.2019.113612

Wang, P., Yang, Y.-Y., Heidrich, O., Chen, L.-Y., Chen, L.-H., Fishman, T., & Chen, W.-Q. (2024). Regional rare-earth element supply and demand balanced with circular economy strategies. Nature Geoscience, 17(1), 94–102. https://doi.org/10.1038/s41561-023-01350-9

Watari, T., McLellan, B. C., Giurco, D., Dominish, E., Yamasue, E., & Nansai, K. (2019). Total material requirement for the global energy transition to 2050: A focus on transport and electricity. Resources, Conservation and Recycling, 148, 91–103. https://doi.org/10.1016/j.resconrec.2019.05.015

Wind Power Monthly. (2018). Rethinking the use of rare-earth elements. https://www.windpowermonthly.com/article/1519221/rethinking-use-rare-earth-elements

Zhou, Y., Li, J., Rechberger, H., Wang, G., Chen, S., Xing, W., & Li, P. (2020). Dynamic criticality of by-products used in thin-film photovoltaic technologies by 2050. Journal of Cleaner Production, 263, 121599. https://doi.org/10.1016/j.jclepro.2020.121599

https://cordis.europa.eu/project/id/656024

https://cordis.europa.eu/project/id/656024