Supplement 2

PET-CT Imaging

The study was performed with dedicated PET/CT scanners (GE Healthcare, 23 patients: Discovery 710; 3 patients: Discovery VCT). All patients fasted for at least 4 hours before each examination. Preparation of study subjects consisted of inserting a suitable indwelling intravenous catheter into a large vein (e.g., antecubital vein), preferably in the subject’s nondominant arm. Correct localization of the indwelling cannula must be ensured by test injection of normal saline solution prior to injecting the study drug.

After injecting the study drug, the cannula and injection system were flushed with 10 ml saline solution. Imaging was acquired 60 min. after an intravenous (i.v.) injection of 4 mCi (148 MBq) of 68Ga-RM2. Scanning time here? All except 3 patients, had concomitant 18F-FCH PET/CT imaging 60 min. after i.v. administration of 6 mCi (222 MBq) 18F-FCH. All patients received standard volume (i.e. 250 ml) of sodium chloride i.v. infusion and 250-500 ml oral fluid before the PET/CT acquisition in order to push tracer excretion for better image quality and reduce the radiation exposure in the urinary system.

PET acquisitions were obtained from the base of the skull to the proximal of the thigh with 2.5 min./bed position acquisition time using time of flight (TOF) modus. All images were reconstructed identically with ordered-subsets expectation maximization algorithm (4 iterations, 18 subsets) followed by a post-reconstruction smoothing Gaussian filter (4.0 mm in full width at one-half maximum).

The CT portion of the 18F-FCH PET/CT procedure was performed after intravenous infusion of 100 ml ionic contrast medium with high beam current modulation (120-330 mA, 0.6 second per rotation, 5.0 mm reconstructed section thickness, 0.5 mm overlap, 512x512 matrix, pitch index 1.5), while a non-contrast CT with low beam current modulation (80–120 mA) was obtained on 68Ga-RM2 PET/CT for localization and attenuation correction. The reformatted, transverse, coronal, and sagittal views were used for interpretation.

Images were read using advanced PET-CT review software (Advantage Windows, version 4.6; GE Medical Systems), which allows simultaneous scrolling through the corresponding PET, CT, and fusion images in the transverse, coronal, and sagittal planes. The prostate gland contour was identified on the PET images based on corresponding CT images. The defined prostate volume on PET images was extracted using a software program provided by the manufacturer.

Supporting Matherial MRI data acquisition

Dynamic contrast enhancement 3D T1-spoiled gradient echo (GE) images were acquired during an intravenous bolus injection of paramagnetic contrast medium at a dose of 0.2 mmol/kg of body weight for examination at 1.5T and 0.1 mmol/kg of body weight for examination at 3T. A common flow rate of 2.0 ml/s was used, followed by a 20 ml saline flush. The images were acquired every 13 seconds at 1.5T and every 7 seconds at 3T, with each sequence lasting for 4:30 and 6:59 minutes at 1.5T and 3T, respectively.

In order to suppress bowel peristalsis, 1.0 mg of glucagon (GlucaGen, Novo Nordisk, Bagsværd, Denmark) subcutaneously was given immediately before the start of the examination. Moreover, patients were instructed to take laxatives in the evening before the mpMRI examination to limit rectal air (Laxoberon [Boehringer Ingelheim, Ingelheim, Germany]) as a part of routine preparation for prostate MRI protocol.

Anatomical triplanar T2-weighted images were acquired using a turbo spin-echo sequence with  repetition time/echo time (TR/TE) 6400-8640/101 ms, field of view (FOV) 200 x 200 mm2, matrix size 320 x 320,  slice thickness 3 mm, no gaps, number of signal averages (NSA) 2 and acquisition time 2.27 min (transverse images), 2.47 min  (sagittal images), 2.78 min (coronal images). Generalized autocalibrating partially parallel acquisitions (GRAPPA) parallel imaging method (1) was used with the acceleration factor of 2 and 32 reference lines for autocalibration. A single shot spin-echo based sequence was used for DWI with TR/TE 5543/80 ms, FOV 260 x 260 mm2, matrix size 128 x 128, slice thickness 3 mm, no gaps NSA 4, a bandwidth 1184Hz/pixel acquiring 6/8 (75%) of k-space in phase-encoding direction, b-values 0, 100, 200 350, 500 s/mm², diffusion gradients in three orthogonal directions for each b-value, GRAPPA with the acceleration factor of 2 and 32 reference lines. The acquisition time for DWI was 5.1 min. Additional DWI with 16 b values  (0, 50, 100, 200, 350, 500, 650, 800, 950, 1100, 1250, 1400, 1550, 1700, 1850, 2000 s/mm²) was acquired with TR/TE 7000/87, FOV 200 x 200 mm2, matrix size 320 x 320,  slice thickness 5 mm, NSA 2 and acquisition time 11.07 min.   The apparent diffusion coefficient maps were calculated automatically by vendor specific software using mono-exponential fit with a noise level of 10,. The three-dimensional 1H-MRS covering whole prostate was performed using point resolved spatially localized spectroscopy (PRESS) sequence. Automatic and manual shimming of whole prostate volume was performed (to optimize the main magnetic field homogeneity) in every patient. Weighted averaging (NSA: 6) of elliptically sampled k-space, Hanning filtering of the signal and zero-filling to a 16 x 16 matrix was performed before Fourier transformation (2). The TR and TE were optimized for the shape of citrate resonance (3,4).  Additional water and lipid signal from adjacent tissues was suppressed with seven outer volume saturation slabs. The following additional imaging parameters were used: acquisition bandwidth 1300 Hz, 512 spectra data points, FOV 96 x 96 x 96 mm2 and matrix size of 12 x 12 x 12 resulting into nominal voxel size of 8x8x8 mm3.  A real voxel size could be best approximated as a sphere with a volume of 1.51 cm3 and diameter of 14.24 mm after apodization (2).  The acquisition time was about 15-20 minutes including manual shimming. Axial dynamic contrast enhanced images were acquired before, during and after injection of contrast agent. Contrast agent (0.1 mmol/kg Dotarem [Guerbet, France] was injected 30 s after the beginning of the sequence through a peripheral vein at a rate of 2 ml/s via a mechanical injector (Spectris, Medrad, Indianola, USA).  In total, 60 time points at a temporal resolution of 6.9 seconds were acquired using a three-dimensional VIBE sequence (5) with the following parameters: TR/TE 5.43/1.87 seconds, 15 degree flip angle, FOV 240 x 240 mm2, matrix size 192 x 192, slice thickness 3 mm, a bandwidth of 260Hz/pixel acquiring 6/8 (75%) of k-space in phase-encoding direction, GRAPPA (1) with the acceleration factor of 2 and 24 reference lines, acquisition time 6.9 min. Before the contrast enhanced MR imaging, images with five different flip angles of 2, 5, 8, 10, 15 degrees were obtained for calculation of pre-contrast longitudinal relaxation time (T10) (6). The total duration of the MRI examination was about 60 minutes.

References

- Griswold MA, Jakob PM, Heidemann RM, et al. Generalized autocalibrating partially parallel acquisitions (GRAPPA). Magn Reson Med 2002; 47:1202-1210.

- Scheenen TW, Klomp DW, Roll SA, Futterer JJ, Barentsz JO, Heerschap A. Fast acquisition-weighted three-dimensional proton MR spectroscopic imaging of the human prostate. Magn Reson Med 2004; 52:80-88.

- Scheenen TW, Heijmink SW, Roell SA, et al. Three-dimensional proton MR spectroscopy of human prostate at 3 T without endorectal coil: feasibility. Radiology 2007; 245:507-516.

- Scheenen TW, Gambarota G, Weiland E, et al. Optimal timing for in vivo 1H-MR spectroscopic imaging of the human prostate at 3T. Magn Reson Med 2005; 53:1268-1274.

- Rofsky NM, Lee VS, Laub G, et al. Abdominal MR imaging with a volumetric interpolated breath-hold examination. Radiology 1999; 212:876-884.

- Wang HZ, Riederer SJ, Lee JN. Optimizing the precision in T1 relaxation estimation using limited flip angles. Magn Reson Med 1987; 5:399-416.