Appendix A. Equations for Time-Domain features

- (Root Mean Square) RMS= 1Ni=1Nxi2

- (Standard Deviation)  σ = 1N-1i=1N(xi-µ)2

- Skewness= 1Ni=1N1σ(xi-µ)3

- Kurtosis= 1Ni=1N1σ(xi-µ)4

- Absolute Mean= 1Ni=1N|xi|

- Crest Factor=max⁡{|xi|} RMS

- Total Energy= i=1Nxi2

Where:

xi: value of the signal at the i-th sample

µ: mean of the signal

σ: standard deviation of the signal

N: total number of samples in the dataset

RMS: root mean square value of the signal

Appendix B. Equations for Frequency-domain features

- Dominant Frequency=argmax (|X(f)|)

- Peak Frequency=argmax (|xk|)

- Peak Value=max (|xk|)

- Bandwidth= k(fk-fc)2.|Xk|2k|Xk|2

- Spectral Centroid= kfk.|Xk|k|Xk|

Where:

Xk: magnitude of the k-th frequency component in the FFT spectrum

fk: frequency corresponding to the k-th component

fc: spectral centroid (used as the reference in bandwidth calculation)

X(f): frequency spectrum of the signal obtained via FFT

sum: summation over all frequency bins (k)

Appendix C. Complete F-value Table for Feature Selection

| Nº | Feature Name | Domain | Channel | F-Value (Ra) | F-Value (Penp) |
| --- | --- | --- | --- | --- | --- |
| 1 | RMS | Time | Ch1 | 34.16 | 74.63 |
| 2 | RMS | Time | Ch2 | 1.32 | 6.81 |
| 3 | RMS | Time | Ch3 | 0.85 | 3.72 |
| 4 | Std Dev | Time | Ch1 | 2.12 | 121.26 |
| 5 | Std Dev | Time | Ch2 | 14.47 | 9.22 |
| 6 | Std Dev | Time | Ch3 | 0.88 | 4.15 |
| 7 | Abs Mean | Time | Ch1 | 0.97 | 4.30 |
| 8 | Abs Mean | Time | Ch2 | 0.59 | 2.20 |
| 9 | Abs Mean | Time | Ch3 | 0.63 | 2.95 |
| 10 | Skewness | Time | Ch1 | 0.79 | 4.88 |
| 11 | Skewness | Time | Ch2 | 1.45 | 23.46 |
| 12 | Skewness | Time | Ch3 | 0.49 | 1.63 |
| 13 | Kurtosis | Time | Ch1 | 0.76 | 3.91 |
| 14 | Kurtosis | Time | Ch2 | 0.61 | 2.61 |
| 15 | Kurtosis | Time | Ch3 | 8.07 | 4.32 |
| 16 | Crest Factor | Time | Ch1 | 0.68 | 3.60 |
| 17 | Crest Factor | Time | Ch2 | 0.62 | 2.18 |
| 18 | Crest Factor | Time | Ch3 | 0.74 | 5.10 |
| 19 | FFT Peak Freq | Frequency | Ch1 | 2.58 | 2.95 |
| 20 | FFT Peak Freq | Frequency | Ch2 | 0.71 | 3.40 |
| 21 | FFT Peak Freq | Frequency | Ch3 | 0.56 | 2.43 |
| 22 | FFT Peak Value | Frequency | Ch1 | 20.10 | 13.67 |
| 23 | FFT Peak Value | Frequency | Ch2 | 1.12 | 5.44 |
| 24 | FFT Peak Value | Frequency | Ch3 | 0.63 | 2.12 |
| 25 | Dominant Frequency | Frequency | Ch1 | 0.91 | 10.59 |
| 26 | Dominant Frequency | Frequency | Ch2 | 0.69 | 2.74 |
| 27 | Dominant Frequency | Frequency | Ch3 | 0.34 | 1.21 |
| 28 | Peak Value | Frequency | Ch1 | 0.48 | 2.00 |
| 29 | Peak Value | Frequency | Ch2 | 0.52 | 2.86 |
| 30 | Peak Value | Frequency | Ch3 | 0.43 | 1.58 |
| 31 | Bandwidth | Frequency | Ch1 | 0.41 | 1.84 |
| 32 | Bandwidth | Frequency | Ch2 | 0.37 | 1.35 |
| 33 | Bandwidth | Frequency | Ch3 | 0.31 | 1.12 |
| 34 | Spectral Centroid | Frequency | Ch1 | 0.39 | 1.74 |
| 35 | Spectral Centroid | Frequency | Ch2 | 0.44 | 2.25 |
| 36 | Spectral Centroid | Frequency | Ch3 | 0.35 | 1.46 |