Manuscript title - Submission of manuscript titled " Developing an Open-access Telegram Bot for Automated, estimation and IA- based interpreted of OpenQuake Ground-Motion intensity measures "

Boumédiène DERRAS1,2

1Civil engineering & hydraulic department, university of Dr Moulay Tahar, Saida, Algeria. boumediene.derras@univ-saida.dz. orcid.org/0000-0002-3979-7083

2Risk Management and assessment (RISAM) laboratory, University of Abou Bekr Belkaid, Tlemcen, Algeria, boumediene.derras@univ-tlemcen.dz

Table S1 : Description of all physical parameters required by GMMs in OpenGSIM

| Effects | Name | Definition | Unit |
| --- | --- | --- | --- |
| Source effect |  | Stress Drop. A parameter describing the stress reduction on the fault during an earthquake. Used in some advanced GMMs. | bar |
|  | mag | Moment Magnitude (Mw). A logarithmic scale measuring the total energy released by an earthquake. | none |
|  | Hypo depth | Depth of the earthquake's hypocenter (point of initiation) below the surface. | km |
|  | hypo_lat | Latitude of the hypocenter. | degrees |
|  | hypo_lon | Longitude of the hypocenter. | degrees |
|  | dip | The angle of the fault plane with respect to the horizontal. | degrees |
|  | rake | The angle describing the direction of slip on the fault plane. Defines the style of faulting. | degrees |
|  | ztor | Depth to the Top of Rupture. The shallowest point on the fault rupture plane. | km |
|  | width | The down-dip width of the fault rupture plane. | km |
|  | in_cshm | in_cshm: A Boolean flag (1=true, 0=false) indicating if the earthquake source is within the Canterbury zone. Canterbury Seismic Hazard Model (CSHM) to apply region-specific adjustments for New Zealand (Bradley, 2013). | boolean (1/0) |
| Path effect | rrup | Rupture Distance. The shortest distance from the site to any point on the fault rupture plane. | km |
|  | rjb | Joyner-Boore Distance. The shortest horizontal distance from the site to the surface projection of the rupture plane. | km |
|  | repi | Epicentral Distance. The horizontal distance from the site to the earthquake's epicenter. | km |
|  | rhypo | Hypocentral Distance. The straight-line distance from the site to the earthquake's hypocenter. | km |
|  | rvolc | (Context-dependent, Japan) Source to site distance passing through surface projection of volcanic zone. | km |
|  | rx | Horizontal distance from the site to the fault trace, measured perpendicular to the trace. Can be positive or negative. | km |
|  | ry0 | Horizontal distance from the site to the fault trace, measured parallel to the trace from the midpoint. | km |
|  | azimuth | The angle of the site relative to the fault strike, measured clockwise from North. | degrees |
|  | rcdpp | Closest Distance to the Centred Predictor Point. A specific geometric parameter used to model the effects of near-fault forward directivity. A smaller value indicates a higher potential for strong directivity pulses. | km |
| Site effect | vs30 | Time-averaged shear-wave velocity in the top 30 meters of soil. It is a key proxy for site amplification, with lower values indicating softer soil. | m/s |
|  | vs30 measured | A Boolean flag (1 for true, 0 for false) indicating if the Vs30 value comes from a direct measurement or an inference. | boolean (1/0) |
|  | z1pt0 | Depth to the shear-wave velocity horizon of 1.0 km/s. It characterizes the basin depth. | m |
|  | z1pt4 | Depth to the shear-wave velocity horizon of 1.4 km/s. Used in some Japanese models. | m |
|  | z2pt5 | Depth to the shear-wave velocity horizon of 2.5 km/s. It characterizes the deeper basin structure. | km |
|  | backarc | A Boolean flag (1 for true, 0 for false) indicating if the site is in a back-arc tectonic setting, which affects wave attenuation. | boolean (1/0) |
|  | xvf | (Context-dependent, Japan) Horizontal distance from the site to the volcanic front line. It's positive in the fore-arc region. | km |
|  | PHV | Amplitude (Peak) of the Horizontal-to-Vertical Spectral Ratio (H/V). Measures the site's primary amplification factor. | none |
|  | THV | Fundamental period (Time) of the site, corresponding to the peak of the H/V ratio. | s |
|  | slope | Ground slope, used for liquefaction models (LSD). Represents the gradient in percent. | % |
|  | Free face ratio | Free-face ratio, used for liquefaction models (LSD). Ratio of the height of a free face to its distance from the site. | none |
|  | T_15 | Cumulative thickness of saturated sandy layers with corrected SPT blow-counts (N1)60 < 15. Used for liquefaction models. | m |
|  | F_15 | Average fines content for the T_15 layers. Used for liquefaction models. | % |
|  | D50_15 | Average mean grain size for the T_15 layers. Used for liquefaction models. | mm |
|  | kappa0 | High-frequency filter parameter representing near-surface attenuation directly beneath the site. | s |
|  | f0 | Fundamental site frequency, or resonance frequency. | Hz |
|  | bas | Basin parameter, typically a boolean flag (1/0) indicating if the site is on a sedimentary basin. | boolean (1/0) |
|  | lat | Site latitude. | degrees |
|  | lon | Site longitude. | degrees |
|  | region | Geologic region string, used for regional adjustments in some GMMs. 1: Iberian Peninsula Includes Spain and Portugal. 2: France and Germany Includes France, Germany, Belgium, the Netherlands, Switzerland and the United Kingdom. It is a region of low to moderate seismicity. 3: Italy and the Western Balkans Includes Italy, Austria, Slovenia, Croatia, Serbia, etc. It is a region with high seismicity. It is a great default choice for our tests because it is seismologically very active. 4: Greece and Turkey Includes Greece, Albania and the western part of Turkey. It is the most seismically active region in Europe. 5: Pannonian Basin Mainly includes Hungary and the surrounding regions. 0 (or unspecified): If you do not specify a region or if you give a value of 0, the model will use the basic coefficients of Kotha et al. (2020), which represent an average for the dataset. | 0 to 6 |
|  | Geology | Gelogic age: CENOZOIC, HOLOCENE, JURASSIC-TRIASSIC, CRETACEOUS, PALEOZOIC, PLEISTOCENE, PRECAMBRIAN, UNKNOWN | Name of Gelogic age |
|  | siteclass | EC8 site category. Class A: Rock or other rock-like geological formation, including at most 5 m of weaker material at the surface. (Vs30 > 800 m/s) Class B: Deposits of very dense sand, gravel, or very stiff clay, at least several tens of meters thick. (360 m/s ≤ Vs30 ≤ 800 m/s) Class C: Deep deposits of dense or medium-dense sand, gravel, or stiff clay. (180 m/s ≤ Vs30 ≤ 360 m/s) Class D: Deposits of loose-to-medium cohesionless soil or soft-to-firm cohesive soil. (Vs30 < 180 m/s) Class E: A soil profile consisting of a surface alluvium layer with Vs30 < 360 m/s, thickness ≥ 20 m, overlying stiffer material. | A to E |
|  | soiltype | A simplified soil classification, Soiltypes - 1 : Hard rock, 2 : very stiff soil, 3 : Dense soil, 4 : Medium-dense soil, 5 : Soft soil, 6 Very soft-. If soiltype > 6, OpenQuake automatically defaults to 1 (hard rock). | 1 to 6 |

Table S2 : Description of all IMs supported in OpenGSIM bot

| N° | Name | Definition | Unit | The number of GMMs Classes by IM |
| --- | --- | --- | --- | --- |
| 1 | PGA | Peak Ground Acceleration. The maximum acceleration experienced by the ground during shaking. | g | 472 |
| 2 | PGV | Peak Ground Velocity. The maximum velocity experienced by the ground during shaking. | cm/s | 256 |
| 3 | PGD | Peak Ground Displacement. The maximum displacement of the ground during shaking. | cm | 6 |
| 4 | SA | Spectral Acceleration. The max acceleration of a single-degree-of-freedom oscillator. Requires a period (T) and 5% damping is assumed unless specified. | g | 462 |
| 5 | IA | Arias Intensity. A measure of the total energy of the ground motion over its entire duration. | m/s | 11 |
| 6 | CAV | Cumulative Absolute Velocity. The integral of the absolute value of the acceleration time series, a measure of damaging potential. | g.s | 7 |
| 7 | MMI | Modified Mercalli Intensity. A qualitative scale (I-XII) that describes the severity of earthquake shaking based on observed effects. | integer | 12 |
| 8 | RSD2080 | Significant Duration between 20% and 80% of cumulative IA | s | 2 |
| 9 | RSD575 | Significant Duration between 5% and 75% of cumulative IA | s | 8 |
| 10 | RSD595 | Significant Duration between 5% and 95% of cumulative IA | s | 7 |
| 11 | FAS | Fourier Amplitude Spectrum. A representation of the ground motion's amplitude at different frequencies. Requires a frequency (f) in Hz. | g.s | 2 |
| 12 | SDi | Inelastic Spectral Displacement. The peak displacement of a structure that behaves inelastically. Requires period (T) and ductility or strength ratio (R). | cm | 2 |
| 13 | LSD | Lateral Spread Displacement. The permanent horizontal ground displacement due to soil liquefaction. Typically, from models like (Youd et al., 2002); (Zhang & Zhao, 2005). | m | 4 |
| 14 | AvgSA | Average Spectral Acceleration over a range of periods. The specific period band must be defined in the model. | g | 3 |
| 15 | JMA | Japan Meteorological Agency seismic intensity scale, an instrumental scale ranging from 0 to 7 used in Japan. | integer | 7 |
| 16 | SA1 | Spectral Acceleration. The max acceleration of a single-degree-of-freedom oscillator. At period (T) equal to 1.0 s and 5% damping is assumed unless specified. | g | 2 |
| 17 | EAS | Effective Amplitude Spectrum, derived from the Fourier Amplitude Spectrum (FAS). Represents a smoothed, band-averaged measure of ground motion amplitude in the frequency domain. Useful for site response and stochastic simulations. | cm/s² | 1 |
| 18 | VHR_SA | VHR is the ratio of the 5 %-damped vertical response spectrum (SA) to the geometric mean of the two horizontal components, all expressed in natural-log units in the regression. It quantifies how much vertical shaking is expected relative to horizontal shaking for a given oscillator period. | none | 1 |
| 19 | VHR_PGA | VHR is the ratio of peak ground acceleration (PGA) to the geometric mean of the two horizontal components, all expressed in natural-log units in the regression. It quantifies how much vertical shaking is expected relative to horizontal shaking for a given oscillator period. | none | 1 |
| 20 | VHR_PGV | VHR is the ratio of peak ground velocity (PGV) to the geometric mean of the two horizontal components, all expressed in natural-log units in the regression. It quantifies how much vertical shaking is expected relative to horizontal shaking for a given oscillator period. | none | 1 |
| 21 | DRVT | DRVT (Duration of a Random Vibration Time-History): A frequency-dependent measure of ground motion duration, representing the time required for a random signal to build up to the observed Fourier amplitude. | s | 1 |