Supplementary information

Table S1: The values of parameters used for the health risk simulation.

| Parameters | Symbols | units | Distribution | Values |
| --- | --- | --- | --- | --- |
| Chemical concentration in soil | Cs | mg/kg | Lognormal | Element specific concentrations |
| Ingestion rate-adult | IngR A | mg/day | Triangular | 25 (0.1-50)a |
| Ingestion rate-children | IngR C | mg/day | Triangular | 100 (5-500)a,b |
| Inhalation rate | InhR | m^3/hr | Lognormal | 1.68 ± 0.62c |
| Exposure frequency | EF | day/year | Triangular | 225 (200-250)c |
| Exposure duration | ED | year | Lognormal | 14.0 ± 9.4c |
|  |  |  |  | Upper transaction limit 44.0 |
| Exposure time | ET | hr/day | point | 8c |
| Exposed skin surface area-Adult | SA A | cm2 | point | 5700c |
| Exposed skin surface area-Children | SA C | cm2 | point | 2800c |
| Soil to skin adherence factor-Adult | AF A | mg/cm^2-day | Lognormal | 0.49 ± 0.54d,e |
| Soil to skin adherence factor-Children | AF C | mg/cm^2-day | Lognormal | 0.65 ± 1.2d,e |
| Soil to air particulate emission factor | PEF | kg/m^3 | point | 1.36E+09c |
| Absorption factor | ABS | unitless | point | 0.001f,g,h |
| Body weight-Adult | Bw A | kg | Lognormal | 77.1 ± 13.5a |
| Body weight-Children | Bw C | kg | Triangular | 15 (6.5-26.1)a |
| Averaging time-non carcinogen = [ED x 365] | AT nc | day | Lognormal | 5110 ± 3431a |
|  |  |  |  | Upper truncation limit 16060 |
| Averaging time-Carcinogen = [70 x 365] | AT ca | day | point | 25550a |

a(USPEA, 1996), b(Finley et al., 1994a), c(USEPA, 2001), d(Ramazanova et al., 2021), e(Finley et al., 1994b), f(USDoE, 2005), g(De Migueal et al., 2007), h(Chabukdhara et al., 2013)

Table S2. Univariate statistics of element (mg/kg) distribution of Fogo Island

| Metal(loid) | Minimum | Median | Mean | Maximum | SD |
| --- | --- | --- | --- | --- | --- |
| As | 0.25 | 0.60 | 0.62 | 2.00 | 0.32 |
| Cd | 0.05 | 0.20 | 0.20 | 2.60 | 0.29 |
| Co | 8.10 | 30.9 | 30.7 | 69.4 | 9.66 |
| Cr | 3.00 | 38.0 | 39.2 | 123 | 20.8 |
| Cu | 15.6 | 56.3 | 63.5 | 726 | 63.1 |
| Hg | 0.01 | 0.01 | 0.01 | 0.09 | 0.02 |
| Mn | 477 | 1130 | 1130 | 2679 | 295 |
| Ni | 2.30 | 39.7 | 49.5 | 193 | 31.5 |
| Pb | 2.50 | 6.45 | 21.8 | 1649 | 149 |
| V | 76.0 | 194 | 194.9 | 395 | 54.2 |
| Zn | 36.0 | 77.5 | 75.9 | 126 | 16.4 |

Table S3: Loadings of first three principal components (PCs) showing the correlation of different minerals with PCs.

|  | PC 1 | PC 2 | PC 3 |
| --- | --- | --- | --- |
| Quartz | 0.2 | 0.0 | 0.2 |
| Feldspar | -0.7 | -0.6 | -0.4 |
| Pyroxene | -0.4 | 0.9 | -0.2 |
| Phyllosilicate | 0.9 | -0.2 | 0.2 |
| Carbonate | 0.0 | -0.1 | 0.9 |
| Fe oxide | 0.5 | -0.1 | -0.1 |

Table S4 – Association of measured metal(oid)s with first three principal components from PCA analysis.

|  | PC 1 |  | PC 2 |  | PC 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
| As |  | -0.1 |  | 0.1 |  | 0.6 |
| Cd |  | 0.1 |  | 0.0 |  | 0.8 |
| Co |  | 0.9 |  | 0.2 |  | 0.0 |
| Cr |  | 0.9 |  | 0.0 |  | -0.2 |
| Cu |  | 0.7 |  | -0.2 |  | -0.2 |
| Hg |  | 0.4 |  | 0.5 |  | 0.5 |
| Mn |  | 0.6 |  | 0.6 |  | 0.2 |
| Ni |  | 0.8 |  | -0.2 |  | -0.1 |
| Pb |  | -0.2 |  | 0.2 |  | 0.3 |
| V |  | 0.8 |  | 0.3 |  | 0.0 |
| Zn |  | 0.0 |  | 0.9 |  | 0.1 |